FLORIDA PUBLIC SERVICE COMMISSION

Capital Circle Office Center  2540 Shumard Oak Boulevard

Tallahassee, Florida  32399-0850

M E M O R A N D U M

February 22, 1996

TO:		DIRECTOR, DIVISION OF RECORDS AND REPORTING (BAYO)

FROM:		DIVISION OF AUDITING & FINANCIAL ANALYSIS (LEE, BASS, HICKS, LESTER, JOHNSON, MERTA)

DIVISION OF ELECTRIC & GAS (WHEELER)

DIVISION OF LEGAL SERVICES (ERSTLING)

RE:		DOCKET NO. 950499-EI - TAMPA ELECTRIC Company - PETITION FOR APPROVAL OF 1995 DEPRECIATION STUDY BY TAMPA ELECTRIC Company

AGENDA:	MARCH 5, 1996 - REGULAR AGENDA - PROPOSED AGENCY ACTION -INTERESTED PERSONS MAY PARTICIPATE

CRITICAL DATES:	  NONE

SPECIAL INSTRUCTIONS:  I:\PSC\AFA\WP\950499.RCM

DISCUSSION OF ISSUES

ISSUE :	Should the current depreciation rates and provision for dismantlement for Tampa Electric Company (TECO or Company) be changed?

RECOMMENDATION:  Yes.  A review of the Company's activity since the prior depreciation study, as well as current planning and provisions for dismantlement, indicate the need for revising depreciation rates and recovery schedules.  (BASS, JOHNSON, LEE)

STAFF ANALYSIS:   The study filed in this docket is the regular quadrennial depreciation study for TECO.  The previous comprehensive study was filed on December 31, 1991, resulting in Order No. 25619.  In addition to setting appropriate recovery provisions for TECO's investment effective January 1, 1991, that order required the Company to file revised site-specific depreciation studies by June 30, 1992 for Big Bend and Gannon Stations.  Revised depreciation rates, based on that study, were ordered for the investment relating to those two sites, effective January 1, 1992.

In this study, the Company has provided production plant investment stratified by Federal Energy Regulatory Commission (FERC) account for each unit at each steam generation site. Formerly, the breakdown of steam production plant investment was by unit for each site.  As a result of this stratification of investment, recovery provisions can be more closely matched to the life characteristics of specific categories of the investment which has been made to provide for steam generation of electric power.  Additionally, this study affords an opportunity to set appropriate recovery rates for the Polk Power Station, slated to come on line in late 1996.  Taken together with changes in net plant balances and updated planning, a need for review and revision of recovery provisions is indicated.

This study also provides an opportunity to review the accrual which has been undertaken to provide for the dismantlement of fossil fueled generation plants following the retirement of those installations.  This matter is addressed in Issue 5.

ISSUE 2:  What should be the implementation date of the recommended rates and schedules?

RECOMMENDATION:  Staff recommends approval of the Company's proposed January 1, 1996, date of implementation for the depreciation rates and schedules.  (BASS)

STAFF ANALYSIS:  Company data and related calculations abut the January 1, 1996, date.  This is the recommended date of implementation, being the earliest practicable date for utilizing the revised rates and schedules.

ISSUE 3:  Should any corrective reserve allocations be made?

RECOMMENDATION:  Yes.  The reserves for production plant should be reallocated from the unit level to an account by unit level as shown on Attachment  A, pages 12 through 14. (LEE)

STAFF ANALYSIS:  Up to this time, the investment and depreciation accrual for production plant have been maintained for each unit at each site.  Stratifying the investment (associated with each of those units into the appropriate FERC accounts) affords the opportunity to provide recovery more closely matched to the life characteristics of investment comprising each account.

In the process of stratifying investment and developing remaining life rates by account by unit, TECO reallocated the total reserve for each unit among the constituent accounts.  Each account's reserve was aligned with it's theoretical reserve, as developed using the rates and parameters proposed in the Company's originally filed study.  Staff recommended reserves are not substantially different from those proposed by the Company; rather they are recommended as a refinement incorporating the depreciation parameters which have been agreed upon in the review process, as discussed further in Issue 4.

ISSUE 4:  What are the appropriate depreciation rates and amortization schedules?

RECOMMENDATION:  The Staff recommended lives, net salvages, reserves and resultant depreciation rates are shown on Attachment B, pages 15 through 19.  Without including recovery for investment at the Polk Power Station, there is a projected decrease in annual depreciation expense of approximately $555,000, based on projected January 1, 1996, investments, as shown on Attachment C, pages 20 through 24.  The annual capital recovery expense and provision for dismantlement relating to the Polk Power Station will be approximately $22 million.  (LEE, BASS, JOHNSON)

STAFF ANALYSIS:  Staff's recommendations are the result of a comprehensive review of the Company's submitted study.  Attachment B shows a comparison of rate components (lives, salvages, and reserves).  Investments and reserve amounts are estimated as of January 1, 1996.  Attachment C, pages 20 through 24, shows the estimated resultant annual expenses.

As a result of the review and analytical process, TECO and Staff have reached basic agreement on life and net salvage parameters recommended in this docket.  There is some difference, however, in the Company proposed reserve allocations for production plant compared to those recommended by Staff, as discussed in Issue 3.  Further, TECO has proposed that official communication equipment be amortized over 10 years with property records maintained on a vintage group basis.  The Company notes that use of amortization is in line with Staff efforts to simplify the depreciation study process, where possible.  Staff is amenable to the Company's proposal as it relates to communications equipment similar to that amortized by telecommunications companies.  The remaining investment however, relates to fiber cable and associated electronics.  While early generations of this technology can be impacted by refinements in the technology and in installation procedures, there is no reason to believe that subsequent generations will not experience longer lives than metallic cables. For this reason, Staff believes that this investment should continue to be studied for the determination of appropriate life and salvage characteristics.  Staff's recommendation for Fiber Communication Equipment, Account 397.25, reflects a composite of a 20-year life for the fiber cable and a 7-year life for the associated electronics .

A summary of the changes to the annual accrual based on January 1, 1996 estimated investments resulting from the recommended depreciation rates and provision for dismantlement are as follows:

($000)

Rates:

Production                        21,301

Transmission                        (252)

Distribution                        (427)

General                             (348)

Total Rates                       20,274

Provision for Dismantlement           	    1,349

Total Change in Annual Expenses    		   21,263

The most significant changes are seen in the production plant area.  In the previous study, depreciation rates were developed by unit for each generating plant.  This study represents a further refinement with development of depreciation rates and allocation of reserves by account by unit by plant site.  Further, the study provides the development of a depreciation rate for the Polk Power Plant scheduled to be in service in October, 1996.

TECO has utilized its continuing property record system to develop stratified categories expected to have homogenous life characteristics.  The life of the account is then arrived at by compositing the life expectations of the various strata.  This approach provides a more accurate determination of the required depreciation components than the historical approach of arriving at the pattern of interim retirement and life expectancy of the generating plant without identifying the contents or quantifying the varying life characteristics of the contained assets.

The life analysis for the Polk Power Plant has been based on the most recent project estimate and assigned lives to categories of plant consistent with like items of other production plants.  A composite life at a plant site level is recommended pending a more detailed study at the next comprehensive review. Since Polk is expected to go in service in October of 1996, the projected impact of recommended rates for that investment is reduced to approximately $5 million for the year ending December 1996.  The projected impact of the revised dismantlement accrual is about $337,000.  As a result, total projected expense for TECO for 1996 is an increase of approximately $5 million.

Except for the account relating to communication equipment as discussed above, the transmission, distribution and general plant accounts are basically status quo.  In other words, recommended remaining lives simply reflect an update of each account's activity since the last review.  Underlying service lives and mortality dispersions are still considered appropriate and reasonable.

ISSUE 5:  Should the current approved annual accrual for dismantlement for Tampa Electric Company be revised?

RECOMMENDATION:  No; the current approved annual accrual for dismantlement of facilities using fossil fuel should be continued for the next 4-year period.  With the addition of Polk, the annual accrual will be $10,118,800.  (LESTER, LEE)

STAFF ANALYSIS:  In Docket No. 890186-EI, the Commission established the methodology for accruing the costs of fossil fuel dismantlement.  (See Order No. 24741, issued July 1, 1991).  Electric companies are required to file dismantlement studies at least once every four years in connection with their depreciation studies.  The methodology depends on three factors:  estimated base costs of dismantling the fossil fuel plants, projected inflation, and a contingency factor.

TECO's preferred accrual is shown in the first column of Attachment D.  This reflects maintaining the currently prescribed accrual level for all plants except for the Polk Power Station which is expected to come on-line in October 1996.  The assumptions inherent in these currently prescribed accruals are base cost estimates resulting from a 1991 site specific dismantlement cost study, a 20% contingency factor and inflation indices based on the 1991 DRI Summer forecast.  The proposed accrual for Polk reflects an estimate based on the cost estimates for Big Bend Unit No. 4 and the Big Bend Unit No. 4 FGD System, a 20% contingency factor and inflation indices based on the DRI Summer 1995 forecast.  A site specific study for Polk will be included in the next dismantlement study.

Since the last study, base cost estimates for the various dismantlement activities have changed.  The 1991 dismantlement study indicated base cost estimates of $83.8 million; current cost estimates are $85.6 million.  At Staff's request, TECO also provided an update using the most recent DRI inflation forecast and a lower contingency factor.  However, the 16% factor, shown in the third column of Attachment D, is an FPL company specific factor not TECO specific.  In Docket No. 941343-EI, FPL developed a weighted average contingency factor for the various tasks detailed in the Atomic Industrial Forum/National Environmental Studies Project report AIF/NESP-036, "Guidelines for Producing Commercial Nuclear Power Plant Decommissioning Cost Estimates" as they related to dismantlement activities.  To this amount a 20% contingency factor was added for asbestos removal.  The result was a 16% weighted average contingency factor specific to FPL.  Staff does not believe that this 16% contingency factor is necessarily appropriate for TECO.

Before the inclusion of Polk, the updated study (current base cost estimates, Summer 1995 DRI inflation forecast and a 20% contingency factor) shows an annual level accrual of $7,860,000, a  decrease of $910,000 from the current annual accrual of $8,770,000.  When one fourth of the $1,304,000 dismantlement accrual attributed to Polk is included in the calculation for 1996, the annual accrual is $8,186,000, still a slight decrease from the current accrual.  In 1997, and thereafter, when the full amount attributable to Polk is included, the accrual will increase slightly above the current accrual to $9,164,000.  The Commission approved the current annual accrual in Docket No. 910686-EI, Order No. 25619, issued January 21, 1992.

TECO proposes no change in the currently approved annual accrual of $8,770,000 for fossil fuel plant dismantlement.  Specifically, the Company states "with the uncertainties inherent in estimating the cost of dismantling a plant fifty years in the future, the company feels it is too early to begin to reduce accruals for this cost."  Further, TECO believes that, if the effect of the decrease in inflation projections is recognized, the 20% contingency factor should be used to mitigate the reduction to the annual accrual for the reasons stated.  The updated study shows increases in the base cost estimates of dismantling the fossil fuel plants but these increases have been more than offset by decreases in projected inflation for a net decrease of $910,000.  (See the second column of Attachment D.)

Staff believes that the possibility of under-accrual exists.  National industry experience with dismantlement of fossil fuel plants is limited.  Experience with estimating the base costs is even more limited.  With the passage of time, estimating the costs of dismantlement likely will become more accurate.  This case is part of only the second round of dismantlement studies for the major electric companies.  As this round of reviews has progressed, Staff and the companies have refined the elements that go into the calculation of the base cost estimates of dismantlement and the calculation of the annual level accrual.

In making this recommendation, Staff considered the alternative of reducing the annual accrual to agree with the updated base cost estimates, the DRI Summer 1995 inflation forecasts and a weighted contingency factor as used in the FPL review.  This would be consistent with the methodology of determining dismantlement accruals that the Commission established by Order No. 24741, above, and subsequent reviews.  However, there is little history by which to determine the adequacy of base cost estimation.  Recognizing that base costs have increased by a very small amount, Staff is concerned about under-accrual and so recommends that the Commission not decrease the Companyy

The annual accrual for fossil fuel dismantlement of $8,770,000.  The annual accrual will increase by $1,034,000 when Polk comes on-line.

ISSUE 6:  Should the current amortization of investment tax credits (ITCs) and the flowback of excess deferred income taxes be revised to reflect the approved depreciation rates and recovery schedules?

RECOMMENDATION:  Yes.  The current amortization of ITCs and the flowback of excess deferred income taxes should be revised to reflect the approved depreciation rates and recovery schedules.  Also, the utility should be required to file detailed calculations of the revised ITC amortization and flowback of excess deferred taxes at the same time it files its January 1996 surveillance report.  (HICKS)

STAFF ANALYSIS:  In issues previously addressed, Staff recommends revisions to TECO's depreciation rates and capital recovery schedules, to be effective January 1, 1996.  Revising a utility's depreciation rates usually results in a change in its rate of ITC amortization and flowback of excess deferred income taxes.

Section 46(f)(6) of the Internal Revenue Code (IRC) states that the amortization of ITCs should be determined by the period of time used in computing depreciation expense for purposes of reflecting regulated operating results of the utility.  Since Staff is recommending a change in depreciation rates, it is also appropriate to change the amortization of ITCs.

Section 203(e) of the Tax Reform Act of 1986 (TRA) prohibits rapid write-back of protected (depreciation related) deferred taxes.  In addition, Rule 25-14.013, Accounting for Deferred Income Taxes under SFAS 109, Florida Administrative Code (F.A.C), prohibits, without good cause shown, excess deferred income taxes associated with temporary differences from being reversed any faster than allowed under Section 203(e).  Therefore, both the TRA and Rule 25-14.013, F.A.C., prohibit faster write-off of protected excess deferred taxes.  Consequently, Staff believes that the flowback of excess deferred taxes should be altered to comply with the TRA and Rule 25-14.013, F.A.C.

The Company has submitted detailed workpapers quantifying the impact of the proposed depreciation rates on the amortization of ITCs and the flowback of excess deferred income taxes.  Staff has reviewed the calculations and found them to be acceptable.  However, the amounts reflected on the workpapers will change based on Staff's recommended depreciation rates.

Staff recommends that the current amortization of ITCs and the flowback of excess deferred income taxes be revised to reflect the approved depreciation rates and recovery schedules.  Also, the utility should be required to file detailed calculations of the revised ITC amortization and flowback of excess deferred taxes at the time it files its January 1996 surveillance report.

ISSUE 7:  Should this docket be closed?

RECOMMENDATION:  Yes. If no substantially affected person timely files a protest to the Commission's notice of proposed agency action, this docket should be closed.  (BASS)

STAFF ANALYSIS:  If no substantially affected person files a timely request for a Section 120.57, Florida Statutes, hearing within twenty-one days, no further action will be required and this docket should be closed.

TAMPA     ELECTRIC     COMPANY

STAFF     RECOMMENDED

RESTATED     RESERVE

1/1/96

RESTATED

ACCOUNT                                        RESERVE

$

STEAM  PRODUCTION

BIG BEND STATION

- Common -

311400 - Structures                  13,608,187

312400 - Boiler Plant                18,276,514

314400 - Turbogenerators              1,408,938

315400 - Access. Elec Eqpt.           5,098,343

316400 - Miscellaneous                1,735,272

TOTAL                       40,127,254

- Unit  1 -

311410 - Structures                   2,995,771

312410 - Boiler Plant                20,940,563

314410 - Turbogenerators             10,763,875

315410 - Access. Elec. Eqpt.          3,805,745

316410 - Miscellaneous                  264,736

TOTAL                       38,770,690

- Unit  2 -

311420 - Structures                   2,504,676

312420 - Boiler Plant                18,838,351

314420 - Turbogenerators              9,690,590

315420 - Access. Elec. Eqpt.          3,136,106

316420 - Miscellaneous                  170,730

TOTAL                       34,340,453

- Unit  3 -

311430 - Structures                   6,330,812

312430 - Boiler Plant                38,065,478

314430 - Turbogenerators             16,886,281

315430 - Access. Elec. Eqpt.          8,063,343

316430 - Miscellaneous                  311,016

TOTAL                       69,656,930

- Unit  4 -

311440 - Structures                  18,229,870

312440 - Boiler Plant                46,840,707

314440 - Turbogenerators             25,663,858

315440 - Access. Elec. Eqpt.         11,223,899

316440 - Miscellaneous                2,000,748

TOTAL                      103,959,082

- Unit   4   FGD  -

311450 - Structures                   5,565,942

312450 - Boiler Plant                45,106,539

315450 - Access. Elec. Eqpt.          5,462,482

316450 - Miscellaneous                   79,579

TOTAL                       56,214,542

TOTAL  BIG BEND  STATION 343,068,951

STEAM  PRODUCTION   (CONT'D)

GANNON  STATION

- Common -

311500 - Structures                  10,051,659

312500 - Boiler Plant                 5,392,081

314500 - Turbogenerators                427,617

315500 - Access. Elec. Eqpt.          1,339,897

316500 - Miscellaneous                1,642,868

TOTAL                       18,854,122

- Unit  1 -

311510 - Structures                   1,956,255

312510 - Boiler Plant                 7,023,120

314510 - Turbogenerators              5,867,962

315510 - Access. Elec. Eqpt.          1,516,013

316510 - Miscellaneous                  219,558

TOTAL                       16,582,908

- Unit  2 -

311520 - Structures                   1,864,168

312520 - Boiler Plant                 6,334,013

314520 - Turbogenerators              7,007,721

315520 - Access. Elec. Eqpt.          1,040,281

316520 - Miscellaneous                   59,727

TOTAL                       16,305,910

- Unit  3 -

311530 - Structures                   1,431,245

312530 - Boiler Plant                10,933,947

314530 - Turbogenerators              7,975,955

315530 - Access. Elec. Eqpt.          1,503,928

316530 - Miscellaneous                   89,069

TOTAL                       21,934,144

- Unit  4 -

311540 - Structures                     971,906

312540 - Boiler Plant                 9,035,049

314540 - Turbogenerators              6,000,641

315540 - Access. Elec. Eqpt.          1,302,628

316540 - Miscellaneous                   34,582

TOTAL                       17,344,806

- Unit  5 -

311550 - Structures                   1,548,389

312550 - Boiler Plant                12,754,799

314550 - Turbogenerators              5,776,887

315550 - Access. Elec. Eqpt.          2,261,916

316550 - Miscellaneous                  111,926

TOTAL                       22,453,917

- Unit  6 -

311560 - Structures                   2,133,836

312560 - Boiler Plant                18,828,348

314560 - Turbogenerators              7,557,822

315560 - Access. Elec. Eqpt.          2,792,549

316560 - Miscellaneous                  176,937

TOTAL                       31,489,492

TOTAL  GANNON  STATION     144,965,299

STEAM  PRODUCTION   (CONT'D)

GANNON   OIL   BACKOUT

- Common -                           20,543,288

- Unit  1 -                          14,902,090

- Unit  2 -                          14,506,218

- Unit  3 -                          17,203,050

- Unit  4 -                          18,678,325

TOTAL  OIL BACKOUT          85,832,971

HOOKER'S  POINT  STATION             46,898,372

DINNER   LAKE   STATION               3,039,727

TOTAL  STEAM  PRODUCTION                    623,805,320

MISCELLANEOUS  PRODUCTION

311010 - Structures & Improvements    2,515,405

TOTAL   MISCELLANEOUS  PRODUCTION             2,515,405

OTHER  PRODUCTION

BIG BEND STATION

- Combustion  Turbine  No. 1 -

341410 - Structures                      79,304

342410 - Boiler Plant                   117,995

344410 - Turbogenerator               1,160,824

345410 - Access. Elec. Eqpt.            162,519

346410 - Miscellaneous                    3,251

TOTAL                        1,523,893

- Combustion  Turbine  No. 2 & 3 -

341420 - Structures                   1,199,286

342420 - Boiler Plant                   795,900

344420 - Turbogenerator              11,038,241

345420 - Access. Elec. Eqpt.          1,789,610

346420 - Miscellaneous                   14,975

TOTAL                       14,838,012

GANNON STATION

- Combustion  Turbine  No. 1 -

341510 - Structures                      66,001

342510 - Boiler Plant                    92,204

344510 - Turbogenerator               1,301,169

345510 - Access. Elec. Eqpt.            228,812

TOTAL                        1,688,186

PHILLIPS STATION                     31,997,042

POLK POWER STATION                            0

TOTAL   OTHER   PRODUCTION                   50,047,133

TOTAL   DEPRECIABLE   PRODUCTION   PLANT         676,367,858

AMORTIZABLE   PRODUCTION   PLANT

STEAM PROD. -- BIG BEND STATION                    2,148,627

STEAM PROD. -- GANNON STATION                      1,152,623

MISC. PROD. - 316010 Misc. Power Plant Equipment   1,801,673

MISC. PROD. - 316170 Amort Power Plant Equipment   1,160,793

TOTAL    PRODUCTION    PLANT              682,631,574

TAMPA    ELECTRIC    COMPANY

1995   STUDY

COMPARISON OF RATES AND COMPONENTS

C U R R E N T                                                 C O M P A N Y     R E V I S E D    P R O P O S A L                                 S T A F F      R E C O M  M E N  D A T I  O N

AVERAGE                            REMAINING                   AVERAGE                                              REMAINING                     AVERAGE                                              REMAINING

REMAINING            NET              LIFE                     REMAINING            NET                                LIFE                     REMAINING              NET                                 LIFE

LIFE             SALVAGE            RATE                       LIFE            SALVAGE            RESERVE            RATE                        LIFE              SALVAGE           RESERVE             RATE

ACCOUNT                              (YRS.)              (%)               (%)                      (YRS.)              (%)               (%)               (%)                       (YRS.)               (%)               (%)                (%)

.

BIG BEND STATION

- Common -

311400 - Structures                     32.0               (11)              2.4                       35.0              (3.0)            30.92               2.1                           35                (3)            32.31                 2.0

312400 - Boiler Plant                   32.0               (11)              2.4                       29.0             (17.0)            37.16               2.8                           29               (17)            36.03                 2.8

314400 - Turbogenerators                32.0               (11)              2.4                       35.0              (9.0)            42.89               1.9                           35                (9)            42.71                 1.9

315400 - Access. Elec Eqpt.             32.0               (11)              2.4                       18.7              (4.0)            40.44               3.4                         18.7                (4)            40.17                 3.4

316400 - Miscellaneous                  32.0               (11)              2.4                       17.1             (17.0)            48.58               4.0                         17.1               (17)            49.48                 3.9

- Unit 1 -

311410 - Structures                     23.0               (11)              2.3                       23.0              (3.0)            49.66               2.3                           23                (3)            46.33                 2.5

312410 - Boiler Plant                   23.0               (11)              2.3                       23.0             (17.0)            42.94               3.2                           23               (17)            41.85                 3.3

314410 - Turbogenerators                23.0               (11)              2.3                       22.0              (9.0)            42.20               3.0                           22                (9)            46.03                 2.9

315410 - Access. Elec. Eqpt.            23.0               (11)              2.3                       23.0              (4.0)            37.49               2.9                           23                (4)            37.98                 2.9

316410 - Miscellaneous                  23.0               (11)              2.3                       24.0             (17.0)            69.22               2.0                           24               (17)            40.94                 3.2

- Unit 2 -

311420 - Structures                     26.0               (11)              2.6                       26.0              (3.0)            42.48               2.3                           26                (3)            37.92                 2.5

312420 - Boiler Plant                   26.0               (11)              2.6                       23.0             (17.0)            43.00               3.2                           23               (17)            43.31                 3.2

314420 - Turbogenerators                26.0               (11)              2.6                       25.0              (9.0)            39.28               2.8                           24                (9)            39.32                 2.9

315420 - Access. Elec. Eqpt.            26.0               (11)              2.6                       22.0              (4.0)            33.96               3.2                           22                (4)            35.73                 3.1

316420 - Miscellaneous                  26.0               (11)              2.6                       26.0             (17.0)            31.34               3.3                           26               (17)            31.14                 3.3

- Unit 3 -

311430 - Structures                     27.0               (11)              2.4                       28.0              (3.0)            43.02               2.1                           28                (3)            41.99                 2.2

312430 - Boiler Plant                   27.0               (11)              2.4                       26.0             (17.0)            47.58               2.7                           25               (17)            46.33                 2.8

314430 - Turbogenerators                27.0               (11)              2.4                       22.0              (9.0)            54.52               2.5                           21                (9)            58.88                 2.4

315430 - Access. Elec. Eqpt.            27.0               (11)              2.4                       23.0              (4.0)            39.66               2.8                           22                (4)            39.41                 2.9

316430 - Miscellaneous                  27.0               (11)              2.4                       29.0             (17.0)            44.25               2.5                           29               (17)            41.99                 2.6

- Unit 4 -

311440 - Structures                     35.0               (11)              2.7                       36.0              (3.0)            22.21               2.2                           36                (3)            29.22                 2.0

312440 - Boiler Plant                   35.0               (11)              2.7                       25.0             (17.0)            29.38               3.5                           25               (17)            23.94                 3.7

314440 - Turbogenerators                35.0               (11)              2.7                       32.0              (9.0)            26.21               2.6                           32                (9)            31.68                 2.4

315440 - Access. Elec. Eqpt.            35.0               (11)              2.7                       28.0              (4.0)            27.27               2.7                           28                (4)            31.43                 2.6

316440 - Miscellaneous                  35.0               (11)              2.7                       27.0             (17.0)            31.07               3.2                           27               (17)            37.57                 2.9

- Unit 4 FGD -

311450 - Structures                     34.0               (11)              2.5                       35.0              (3.0)            25.31               2.2                           35                (3)            26.23                 2.2

312450 - Boiler Plant                   34.0               (11)              2.5                       31.0             (17.0)            31.71               2.8                           31               (17)            31.59                 2.8

315450 - Access. Elec. Eqpt.            34.0               (11)              2.5                       29.0              (4.0)            30.05               2.6                           29                (4)            29.96                 2.6

316450 - Miscellaneous                  34.0               (11)              2.5                       27.0             (17.0)            36.59               3.0                           27               (17)            32.52                 3.1

STEAM   PRODUCTION    (CONT'D)

GANNON  STATION

- Common -

311500 - Structures                     22.0               (22)              3.7                       20.7              (3.0)            31.84               3.4                         21                (3)            33.61               3.3

312500 - Boiler Plant                   22.0               (22)              3.7                       20.0             (17.0)            39.38               3.9                         20               (17)            38.89               3.9

314500 - Turbogenerators                22.0               (22)              3.7                       26.1              (9.0)            30.40               3.0                         26                (9)            25.73               3.2

315500 - Access. Elec. Eqpt.            22.0               (22)              3.7                       26.2              (4.0)            25.63               3.0                         26                (4)            20.74               3.2

316500 - Miscellaneous                  22.0               (22)              3.7                       13.7             (17.0)            54.61               4.6                       13.7               (17)            52.46               4.7

- Unit 1 -

311510 - Structures                     12.9               (22)              3.3                       11.3              (3.0)            78.87               2.1                       11.3                (3)            75.18               2.5

312510 - Boiler Plant                   12.9               (22)              3.3                       11.5             (17.0)            73.54               3.8                       11.5               (17)            76.05               3.6

314510 - Turbogenerators                12.9               (22)              3.3                       10.9              (9.0)            77.12               2.9                       10.9                (9)            76.61               3.0

315510 - Access. Elec. Eqpt.            12.9               (22)              3.3                        9.8              (4.0)            76.53               2.8                        9.8                (4)            73.78               3.1

316510 - Miscellaneous                  12.9               (22)              3.3                       11.4             (17.0)           102.34               1.3                       11.4               (17)            86.40               2.7

- Unit 2 -

311520 - Structures                     13.0               (22)              3.7                       12.2              (3.0)            65.13               3.1                       12.2                (3)            65.72               3.1

312520 - Boiler Plant                   13.0               (22)              3.7                       11.6             (17.0)            62.46               4.7                       11.6               (17)            68.61               4.2

314520 - Turbogenerators                13.0               (22)              3.7                       12.7              (9.0)            69.87               3.1                       12.7                (9)            65.20               3.4

315520 - Access. Elec. Eqpt.            13.0               (22)              3.7                       11.7              (4.0)            67.89               3.1                       11.7                (4)            63.55               3.5

316520 - Miscellaneous                  13.0               (22)              3.7                       12.4             (17.0)            95.41               1.7                       12.4               (17)            78.76               3.1

- Unit 3 -

311530 - Structures                     15.9               (22)              3.8                       13.7              (3.0)            73.65               2.1                       13.7                (3)            64.98               2.8

312530 - Boiler Plant                   15.9               (22)              3.8                       14.6             (17.0)            56.46               4.1                       14.6               (17)            60.74               3.9

314530 - Turbogenerators                15.9               (22)              3.8                       12.8              (9.0)            72.29               2.9                       12.8                (9)            68.09               3.2

315530 - Access. Elec. Eqpt.            15.9               (22)              3.8                       11.7              (4.0)            67.63               3.1                       11.7                (4)            64.19               3.4

316530 - Miscellaneous                  15.9               (22)              3.8                       10.0             (17.0)            91.32               2.6                       10.0               (17)            84.87               3.2

- Unit 4 -

311540 - Structures                     14.3               (22)              3.4                       17.0              (3.0)            64.90               2.2                       17.0                (3)            56.55               2.7

312540 - Boiler Plant                   14.3               (22)              3.4                       17.6             (17.0)            45.79               4.0                       17.6               (17)            49.36               3.8

314540 - Turbogenerators                14.3               (22)              3.4                       14.2              (9.0)            75.08               2.4                       14.2                (9)            69.22               2.8

315540 - Access. Elec. Eqpt.            14.3               (22)              3.4                       13.1              (4.0)            57.90               3.5                       13.1                (4)            58.02               3.5

316540 - Miscellaneous                  14.3               (22)              3.4                       17.0             (17.0)            79.57               2.2                       17.0               (17)            69.10               2.8

- Unit 5 -

311550 - Structures                     19.0               (11)              3.4                       19.0              (3.0)            48.43               2.9                       19.0                (3)            42.95               3.2

312550 - Boiler Plant                   19.0               (11)              3.4                       19.0             (17.0)            45.24               3.8                       19.0               (17)            45.28               3.8

314550 - Turbogenerators                19.0               (11)              3.4                       19.0              (9.0)            43.52               3.4                       19.0                (9)            44.92               3.4

315550 - Access. Elec. Eqpt.            19.0               (11)              3.4                       16.4              (4.0)            38.25               4.0                       16.4                (4)            38.84               4.0

316550 - Miscellaneous                  19.0               (11)              3.4                       22.0             (17.0)            39.70               3.5                         22               (17)            31.92               3.9

-Unit 6 -

311560 - Structures                     22.0               (11)              3.2                       21.0              (3.0)            49.32               2.6                         21                (3)            46.48               2.7

312560 - Boiler Plant                   22.0               (11)              3.2                       20.0             (17.0)            43.44               3.7                         20               (17)            45.10               3.6

314560 - Turbogenerators                22.0               (11)              3.2                       22.0              (9.0)            33.63               3.4                         22                (9)            31.48               3.5

315560 - Access. Elec. Eqpt.            22.0               (11)              3.2                       16.4              (4.0)            43.69               3.7                       16.4                (4)            42.89               3.7

316560 - Miscellaneous                  22.0               (11)              3.2                       17.5             (17.0)            58.23               3.4                       17.5               (17)            59.37               3.3

STEAM   PRODUCTION    (CONT'D)

GANNON   OIL   BACKOUT

- Common -                                                                   5.0                       18.7               (12)            66.00               2.5                       18.7               (12)            46.88               3.5

- Unit 1 -                                                                   5.0                       11.4               (12)            56.27               4.9                       11.4               (12)            78.96               2.9

- Unit 2 -                                                                   5.0                       12.3               (12)            54.45               4.7                       12.3               (12)            68.77               3.5

- Unit 3 -                                                                   5.0                       14.5               (12)            59.20               3.6                       14.5               (12)            68.67               3.0

- Unit 4 -                                                                   5.0                       17.0               (12)            63.49               2.9                       17.0               (12)            59.36               3.1

HOOKER'S  POINT  STATION                50.0               (11)              2.2                        7.5                (2)            89.16               1.7                        7.5                (2)            89.16               1.7

DINNER   LAKE   STATION                 35.0               (20)              3.4                        8.7               (12)            82.16               3.4                        8.7               (12)            82.16               3.4

MISCELLANEOUS  PRODUCTION

311010 - Structures & Improvements      25.0               (11)              3.3                       21.0              (3.0)            37.68               3.1                         21                (3)            37.68               3.1

OTHER  PRODUCTION

BIG BEND STATION

- Combustion Turbine No. 1  -

341410 - Structures                     15.8               (11)              2.0                       13.4              (3.0)            93.47               0.7                       13.4                (3)            95.46               0.6

342410 - Boiler Plant                   15.8               (11)              2.0                       13.4             (17.0)            78.86               2.8                       13.4               (17)           103.81               1.0

344410 - Turbogenerator                 15.8               (11)              2.0                       12.6              (9.0)            85.88               1.8                       12.6                (9)            83.84               2.0

345410 - Access.  Elec. Eqpt.           15.8               (11)              2.0                       13.4              (4.0)            94.49               0.7                       13.4                (4)            93.11               0.8

346410 - Miscellaneous                  15.8               (11)              2.0                        6.2             (17.0)            96.81               3.3                        6.2               (17)           123.05              (1.0)

- Combustion Turbine No. 2 & 3  -

341420 - Structures                     11.7               (11)              4.4                        5.4              (3.0)            84.38               3.4                        5.4                (3)            83.72               3.6

342420 - Boiler Plant                   11.7               (11)              4.4                        8.9             (17.0)            83.13               3.8                        8.9               (17)            84.28               3.7

344420 - Turbogenerator                 11.7               (11)              4.4                        9.9              (9.0)            70.90               3.8                        9.9                (9)            69.81               4.0

345420 - Access. Elec. Eqpt.            11.7               (11)              4.4                        6.3              (4.0)            67.87               5.7                        6.3                (4)            75.07               4.6

346420 - Miscellaneous                  11.7               (11)              4.4                        6.4             (17.0)            93.02               3.7                        6.4               (17)            89.99               4.2

OTHER    PRODUCTION    (CONT'D)

GANNON STATION

-  Combustion  Turbine No. 1  -

341510 - Structures                                  15.8               (11)              1.6                       13.4              (3.0)            98.84               0.3                       13.4                (3)            87.58               1.2

342510 - Boiler Plant                                15.8               (11)              1.6                       13.4             (17.0)            83.39               2.5                       13.4               (17)            99.14               1.3

344510 - Turbogenerator                              15.8               (11)              1.6                       13.4              (9.0)            92.77               1.2                       13.4                (9)            93.81               1.1

345510 - Access.  Elec. Eqpt.                        15.8               (11)              1.6                       13.4              (4.0)            96.91               0.5                       13.4                (4)            88.92               1.1

346510 - Miscellaneous                               15.8               (11)              1.6                        0.0             (17.0)             0.00               0.0                         --               (17)               --                --

PHILLIPS  STATION                                    28.0               (20)              4.3                       15.4             (12.0)            53.66               3.8                       15.4               (12)            53.66               3.8

POLK  POWER  STATION                                                                                                  26             (12.0)             0.00               4.3                         26               (12)             0.00               4.3

AMORTIZABLE   PRODUCTION   PLANT

STEAM PROD. -- BIG BEND STATION                         7               YEAR     AMORTIZATION                          7               YEAR     AMORTIZATION                                            7               YEAR     AMORTIZATION

STEAM PROD. -- GANNON STATION                           7               YEAR     AMORTIZATION                          7               YEAR     AMORTIZATION                                            7               YEAR     AMORTIZATION

MISC. PROD. - 316010 Misc. Power Plant Equipment     14.3                 0               4.1                          7               YEAR     AMORTIZATION                                            7               YEAR     AMORTIZATION

MISC. PROD. - 316170 Misc. Power Plant Equipment        7               YEAR     AMORTIZATION                          7               YEAR     AMORTIZATION                                            7               YEAR     AMORTIZATION

TAMPA ELECTRIC COMPANY

1995 STUDY

COMPARISON OF RATES AND COMPONENTS

ACCOUNT            CURRENT                                                                COMPANY PROPOSAL                                                        STAFF RECOMMENDATION

AVERAGE                                              REMAINING          AVERAGE                                              REMAINING          AVERAGE                                              REMAINING

REMAINING            NET                                LIFE            REMAINING            NET                                LIFE            REMAINING            NET                                 LIFE

LIFE             SALVAGE           RESERVE            RATE              LIFE             SALVAGE           RESERVE            RATE              LIFE             SALVAGE           RESERVE             RATE

(YRS.)              (%)               (%)               (%)             (YRS.)              (%)               (%)               (%)             (YRS.)              (%)               (%)                (%)

TRANSMISSION

350 - Land Rights                            33.0               0.0             28.00               2.2              33.0               0.0             27.87               2.2              33.0               0.0             27.87                 2.2

352 - Structures and Improvements            37.0              (3.0)            20.65               2.2              39.0              (3.0)            22.39               2.1              39.0              (3.0)            22.39                 2.1

353 - Station Equipment                      26.0             (15.0)            28.78               3.3              28.0             (15.0)            34.11               2.9              28.0             (15.0)            34.11                 2.9

354 - Towers and Fixtures                    29.0             (15.0)            43.05               2.5              24.0             (15.0)            55.69               2.5              24.0             (15.0)            55.69                 2.5

355 - Poles and Fixtures                     24.0             (30.0)            24.03               4.4              24.0             (35.0)            23.63               4.6              24.0             (35.0)            23.63                 4.6

356 - Overhead Conductors and Devices        24.0             (15.0)            35.20               3.3              24.0             (15.0)            33.76               3.4              24.0             (15.0)            33.76                 3.4

356.01 - Clearing Rights-of-Way              31.0               0.0             34.44               2.1              29.0               0.0             40.82               2.0              29.0               0.0             40.82                 2.0

357 - Underground Conduit                    21.0               0.0             53.93               2.2              45.0               0.0              9.88               2.0              45.0               0.0              9.88                 2.0

358 - Underground Conductors & Devices       12.9               0.0             63.48               2.8               9.4               0.0             76.32               2.5               9.4               0.0             76.32                 2.5

359 - Roads and Trails                       36.0               0.0             26.15               2.1              36.0               0.0             25.35               2.1              36.0               0.0             25.35                 2.1

DISTRIBUTION

361 - Structures and Improvements            31.0              (3.0)            28.38               2.4              29.0              (3.0)            33.14               2.4              29.0              (3.0)            33.14                 2.4

362 - Station Equipment                      25.0             (15.0)            28.17               3.5              24.0             (15.0)            32.80               3.4              24.0             (15.0)            32.80                 3.4

364 - Poles, Towers, and Fixtures            23.0             (27.0)            26.47               4.4              26.0             (35.0)            30.29               4.0              26.0             (35.0)            30.29                 4.0

365 - Overhead Conductors and Devices        21.0              (8.0)            34.49               3.5              24.0             (20.0)            40.75               3.3              24.0             (20.0)            40.75                 3.3

366 - Underground Conduit                    41.0               0.0             18.10               2.0              39.0               0.0             22.20               2.0              39.0               0.0             22.20                 2.0

367 - Underground Conductors & Devices       25.0               0.0             23.24               3.1              24.0               0.0             28.16               3.0              24.0               0.0             28.16                 3.0

368 - Line Transformers                      10.3              30.0             27.25               4.2               9.3              30.0             34.03               3.9               9.3              30.0             34.03                 3.9

369.01 - Overhead Services                   25.0              (5.0)            21.75               3.3              26.0             (50.0)            28.58               4.7              26.0             (50.0)            28.58                 4.7

369.01 - Underground Services                29.0             (15.0)            21.06               3.2              27.0             (15.0)            27.66               3.2              27.0             (15.0)            27.66                 3.2

370 - Meters                                 18.0             (19.0)            24.80               5.2              17.0             (20.0)            29.27               5.3              17.0             (20.0)            29.27                 5.3

373 - Street Lighting and Signal Systems     16.1               9.0             22.77               4.2              15.1               0.0             26.65               4.9              15.1               0.0             26.65                 4.9

GENERAL PLANT

390 - Structures and Improvements            31.0             (20.0)            16.87               3.3              29.0             (20.0)            20.35               3.4              29.0             (20.0)            20.35                 3.4

392.1 - Automobiles                           3.0              20.0             42.38              12.5               2.7              24.0             16.09              22.2               2.7              24.0             16.09                22.2

392.2 - Light Trucks                          3.8              20.0             34.85              11.9               3.7              20.0             45.77               9.3               3.7              20.0             45.77                 9.3

392.3 - Heavy Trucks                          9.1              25.0             31.14               4.8               9.3              20.0             41.66               4.1               9.3              20.0             41.66                 4.1

393.01 - Stores Equipment                    21.0               0.0             39.99               2.9              18.3               0.0             54.21               2.5              18.3               0.0             54.21                 2.5

394.01 - Tools, Shop & Garage Equipment       7.1               5.0             67.48               3.9               8.2               5.0             52.75               5.2               8.2               5.0             52.75                 5.2

395.01 - Laboratory Equipment                21.0               0.0             31.72               3.3              17.6               0.0             47.38               3.0              17.6               0.0             47.38                 3.0

396 - Power Operated Equipment               11.0              10.0             55.93               3.1               8.8              10.0             56.77               3.8               8.8              10.0             56.77                 3.8

397 - Communication Equipment                 5.9               0.0             34.49              11.1                   10 Year Amortization                                                             10 Year Amortization

397.01 - Energy Management Systems           13.6               0.0             11.81               6.5               4.5               0.0             57.59               9.4               4.5               0.0             57.59                 9.4

397.25 - Communication Equipment - Fiber      5.9               0.0             11.81              14.9                   10 Year Amortization                                               11.4             (10.0)            38.45                 6.3

391.01 - Office Furniture and Equipment                    7 Year Amortization                                            7 Year Amortization                                                     7 Year Amortization

391.02 - Computer Equipment                                5 Year Amortization                                            5 Year Amortization                                                     5 Year Amortization

393.00 - Stores Equipment                                  7 Year Amortization                                            7 Year Amortization                                                     7 Year Amortization

394.00 - Tools, Shop and Garage Equipment                  7 Year Amortization                                            7 Year Amortization                                                     7 Year Amortization

395.00 - Laboratory Equipment                              7 Year Amortization                                            7 Year Amortization                                                     7 Year Amortization

398.00 - Miscellaneous Equipment                           7 Year Amortization                                            7 Year Amortization                                                     7 Year Amortization

**Denotes restated reserve

TAMPA ELECTRIC COMPANY

1995 STUDY

COMPARISON OF EXPENSES

C U R R E N T                 C O M P A N Y   R E V I S E D   P R O P O S A L                   S T A F F    R E C O M M E N D A T I O N

ESTIMATED            ESTIMATED                                                                                            CHANGE

INVESTMENT            RESERVE                                                                                               IN

ACCOUNT                                1/1/96               1/1/96                RATE            EXPENSES             RATE              EXPENSES             EXPENSES                RATE                 EXPENSES             EXPENSES

(%)                 ($)                 (%)                ($)                  ($)                     %                     $                    $

STEAM  PRODUCTION

BIG BEND STATION

- Common -

311400 - Structures                  42,121,057           13,608,187               2.4            1,010,905               2.1              884,542             (126,363)                      2              842,421             (168,484)

312400 - Boiler Plant                50,724,583           18,276,514               2.4            1,217,390               2.8            1,420,288              202,898                     2.8            1,420,288              202,898

314400 - Turbogenerators              3,298,742            1,408,938               2.4               79,170               1.9               62,676              (16,494)                    1.9               62,676              (16,494)

315400 - Access. Elec Eqpt.          12,691,729            5,098,343               2.4              304,601               3.4              431,519              126,918                     3.4              431,519              126,918

316400 - Miscellaneous                3,507,053            1,735,272               2.4               84,169                 4              140,282               56,113                     3.9              136,775               52,606

- Unit 1 -

311410 - Structures                   6,465,777            2,995,771               2.3              148,713               2.3              148,713                    0                     2.5              161,644               12,931

312410 - Boiler Plant                50,034,557           20,940,563               2.3            1,150,795               3.2            1,601,106              450,311                     3.3            1,651,140              500,345

314410 - Turbogenerators             23,385,883           10,763,875               2.3              537,875                 3              701,576              163,701                     2.9              678,191              140,316

315410 - Access. Elec. Eqpt.         10,019,691            3,805,745               2.3              230,453               2.9              290,571               60,118                     2.9              290,571               60,118

316410 - Miscellaneous                  646,712              264,736               2.3               14,874                 2               12,934               (1,940)                    3.2               20,695                5,821

- Unit 2 -

311420 - Structures                   6,604,619            2,504,676               2.6              171,720               2.3              151,906              (19,814)                    2.5              165,115               (6,605)

312420 - Boiler Plant                43,494,360           18,838,351               2.6            1,130,853               3.2            1,391,820              260,967                     3.2            1,391,820              260,967

314420 - Turbogenerators             24,645,281            9,690,590               2.6              640,777               2.8              690,068               49,291                     2.9              714,713               73,936

315420 - Access. Elec. Eqpt.          8,777,837            3,136,106               2.6              228,224               3.2              280,891               52,667                     3.1              272,113               43,889

316420 - Miscellaneous                  548,322              170,730               2.6               14,256               3.3               18,095                3,839                     3.3               18,095                3,839

- Unit 3 -

311430 - Structures                  15,075,910            6,330,812               2.4              361,822               2.1              316,594              (45,228)                    2.2              331,670              (30,152)

312430 - Boiler Plant                82,156,385           38,065,478               2.4            1,971,753               2.7            2,218,222              246,469                     2.8            2,300,379              328,626

314430 - Turbogenerators             28,677,246           16,886,281               2.4              688,254               2.5              716,931               28,677                     2.4              688,254                    0

315430 - Access. Elec. Eqpt.         20,458,936            8,063,343               2.4              491,014               2.8              572,850               81,836                     2.9              593,309              102,295

316430 - Miscellaneous                  740,640              311,016               2.4               17,775               2.5               18,516                  741                     2.6               19,257                1,482

- Unit 4 -

311440 - Structures                  62,381,029           18,229,870               2.7            1,684,288               2.2            1,372,383             (311,905)                      2            1,247,621             (436,667)

312440 - Boiler Plant               195,629,688           46,840,707               2.7            5,282,002               3.5            6,847,039            1,565,037                     3.7            7,238,298            1,956,296

314440 - Turbogenerators             81,011,780           25,663,858               2.7            2,187,318               2.6            2,106,306              (81,012)                    2.4            1,944,283             (243,035)

315440 - Access. Elec. Eqpt.         35,706,700           11,223,899               2.7              964,081               2.7              964,081                    0                     2.6              928,374              (35,707)

316440 - Miscellaneous                5,324,966            2,000,748               2.7              143,774               3.2              170,399               26,625                     2.9              154,424               10,650

- Unit 4  FGD -

311450 - Structures                  21,221,962            5,565,942               2.5              530,549               2.2              466,883              (63,666)                    2.2              466,883              (63,666)

312450 - Boiler Plant               142,790,572           45,106,539               2.5            3,569,764               2.8            3,998,136              428,372                     2.8            3,998,136              428,372

315450 - Access. Elec. Eqpt.         18,234,179            5,462,482               2.5              455,854               2.6              474,089               18,235                     2.6              474,089               18,235

316450 - Miscellaneous                  244,694               79,579               2.5                6,117                 3                7,341                1,224                     3.1                7,586                1,469

TOTAL  BIG BEND  STATION 996,620,890          343,068,951                             25,319,140                             28,476,757            3,157,617                                   28,650,339            3,331,199

STEAM  PRODUCTION  (CONT'D)

GANNON  STATION

- Common -

311500 - Structures           29,910,870           10,051,659               3.7            1,106,702               3.4            1,016,970           (89,732)              3.3              987,059          (119,643)

312500 - Boiler Plant         13,864,779            5,392,081               3.7              512,997               3.9              540,726            27,729               3.9              540,726            27,729

314500 - Turbogenerators       1,662,098              427,617               3.7               61,498                 3               49,863           (11,635)              3.2               53,187            (8,311)

315500 - Access. Elec. Eqpt.   6,459,954            1,339,897               3.7              239,018                 3              193,799           (45,219)              3.2              206,719           (32,299)

316500 - Miscellaneous         3,131,523            1,642,868               3.7              115,866               4.6              144,050            28,184               4.7              147,182            31,316

- Unit  1 -

311510 - Structures            2,602,102            1,956,255               3.3               85,869               2.1               54,644           (31,225)              2.5               65,053           (20,816)

312510 - Boiler Plant          9,235,039            7,023,120               3.3              304,756               3.8              350,931            46,175               3.6              332,461            27,705

314510 - Turbogenerators       7,659,040            5,867,962               3.3              252,748               2.9              222,112           (30,636)                3              229,771           (22,977)

315510 - Access. Elec. Eqpt.   2,054,670            1,516,013               3.3               67,804               2.8               57,531           (10,273)              3.1               63,695            (4,109)

316510 - Miscellaneous           254,128              219,558               3.3                8,386               1.3                3,304            (5,082)              2.7                6,861            (1,525)

- Unit  2 -

311520 - Structures            2,836,621            1,864,168               3.7              104,955               3.1               87,935           (17,020)              3.1               87,935           (17,020)

312520 - Boiler Plant          9,231,357            6,334,013               3.7              341,560               4.7              433,874            92,314               4.2              387,717            46,157

314520 - Turbogenerators      10,747,571            7,007,721               3.7              397,660               3.1              333,175           (64,485)              3.4              365,417           (32,243)

315520 - Access. Elec. Eqpt.   1,636,932            1,040,281               3.7               60,566               3.1               50,745            (9,821)              3.5               57,293            (3,273)

316520 - Miscellaneous            75,834               59,727               3.7                2,806               1.7                1,289            (1,517)              3.1                2,351              (455)

- Unit  3 -

311530 - Structures            2,202,515            1,431,245               3.8               83,696               2.1               46,253           (37,443)              2.8               61,670           (22,026)

312530 - Boiler Plant         18,001,574           10,933,947               3.8              684,060               4.1              738,065            54,005               3.9              702,061            18,001

314530 - Turbogenerators      11,714,241            7,975,955               3.8              445,141               2.9              339,713          (105,428)              3.2              374,856           (70,285)

315530 - Access. Elec. Eqpt.   2,342,879            1,503,928               3.8               89,029               3.1               72,629           (16,400)              3.4               79,658            (9,371)

316530 - Miscellaneous           104,950               89,069               3.8                3,988               2.6                2,729            (1,259)              3.2                3,358              (630)

- Unit  4 -

311540 - Structures            1,718,758              971,906               3.4               58,438               2.2               37,813           (20,625)              2.7               46,406           (12,032)

312540 - Boiler Plant         18,303,944            9,035,049               3.4              622,334                 4              732,158           109,824               3.8              695,550            73,216

314540 - Turbogenerators       8,668,427            6,000,641               3.4              294,727               2.4              208,042           (86,685)              2.8              242,716           (52,011)

315540 - Access. Elec. Eqpt.   2,245,260            1,302,628               3.4               76,339               3.5               78,584             2,245               3.5               78,584             2,245

316540 - Miscellaneous            50,046               34,582               3.4                1,702               2.2                1,101              (601)              2.8                1,401              (301)

- Unit  5 -

311550 - Structures            3,605,278            1,548,389               3.4              122,579               2.9              104,553           (18,026)              3.2              115,369            (7,210)

312550 - Boiler Plant         28,169,318           12,754,799               3.4              957,757               3.8            1,070,434           112,677               3.8            1,070,434           112,677

314550 - Turbogenerators      12,860,275            5,776,887               3.4              437,249               3.4              437,249                 0               3.4              437,249                 0

315550 - Access. Elec. Eqpt.   5,823,473            2,261,916               3.4              197,998                 4              232,939            34,941                 4              232,939            34,941

316550 - Miscellaneous           350,652              111,926               3.4               11,922               3.5               12,273               351               3.9               13,675             1,753

- Unit  6 -

311560 - Structures            4,590,509            2,133,836               3.2              146,896               2.6              119,353           (27,543)              2.7              123,944           (22,952)

312560 - Boiler Plant         41,745,273           18,828,348               3.2            1,335,849               3.7            1,544,575           208,726               3.6            1,502,830           166,981

314560 - Turbogenerators      24,008,318            7,557,822               3.2              768,266               3.4              816,283            48,017               3.5              840,291            72,025

315560 - Access. Elec. Eqpt.   6,510,375            2,792,549               3.2              208,332               3.7              240,884            32,552               3.7              240,884            32,552

316560 - Miscellaneous           298,023              176,937               3.2                9,537               3.4               10,133               596               3.3                9,835               298

294,676,606          144,965,299                             10,219,030                             10,386,711           167,681                             10,407,137           188,107

STEAM  PRODUCTION  (CONT'D)

GANNON   OIL   BACKOUT

- Common -                                 43,818,293           20,543,288                 5            2,190,915               2.5            1,095,457            (1,095,458)              3.5            1,533,640              (657,275)

- Unit 1 -                                 18,873,074           14,902,090                 5              943,654               4.9              924,781               (18,873)              2.9              547,319              (396,335)

- Unit 2 -                                 21,092,364           14,506,218                 5            1,054,618               4.7              991,341               (63,277)              3.5              738,233              (316,385)

- Unit 3 -                                 25,053,513           17,203,050                 5            1,252,676               3.6              901,926              (350,750)                3              751,605              (501,071)

- Unit 4 -                                 31,468,017           18,678,325                 5            1,573,401               2.9              912,572              (660,829)              3.1              975,509              (597,892)

TOTAL  OIL BACKOUT               140,305,261           85,832,971                              7,015,264                              4,826,077            (2,189,187)                             4,546,306            (2,468,958)

HOOKER'S  POINT  STATION                   52,601,551           46,898,372               2.2            1,157,234               1.7              894,226              (263,008)              1.7              894,226              (263,008)

DINNER  LAKE  STATION                       3,699,556            3,039,727               3.4              125,785               3.4              125,785                     0               3.4              125,785                     0

TOTAL  STEAM  PRODUCTION           1,487,903,864          623,805,320                             43,836,453                             44,709,556               873,103                             44,623,793               787,340

MISCELLANEOUS    PRODUCTION

311010 - Structures & Improvements          6,676,569            2,515,405               3.3              220,327               3.1              206,974               (13,353)              3.1              206,974               (13,353)

TOTAL  MISCELLANEOUS  PRODUCTION       6,676,569            2,515,405                                220,327                                206,974               (13,353)                               206,974               (13,353)

OTHER    PRODUCTION

BIG BEND STATION

- Combustion Turbine No. 1 -

341410 - Structures                            83,072               79,304                 2                1,661               0.7                  582                (1,079)              0.6                  498                (1,163)

342410 - Boiler Plant                         113,663              117,995                 2                2,273               2.8                3,183                   910                 1                1,137                (1,136)

344410 - Turbogenerator                     1,384,582            1,160,824                 2               27,692               1.8               24,922                (2,770)                2               27,692                     0

345410 - Access.  Elec.  Eqpt.                174,543              162,519                 2                3,491               0.7                1,222                (2,269)              0.8                1,396                (2,095)

346410 - Miscellaneous                          2,642                3,251                 2                   53               3.3                   87                    34                -1                  (26)                  (79)

- Combustion Turbine No. 2 & 3 -

341420 - Structures                         1,432,475            1,199,286               4.4               63,029               3.4               48,704               (14,325)              3.6               51,569               (11,460)

342420 - Boiler Plant                         944,348              795,900               4.4               41,551               3.8               35,885                (5,666)              3.7               34,941                (6,610)

344420 - Turbogenerator                    15,812,806           11,038,241               4.4              695,763               3.8              600,887               (94,876)                4              632,512               (63,251)

345420 - Access.  Elec. Eqpt.               2,384,066            1,789,610               4.4              104,899               5.7              135,892                30,993               4.6              109,667                 4,768

346420 - Miscellaneous                         16,640               14,975               4.4                  732               3.7                  616                  (116)              4.2                  699                   (33)

OTHER    PRODUCTION     (CONT'D)

GANNON STATION

- Combustion Turbine No. 1 -

341510 - Structures                                       75,362               66,001               1.6                1,206               0.3                  226                 (980)              1.2                  904                 (302)

342510 - Boiler Plant                                     93,008               92,204               1.6                1,488               2.5                2,325                  837               1.3                1,209                 (279)

344510 - Turbogenerator                                1,387,006            1,301,169               1.6               22,192               1.2               16,644               (5,548)              1.1               15,257               (6,935)

345510 - Access.  Elec.  Eqpt.                           257,335              228,812               1.6                4,117               0.5                1,287               (2,830)              1.1                2,831               (1,286)

346510 - Miscellaneous                                         0                    0               1.6                    0                 0                    0                    0                --                    0                    0

PHILLIPS  STATION                                     59,626,801           31,997,042               4.3            2,563,952               3.8            2,265,818             (298,134)              3.8            2,265,818             (298,134)

POLK  POWER  STATION                                 484,400,000                    0                                                      4.3           20,829,200           20,829,200               4.3           20,829,200           20,829,200

TOTAL  OTHER  PRODUCTION                        568,188,349           50,047,133                              3,534,099                             23,967,480           20,433,381                             23,975,304           20,441,205

TOTAL   DEPRECIABLE   PRODUCTION   PLANT           2,062,768,782          676,367,858                             47,590,879                             68,884,010           21,293,131                             68,806,071           21,215,192

AMORTIZABLE   PRODUCTION   PLANT

STEAM PROD. -- BIG BEND STATION                        3,407,559            2,148,627            7  YR  AMORT        487,281            7  YR  AMRT         487,281                    0            7  YR  AMRT         487,281                    0

STEAM PROD. -- GANNON STATION                          1,990,267            1,152,623            7  YR  AMORT        284,608            7  YR  AMRT         284,608                    0            7  YR  AMRT         284,608                    0

MISC. PROD. - 316010 Misc. Power Plant Equipment       3,366,124            1,801,673               4.1              138,011            7  YR  AMRT         223,717               85,706            7  YR  AMRT         223,717               85,706

MISC. PROD. - 316010 Misc. Power Plant Equipment       1,997,953            1,160,793            7  YR  AMORT        285,707            7  YR  AMRT         285,707                    0            7  YR  AMRT         285,707                    0

TOTAL   AMORTIZABLE   PRODUCTION   PLANT              10,761,903            6,263,716                              1,195,607                              1,281,313               85,706                              1,281,313               85,706

TOTAL    PRODUCTION    PLANT                2,073,530,685          682,631,574                             48,786,486                             70,165,323           21,378,837                             70,087,384           21,300,898

TAMPA ELECTRIC COMPANY

1995 STUDY

COMPARISON OF EXPENSES

CURRENT                               COMPANY REVISED PROPOSAL                                    STAFF RECOMMENDATION

ESTIMATED                ESTIMATED                                                                                              CHANGE                                                      CHANGE

ACCOUNT                                                    INVESTMENT                RESERVE                                                                                                 IN                                                          IN

1/1/96                   1/1/96                RATE              EXPENSES             RATE            EXPENSES               EXPENSES             RATE              EXPENSES             EXPENSES

(%)                ($)                 (%)                ($)                  ($)                 (%)                ($)                   ($)

TRANSMISSION PLANT

350 - Land Rights                                     4,284,474                1,194,058               2.2               94,258               2.2               94,258                    0               2.2               94,258                     0

352 - Structures and Improvements                     1,308,745                  293,067               2.2               28,792               2.1               27,484               (1,308)              2.1               27,484                (1,308)

353 - Station Equipment                             101,027,203               34,464,399               3.3            3,333,898               2.9            2,929,789             (404,109)              2.9            2,929,789              (404,109)

354 - Towers and Fixtures                             4,342,275                2,418,394               2.5              108,557               2.5              108,557                    0               2.5              108,557                     0

355 - Poles and Fixtures                             56,702,156               13,399,544               4.4            2,494,895               4.6            2,608,299              113,404               4.6            2,608,299               113,404

356 - Overhead Conductors and Devices                56,835,865               19,188,198               3.3            1,875,584               3.4            1,932,419               56,835               3.4            1,932,419                56,835

356.01 - Clearing Rights-of-Way                       1,820,514                  743,050               2.1               38,231                 2               36,410               (1,821)                2               36,410                (1,821)

357 - Underground Conduit                             6,203,994                  613,051               2.2              136,488                 2              124,080              (12,408)                2              124,080               (12,408)

358 - Underground Conductors & Devices                  931,348                  710,788               2.8               26,078               2.5               23,284               (2,794)              2.5               23,284                (2,794)

359 - Roads and Trails                                2,360,433                  598,276               2.1               49,569               2.1               49,569                    0               2.1               49,569                     0

Total Transmission Plant             235,817,006               73,622,825                              8,186,350                              7,934,149             (252,201)                               7934149              (252,201)

DISTRIBUTION PLANT

361 - Structures and Improvements                       698,706                  231,564               2.4               16,769               2.4               16,769                    0               2.4               16,769                     0

362 - Station Equipment                              93,824,723               30,771,801               3.5            3,283,865               3.4            3,190,041              (93,824)              3.4            3,190,041               (93,824)

364 - Poles, Towers, and Fixtures                   106,231,622               32,177,160               4.4            4,674,191                 4            4,249,265             (424,926)                4            4,249,265              (424,926)

365 - Overhead Conductors and Devices               132,373,718               53,940,955               3.5            4,633,080               3.3            4,368,333             (264,747)              3.3            4,368,333              (264,747)

366 - Underground Conduit                            61,420,490               13,636,372                 2            1,228,410                 2            1,228,410                    0                 2            1,228,410                     0

367 - Underground Conductors & Devices               76,626,435               21,579,747               3.1            2,375,419                 3            2,298,793              (76,626)                3            2,298,793               (76,626)

368 - Line Transformers                             201,378,222               68,522,400               4.2            8,457,885               3.9            7,853,751             (604,134)              3.9            7,853,751              (604,134)

369.01 - Overhead Services                           41,225,145               11,781,543               3.3            1,360,430               4.7            1,937,582              577,152               4.7            1,937,582               577,152

369.01 - Underground Services                        39,456,265               10,914,024               3.2            1,262,600               3.2            1,262,600                    0               3.2            1,262,600                     0

370 - Meters                                         37,302,287               10,919,315               5.2            1,939,719               5.3            1,977,021               37,302               5.3            1,977,021                37,302

373 - Street Lighting and Signal Systems             60,394,836               16,096,222               4.2            2,536,583               4.9            2,959,347              422,764               4.9            2,959,347               422,764

Total Distribution Plant            850,932,449              270,571,103                             31,768,951                             31,341,912             (427,039)                            31,341,912              (427,039)

GENERAL PLANT

390 - Structures and Improvements                    56,256,971               11,446,229               3.3            1,856,480               3.4            1,912,737               56,257               3.4            1,912,737                56,257

392.1 - Automobiles                                     961,475                  447,298              12.5              120,184              22.2              213,447               93,263              22.2              213,447                93,263

392.2 - Light Trucks                                  5,820,499                3,196,894              11.9              692,639               9.3              541,306             (151,333)              9.3              541,306              (151,333)

392.3 - Heavy Trucks                                 26,926,179                9,483,265               4.8            1,292,457               4.1            1,103,973             (188,484)              4.1            1,103,973              (188,484)

393.01 - Stores Equipment                               578,179                  313,411               2.9               16,767               2.5               14,454               (2,313)              2.5               14,454                (2,313)

394.01 - Tools, Shop & Garage Equipment                 728,812                  384,469               3.9               28,424               5.2               37,898                9,474               5.2               37,898                 9,474

395.01 - Laboratory Equipment                         1,198,928                  568,059               3.3               39,565               3.0               35,968               (3,597)              3.0               35,968                (3,597)

396 - Power Operated Equipment                          872,854                  495,488               3.1               27,058               3.8               33,168                6,110               3.8               33,168                 6,110

397 - Communication Equipment                        48,344,980               23,531,707              11.1            5,366,293          10 Yr. Amort.       4,834,498             (531,795)         10 Yr. Amort.       4,834,498              (531,795)

397.01 - Energy Management Systems                   33,582,172               19,339,617               6.5            2,182,841               9.4            3,156,724              973,883               9.4            3,156,724               973,883

397.25 - Communication Equipment - Fiber              7,078,287                2,721,399              14.9            1,054,665          10 Yr. Amort.       707,828.7             (346,836)              6.3              445,932              (608,733)

Subtotal                             182,349,337               71,927,836                             12,677,373                             12,592,002              (85,371)                            12,330,105              (347,268)

391.01 - Office Furniture and Equipment               4,943,992                2,745,749          7 Yr Amort.           706,991          7 Yr Amort.           706,991                    0          7 Yr Amort.           706,991                     0

391.02 - Computer Equipment                          17,578,819               11,027,104          5 Yr Amort.         3,515,764          5 Yr Amort.         3,515,764                    0          5 Yr Amort.         3,515,764                     0

393.00 - Stores Equipment                               129,403                   77,675          7 Yr Amort.            18,505          7 Yr Amort.            18,505                    0          7 Yr Amort.            18,505                     0

394.00 - Tools, Shop and Garage Equipment             3,207,858                1,680,791          7 Yr Amort.           458,724          7 Yr Amort.           458,724                    0          7 Yr Amort.           458,724                     0

395.00 - Laboratory Equipment                           899,696                  353,203          7 Yr Amort.           128,657          7 Yr Amort.           128,657                    0          7 Yr Amort.           128,657                     0

398.00 - Miscellaneous Equipment                        475,552                  152,459          7 Yr Amort.            68,004          7 Yr Amort.            68,004                    0          7 Yr Amort.            68,004                     0

Subtotal                              27,235,320               16,036,981                              4,896,645                              4,896,645                    0                              4,896,645                     0

Total General Plant       209,584,657               87,964,817                             17,574,018                             17,488,647              (85,371)                            17,226,750              (347,268)

Total Transmission, Distribution, General Plant   1,296,334,112              432,158,746                             57,529,319                             56,764,708             (764,611)                            56,502,811            (1,026,508)

Total Production Plant                            2,073,530,685              682,631,574                             48,786,486                             70,165,323           21,378,837                             70,087,384            21,300,898

TOTAL DEPRECIABLE PLANT                           3,369,864,797            1,114,790,320                            106,315,805                            126,930,031           20,614,226                            126,590,195            20,274,390

Tampa Electric Company                                                                   Attachment D

Production Dismantlement Study                                                           Page 1 of 1

Estimate as of 12/31/94

Current       DRI + 20%     DRI + 16%

Annual         Annual        Annual

Accrual        Accrual       Accrual

Big Bend Common                                                533,188.00     493,340.00   476,083.00

Big Bend Unit No. 1                                            906,800.00     798,108.00   769,273.00

Big Bend Unit No. 2                                            606,322.00     559,615.00   539,661.00

Big Bend Unit No. 3                                            543,937.00     518,080.00   499,742.00

Big Bend Unit No. 4                                          1,019,485.00     980,164.00   944,798.00

Big Bend Unit No. 4 FGD                                        449,632.00     392,733.00   378,526.00

Gannon Common                                                  362,504.00     467,917.00   450,742.00

Gannon Unit No. 1                                              812,947.00     729,975.00   698,991.00

Gannon Unit No. 2                                              637,700.00     555,348.00   532,059.00

Gannon Unit No. 3                                              628,587.00     551,895.00   529,456.00

Gannon Unit No. 4                                              535,371.00     467,277.00   448,873.00

Gannon Unit No. 5                                              496,041.00     434,039.00   417,215.00

Gannon Unit No. 6                                              488,561.00     441,189.00   424,344.00

Hookers Point                                                  570,969.00     207,139.00   170,325.00

Dinner Lake                                                     41,982.00      46,673.00    44,676.00

Big Bend CT No. 1, 2 & 3                                        26,622.00      82,269.00    79,203.00

Gannon CT No. 1                                                  5,465.00      18,787.00    18,102.00

Phillips Station                                               103,923.00     115,406.00   110,971.00

Total                                                        8,770,036.00   7,859,954.00 7,533,040.00

Polk Power Station                                           1,348,764.00   1,348,764.00 1,303,805.00

Total                                                       10,118,800.00   9,208,718.00 8,836,845.00

Note:       Current Annual Accrual reflects current annual accruals for all units and

proposed annual accrual for Polk Power Station including contingency

of 20%.