Attachment 2

Network Elements and Other Services

TABLE OF CONTENTS

1.	Introduction	3

2.	Unbundled Loops	4

3.	Integrated Digital Loop Carriers	10

4.	Network Interface Device	11

5.	Unbundled Loop Concentration (ULC) System	12

6.	Sub-Loop Elements	13

7.  	Local Switching	17

8.	Interoffice Transmission Facilities	24

9.	Tandem Switching	30

10.	 Combinations	32

11.	Operator Systems	38

12.	Signaling	43

13.	Signaling Transfer Points (STPs)	45

14.	Service Control Points/DataBases	49

15.	Dark Fiber	56

16.	SS7 Network Interconnection	57

17.	Basic 911 and E911	61

18.	Rates	62

1.00  DEFINITIONS	74

2.0	ATTACHMENT	74

3.00  PHYSICAL CONNECTION AND COMPENSATION	75

4.00  CNAM RECORD INITIAL LOAD AND UPDATES	75

EXHIBIT A – LIDB STORAGE AGREEMENT……………………………………EXHIBIT A

EXHIBIT B – CNAM DATABASE SERVICES……………………………………EXHIBIT B

EXHIBIT C – RATES .………….…………………………………………………..EXHIBIT C

ACCESS TO NETWORK ELEMENTS AND OTHER SERVICES

# Introduction

1.1	Network Element is defined to mean a facility or equipment used in the provision of a telecommunications service.  Such term may include, but is not limited to, features, functions, and capabilities that are provided by means of such facility or equipment, including but not limited to, subscriber numbers, databases, signaling systems, and information sufficient for billing and collection or used in the transmission, routing, or other provision of a telecommunications service.  BellSouth offers access to the Network Elements, unbundled loops; network interface device; sub-loop elements; local switching; transport; tandem switching; operator systems; signaling; access to call-related databases; dark fiber as set forth in this Attachment.

### 1.2	BellSouth shall, upon request of Cbeyond, and to the extent technically feasible, provide to Cbeyond access to its  network elements for the provision of Cbeyond’s telecommunications service.  If no rate is identified in the contract, the rate for the specific service or function will be as set forth in applicable BellSouth tariff or as negotiated by the Parties upon request by either Party.

### 1.3	Cbeyond may purchase network elements and other services from BellSouth for the purpose of combining such network elements in any manner Cbeyond chooses to provide telecommunication services to its intended users, including recreating existing BellSouth services.  With the exception of the sub-loop elements which are located outside of the central office, BellSouth shall deliver the  network elements purchased by Cbeyond for combining to the designated Cbeyond collocation space.  The network elements shall be provided as set forth in this Attachment.

### 1.4	BellSouth will provide the following combined  network elements for purchase by Cbeyond.  The rate of the following combined  network elements is the sum of the individual element prices as set forth in this Attachment.  Order Coordination as defined in Section 2 of Attachment 2 of this Agreement is available for each of these combinations:

- SL1 or SL2 loop and cross connect

- Port and cross connect

- Port and cross connect and common (shared) transport

- Port and vertical features

- SL2 Loop with loop concentration

- Port and common (shared) transport

- SL1 or SL2 Loop and LNP

### 1.5	BellSouth shall comply with the requirements as set forth in the technical references within Attachment 2 to the extent that they are consistent with the greater of BellSouth’s actual performance or applicable industry standards.

1.6	Cbeyond will adopt and adhere to the standards contained in the applicable CLEC Work Center Operational Understanding Agreement regarding maintenance and installation of service, provided however that nothing required in this Work Center Operational Understanding Agreement shall override Cbeyond’s rights or BellSouth’s obligations under this Agreement.

# 2.	Unbundled Loops

### 2.1	BellSouth agrees to offer access to loops pursuant to the following terms and conditions and at the rates set forth in this Attachment.

## 2.2	Definition

### The loop is the physical medium or functional path on which a subscriber’s traffic is carried from the MDF or similar terminating device in a central office up to the termination at the NID at the customer’s premise.  Each  loop will be provisioned with a NID.

### The provisioning of service to a CLEC will require cross-office cabling and cross-connections within the central office to connect the loop to a local switch or to other transmission equipment in collocation space.  These cross-connects are a separate element and are not considered a part of the loop.

2.2.3  	BellSouth Order Coordination referenced in Attachment 2 includes two types:  “Order Coordination” and “Order Coordination - Time Specific.”

2.2.4	“Order Coordination” refers to standard BellSouth service order coordination involving SL2 voice loops and all digital loops. Order coordination for physical cut-overs will be scheduled as follows:

      - For a coordinated conversion i.e. stand alone INP, INP and LNP   with loop or stand alone loop where order coordination is provided for in this agreement, BellSouth shall verbally coordinate the disconnect with Cbeyond and perform any switch translations so as to limit end user service outage.  BellSouth and Cbeyond will mutually agree upon a cut-over time 24 to 48 hours prior to the actual conversion.  Cbeyond may designate the conversion time when the conversion involves a loop with ILNP or LNP by ordering time specific conversion at rates designated in this agreement.  For time specific conversions, BellSouth will verify the cut-over time designated by Cbeyond 24 to 48 hours in advance to ensure that the conversion is to be completed as ordered.   Both parties will use best efforts to ensure mutually agreed to conversion times, as identified in this paragraph, will commence within 15 minutes of the agreed time.  For coordinated conversions, BellSouth’s target intervals for service disruption to the end-user is 15 minutes or less for each loop.

2.2.4.2  	Order coordination is not provided for in the provisioning of an SL1 loop.  BellSouth will however, provide a notifier to the Cbeyond when the physical wirework is completed for an SL1 loop with LNP.  This notification will allow the Cbeyond to ensure minimal end user loss of service, provided that Cbeyond promptly sends the activate message to NPAC to port the number.  BellSouth will use best efforts to notify Cbeyond within thirty (30) minutes of the completion of the physical wire work.

2.2.4.3	BellSouth normal hours of operation are defined in Attachment 6.  Provisioning outside of these hours will be billed at overtime rates for the number of employees supporting the after hours conversion.

2.2.4.4  		Testing

2.2.4.4.1  	BellSouth will perform the appropriate pre-service tests to ensure Cbeyond dial tone is delivered to the appropriate connecting point.  The timing of the test is based on the overall interval and type of the service being provisioned.  Under normal intervals, testing for designed services are normally completed 24 hours in advance of the conversion. For non-designed services, dial tone is verified in time frames consistent with the same time frames that BellSouth uses to activate POTS services for it’s own end users.  In any event, BellSouth will advise Cbeyond whenever connectivity cannot be verified with Cbeyond and will work cooperatively with Cbeyond to correct the problem.  BellSouth will advise Cbeyond at completion of the conversion or turn up of new services in order for Cbeyond to accept or reject the services being provisioned.  BellSouth will work cooperatively with Cbeyond to ensure end user service outage is minimal.

2.2.4.4.2  	Where a field visit is required to provision the loop, BellSouth will test the loop ordered by Cbeyond to the NID.  Testing requested by Cbeyond to points beyond the NID will be billed a time and material charge at the same increments BellSouth charges it’s own end users.  Requests for field-testing where a dispatch is not required may be made by Cbeyond and where mutually agreed to, BellSouth will dispatch to perform additional field testing at rates billed on a time and material basis as mentioned in the previous paragraph.

BellSouth will place a tag on all unbundled loops that require a technician to be dispatched to the end user's premises during the provisioning process.  The loop tag will include the CLEC's name and the circuit ID number.  Otherwise, the loop will be tagged by BellSouth during the next scheduled maintenance or repair visit to the customer's location for that loop; or the loop may be tagged by the CLEC during their dispatch to that customer's location.

2.2.4.4.3	Cut-over intervals for ILNP, ILNP with loop and LNP with loop will be at parity with the intervals experienced by BellSouth end users, BellSouth itself or any other Cbeyond as indicated in the results of the Service Quality Measurements published by BellSouth.  In any event, BellSouth will use best efforts to convert each loop within fifteen (15) minutes.

2.2.4.4.4	BellSouth and Cbeyond will jointly develop additional processes or procedures as the need arises to improve service delivery during the life of the agreement.

    - “Order Coordination – Time Specific” refers to service order coordination in which Cbeyond requests a specific time for a service order conversion to take place.  Loops on a single service order of 14 or more loops will be provisioned on a project basis.  This is a chargeable option for any coordinated order and is billed in addition to the OC charge.  Cbeyond may specify a time between 8:00 a.m. and 5:00 p.m. (location time) Monday through Friday (excluding holidays).  If Cbeyond specifies a time outside this window, or selects a time or quantity of loops that requires BellSouth technicians to work outside normal work hours, overtime charges will apply in addition to the OC and OC-TS charges.  Overtime charges will be applied according to actual costs based on type of force group required to perform the work, overtime hours worked and any special circumstances.

    - Where facilities are available, BellSouth will install unbundled loops at the same intervals that it does for itself, its end-users, and other CLECs at  parity as described above. Where BellSouth does not provide intervals based on the above, BellSouth will be subject to the terms and conditions of the performance measures in accordance with Attachment 9.  Some  loops require a Service Inquiry (SI) to determine if facilities are available prior to issuing the order.  The interval for the SI process is separate from the installation interval.  For expedite requests by Cbeyond, expedite charges will apply for intervals less than 5 days.  The charges outlined in BST’s FCC # 1 Tariff, Section 5.1.1, will apply. BellSouth will bill expedite charges the same as BellSouth bills its wholesale customers and other CLECs.  If Cbeyond cancels an order for network elements and other services, any costs incurred by BellSouth in conjunction with the provisioning of that order will be recovered in accordance with FCC #1 Tariff, Section 5.4.

2.2.7 	If Cbeyond modifies an order after being sent a Firm Order Confirmation (FOC) from BellSouth, reasonable costs incurred by BellSouth to accommodate the modification will be reimbursed by Cbeyond. Upon request BellSouth will provide Cbeyond an invoice detailing such charges.

2.2.8  	BellSouth will offer Unbundled Voice Loops (UVL) in two different service levels - Service Level One (SL1) and Service Level Two (SL2).

2.2.8.1 	SL1 loops will be non-designed, will not have test points, and will not come with any Order Coordination (OC) or engineering information/circuit make-up data.  Upon issuance of an order in the service order system, SL1 loops will be activated on the due date in the same manner and time frames that BellSouth normally activates POTS-type loops for its customers.  If Cbeyond requests work to be done for SL1s that requires BellSouth technicians to work outside normal work hours, overtime charges will be applied according to actual costs based on type of force group required to perform the work, overtime hours worked and any special circumstances, such charges will be consistent with BellSouth’s applicable tariffs.

2.2.8.2  	SL2 loops have test points, will be designed with a Design Layout Record provided to Cbeyond, and will be provided with Order Coordination.  The OC feature will allow Cbeyond to coordinate the installation of the loop with the disconnect of an existing customer’s service and/or number portability service.  In these cases, BellSouth will perform the order conversion with standard order coordination at the mutually agreed upon time during normal working hours.

2.2.9	BellSouth will also offer Unbundled Digital Loops (UDL).  They will be designed, will be provisioned with test points (where appropriate), and will come standard with Order Coordination and a Designed Layout Record (DLR).

2.2.10  	In addition to the UVLs and UDLs, BellSouth shall make available an Unbundled Copper Loop (UCL).  The UCL will be a copper twisted pair loop that is unencumbered by any intervening equipment (e.g., filters, load coils, range extenders, digital loop carrier, or repeaters).   The UCL will be offered in two versions - Short and Long.  A short UCL (18 kft or less) will be provisioned according to Resistance Design parameters.  The long UCL (beyond 18kft) will be used when a CLEC wants to condition copper loops longer than 18kft by removing load coils and other intervening equipment.  BST will only ensure electrical continuity and balance relative to tip and ring on UCLs.

      - The UCL will be a designed circuit, with or without conditioning, provisioned with a test point and come standard with a DLR.  OC will be offered as a chargeable option on all UCL loops.  Order Coordination - Time Specific (OC-TS) will not be offered on UCLs.

### 2.2.11	As a chargeable option on all  loops except UVL-SL1, BellSouth will offer Order Coordination - Time Specific (OC-TS).  This will allow Cbeyond the ability to specify the time that the coordinated conversion takes place.  The OC-TS charge for orders due on the same day at the same location will be applied on a per Local Service Request (LSR) basis Cbeyond will be responsible for testing and isolating troubles on the loops.  Once Cbeyond has isolated a trouble to the BellSouth provided loop, Cbeyond will issue a trouble to BellSouth on the loop.  BellSouth will take the actions necessary to repair the loop if a trouble actually exists.  BellSouth will repair these loops in the same time frames that BellSouth repairs similarly situated loops to its customers.

2.2.12 	If Cbeyond reports a trouble on loops and no trouble actually exists, BellSouth will charge Cbeyond for any dispatching and testing (both inside and outside the CO) required by BellSouth in order to confirm the loop's working status.  Failure of BellSouth personnel to find trouble in BellSouth facilities will result in no charge if trouble is actually in those facilities but not discovered at the time.

2.3		Technical Requirements

2.3.1  	To the extent available within BST's Network at a particular location, BellSouth will offer loops capable of supporting telecommunications services such as: Voice Grade (designed and non-designed), basic rate ISDN (even if the loop uses digital loop carrier), ADSL, HDSL (2 and 4 wire), DS1, digital data (up to 64 kbps), primary rate ISDN, and copper loops that are capable of supporting xDSL services.  If a requested loop type is not available, Cbeyond can use the Special Construction process to request that BellSouth place facilities or otherwise modify facilities in order to meet Cbeyond's request.

2.3.1.1	These loop types may also support other telecommunications services that the CLEC may offer, including, but not limited to, Centrex, PBX (analog and data), and N x 64 kbps.

The loop will support the transmission, signaling, performance and interface requirements of the services described in 2.3.1 above.  It is recognized that the requirements of different services are different, and that a number of types or grades of loops are required to support these services.  Services provided over the loop by Cbeyond will be consistent with industry standards and BellSouth TR73600.

      - Cbeyond may utilize the unbundled loops to provide any telecommunication service it wishes.  However, BellSouth will only provision, maintain and repair the loops to the standards that are consistent with the type of loop ordered provided, however  BellSouth will condition the loops consistent with Cbeyond’s request.  For example, if Cbeyond orders an ISDN-capable loop but wants to use the loop for a service other than ISDN, BellSouth will only support that the loop is capable of providing ISDN service. For non-service specific loops (e.g. UCL, loops modified by Cbeyond using the Special Construction process), BellSouth will only support that the loop has electrical continuity and balanced tip-and-ring.

2.3.1.3	In those cases where Cbeyond has requested that BellSouth modify a loop so that it no longer meets the technical parameters for a specific loop (e.g., voice grade, ISDN, ADSL, etc.), the resulting modified loop will be ordered and maintained as a Unbundled Cooper Loop.

### 2.3.2	The loop shall be provided to Cbeyond in accordance with the following Technical References:

BellSouth’s TR73600, Unbundled Local Loop Technical Specification

#### 2.3.2.1	Telcordia (formerly BellCore) TR-NWT-000057, Functional Criteria for Digital Loop Carrier Systems, Issue 2, January 1993.

#### 2.3.2.2	Telcordia (formerly BellCore) TR-NWT-000393, Generic Requirements for ISDN Basic Access Digital Subscriber Lines.

#### 2.3.2.3	ANSI T1.102 - 1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces.

#### 2.3.2.4	ANSI T1.403 - 1989, American National Standard for Telecommunications - Carrier to Customer Installation, DS1 Metallic Interface Specification.

2.3.3		Universal Digital Channel (UDC) Loop

2.3.3.1	Due to technical limitations associated with certain DLC systems, some ISDN-capable loops that are provisioned using DLC systems may not support IDSL service.  Effective with this agreement, BellSouth will no longer reconfigure its ISDN-capable loop to support IDSL service.

2.3.3.2	Instead, BellSouth agrees to offer the Universal Digital Channel (UDC) loop as a part of their Unbundled Digital Loop offerings.  The UDC loop is intended to be compatible with IDSL service and has the same physical characteristics and transmission specifications as BellSouth’s ISDN-capable loop. These specifications are listed in BellSouth’s TR73600.

2.3.3.3	Like the ISDN-capable loop, the UDC loop may be provisioned on copper or through a DLC system.  However, when UDC loops are provisioned using a DLC system, BellSouth will ensure that they are only provisioned on time slots that are compatible with data-only services such as IDSL.

2.4  		Loop Make-Up Service Inquiry

2.4.1	As an interim process until electronic access to loop make-up information is available,  BellSouth shall make available to Cbeyond a Loop Make-Up Service Inquiry process that will provide a description of the loop facility for a specific telephone number or the loop facility(ies)  (DLC and/or copper) serving a specific address.   This information will allow Cbeyond to make a determination of what type of loop to order and what loop conditioning activities (using BellSouth's Unbundled Loop Modification product), if any, are desired by Cbeyond.

2.4.2 	The information provided via this process includes 1) the portion of the loop serviced by Digital Loop Carrier (if applicable), 2) cable lengths and gauges, 3) the presence and location of load coils, 4) the presence, location and length of bridged taps.

2.5		Unbundled Loop Modifications (Loop Conditioning)

2.5.1	Subject to applicable and effective FCC rules and orders, BellSouth shall condition loops, as requested by Cbeyond, whether or not BellSouth offers advanced services to the End User on that loop.

2.5.2	Loop conditioning is defined as the removal from the loop of any devices that may diminish the capability of the loop to deliver high-speed switched wireline telecommunications capability, including xDSL service.  Such devices include, but are not limited to, bridge taps, load coils, low pass filters, and range extenders, and repeaters

2.5.3   	BellSouth shall recover the cost of line conditioning requested by Cbeyond through a recurring charge and/or nonrecurring charge(s) in accordance with the FCC’s forward-looking pricing principles promulgated pursuant to section 252 (d) (1) of the Act and in compliance with FCC Rule 51.507 (e).  Until such time as charges for loop conditioning have been approved by the Commission, Cbeyond shall pay to BellSouth interim cost-based charges as set forth in this Attachment.  Such charges shall be subject to true up, in accordance with Section 18.3 of this Attachment.

# 3.	Integrated Digital Loop Carriers

### The feeder portion of some loops may be provide by means of Integrated Digital Loop Carrier (IDLC). IDLC provides a fiber optic cable transmission path that travels directly into BellSouth’s central office local switch. Where BellSouth uses IDLC ,if technically feasible and capacity does exist, BST will provide Cbeyond with a Designed DS0 UVL by using alternative provisioning techniques including but not limited to such as “hairpinning” and DAC grooming.  Alternative provisioning techniques will be provided at no additional cost to Cbeyond.  Hairpinning involves providing a DS0 signal from an IDLC-served loop to Cbeyond’s collocation equipment by using a dedicated pathway that traverses BellSouth’s central office switch.  BellSouth will provide such DS0 signal to Cbeyond by establishing a copper cross connect between the BellSouth switch and Cbeyond’s collocation equipment.

# 4.	Network Interface Device

4.1		The NID is defined as any means of interconnection of end-user customer premises wiring to BellSouth’s distribution plant, such as a cross-connect device used for that purpose.  The NID is a single-line termination device or that portion of a multiple-line termination device required to terminate a single line or circuit at the premises. The NID features two independent chambers or divisions that separate the service provider’s network from the End User’s on-premises wiring.  Each chamber or division contains the appropriate connection points or posts to which the service provider and the End User each make their connections.  The NID provides a protective ground connection and is capable of terminating cables such as twisted pair cable.

4.1.1  	BellSouth shall permit Cbeyond to connect Cbeyond’s loop facilities to on-premises wiring through the BellSouth NID or at any other technically feasible point.

## 4.2  		Access to Network Interface Device (NID)

#### 4.2.1  	Due to the wide variety of NIDs utilized by BellSouth (based on subscriber size and environmental considerations), Cbeyond may access the on-premises wiring by any of the following means: BellSouth shall allow Cbeyond to connect its loops directly to BellSouth’s multi-line residential NID enclosures that have additional space and are not used by BellSouth or any other telecommunications carriers to provide service to the premise.  Cbeyond agrees to install compatible protectors and test jacks and to maintain the protection system and equipment and to indemnify BellSouth pursuant to Section  9.4 of the General Terms and Conditions of this Agreement.

### 4.2.2  	Where an adequate length of on-premises wiring is present and environmental conditions permit, either Party may remove the on-premises wiring from the other Party’s NID and connect that wire to that Party’s own NID; or

### 4.2.3  	Enter the subscriber access chamber or “side” of “dual chamber” NID enclosures for the purpose of extending a connecterized or spliced jumper wire from the on-premises wiring through a suitable “punch-out” hole of such NID enclosures; or

### 4.2.4  	Request BellSouth to make other rearrangements to the on-premises wiring terminations or terminal enclosure on a time and materials cost basis to be charged to the requesting Party (i.e., Cbeyond, its agent, the building owner or the subscriber).  Such charges will be billed to the requesting Party.

### 4.2.5	In no case shall either Party remove or disconnect the other Party’s loop facilities from either Party’s NIDs, enclosures, or protectors, without state regulatory requirement, without providing prior notice to the other Party, and without appropriately re-grounding the other Party’s loop.  In such cases, it shall be the responsibility of the disconnecting party to properly ground the other party’s loop, maintain the NID, and assume full liability for its action and any adverse consequences.

### 4.2.6	In no case shall either Party remove or disconnect ground wires from BellSouth’s NIDs, enclosures, or protectors.

### 4.2.7  	In no case shall either Party remove or disconnect NID modules, protectors, or terminals from BellSouth’s NID enclosures.

### 4.2.8  	Due to the wide variety of NID enclosures and outside plant environments BellSouth will work with Cbeyond to develop specific procedures to establish the most effective means of implementing this Section .

4.3		Technical Requirements

4.3.1  	The NID shall provide an accessible point of interconnection and shall maintain a connection to ground.

4.3.2  	The NID shall be capable of transferring electrical analog or digital signals between the subscriber’s inside wiring and the Distribution Media and/or cross connect to Cbeyond’s NID, consistent with the NID’s function at the Effective Date of this Agreement.

##### 4.3.3 	Where a BellSouth NID exists, it is provided in its “as is” condition.  If such NID is not functioning properly, BellSouth will repair or replace it at BellSouth’s expense.

##### 4.3.4	When Cbeyond deploys its own local loops with respect to multiple-line termination devices, Cbeyond shall Order the quantity of NIDs connections that it requires within such device.

## 4.4  	Interface Requirements

4.4.1 	The NID shall be equal to or better than all of the requirements for NIDs set forth in the applicable industry standard technical references.

# 5.	Unbundled Loop Concentration (ULC) System

### 5.1	BellSouth will provide to Cbeyond loop concentration (ULC).  Loop concentration systems in the central office concentrate the signals transmitted over local loops onto a digital loop carrier system.  The concentration device is placed inside a BellSouth central office. BellSouth will offer ULC with a TR008 interface or a TR303 interface.

5.2	ULC will be offered in two sizes.  System A will allow up to 96 BellSouth loops to be concentrated onto multiple DS1s.  The high speed connection from the concentrator will be at the electrical DS1 level and may connect to Cbeyond at Cbeyond’s collocation site.  System B will allow up to 192 BellSouth loops to be concentrated onto multiple DS1s.  System A may be upgraded to a System B.  A minimum of two DS1s is required for each system (i.e., System A requires two DS1s and System B would require an additional two DS1s or four in total ).  All DS1 interfaces will terminate to the CLEC’s collocation space.  ULC service is offered with or without concentration and with or without protection. A Line Interface element will be required for each loop that is terminated onto the ULC system.  Rates for ULC are as set forth in this Attachment.

# 6.	Sub-Loop Elements

## 6.1 	Where facilities permit and subject to applicable and effective FCC rules and orders, BellSouth shall offer access to its Unbundled Sub-Loop (USL), Unbundled Sub-Loop Concentration (USLC) System and Unbundled Network Terminating Wire (UNTW) elements.  BellSouth shall provide non-discriminatory access, in accordance with 51.311 and section 251 (c)(3) of the Act, to the Sub-Loop. On an unbundled basis and pursuant to the following terms and conditions and the rates approved by the Commission and set forth in this Attachment.  Until such time as rates for Sub Loop elements have been approved by the Commission, Cbeyond shall pay to BellSouth interim cost-based rates as  set forth in this Attachment, such rates to be subject to true-up in accordance with Section 18.3 of this Attachment.

6.2  		Sub-Loop components include but are not limited to the following:

6.2.1  		Unbundled Sub-Loop Distribution;

6.2.2  		Unbundled Sub-Loop Concentration/Multiplexing Functionality; and

    - Feeder, Unbundled Network Terminating Wire; and

6.2.4  		Unbundled Sub-Loop Feeder.

6.3 		Unbundled Sub-Loop (distribution facilities)

6.3.1  		Definition

6.3.2  	Subject to applicable and effective FCC rules and orders, the unbundled sub-loop distribution facility is dedicated transmission facility that BellSouth provides from a customer’s point of demarcation to a BellSouth cross-connect device.  The BellSouth cross-connect device may be located within a remote terminal (RT) or a stand-alone cross-box in the field or in the equipment room of a building.  There are two offerings available for Unbundled Sub-Loops (USL):

6.3.3  	Unbundled Sub-Loop Distribution (USL-D) will include the sub-loop facility from the cross-box in the field up to and including the point of demarcation.

6.3.4	BellSouth will also provide sub-loop interconnection to the intrabuilding network cable (INC) (riser cable).  INC is the distribution facility inside a subscriber’s building or between buildings on one customer’s same premises (continuous property not separated by a public street or road).  USL-INC (riser cable) will include the facility from the cross-connect device in the building equipment room up to an including the point of demarcation.

### 6.3.5  	Requirements for Unbundled Sub-Loops Distribution Facilities

#### 6.3.5.1	Unbundled Sub-Loop distribution facilities were originally built as part of the entire voice grade loop from the BellSouth central office to the customer network interface.  Therefore, the Unbundled Sub-Loop may have load coils, which are necessary for transmission of voice grade services.  The Unbundled Sub-Loops will be provided in accordance with technical reference TR73600. Cbeyond may request that the sub-loop be conditioned in accordance with section 2.4.

### 6.3.5.2  	USL distribution facilities shall support functions associated with provisioning, maintenance and testing of the Unbundled Sub-Loop.  In a scenario that involves connection at a BellSouth cross-box located in the field, Cbeyond would be required to deliver a cable to the BellSouth remote terminal or cross-box to provide continuity to Cbeyond’s feeder facilities.  This cable would be connected, by a BellSouth technician, to a cross-connect panel within the BellSouth RT/cross-box.  Cbeyond’s cable pairs can then be connected to BellSouth’s USL within the BellSouth cross-box by the BellSouth technician.  In a scenario that requires connection in a building equipment room, BellSouth will install a cross connect panel on which access to the requested sub-loops will be connected.  The CLEC’s cable pairs can then be connected to the Unbundled Sub-Loop pairs on this cross-connect panel by the BellSouth technician.

6.4	BellSouth will provide Unbundled Sub-Loops where possible.  Through the Service Inquiry (SI) process, BellSouth will determine if it is feasible to place the required facilities where Cbeyond has requested access to Unbundled Sub-Loops.  If existing capacity is sufficient to meet the CLEC demand, then BellSouth will perform the set-up work as described in the next section.  If any work must be done to modify existing BellSouth facilities or add new facilities (other than adding the cross-connect panel in a building equipment room as noted in 6.5) to accommodate Cbeyond’s request for Unbundled Sub-Loops, BellSouth will use its Special Construction (SC) process to determine the additional costs required to provision the Unbundled Sub-Loops.  Cbeyond will then have the option of paying the one-time SC charge to modify the facilities to meet Cbeyond’s request. In the event that Cbeyond invokes the dispute resolution process in connection with a request from Cbeyond for Sub-Loops as described in this section, BellSouth shall have the burden of demonstrating that facilities are not available as requested by Cbeyond.

6.5	During the initial set-up in a BellSouth cross-connect box in the field, the BellSouth technician will perform the necessary work to splice the CLEC’s cable into the cross connect box.  For the set-up inside a building equipment room, BellSouth will perform the necessary work to install the cross-connect panel that will be used to provide access to the requested USLs.  Once the set-up is complete, the CLEC requested sub-loop pairs would be provisioned through the service order process based on the submission of a LSR to the LCSC.

### 6.6  	Interface Requirements

#### 6.6.1	Unbundled Sub-Loop shall be equal to or better than each of the applicable requirements set forth in the applicable industry standard technical references.

#### 6.7	Unbundled Sub-Loop Concentration System (USLC)

### 6.7.1	Where facilities permit, BellSouth will provide to Cbeyond with the ability to concentrate its sub-loops onto one or more DS1s back to the BellSouth Central Office.  The DS1s will then be terminated into Cbeyond’s collocation space. TR-008 and TR303 interface standards are available.

### 6.7.2  	USLC, using the Lucent Series 5 equipment, will be offered in two different systems.  System A will allow up to 96 of Cbeyond’s sub-loops to be concentrated onto multiple DS1s.  System B will allow an additional 96 of Cbeyond’s sub-loops to be concentrated onto multiple DS1s.  One System A may be supplemented with one System B and they both must be physically located in a single Series 5 dual channel bank.  A minimum of two DS1s is required for each system (i.e., System A requires two DS1s and System B would require an additional two DS1s or four in total).  The DS1 level facility that connects the RT site with the serving wire center is known as a Feeder Interface.  All DS1 Feeder Interfaces will terminate to the CLEC’s collocation space within the SWC that serves the RT where the CLEC’s sub-loops are connected.  USLC service is offered with or without concentration and with or without a protection DS1.

### 6.7.3	In these scenarios Cbeyond would be required to place a cross-box, remote terminal (RT), or other similar device and deliver a cable to the BellSouth remote terminal.  This cable would be connected, by a BellSouth technician, to a cross-connect panel within the BellSouth RT/cross-box and would allow Cbeyond’s sub-loops to then be placed on the ULSC and transported to their collocation space at a DS1 level.

## 6.8  	Unbundled Network Terminating Wire (UNTW)

### 6.8.1	BellSouth agrees to offer its Unbundled Network Terminating Wire (UNTW) to Cbeyond pursuant to the following terms and conditions at rates as set forth in this Attachment.

## 6.8.2  	Definition

### 6.8.2.1 	Subject to applicable and effective FCC rules and orders, UNTW is a dedicated transmission facility that BellSouth provides from the Wiring Closet /Garden Terminal (or other type of cross-connect point) at the point of termination of BellSouth’s loop distribution facilities to the end user’s point of demarcation.

## 6.8.3 	Requirements

### 6.8.3.1 	BellSouth will offer spare pairs that are available to an end user's premises to Cbeyond. Available spare pairs are defined as pairs that are not being utilized by BellSouth or by a third party to provide an end user with working service at the time of Cbeyond’s request for UNTW.  If no spare pairs are available and the end user is no longer using BellSouth’s local service, BellSouth will relinquish the first pair to Cbeyond.  If after BellSouth has relinquished the first pair to Cbeyond and the end user decides to change from Cbeyond to another carrier, Cbeyond will relinquish the first pair back to BellSouth.

6.8.3.2. 	Notwithstanding the foregoing, should BellSouth subsequently require the use of additional pair(s) to provide for the activation of additional lines in an end users premises in response to a request from such end user, Cbeyond agrees to surrender it available spare pair(s) upon request by BellSouth.

6.8.3.3 	If an end user of Cbeyond desires to receive local exchange service from a service provider who is not a Party to this Agreement, and such third party service provider needs access to the BellSouth UNTW to provide local exchange service to the end user, then Cbeyond agrees to surrender the requisite number of its inactive spare pair(s) if no other spare pair is available and upon request by BellSouth.

6.8.3.4 	If Cbeyond has placed NTW at a location and an end user desires to receive local exchange service from BellSouth and BellSouth needs access to Cbeyond’s NTW to provide local exchange service to the end user, then Cbeyond agrees to make available to BellSouth the requisite number of its spare pair(s), upon request by BellSouth, at rates determined by Cbeyond.

6.8.3.5 	In new construction, where possible, both Parties may at their option and with the property owner’s agreement install their own NTW.  In existing construction, BellSouth shall not be required to install new or additional NTW beyond existing NTW to provision the services of the Cbeyond.

## 6.9  	Technical Requirements

### 6.9.1 	In these scenarios, BellSouth will connect the requested UNTW pairs to a single point of interconnection (SPOI)  designed for Cbeyond access to BellSouth’s NTW.  The SPOI will be installed either near BellSouth’s garden terminal or wiring closet. Cbeyond will be required to place a cross-box, terminal or other similar device and deliver a cable to this SPOI.  Cbeyond will then connect their cable to the cross-connect panel to access the requested UNTW pairs.

# 7.  	Local Switching

7.1  	BellSouth shall provide non-discriminatory access to local circuit switching capability, and local tandem switching capability, on an unbundled basis, except as set forth below in Section 7.1.2 to Cbeyond for the provision of a telecommunications service.  BellSouth shall provide non-discriminatory access to packet switching capability on an unbundled basis to Cbeyond for the provision of a telecommunications service only in the limited circumstance described below in Section 7.1.2.

7.1.1 	Except as otherwise provided herein, BellSouth shall not impose any restrictions on Cbeyond regarding the use of Switching Capabilities purchased from BellSouth provided such use does not result in demonstrable harm to either the BellSouth network or personnel or the use of the BellSouth network by BellSouth or any other telecommunication carrier.

7.1.2	Local Circuit Switching Capability, including Tandem Switching Capability

## 7.1.2.1		Definition

### Local Circuit Switching Capability is defined as: (A) line-side facilities, which include, but are not limited to, the connection between a loop termination at a main distribution frame and a switch line card; (B) trunk-side facilities, which include, but are not limited to, the connection between trunk termination at a trunk-side cross-connect panel and a switch trunk card; and (C) All features, functions, and capabilities of the switch, which include, but are not limited to: (1) the basic switching function of connecting lines to lines, line to trunks, trunks to lines, and trunks to trunks, as well as the same basic capabilities made available to BellSouth’s customers, such as a telephone number, white page listings, and dial tone; and (2) all other features that the switch is capable of providing, including but not limited to customer calling, customer local area signaling service features, and Centrex, as well as any technically feasible customized routing functions provided by the switch; (D) switching provided by remote switching modules.

7.1.3  	Notwithstanding BellSouth’s general duty to unbundle local circuit switching, BellSouth shall not be required to unbundle local circuit switching for Cbeyond when Cbeyond serves end-users with four (4) or more voice-grade (DS-0) equivalents or lines in locations served by BellSouth’s local circuit switches, which are in the following MSAs: Atlanta, GA; Miami, FL; Orlando, FL; Ft. Lauderdale, FL; Charlotte-Gastonia-Rock Hill, NC; Greensboro-Winston Salem-High Point, NC; Nashville, TN; and New Orleans, LA, and BellSouth has provided non-discriminatory cost based access to the Enhanced Extended Link (EEL) throughout Density Zone 1 as determined by NECA Tariff No. 4 as in effect on January 1, 1999.

    - In the event that Cbeyond orders local circuit switching for a single end user account name at a single physical end user location with four (4) or more two (2) wire voice-grade loops from a BellSouth central office located in Density Zone 1, as determined by NECA Tariff No. 4 as in effect on January 1, 1999, of the Atlanta, GA; Miami, FL; Orlando, FL; Ft. Lauderdale, FL; Charlotte-Gastonia-Rock Hill, NC; Greensboro-Winston Salem-High Point, NC; Nashville, TN; and New Orleans, LA MSAs, BellSouth’s sole recourse shall be to charge Cbeyond a rate to be negotiated for use of the local circuit switching functionality for the affected facilities, or in the alternative, to charge Cbeyond the local services resale rate for use of all Combinations used to provide the affected facilities to Cbeyond.

### A featureless port is one that has a line port, switching facilities, and an interoffice port.  A featured port is a port that includes all features then capable or a number of then capable features specifically requested by Cbeyond.  Any features that are not currently then capable at the time of the request, but are technically feasible through the switch can be requested through the BFR process.

    - BellSouth will provide to Cbeyond customized routing of calls:  (i) to a requested directory assistance services platform; (ii) to an operator services platform pursuant to Section 11 of Attachment 2; (iii) for Cbeyond’s PIC’ed toll traffic in a two (2) PIC environment to an alternative OS/DA platform designated by Cbeyond.  Cbeyond customers may use the same dialing arrangements as BellSouth customers.

    - Remote Switching Module functionality is included in Switching Capability.  The switching capabilities used will be based on the line side features they support.

### Switching Capability will also be capable of routing (1)local, intraLATA, interLATA, and calls to international customer’s preferred carrier; (2)call features (e.g., call forwarding) and (3) Centrex capabilities.

### 7.1.9	Where required to do so in order to comply with an effective Commission order, BellSouth will provide to Cbeyond purchasing  local BellSouth switching and reselling BellSouth local exchange service under Attachment 1, selective routing of calls to a requested directory assistance services platform or operator services platform.  Cbeyond customers may use the same dialing arrangements as BellSouth customers, but obtain a Cbeyond branded service.

## 7.2	Technical Requirements

### 7.2.1	The requirements set forth in this Section apply to Local Switching, but not to the Data Switching function of Local Switching.

#### 7.2.1.1	Local Switching shall be equal to or better than the requirements for Local Switching set forth in Telcordia (formerly BellCore)’s Local Switching Systems General Requirements (FR-NWT-000064).

#### 7.2.1.2	When applicable, BellSouth shall route calls to the appropriate trunk or lines for call origination or termination.

#### 7.2.1.3	Subject to this section, BellSouth shall route calls on a per line or per screening class basis to (1) BellSouth platforms providing Network Elements or additional requirements (2) Operator Services platforms, (3) Directory Assistance platforms, and (4) Repair Centers.  Any other routing requests by Cbeyond will be made pursuant to the Bona Fide Request/ New Business Request Process as set forth in General Terms and Conditions.

#### 7.2.1.4	BellSouth shall provide unbranded recorded announcements and call progress tones to alert callers of call progress and disposition.

#### 7.2.1.5	BellSouth shall activate service for a Cbeyond customer or network interconnection on any of the Local Switching interfaces.  This includes provisioning changes to change a customer from BellSouth’s services to Cbeyond’s services without loss of switch feature functionality as defined in this Agreement.

#### 7.2.1.6	BellSouth shall perform routine testing (e.g., Mechanized Loop Tests (MLT) and test calls such as 105, 107 and 108 type calls) and fault isolation on a mutually agreed upon schedule.

#### 7.2.1.7	BellSouth shall repair and restore any equipment or any other maintainable component that may adversely impact Local Switching.

#### 7.2.1.8	BellSouth shall control congestion points such as those caused by radio station call-ins, and network routing abnormalities.  All traffic shall be restricted in a non discriminatory manner.

#### 7.2.1.9	BellSouth shall perform manual call trace and permit customer originated call trace.

#### 7.2.1.10	Special Services provided by BellSouth will include the following:

##### 7.2.1.10.1	Telephone Service Prioritization;

##### 7.2.1.10.2	Related services for handicapped;

##### 7.2.1.10.3	Soft dial tone where required by law;

##### Any other service required by law; and

#### 7.2.1.11	BellSouth shall provide Switching Service Point (SSP) capabilities and signaling software to interconnect the signaling links destined to the Signaling Transfer Point Switch (STP).  These capabilities shall adhere to Telcordia (formerly BellCore) specifications - TCAP (GR-1432-CORE), ISUP(GR-905-CORE), Call Management (GR-1429-CORE), Switched Fractional DS1 (GR-1357-CORE), Toll Free Service (GR-1428-CORE), Calling Name (GR-1597-CORE), Line Information Database (GR-954-CORE), and Advanced Intelligent Network (GR-2863-CORE).

#### 7.2.1.12	BellSouth shall provide interfaces to adjuncts through Telcordia (formerly BellCore) standard interfaces.  These adjuncts can include, but are not limited to, the Service Circuit Node and Automatic Call Distributors.

#### 7.2.1.13	BellSouth shall provide performance data regarding a customer line, traffic characteristics or other measurable elements to Cbeyond, upon a reasonable request from Cbeyond.  CLEC will pay BellSouth for all costs incurred to provide such performance data through the Business Opportunity Request process.

#### 7.2.1.14	BellSouth shall offer Local Switching that provides feature offerings at parity to those provided by BellSouth to itself or any other Party.  Such feature offerings shall include but are not limited to:

##### 7.2.1.14.1	Basic and primary rate ISDN;

##### 7.2.1.14.2	Residential features;

##### 7.2.1.14.3	Customer Local Area Signaling Services (CLASS/LASS);

##### 7.2.1.14.4	CENTREX (including equivalent administrative capabilities, such as customer accessible reconfiguration and detailed message recording); and

##### 7.2.1.14.5	Advanced intelligent network triggers supporting Cbeyond and BellSouth service applications.

BellSouth shall offer to Cbeyond all AIN triggers in connection with its SMS/SCE offering which are supported by BellSouth for offering AIN-based services. Triggers that are currently available are:

###### 7.2.1.14.5.1	Off-Hook Immediate

###### 7.2.1.14.5.2	Off-Hook Delay

###### 7.2.1.14.5.3	Termination Attempt

###### 7.2.1.14.5.4	6/10 Public Office Dialing Plan

###### 7.2.1.14.5.5	Feature Code Dialing

###### 7.2.1.14.5.6	Customer Dialing Plan

##### 7.2.1.14.6	When the following triggers are supported by BellSouth, BellSouth will make these triggers available to Cbeyond:

###### 7.2.1.14.6.1	Private EAMF Trunk

###### 7.2.1.14.6.2	Shared Interoffice Trunk (EAMF, SS7)

###### 7.2.1.14.6.3	N11

###### 7.2.1.14.6.4	Automatic Route Selection

7.2.1.14.6.5	9XX Blocking and toll blocking

#### 7.2.1.15	Where capacity exists, BellSouth shall assign each Cbeyond customer line the class of service designated by Cbeyond (e.g., using line class codes or other switch specific provisioning methods), and shall route directory assistance calls from Cbeyond customers to Cbeyond directory assistance operators at Cbeyond’s option.

#### 7.2.1.16	Where capacity exists, BellSouth shall assign each Cbeyond customer line the class of services designated by Cbeyond (e.g., using line class codes or other switch specific provisioning methods) and shall route operator calls from Cbeyond customers to Cbeyond operators at Cbeyond’s option.  For example, BellSouth may translate 0- and 0+ intraLATA traffic, and route the call through appropriate trunks to an Cbeyond Operator Services Position System (OSPS).  Calls from Local Switching must pass the ANI-II digits unchanged.

#### 7.2.1.17	Local Switching shall be offered in accordance with the requirements of the following technical references:

##### 7.2.1.17.1	Telcordia (formerly BellCore) GR-1298-CORE, AIN Switching System Generic Requirements, as implemented in BellSouth’s switching equipment;

##### 7.2.1.17.2	Telcordia (formerly BellCore) GR-1299-CORE, AIN Switch-Service Control Point (SCP)/Adjunct Interface Generic Requirements;

##### 7.2.1.17.3	Telcordia (formerly BellCore) TR-NWT-001284, AIN 0.1 Switching System Generic Requirements;

##### 7.2.1.17.4	Telcordia (formerly BellCore) SR-NWT-002247, AIN Release 1 Update.

### 7.2.2	Interface Requirements

#### 7.2.2.1	BellSouth shall provide the following interfaces to loops:

#### 7.2.2.2	Standard Tip/Ring interface including loopstart or groundstart, on-hook signaling (e.g., for calling number, calling name and message waiting lamp);

#### 7.2.2.3	Coin phone signaling;

#### 7.2.2.4	Basic Rate Interface ISDN adhering to appropriate Telcordia (formerly BellCore) Technical Requirements;

#### 7.2.2.5	Two-wire analog interface to PBX;

##### 7.2.2.5.1	Four-wire analog interface to PBX;

#### 7.2.2.6	Four-wire DS1 interface to PBX or customer provided equipment (e.g. computers and voice response systems);

#### 7.2.2.7	Primary Rate ISDN to PBX adhering to ANSI standards Q.931, Q.932 and appropriate Telcordia (formerly BellCore) Technical Requirements;

#### 7.2.2.8	Switched Fractional DS1 with capabilities to configure Nx64 channels (where N = 1 to 24); and

#### 7.2.2.9	Loops adhering to Telcordia (formerly BellCore) TR-NWT-08 and TR-NWT-303 specifications to interconnect Digital Loop Carriers.

#### 7.2.2.10	BellSouth shall provide access to the following but not limited to:

#### 7.2.2.11	SS7 Signaling Network or Multi-Frequency trunking if requested by  Cbeyond;

#### 7.2.2.12	Interface to Cbeyond operator services systems or Operator Services through appropriate trunk interconnections for the system; and

#### 7.2.2.13	Interface to Cbeyond directory assistance services through the Cbeyond switched network or to Directory Assistance Services through the appropriate trunk interconnections for the system; and 950 access or other Cbeyond required access to interexchange carriers as requested through appropriate trunk interfaces.

7.2.2.14. 	Packet Switching Capability

7.2.2.14.1	Definition

Packet Switching Capability. The packet switching capability network element is defined as the basic packet switching function of routing or forwarding packets, frames, cells or other data units based on address or other routing information contained in the packets, frames, cells or other data units, and the functions that are performed by Digital Subscriber Line Access Multiplexers, including but not limited to:

7.2.2.14.1.1	The ability to terminate copper customer loops (which includes both a low band voice channel and a high-band data channel, or solely a data channel);

7.2.2.14.1.2	The ability to forward the voice channels, if present, to a circuit switch or multiple circuit switches;

7.2.2.14.3	The ability to extract data units from the data channels on the loops, and

7.2.2.14.4	The ability to combine data units from multiple loops onto one or more trunks connecting to a packet switch or packet switches.

7.2.2.14.5	BellSouth shall be required to provide non-discriminatory access to unbundled packet switching capability only where each of the following conditions are satisfied:

7.2.2.14.5.1	BellSouth has deployed digital loop carrier systems, including but not limited to, integrated digital loop carrier or universal digital loop carrier systems; or has deployed any other system in which fiber optic facilities replace copper facilities in the distribution section (e.g., end office to remote terminal, pedestal or environmentally controlled vault);

7.2.2.14.5.2	There are no spare copper loops capable of supporting the xDSL services Cbeyond seeks to offer;

7.2.2.14.5.3	BellSouth has not permitted Cbeyond to deploy a Digital Subscriber Line Access Multiplexer at the remote terminal, pedestal or environmentally controlled vault or other interconnection point, nor has the Cbeyond obtained a virtual collocation arrangement at these Sub-Loop interconnection points as defined by 47 C.F.R. § 51.319 (b); and

7.2.2.14.5.4	BellSouth has deployed packet switching capability for its own use.

7.2.2.14.5.5	If there is a dispute as to whether BellSouth must provide Packet Switching , such dispute will be resolved according to the dispute resolution process set forth in Section 15 of the General Terms and Conditions of this Agreement, incorporated herein by this reference.

# 8.	Interoffice Transmission Facilities

BellSouth shall provide nondiscriminatory access, in accordance with FCC Rule 51.311 and Section 251(c)(3) of the Act, to interoffice transmission facilities on an unbundled basis to Cbeyond for the provision of a telecommunications service.

8.1	Interoffice transmission facility network elements include:

1.	Dedicated transport, defined as BellSouth’s transmission facilities, including all technically feasible capacity-related services including, but not limited to, DS1, DS3 and OCn levels, dedicated to a particular customer or carrier, that provide telecommunications between wire centers or switches owned by BellSouth, or between wire centers and switches owned by BellSouth and Cbeyond;

2.	Dark Fiber transport, defined as BellSouth’s optical transmission facilities without attached multiplexing, aggregation or other electronics;

3.	Shared transport, defined as transmission facilities shared by more than one carrier, including BellSouth, between end office switches, between end office switches and tandem switches, and between tandem switches, in BellSouth’s network.

8.1.1	BellSouth shall:

- Provide Cbeyond exclusive use of interoffice transmission facilities dedicated to a particular customer or carrier, or shared use of the features, functions, and capabilities of interoffice transmission facilities shared by more than one customer or carrier;

- Provide all technically feasible transmission facilities, features, functions, and capabilities that Cbeyond could use to provide telecommunications services;

- Permit, to the extent technically feasible, Cbeyond to connect such interoffice facilities to equipment designated by Cbeyond, including but not limited to, Cbeyond’s collocated facilities; and

- Permit, to the extent technically feasible, Cbeyond to obtain the functionality provided by BellSouth’s digital cross-connect systems in the same manner that BellSouth provides such functionality to interexchange carriers.

8.1.2	For the conversion of individual UNEs, and in accordance with the FCC’s Supplemental Order Clarification, released June 2, 2000, in CC Docket No. 96-98, if applicable, Cbeyond shall be entitled to convert existing interoffice transmission facilities (i.e., special access) to the corresponding interoffice transport network element option.

## 8.2	Technical Requirements of Common (Shared) Transport

### 8.2.1	Common (Shared) Transport provided on DS1 or VT1.5 circuits, shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for Central Office to Central Office (“CO to CO”) connections in the appropriate industry standards.

### 8.2.2	Common (Shared) Transport provided on DS3 circuits, STS-1 circuits, and higher transmission bit rate circuits, shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for CO to CO connections in the appropriate industry standards.

### 8.2.3	BellSouth shall be responsible for the engineering, provisioning, and maintenance of the underlying equipment and facilities that are used to provide Common (Shared) Transport.

### 8.2.4	At a minimum, Common (Shared) Transport shall meet all of the requirements set forth in the following technical references (as applicable for the transport technology being used):

#### 8.2.4.1	ANSI T1.101-1994, American National Standard for Telecommunications - Synchronization Interface Standard Performance and Availability;

#### 8.2.4.2	ANSI T1.102-1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces;

#### 8.2.4.3	ANSI T1.102.01-199x, American National Standard for Telecommunications - Digital Hierarchy - VT1.5;

#### 8.2.4.4	ANSI T1.105-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Basic Description including Multiplex Structure, Rates and Formats;

#### 8.2.4.5	ANSI T1.105.01-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Automatic Protection Switching;

#### 8.2.4.6	ANSI T1.105.02-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Payload Mappings;

#### 8.2.4.7	ANSI T1.105.03-1994, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Jitter at Network Interfaces;

#### 8.2.4.8	ANSI T1.105.03a-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET): Jitter at Network Interfaces - DS1 Supplement;

#### 8.2.4.9	ANSI T1.105.05-1994, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Tandem Connection;

#### 8.2.4.10	ANSI T1.105.06-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Physical Layer Specifications;

#### 8.2.4.11	ANSI T1.105.07-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Sub STS-1 Interface Rates and Formats;

#### 8.2.4.12	ANSI T1.105.09-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Network Element Timing and Synchronization;

#### 8.2.4.13	ANSI T1.106-1988, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (Single Mode);

#### 8.2.4.14	ANSI T1.107-1988, American National Standard for Telecommunications - Digital Hierarchy - Formats Specifications;

#### 8.2.4.15	ANSI T1.107a-1990 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications (DS3 Format Applications);

#### 8.2.4.16	ANSI T1.107b-1991 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications;

#### 8.2.4.17	ANSI T1.117-1991, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (SONET) (Single Mode - Short Reach);

#### 8.2.4.18	ANSI T1.403-1989, Carrier to Customer Installation, DS1 Metallic Interface Specification;

#### 8.2.4.19	ANSI T1.404-1994, Network-to-Customer Installation - DS3 Metallic Interface Specification;

#### 8.2.4.20	ITU Recommendation G.707, Network node interface for the synchronous digital hierarchy (SDH);

#### 8.2.4.21	ITU Recommendation G.704, Synchronous frame structures used at 1544, 6312, 2048, 8488 and 44736 kbps hierarchical levels;

#### 8.2.4.22	Telcordia (formerly BellCore) FR-440 and TR-NWT-000499, Transport Systems Generic Requirements (TSGR):  Common Requirements;

#### 8.2.4.23	Telcordia (formerly BellCore) GR-820-CORE, Generic Transmission Surveillance:  DS1 & DS3 Performance;

#### 8.2.4.24	Telcordia (formerly BellCore) GR-253-CORE, Synchronous Optical Network Systems (SONET);  Common Generic Criteria;

#### 8.2.4.25	Telcordia (formerly BellCore) TR-NWT 000507, Transmission, Section 7, Issue 5 (Telcordia (formerly BellCore), December 1993). (A module of LSSGR, FR-NWT-000064.);

#### 8.2.4.26	Telcordia (formerly BellCore) TR-NWT-000776, Network Interface Description for ISDN Customer Access;

#### 8.2.4.27	Telcordia (formerly BellCore) TR-INS-000342, High-Capacity Digital Special Access Service-Transmission Parameter Limits and Interface Combinations, Issue 1 February 1991;

#### 8.2.4.28	Telcordia (formerly BellCore) ST-TEC 000052, Telecommunications Transmission Engineering Textbook, Volume 2: Facilities, Third Edition, Issue I May 1989;

#### 8.2.4.29	Telcordia (formerly BellCore) ST-TEC-000051, Telecommunications Transmission Engineering Textbook Volume 1:  Principles, Third Edition. Issue 1 August 1987.

## 8.3	Dedicated Transport

#### 8.3.1.	BellSouth shall offer Dedicated Transport in each of the following ways:

##### 8.3.1.1	As capacity on a shared facility.

##### 8.3.1.2	As a circuit (e.g., DS0, DS1 or DS3) dedicated to Cbeyond .

#### 8.3.2	When Dedicated Transport is provided as a system it shall include:

##### 8.3.2.1	Transmission equipment such as multiplexers, line terminating equipment, amplifiers, and regenerators;

##### 8.3.2.2	Inter-office transmission facilities such as optical fiber, copper twisted pair, and coaxial cable.

### 8.3.3	Unbundled Local Channel

8.3.3.1	The Unbundled Local Channel is the dedicated transmission path between Cbeyond’s Point of Presence and the BellSouth Serving Wire Center.

8.3.3.2	BellSouth currently offers Unbundled Local Channels for switched traffic.  Rates for these elements are listed in this Attachment.  For those states that do not contain rates in this Attachment for DS1 and DS3 switched Local Channels, the rates in the applicable State Access Tariff will apply as interim rates.  When final rates are developed, these interim rates will be subject to true-up, and the Parties will amend the Agreement to reflect the new rates.

8.3.3.3	BellSouth currently offers Unbundled Local Channels for non-switched traffic at DS1 and DS3 levels at rates as set forth in Exhibit C to this Attachment.

### 8.3.4	Technical Requirements

This Section sets forth technical requirements for all Dedicated Transport.

#### 8.3.4.1	When BellSouth provides Dedicated Transport as a circuit or a system, the entire designated transmission circuit or system (e.g., DS0, DS1,DS3) shall be dedicated to Cbeyond  designated traffic.

#### 8.3.4.2	BellSouth shall offer Dedicated Transport in all technologies that become available including, but not limited to, DS1 and DS3 transport systems, SONET (or SDH) Bi-directional Line Switched Rings, SONET (or SDH) Unidirectional Path Switched Rings, and SONET (or SDH) point-to-point transport systems (including linear add-drop systems), at all available transmission bit rates.  While SONET Ring facilities are not available in every application, they are typically available in the major metropolitan areas.

#### 8.3.4.3	For DS1 or VT1.5 circuits, Dedicated Transport shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for Customer Interface to Central Office (“CI to CO”) connections in the appropriate industry standards.

#### 8.3.4.4	Where applicable, for DS3 circuits, Dedicated Transport shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for CI to CO connections in the appropriate industry standards.

#### 8.3.4.5	BellSouth shall offer the following interface transmission rates for Dedicated Transport:

##### 8.3.4.5.1	DS0 Equivalent;

##### 8.3.4.5.2	DS1 (Extended SuperFrame - ESF and D4 channel bank shall be provided);

##### 8.3.4.5.3	DS3 where applicable (M13 multiplexer shall be provided);

##### 8.3.4.5.4	SDH Standard interface rates in accordance with International Telecommunications Union (ITU) Recommendation G.707 and Plesiochronous Digital Hierarchy (PDH) rates per ITU Recommendation G.704.

#### 8.3.4.6	When Dedicated Transport is provided as a system, BellSouth shall design the system according to our network infrastructure to allow for the termination points specified by Cbeyond .

8.3.5	At a minimum, Dedicated Transport shall meet each of the requirements set forth in the following technical references:

#### 8.3.5.1	ANSI T1.231-1993 -American National Standard for Telecommunications - Digital Hierarchy - Layer 1 In-Service Digital Transmission performance monitoring.

##### 8.3.5.1.1	ANSI T1.102-1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces;

##### 8.3.5.1.2	ANSI T1.106-1988, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (Single Mode);

##### 8.3.5.1.3	ANSI T1.107-1988, American National Standard for Telecommunications - Digital Hierarchy - Formats Specifications;

##### 8.3.5.1.4	ANSI T1.107a-1990 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications (DS3 Format Applications);

##### 8.3.5.1.5	ANSI T1.107b-1991 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications;

##### 8.3.5.1.6	Telcordia (formerly BellCore) FR-440 and TR-NWT-000499, Transport Systems Generic Requirements (TSGR):  Common Requirements;

##### 8.3.5.1.7	Telcordia (formerly BellCore) GR-820-CORE, Generic Transmission Surveillance:  DS1 & DS3 Performance;

##### 8.3.5.1.8	Telcordia (formerly BellCore) TR-NWT 000507, Transmission, Section 7, Issue 5 (Telcordia (formerly BellCore), December 1993). (A module of LSSGR, FR-NWT-000064.);

##### 8.3.5.1.9	Telcordia (formerly BellCore) TR-INS-000342, High-Capacity Digital Special Access Service-Transmission Parameter Limits and Interface Combinations, Issue 1 February 1991;

##### 8.3.5.1.10	Telcordia (formerly BellCore) ST-TEC 000052, Telecommunications Transmission Engineering Textbook, Volume 2: Facilities, Third Edition, Issue I May 1989;

##### 8.3.5.1.11	Telcordia (formerly BellCore) ST-TEC-000051, Telecommunications Transmission Engineering Textbook Volume 1:  Principles, Third Edition. Issue 1 August 1987.

# 9.	Tandem Switching

## 9.1	Definition

Tandem Switching is the function that establishes a communications path between two switching offices through a third switching office (the Tandem switch).

## 9.2	Technical Requirements

#### 9.2.1	Tandem Switching shall have the same capabilities or equivalent capabilities as those described in Bell Communications Research TR-TSY-000540 Issue 2R2, Tandem Supplement, 6/1/90.  The requirements for Tandem Switching include, but are not limited to the following:

##### 9.2.1.1	Tandem Switching shall provide signaling to establish a tandem connection;

##### 9.2.1.2	Tandem Switching will provide screening as jointly agreed to by Cbeyond and BellSouth;

##### 9.2.1.3	Tandem Switching shall provide Advanced Intelligent Network triggers supporting AIN features where such routing is not available from the originating end office switch, to the extent such Tandem switch has such capability;

##### 9.2.1.4	Tandem Switching shall provide access to Toll Free number portability database as designated by Cbeyond;

##### 9.2.1.5	Tandem Switching shall provide all trunk interconnections discussed under the “Network Interconnection” section (e.g., SS7, MF, DTMF, DialPulse, PRI-ISDN, DID, and CAMA-ANI (if appropriate for 911));

##### 9.2.1.6	Tandem Switching shall provide connectivity to PSAPs where 911 solutions are deployed and the tandem is used for 911; and

##### 9.2.1.7	Where appropriate, Tandem Switching shall provide connectivity to transit traffic to and from other carriers.

#### 9.2.2	Tandem Switching shall accept connections (including the necessary signaling and trunking interconnections) between end offices, other tandems, IXCs, ICOs, CAPs and CLEC switches.

#### 9.2.3	Tandem Switching shall provide local tandem functionality between two end offices including two offices belonging to different CLEC's (e.g., between a CLEC end office and the end office of another CLEC).

#### 9.2.4	Tandem Switching shall preserve CLASS/LASS features and Caller ID as traffic is processed.

#### 9.2.5	Tandem Switching shall record billable events and send them to the area billing centers designated by Cbeyond.  Tandem Switching will provide recording of all billable events as jointly agreed to by Cbeyond and BellSouth.

#### 9.2.6	Upon a reasonable request from Cbeyond, BellSouth shall perform routine testing and fault isolation on the underlying switch that is providing Tandem Switching and all its interconnections.  The results and reports of the testing shall be made immediately available to Cbeyond.

#### 9.2.7	BellSouth shall maintain Cbeyond’s trunks and interconnections associated with Tandem Switching at least at parity to its own trunks and interconnections.

#### 9.2.8	BellSouth shall control congestion points and network abnormalities.  All traffic will be restricted in a non discriminatory manner.

#### 9.2.9	Selective Call Routing through the use of line class codes is not available through the use of tandem switching.  Selective Call Routing through the use of line class codes is an end office capability only.  Detailed primary and overflow routing plans for all interfaces available within BellSouth switching network shall be mutually agreed to by Cbeyond and BellSouth.

#### 9.2.10	Tandem Switching shall process originating toll-free traffic received from Cbeyond local switch.

#### 9.2.11	In support of AIN triggers and features, Tandem Switching shall provide SSP capabilities when these capabilities are not available from the Local Switching Network Element, to the extent such Tandem Switch has such capability.

### 9.3	Interface Requirements

#### 9.3.1	Tandem Switching shall provide interconnection to the E911 PSAP where the underlying Tandem is acting as the E911 Tandem.

#### 9.3.2	Tandem Switching shall interconnect, with direct trunks, to all carriers with which BellSouth interconnects.

#### 9.3.3	BellSouth shall provide all signaling necessary to provide Tandem Switching with no loss of feature functionality.

#### 9.3.4	Tandem Switching shall interconnect with Cbeyond's switch, using two-way trunks, for traffic that is transiting via BellSouth network to interLATA or intraLATA carriers.  At Cbeyond’s request, Tandem Switching shall record and keep records of traffic for billing.

#### 9.3.5	Tandem Switching shall provide an alternate final routing pattern for Cbeyond traffic overflowing from direct end office high usage trunk groups.

### 9.4	Tandem Switching shall meet or exceed (i.e., be more favorable to Cbeyond) each of the requirements for Tandem Switching set forth in the following technical references:

#### 9.4.1	Bell Communications Research TR-TSY-000540 Issue 2R2, Tandem Supplement, 6/1/90;

#### 9.4.2	GR-905-CORE covering CCSNIS;

#### 9.4.3	GR-1429-CORE for call management features; and

#### 9.4.4	GR-2863-CORE and Telcordia (formerly BellCore) GR-2902-CORE covering CCS AIN interconnection

# 10.		Combinations

10.1	For purposes of this Section, references to “Existing Combinations” of network elements shall mean that such network elements are in fact already combined by BellSouth in the BellSouth network to provide service to a particular end user at a particular location.

## 10.2	EELs

10.2.1	Where facilities permit and where necessary to comply with an effective FCC and/or State Commission order, BellSouth shall offer access to loop and transport combinations, also known as the Enhanced Extended Link (“EEL”) as defined in Section 10.3 below.

10.2.2	Subject to Section 10.2.3 below, BellSouth will provide access to the EEL in the  combinations set forth in 10.3 following.  This offering is intended to provide connectivity from an end user’s location through that end user’s SWC to Cbeyond’s POP serving wire center.  The circuit must be connected to Cbeyond’s switch for the purpose of provisioning telecommunications services, including but not limited to telephone exchange services, to Cbeyond’s end-user customers.  Except as provided for in paragraph 22 of the FCC’s Supplemental Order Clarification, released June 2, 2000, in CC Docket No. 96-98 (“June 2, 2000 Order”), the EEL will be connected to Cbeyond’s facilities in Cbeyond’s collocation space at the POP SWC.  Cbeyond may purchase BellSouth’s access facilities between Cbeyond’s POP and Cbeyond’s collocation space at the POP SWC.

10.2.3	BellSouth shall provide EEL combinations to Cbeyond in the state of Georgia regardless of whether or not such EELs constitute Existing Combinations so long as such combinations are ordinarily combined in BellSouth’s network.  In all other states, BellSouth shall make available to Cbeyond those EEL combinations described in Section 10.3 below only to the extent such combinations are Existing Combinations.

10.2.4	BellSouth will make available EEL combinations to Cbeyond in density Zone 1, as defined in 47 C.F.R. 69.123 as of January 1, 1999, of the Miami, Orlando, Fort Lauderdale, Charlotte, New Orleans, Greensboro and Nashville MSAs, regardless of whether or not such EELs constitute Existing Combinations.

10.2.5	Additionally, BellSouth shall make available to Cbeyond a combination of an unbundled loop and special access interoffice facilities.  To the extent Cbeyond will require multiplexing functionality in connection with such combination, BellSouth will provide access to multiplexing within the central office pursuant to the terms, conditions and rates set forth in its Access Services Tariffs.  Where multiplexing functionality is required in connection with loop and transport combinations, such multiplexing will be provided at the rates and on the terms set forth in this Agreement.

## 10.3	EEL Combinations

10.3.1		DS1 Interoffice Channel + DS1 Channelization + 2-wire VG Local Loop

10.3.2		DS1 Interoffice Channel + DS1 Channelization + 4-wire VG Local Loop

10.3.3		DS1 Interoffice Channel + DS1 Channelization + 2-wire ISDN Local Loop

10.3.4		DS1 Interoffice Channel + DS1 Channelization + 4-wire 56 kbps Local Loop

10.3.5		DS1 Interoffice Channel + DS1 Channelization + 4-wire 64 kbps Local Loop

10.3.6		DS1 Interoffice Channel + DS1 Local Loop

10.3.7		DS3 Interoffice Channel + DS3 Local Loop

10.3.8		STS-1 Interoffice Channel + STS-1 Local Loop

10.3.9		DS3 Interoffice Channel + DS3 Channelization + DS1 Local Loop

10.3.10	STS-1 Interoffice Channel + DS3 Channelization + DS1 Local Loop

10.3.11	2-wire VG Interoffice Channel + 2-wire VG Local Loop

10.3.12	4wire VG Interoffice Channel + 4-wire VG Local Loop

10.3.13	4-wire 56 kbps Interoffice Channel + 4-wire 56 kbps Local Loop

10.3.14	4-wire 64 kbps Interoffice Channel + 4-wire 64 kbps Local Loop

10.4	Other Network Element Combinations

In the state of Georgia, BellSouth shall make available to Cbeyond, at the rates set forth in Section 10.6 below: (1) Existing Combinations of network elements other than EELs; and (2) combinations of network elements other than EELs that are not Existing Combinations but that BellSouth ordinarily combines in its network.  In all other states, BellSouth shall make available to Cbeyond, at the rates set forth in Section 10.6 below, combinations of network elements other than EELs only to the extent such combinations are Existing Combinations.

## 10.5	Special Access Service Conversions

10.5.1	Cbeyond may not convert special access services to combinations of loop and transport network elements, whether or not Cbeyond self-provides its entrance facilities (or obtains entrance facilities from a third party), unless Cbeyond uses the combination to provide a “significant amount of local exchange service” (as described in Section 10.5.2 below), in addition to exchange access service, to a particular customer.

10.5.2	For the purpose of special access conversions, a “significant amount of local exchange service” is as defined in the FCC’s Supplemental Order Clarification, released June 2, 2000, in CC Docket No. 96-98 (“June 2, 2000 Order”).  The Parties agree to incorporate by reference paragraph 22 of the June 2, 2000 Order.  When Cbeyond requests conversion of special access circuits, Cbeyond will self-certify to BellSouth in the manner specified in paragraph 29 of the June 2, 2000 Order that the circuits to be converted qualify for conversion.  In addition there may be extraordinary circumstances where Cbeyond is providing a significant amount of local exchange service, but does not qualify under any of the three options set forth in paragraph 22 of June 2, 2000 Order.  In such case, Cbeyond may petition the FCC for a waiver of the local usage options set forth in the June 2, 2000 Order.  If a waiver is granted, then upon Cbeyond’s request the Parties shall amend this Agreement to the extent necessary to incorporate the terms of such waiver for such extraordinary circumstance.

10.5.3	Upon request for conversions of up to 15 circuits from special access to EELs, BellSouth shall perform such conversions within seven (7) days from BellSouth’s receipt of a valid, error free service order from Cbeyond.  Requests for conversions of fifteen (15) or more circuits from special access to EELs will be provisioned on a project basis.  Conversions should not require the special access circuit to be disconnected and reconnected because only the billing information or other administrative information associated with the circuit will change when Cbeyond requests a conversion.  The Access Service Request process will be used for conversion requests.

10.5.4	BellSouth may, at its sole expense, and upon thirty (30) days notice to Cbeyond, audit Cbeyonds records not more than one in any twelve month period, unless an audit finds non-compliance with the local usage options referenced in the June 2, 2000 Order, in order to verify the type of traffic being transmitted over combinations of loop and transport network elements.  If, based on its audits, BellSouth concludes that Cbeyond is not providing a significant amount of local exchange traffic over the combinations of loop and transport network elements, BellSouth may file a complaint with the appropriate Commission, pursuant to the dispute resolution process as set forth in this Agreement.  In the event that BellSouth prevails, BellSouth may convert such combinations of loop and transport network elements to special access services and may seek appropriate retroactive reimbursement from Cbeyond.

10.6	Rates

10.6.1	Georgia

10.6.1.1	The non-recurring and recurring rates for the EEL combinations set forth in 10.3, whether or not such EELs are Existing Combinations, are as set forth in Exhibit A of this Attachment.

10.6.1.2	On an interim basis, for combinations of loop and transport network elements not set forth in Section 10.3, where the elements are not Existing Combinations but are ordinarily combined in BellSouth’s network, the non-recurring and recurring charges for such UNE combinations shall be the sum of the stand-alone non-recurring and recurring charges of the network elements which make up the combination.  These interim rates shall be subject to true-up based on the Commission’s review of BellSouth’s cost studies.

10.6.1.3	To the extent that Cbeyond seeks to obtain other combinations of network elements that BellSouth ordinarily combines in its network which have not been specifically priced by the Commission when purchased in combined form, Cbeyond, at its option, can request that such rates be determined pursuant to the Bona Fide Request/New Business Request (NBR) process set forth in this Agreement.

10.6.2	All Other States

## 10.6.2.1	Subject to Section 10.2.3 and 10.4 preceding, for all other states, the non-recurring and recurring rates for the Existing Combinations of EELs set forth in Section 10.3 and other Existing Combinations of network elements will be the sum of the recurring rates for the individual network elements plus an appropriate cost-based records change charge unless otherwise negotiated by the parties.

## 10.6.2.2	The non-recurring and recurring rates for EELs made available pursuant to Section 10.2.4 above will be the sum of the nonrecurring and recurring rates for the individual network elements, unless otherwise established by the Commission.

## 10.7	Port/Loop Combinations

10.7.1	Except as specified in Sections 10.7.1.1 and 10.7.1.2 below, at Cbeyond’s request, BellSouth shall provide access to Existing Combinations of port and loop network elements, as set forth in Section 10.7.4 below.  Such port and loop combinations will provide local exchange service for the origination and termination of calls.

10.7.1.1	BellSouth shall not provide access to combinations of port and loop network elements in locations where, pursuant to FCC rules,  BellSouth is not required to provide circuit switching as an unbundled network element.

10.7.1.2	In accordance with effective and applicable FCC rules, BellSouth shall not provide unbundled access to circuit switching in density Zone 1, as defined in 47 C.F.R. 69.123 as of January 1, 1999, of the Atlanta, Miami, Orlando, Fort Lauderdale, Charlotte, New Orleans, Greensboro and Nashville MSAs to Cbeyond if Cbeyond’s customer has 4 or more DS0 equivalent lines.

10.7.2	In Georgia, BellSouth shall provide combinations of port and loop network elements to Cbeyond to the extent such elements are ordinarily combined in BellSouth’s network, regardless of whether or not such combinations are Existing Combinations, except in those locations where BellSouth is not required to provide circuit switching, as set forth in Section 10.7.1.2 above.  In all other states, and subject to Sections 10.7.1.1 and 10.7.1.2 above, BellSouth shall provide combinations of port and loop network elements to Cbeyond only to the extent such elements constitute Existing Combinations.

## 10.7.3		Rates for Combinations of Loop and Port Network Elements

10.7.3.1	Rates for combinations of loop and port network elements, as set forth in Section 10.7.4, are provided in Exhibit A of this Attachment.  Subject to Sections 10.7.1.1 and 10.7.1.2 above, to the extent Cbeyond seeks to obtain other Existing Combinations of ports and loops that are not listed in Section 10.7.4, or in the state of Georgia, to the extent Cbeyond seeks to obtain other port and loop combinations that are not Existing Combinations but that are ordinarily combined in BellSouth’s network, the rate for such combinations shall be the sum of the recurring rates for the individual network elements plus an appropriate cost-based records change charge.

## 10.7.3.2		Rates for Circuit Switching

10.7.3.2.1	Rates for circuit switching, where BellSouth is not required, pursuant to Sections 10.7.1.1 and 10.7.1.2, to provide unbundled access to circuit switching, are as set forth in Exhibit A of this Attachment.

10.7.4	Port/Loop Combination Offerings

10.7.4.1	2-wire voice grade port, voice grade loop, virtual cross connect, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

10.7.4.2	2-wire voice grade DID port, voice grade loop, virtual cross connect, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

10.7.4.3	2-wire CENTREX port, voice grade loop virtual cross connect, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

10.7.4.4.	2-wire ISDN Basic Rate Interface, voice grade loop virtual cross connect, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

10.7.4.5	2-wire ISDN Primary Rate Interface, DS1 loop virtual cross connect, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

10.7.4.6	4-wire DS1 Trunk port, DS1 Loop virtual cross connect, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

# 11.	Operator Systems

BellSouth agrees to offer access to operator systems pursuant to the terms and conditions following and at the rates set forth in this Attachment.

## 11.1	Definition

Operator Systems is the Network Element that provides operator and automated call handling and billing, special services, end user telephone listings and optional call completion services.  The Operator Systems, Network Element provides two types of functions:  Operator Service functions and Directory Assistance Service functions, each of which are described in detail below.

## 11.2	Operator Service

### 11.2.1	Definition

Operator Service provides:  (1) operator handling for call completion (for example, collect, third number billing, and manual credit card calls), (2) operator or automated assistance for billing after the end user has dialed the called number (for example, credit card calls); and (3) special services including but not limited to Busy Line Verification and Emergency Line Interrupt (ELI), Emergency Agency Call, Operator-assisted Directory Assistance, and Rate Quotes.

### 11.2.2	Requirements

#### 11.2.2.1	When Cbeyond requests BellSouth to provide Operator Services, the following requirements apply:

##### 11.2.2.1.1	BellSouth shall complete 0+ and 0- dialed local calls.

##### 11.2.2.1.2	BellSouth shall complete 0+ intraLATA toll calls.

##### 11.2.2.1.3	BellSouth shall complete calls that are billed to Cbeyond end user's calling card that can be validated by BellSouth.

##### 11.2.2.1.4	BellSouth shall complete person-to-person calls.

##### 11.2.2.1.5	BellSouth shall complete collect calls.

##### 11.2.2.1.6	BellSouth shall provide the capability for callers to bill to a third party and complete such calls.

##### 11.2.2.1.7	BellSouth shall complete station-to-station calls.

##### 11.2.2.1.8	BellSouth shall process emergency calls.

##### 11.2.2.1.9	BellSouth shall process Busy Line Verify and Emergency Line Interrupt requests.

##### 11.2.2.1.10	BellSouth shall process emergency call trace, as they do for their End users prior to the Effective Date.  Call must originate from a 911 provider.

##### 11.2.2.1.11	BellSouth shall process operator-assisted directory assistance calls.

#### 11.2.2.2	BellSouth shall adhere to equal access requirements, providing Cbeyond local end users the same IXC access as provided to BellSouth end users.

#### 11.2.2.3	BellSouth shall exercise at least the same level of fraud control in providing Operator Service to Cbeyond that BellSouth provides for its own operator service.

#### 11.2.2.4	BellSouth shall perform Billed Number Screening when handling Collect, Person-to-Person, and Billed-to-Third-Party calls.

#### 11.2.2.5	BellSouth shall direct customer account and other similar inquiries to the customer service center designated by Cbeyond.

#### 10.2.2.6	BellSouth shall provide a feed of customer call records in “EMI” format to Cbeyond in accordance with CLECODUF standards specified in Attachment 7.

### 11.2.3	Interface Requirements

With respect to Operator Services for calls that originate on local switching capability provided by or on behalf of Cbeyond, the interface requirements shall conform to the then current established system interface specifications for the platform used to provide Operator Service and the interface shall conform to industry standards.

## 11.3	Directory Assistance Service

### 11.3.1	Definition

Directory Assistance Service provides local end user telephone number listings with the option to complete the call at the callers direction separate and distinct from local switching.

### 11.3.2	Requirements

#### 11.3.2.1	Directory Assistance Service shall provide up to two listing requests per call.  If available and if requested by Cbeyond’s end user, BellSouth shall provide caller-optional directory assistance call completion service at rates contained in this Attachment to one of the provided listings, equal to that which BellSouth provides its end users.  In providing call completion service, and at such time as functionality is available in the BellSouth network, BellSouth shall route the call to Cbeyond’s network for call completion.  Rates for such functionality shall be established at the time such functionality becomes available.  If not available, Cbeyond may request such requirement pursuant to the Bona Fide Request/New Business Process as set forth in General Terms and Conditions.

#### 11.3.2.2	Directory Assistance Service Updates

##### 11.3.2.2.1	BellSouth shall update end user listings changes daily.  These changes include:

###### 11.3.2.2.1.1	New end user connections:  BellSouth will provide service to Cbeyond that is equal to the service it provides to itself and its end users;

###### 11.3.2.2.1.2	End user disconnections:  BellSouth will provide service to Cbeyond that is equal to the service it provides to itself and its end users; and

###### 11.3.2.2.1.3	End user address changes:  BellSouth will provide service to Cbeyond that is equal to the service it provides to itself and its end users;

#### 11.3.2.3	These updates shall also be provided for non-listed and non-published numbers for use in emergencies.

## 11.4	Branding for Operator Call Processing and Directory Assistance

### 11.4.1	The BellSouth Operator Systems Branding Feature provides a definable announcement to Cbeyond end users using Directory Assistance (DA)/Operator Call Processing (OCP) prior to placing them in queue or connecting them to an available operator or automated operator system. This feature allows Cbeyond to have its calls custom branded with Cbeyond name on whose behalf BellSouth is providing Directory Assistance and/or Operator Call Processing.  Rates for Custom Branding, Operator Call Process and Directory Assistance are set forth in this Attachment.

### 11.4.2	BellSouth offers four service levels of branding to Cbeyond when ordering Directory Assistance and/or Operator Call Processing.

#### 11.4.2.1	Service Level 1 - BellSouth Branding

#### 11.4.2.2	Service Level 2 - Unbranded

#### 11.4.2.3	Service Level 3 - Custom Branding

#### 11.4.2.4	Service Level 4 - Self Branding (applicable only to Cbeyond for Resale or use with an Unbundled Port when routing to an operator service provider other than BellSouth).

### 11.4.3	For Resellers and Use with an Unbundled Port

#### 11.4.3.1	BellSouth Branding is the Default Service Level.

#### 11.4.3.2	Unbranding, Custom Branding, and Self Branding require Cbeyond to order selective routing for each originating BellSouth end office identified by Cbeyond.  Rates for Selective Routing are set forth in this Attachment.

#### 11.4.3.3	Customer Branding and Self Branding require Cbeyond to order dedicated trunking from each BellSouth end office identified by Cbeyond, to either the BellSouth Traffic Operator Position System (TOPS) or Cbeyond Operator Service Provider.  Rates for trunks are set forth in applicable BellSouth tariffs.

#### 11.4.3.4	Unbranding - Unbranded Directory Assistance and/or Operator Call Processing calls ride common trunk groups provisioned by BellSouth from those end offices identified by Cbeyond to the BellSouth TOPS.  These calls are routed to “No Announcement.”

### 11.4.4	For Facilities Based Carriers

#### 11.4.4.1	All Service Levels require Cbeyond to order dedicated trunking from their end office(s) point of interface to the BellSouth TOPS Switches. Rates for trunks are set forth in applicable BellSouth tariffs.

#### 11.4.4.2	Customized Branding includes charges for the recording of the branding announcement and the loading of the audio units in each TOPS Switch, IVS and NAV equipment for which Cbeyond requires service

Directory Assistance customized branding uses:

- the recording of the name;

- the front-end loading of the Digital Recorded Announcement Machine (DRAM) in each TOPS switch.

Operator Call Processing customized branding uses:

- the recording of the name;

- the front-end loading of the DRAM in the TOPS Switch;

- the back-end loading in the audio units in the Automated Alternate Billing System (AABS) in the Interactive Voice Subsystem (IVS);

- the 0- automation loading for the audio units in the Enhanced Billing and Access Service (EBAS) in the Network Applications Vehicle (NAV).

11.4.4.3	BellSouth will provide to Cbeyond purchasing  local BellSouth switching and reselling BellSouth local exchange service, selective routing of calls to a requested directory assistance services platform or operator services platform.  Cbeyond end users may use the same dialing arrangements as BellSouth end users, but obtain a Cbeyond branded service.

## 11.5	Directory Assistance Database Service (DADS)

### 11.5.1	BellSouth shall make its Directory Assistance Database Service (DADS) available solely for the expressed purpose of providing Directory Assistance type services to Cbeyond end users.  The term “end user” denotes any entity which obtains Directory Assistance type services for its own use from a DADS customer.  Directory Assistance type service is defined as Voice Directory Assistance (DA Operator assisted and Electronic Directory Assistance (Data System assisted)).  Cbeyond agrees that Directory Assistance Database Service (DADS) will not be used for any purpose which violates federal or state laws, statutes, regulatory orders or tariffs.  Except for the permitted users, Cbeyond agrees not to disclose DADS to others and shall provide due care in providing for the security and confidentiality of DADS. Further, Cbeyond authorizes the inclusion of Cbeyond Subscriber listings in the BellSouth Directory Assistance products.

### 11.5.2	BellSouth shall provide Cbeyond initially with a base file of subscriber listings which reflect all listing change activity occurring since Cbeyond’s most recent update via magnetic tape, and subsequently using electronic connectivity such as Network Data Mover to be developed mutually by Cbeyond and BellSouth.  Cbeyond agrees to assume the costs associated with CONNECT: Direct TM connectivity, which will vary depending upon volume and mileage.

### 11.5.3	BellSouth will require approximately one month after receiving an order to prepare the Base File.  BellSouth will provide daily updates which will reflect all listing change activity occurring since CLEC’s most recent update.  BellSouth shall provide updates to Cbeyond on a Business, Residence, or combined Business and Residence basis.  Cbeyond agrees that the updates shall be used solely to keep the information current.  Delivery of Daily Updates will commence the day after Cbeyond receives the Base File.

### 11.5.4	BellSouth is authorized to include Cbeyond Subscriber List Information in its Directory Assistance Database Service (DADS) and its Directory Publishers Database Service (DPDS).  Any other use by BellSouth of Cbeyond Subscriber List Information is not authorized and with the exception of a request for DADS or DPDS, BellSouth shall refer any request for such information to Cbeyond.

### 11.5.5	Rates for DADS are as set forth in this Attachment.

## 11.6	Direct Access to Directory Assistance Service

### 11.6.1	Direct Access to Directory Assistance Service (DADAS) will provide Cbeyond’s directory assistance operators with the ability to search all available BellSouth’s subscriber listings using the Directory Assistance search format.  Subscription to DADAS will allow Cbeyond to utilize its own switch, operator workstations and optional audio subsystems.

### 11.6.2	BellSouth will provide DADAS from its DA location.  Cbeyond will access the DADAS system via a telephone company provided point of availability.  Cbeyond has the responsibility of providing the physical links required to connect to the point of availability.  These facilities may be purchased from the telephone company as rates and charges billed separately from the charges associated with this offering.

### 11.6.3	A specified interface to each Cbeyond subsystem will be provided by BellSouth.  Interconnection between Cbeyond system and a specified BellSouth location will be pursuant to the use of Cbeyond owned or Cbeyond leased facilities and shall be appropriate sized based upon the volume of queries being generated by Cbeyond.

### 11.6.4	The specifications for the three interfaces necessary for interconnection are available in the following documents:

#### 11.6.4.1	DADAS to Subscriber Operator Position System—Northern Telecom Document CSI-2300-07; Universal Gateway/ Position Message Interface Format Specification

#### 11.6.4.2	DADAS to Subscriber Switch—Northern Telecom Document Q210-1 Version A107; NTDMS/CCIDAS System Application Protocol; and AT&T Document 250-900-535 Operator Services Position System Listing Service and Application Call Processing Data Link Interface Specification

#### 11.6.4.3	DADAS to Audio Subsystem (Optional)—Directory One Call Control to Audio Response Unit system interface specifications are available through Northern Telecom as a licensed access protocol—Northern Telecom Document 355-004424 and Gateway/Interactive Voice subsystem Protocol Specification

### 11.6.5	Rates for DADAS are as set forth in this Attachment.

# 12.	Signaling

BellSouth agrees to offer access to  signaling and access to BellSouth’s signaling databases subject to compatibility testing and at the rates set forth in this Attachment.  BellSouth may provide mediated access to BellSouth signaling systems and databases.  Available signaling elements include signaling links, signal transfer points and service control points.  Signaling functionality will be available with both A-link and B-link connectivity.

## 12.1	Definition of Signaling Link Transport

Signaling Link Transport is a set of two or four dedicated 56 Kbps. transmission paths between CLEC-designated Signaling Points of Interconnection (SPOI) that provides appropriate physical diversity.

## 12.2	Technical Requirements

### 12.2.1	Signaling Link Transport shall consist of full duplex mode 56 kbps transmission paths.

### 12.2.2	Of the various options available, Signaling Link Transport shall perform in the following two ways:

#### 12.2.2.1	As an “A-link” which is a connection between a switch or SCP and a home Signaling Transfer Point Switch (STP) pair; and

#### 12.2.2.2	As a “B-link” which is a connection between two STP pairs in different company networks (e.g., between two STP pairs for two Competitive Local Exchange Carriers (CLECs)).

### 12.2.3	Signaling Link Transport shall consist of two or more signaling link layers as follows:

#### 12.2.3.1	An A-link layer shall consist of two links.

#### 12.2.3.2	A B-link layer shall consist of four links.

### 12.2.4	A signaling link layer shall satisfy a performance objective such that:

#### 12.2.4.1	There shall be no more than two minutes down time per year for an A-link layer; and

#### 12.2.4.2	There shall be negligible (less than 2 seconds) down time per year for a B-link layer.

### 12.2.5	A signaling link layer shall satisfy interoffice and intraoffice diversity of facilities and equipment, such that:

#### 12.2.5.1	No single failure of facilities or equipment causes the failure of both links in an A-link layer (i.e., the links should be provided on a minimum of two separate physical paths end-to-end); and

#### 12.2.5.2	No two concurrent failures of facilities or equipment shall cause the failure of all four links in a B-link layer (i.e., the links should be provided on a minimum of three separate physical paths end-to-end).

## 12.3	Interface Requirements

### 12.3.1	There shall be a DS1 (1.544 Mbps) interface at the Cbeyond-designated SPOIs.  Each 56 kbps transmission path shall appear as a DS0 channel within the DS1 interface.

# 13.	Signaling Transfer Points (STPs)

## 13.1	Definition - Signaling Transfer Points is a signaling network function that includes all of the capabilities provided by the signaling transfer point switches (STPs) and their associated signaling links which enable the exchange of SS7 messages among and between switching elements, database elements and signaling transfer point switches

## 13.2	Technical Requirements

### 13.2.1	STPs shall provide access to Network Elements connected to BellSouth SS7 network. These include:

#### 13.2.1.1	BellSouth Local Switching or Tandem Switching;

#### 13.2.1.2	BellSouth Service Control Points/DataBases;

#### 132.2.1.3	Third-party local or tandem switching;

#### 13.2.1.4	Third-party-provided STPs.

### 13.2.2	The connectivity provided by STPs shall fully support the functions of all other Network Elements connected to BellSouth SS7 network.  This explicitly includes the use of BellSouth SS7 network to convey messages which neither originate nor terminate at a signaling end point directly connected to BellSouth SS7 network (i.e., transient messages).  When BellSouth SS7 network is used to convey transient messages, there shall be no alteration of the Integrated Services Digital Network User Part (ISDNUP) or Transaction Capabilities Application Part (TCAP) user data that constitutes the content of the message.

### 13.2.3	If a BellSouth tandem switch routes calling traffic, based on dialed or translated digits, on SS7 trunks between an Cbeyond local switch and third party local switch, BellSouth SS7 network shall convey the TCAP messages that are necessary to provide Call Management features (Automatic Callback, Automatic Recall, and Screening List Editing) between Cbeyond local STPs and the STPs that provide connectivity with the third party local switch, even if the third party local switch is not directly connected to BellSouth STPs.

### 13.2.4	STPs shall provide all functions of the MTP as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.  This includes:

#### 13.2.4.1	Signaling Data Link functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements,

#### 13.2.4.2	Signaling Link functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements, and

#### 13.2.4.3	Signaling Network Management functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.

### 13.2.5	STPs shall provide all functions of the SCCP necessary for Class 0 (basic connectionless) service, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.  In particular, this includes Global Title Translation (GTT) and SCCP Management procedures, as specified in T1.112.4.  In cases where the destination signaling point is a Cbeyond or third party local or tandem switching system directly connected to BellSouth SS7 network, BellSouth shall perform final GTT of messages to the destination and SCCP Subsystem Management of the destination.  In all other cases, BellSouth shall perform intermediate GTT of messages to a gateway pair of STPs in an SS7 network connected with BellSouth SS7 network, and shall not perform SCCP Subsystem Management of the destination.  If BellSouth performs final GTT to a Cbeyond database, then Cbeyond agrees to provide BellSouth with the Destination Point Code for the Cbeyond database.

### 13.2.6	STPs shall provide on a non-discriminatory basis all functions of the OMAP commonly provided by STPs, as specified in the reference in Section 12.4.5 of this Attachment.  All OMAP functions will be on a "where available" basis and can include:

#### 13.2.6.1	MTP Routing Verification Test (MRVT) and

#### 13.2.6.2	SCCP Routing Verification Test (SRVT).

### 13.2.7	In cases where the destination signaling point is a BellSouth local or tandem switching system or database, or is a Cbeyond or third party local or tandem switching system directly connected to the BellSouth SS7 network, STPs shall perform MRVT and SRVT to the destination signaling point.  In all other cases, STPs shall perform MRVT and SRVT to a gateway pair of STPs in an SS7 network connected with the BellSouth SS7 network.  This requirement shall be superseded by the specifications for Internetwork MRVT and SRVT if and when these become approved ANSI standards and available capabilities of BellSouth STPs, and if mutually agreed upon by Cbeyond and BellSouth.

### 13.2.8	STPs shall be on parity with BellSouth.

### 13.2.9	SS7 Advanced Intelligent Network (AIN) Access

#### 13.2.9.1	When technically feasible and upon request by Cbeyond, SS7 Access shall be made available in association with  switching.  SS7 AIN Access is the provisioning of AIN 0.1 triggers in an equipped BellSouth local switch and interconnection of the BellSouth SS7 network with the Cbeyond SS7 network to exchange TCAP queries and responses with a Cbeyond SCP.

#### 13.2.9.2	SS7 AIN Access shall provide Cbeyond SCP access to BellSouth local switch in association with  switching via interconnection of BellSouth SS7 and Cbeyond SS7 Networks.  BellSouth shall offer SS7 access through its STPs.  If BellSouth requires a mediation device on any part of its network specific to this form of access, BellSouth must route its messages in the same manner.  The interconnection arrangement shall result in the BellSouth local switch recognizing the Cbeyond SCP as at least at parity with BellSouth’s SCPs in terms of interfaces, performance and capabilities.

## 13.3	Interface Requirements

### 13.3.1	BellSouth shall provide the following STPs options to connect Cbeyond or Cbeyond-designated local switching systems or STPs to BellSouth SS7 network:

#### 13.3.1.1	An A-link interface from Cbeyond local switching systems; and,

#### 13.3.1.2	A B-link interface from Cbeyond local STPs.

### 13.3.2	Each type of interface shall be provided by one or more sets (layers) of signaling links.

### 13.3.3	The Signaling Point of Interconnection (SPOI) for each link shall be located at a cross-connect element, such as a DSX-1, in the Central Office (CO) where BellSouth STP is located.  There shall be a DS1 or higher rate transport interface at each of the SPOIs.  Each signaling link shall appear as a DS0 channel within the DS1 or higher rate interface. BellSouth shall offer higher rate DS1 signaling for interconnecting Cbeyond local switching systems or STPs with BellSouth STPs as soon as these become approved ANSI standards and available capabilities of BellSouth STPs.  BellSouth and Cbeyond will work jointly to establish mutually acceptable SPOIs.

### 13.3.4	BellSouth CO shall provide intraoffice diversity between the SPOIs and BellSouth STPs, so that no single failure of intraoffice facilities or equipment shall cause the failure of both B-links in a layer connecting to a BellSouth STP.  BellSouth and Cbeyond will work jointly to establish mutually acceptable SPOIs.

### 13.3.5	BellSouth shall provide MTP and SCCP protocol interfaces that shall conform to all sections relevant to the MTP or SCCP in the following specifications:

#### 13.3.5.1	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

#### 13.3.5.2	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

### 13.3.6	Message Screening

#### 13.3.6.1	BellSouth shall set message screening parameters so as to accept valid messages from Cbeyond local or tandem switching systems destined to any signaling point within BellSouth’s SS7 network where the Cbeyond switching system has a legitimate signaling relation.

#### 13.3.6.2	BellSouth shall set message screening parameters so as to pass valid messages from Cbeyond local or tandem switching systems destined to any signaling point or network accessed through BellSouth’s SS7 network where the Cbeyond switching system has a legitimate signaling relation.

#### 13.3.6.3	BellSouth shall set message screening parameters so as to accept and pass/send valid messages destined to and from Cbeyond from any signaling point or network interconnected through BellSouth’s SS7 network where the Cbeyond SCP has a legitimate signaling relation.

## 13.4	STPs shall be equal to or better than all of the requirements for STPs set forth in the following technical references:

### 13.4.1	ANSI T1.111-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP);

### 13.4.2	ANSI T1.111A-1994 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP) Supplement;

### 13.4.3	ANSI T1.112-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Signaling Connection Control Part (SCCP);

### 13.4.4	ANSI T1.115-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Monitoring and Measurements for Networks;

### 13.4.5	ANSI T1.116-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Operations, Maintenance and Administration Part (OMAP);

### 13.4.6	ANSI T1.118-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Intermediate Signaling Network Identification (ISNI);

### 13.4.7	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP); and

### 13.4.8	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

# 14.	Service Control Points/DataBases

## 14.1	Definition

### 14.1.1	Databases are the Network Elements that provide the functionality for storage of, access to, and manipulation of information required to offer a particular service and/or capability.  Databases include, but are not limited to:  Local Number Portability, LIDB, Toll Free Number Database, Automatic Location Identification/Data Management System, Calling Name Database, access to Service Creation Environment and Service Management System (SCE/SMS) application databases and Directory Assistance.

### 14.1.2	A Service Control Point (SCP) is a specific type of Database functionality deployed in a Signaling System 7 (SS7) network that executes service application logic in response to SS7 queries sent to it by a switching system also connected to the SS7 network.  Service Management Systems provide operational interfaces to allow for provisioning, administration and maintenance of subscriber data and service application data stored in SCPs.

## 14.2	Technical Requirements for SCPs/Databases

Requirements for SCPs/Databases within this section address storage of information, access to information (e.g. signaling protocols, response times), and administration of information (e.g., provisioning, administration, and maintenance).  All SCPs/Databases shall be provided to Cbeyond in accordance with the following requirements.

### 14.2.1	BellSouth shall provide physical access to SCPs through the SS7 network and protocols with TCAP as the application layer protocol.

### 14.2.2	BellSouth shall provide physical interconnection to databases via industry standard interfaces and protocols (e.g. SS7, ISDN and X.25).

### 14.2.3	The reliability of interconnection options shall be consistent with requirements for diversity and survivability.

### 14.2.4	Database Availability

Call processing databases shall have a maximum unscheduled availability of 30 minutes per year.  Unavailability due to software and hardware upgrades shall be scheduled during minimal usage periods and only be undertaken upon proper notification to providers which might be impacted.  Any downtime associated with the provision of call processing related databases will impact all service providers, including BellSouth, equally.

### 14.2.5	The operational interface provided by BellSouth shall complete Database transactions (i.e., add, modify, delete) for Cbeyond customer records stored in BellSouth databases within 3 days, or sooner where BellSouth provisions its own customer records within a shorter interval.

## 14.3	Local Number Portability Database

### 14.3.1	Definition

The Permanent Number Portability (PNP) database supplies routing numbers for calls involving numbers that have been ported from one local service provider to another.  PNP is currently being worked in industry forums. The results of these forums will dictate the industry direction of PNP.  BellSouth agrees to provide access to the PNP database at rates, terms and conditions as set forth by BellSouth and in accordance with an effective FCC or Commission directive.

## 14.4	Line Information Database (LIDB)

BellSouth will store in its LIDB only records relating to service in the BellSouth region.   The LIDB Storage Agreement is included in this Attachment.

### 14.4.1	Definition

The Line Information Database (LIDB) is a transaction-oriented database accessible through Common Channel Signaling (CCS) networks.  It contains records associated with end user Line Numbers and Special Billing Numbers.  LIDB accepts queries from other Network Elements and provides appropriate responses.  The query originator need not be the owner of LIDB data.  LIDB queries include functions such as screening billed numbers that provides the ability to accept Collect or Third Number Billing calls and validation of Telephone Line Number based non-proprietary calling cards.  The interface for the LIDB functionality is the interface between BellSouth CCS network and other CCS networks.  LIDB also interfaces to administrative systems.

### 14.4.2	Technical Requirements

BellSouth will offer to Cbeyond any additional capabilities that are developed for LIDB during the life of this Agreement.

#### 14.4.2.1	BellSouth shall process Cbeyond’s Customer records in LIDB at least at parity with BellSouth customer records, with respect to other LIDB functions.  BellSouth shall indicate to Cbeyond what additional functions (if any) are performed by LIDB in the BellSouth network.

#### 14.4.2.2	Within two (2) weeks after a request by Cbeyond, BellSouth shall provide Cbeyond with a list of the customer data items which Cbeyond would have to provide in order to support each required LIDB function.  The list shall indicate which data items are essential to LIDB function, and which are required only to support certain services.  For each data item, the list shall show the data formats, the acceptable values of the data item and the meaning of those values.

#### 14.4.2.3	BellSouth shall provide LIDB systems for which operating deficiencies that would result in calls being blocked shall not exceed 30 minutes per year.

#### 14.4.2.4	BellSouth shall provide LIDB systems for which operating deficiencies that would not result in calls being blocked shall not exceed 12 hours per year.

#### 14.4.2.5	BellSouth shall provide LIDB systems for which the LIDB function shall be in overload no more than 12 hours per year.

#### 14.4.2.6	All additions, updates and deletions of Cbeyond data to the LIDB shall be solely at the direction of Cbeyond.  Such direction from Cbeyond will not be required where the addition, update or deletion is necessary to perform standard fraud control measures (e.g., calling card auto-deactivation).

#### 14.4.2.7	BellSouth shall provide priority updates to LIDB for Cbeyond data upon Cbeyond’s request (e.g., to support fraud detection), via password-protected telephone card, facsimile, or electronic mail within one hour of notice from the established BellSouth contact.

#### 14.4.2.8	BellSouth shall provide LIDB systems such that no more than 0.01% of Cbeyond customer records will be missing from LIDB, as measured by Cbeyond audits.  BellSouth will audit Cbeyond records in LIDB against DBAS to identify record mismatches and provide this data to a designated Cbeyond contact person to resolve the status of the records and BellSouth will update system appropriately.  BellSouth will refer record of mismatches to Cbeyond within one business day of audit.  Once reconciled records are received back from Cbeyond, BellSouth will update LIDB the same business day if less than 500 records are received before 1:00PM Central Time.  If more than 500 records are received, BellSouth will contact Cbeyond to negotiate a time frame for the updates, not to exceed three business days.

#### 14.4.2.9	BellSouth shall perform backup and recovery of all of Cbeyond’s data in LIDB including sending to LIDB all changes made since the date of the most recent backup copy, in at least the same time frame BellSouth performs backup and recovery of BellSouth data in LIDB for itself.  Currently, BellSouth performs backups of the LIDB for itself on a weekly basis and when a new software release is scheduled, a backup is performed prior to loading the new release.

#### 14.4.2.10	BellSouth shall provide Cbeyond with LIDB reports of data which are missing or contain errors, as well as any misrouted errors, within a reasonable time period as negotiated between Cbeyond and BellSouth.

#### 14.4.2.11	BellSouth shall prevent any access to or use of Cbeyond data in LIDB by BellSouth personnel that are outside of established administrative and fraud control personnel, or by any other Party that is not authorized by Cbeyond in writing.

#### 14.4.2.12	BellSouth shall provide Cbeyond performance of the LIDB Data Screening function, which allows a LIDB to completely or partially deny specific query originators access to LIDB data owned by specific data owners, for Customer Data that is part of an NPA-NXX or RAO-0/1XX wholly or partially owned by Cbeyond at least at parity with BellSouth Customer Data. BellSouth shall obtain from Cbeyond the screening information associated with LIDB Data Screening of Cbeyond data in accordance with this requirement.  BellSouth currently does not have LIDB Data Screening capabilities.  When such capability is available, BellSouth shall offer it to Cbeyond under the Bona Fide Request/New Business Process as set forth in General Terms and Conditions .

#### 14.4.2.13	BellSouth shall accept queries to LIDB associated with Cbeyond customer records, and shall return responses in accordance with industry standards.

#### 14.4.2.14	BellSouth shall provide mean processing time at the LIDB within 0.50 seconds under normal conditions as defined in industry standards.

#### 14.4.2.15	BellSouth shall provide processing time at the LIDB within 1 second for 99% of all messages under normal conditions as defined in industry standards.

### 14.4.3	Interface Requirements

BellSouth shall offer LIDB in accordance with the requirements of this subsection.

#### 14.4.3.1	The interface to LIDB shall be in accordance with the technical references contained within.

#### 14.4.3.2	The CCS interface to LIDB shall be the standard interface described herein.

#### 14.4.3.3	The LIDB Data Base interpretation of the ANSI-TCAP messages shall comply with the technical reference herein.  Global Title Translation shall be maintained in the signaling network in order to support signaling network routing to the LIDB.

## 14.5	Toll Free Number Database

The Toll Free Number Database is a SCP that provides functionality necessary for toll free (e.g., 800 and 888) number services by providing routing information and additional so-called vertical features during call set-up in response to queries from SSPs. BellSouth shall provide the Toll Free Number Database in accordance with the following:

### 14.5.1	Technical Requirements

#### 14.5.1.1	BellSouth shall make BellSouth Toll Free Number Database available for Cbeyond to query with a toll-free number and originating information.

#### 14.5.1.2 	The Toll Free Number Database shall return carrier identification and, where applicable, the queried toll free number, translated numbers and instructions as it would in response to a query from a BellSouth switch.

#### 14.5.1.3	The SCP shall also provide, at Cbeyond's option, such additional feature as described in SR-TSV-002275 (BOC Notes on BellSouth Networks, SR-TSV-002275, Issue 2, (Telcordia (formerly BellCore), April 1994)) as are available to BellSouth.  These may include but are not limited to:

##### 14.5.1.3.1	Network Management;

##### 14.5.1.3.2	Customer Sample Collection; and

##### 14.5.1.3.3	Service Maintenance

## 14.6	Automatic Location Identification/Data Management System (ALI/DMS)

The ALI/DMS Database contains end user information (including name, address, telephone information, and sometimes special information from the local service provider or end user) used to determine to which Public Safety Answering Point  (PSAP) to route the call.  The ALI/DMS database is used to provide more routing flexibility for E911 calls than Basic 911.  BellSouth shall provide the Emergency Services Database in accordance with the following:

### 14.6.1	Technical Requirements

#### 14.6.1.1	BellSouth shall offer Cbeyond a data link to the ALI/DMS database or permit Cbeyond to provide its own data link to the ALI/DMS database. BellSouth shall provide error reports from the ALI/DMS database to Cbeyond immediately after Cbeyond inputs information into the ALI/DMS database.  Alternately, Cbeyond may utilize BellSouth, to enter end user information into the data base on a demand basis, and validate end user information on a demand basis.

#### 14.6.1.2	The ALI/DMS database shall contain the following end user information:

##### 14.6.1.2.1	Name;

##### 14.6.1.2.2	Address;

##### 14.6.1.2.3	Telephone number; and

##### 14.6.1.2.4	Other information as appropriate (e.g., whether a end user is blind or deaf or has another disability).

#### 14.6.1.3	When BellSouth is responsible for administering the ALI/DMS database in its entirety, ported number NXXs entries for the ported numbers should be maintained unless Cbeyond requests otherwise and shall be updated if Cbeyond requests, provided Cbeyond supplies BellSouth with the updates.

#### 14.6.1.4	When Remote Call Forwarding (RCF) is used to provide number portability to the local end user and a remark or other appropriate field information is available in the database, the shadow or “forwarded-to” number and an indication that the number is ported shall be added to the customer record.

#### 14.6.1.5	If BellSouth is responsible for configuring PSAP features (for cases when the PSAP or BellSouth supports an ISDN interface) it shall ensure that CLASS Automatic Recall (Call Return) is not used to call back to the ported number.  Although BellSouth currently does not have ISDN interface, BellSouth agrees to comply with this requirement once ISDN interfaces are in place.

### 14.6.2	Interface Requirements

The interface between the E911 Switch or Tandem and the ALI/DMS database for Cbeyond end users shall meet industry standards.

## 14.7	Directory Assistance Database

BellSouth shall make its directory assistance database available to Cbeyond in order to allow Cbeyond to provide its end users with the same directory assistance telecommunications services BellSouth provides to BellSouth end users.  BellSouth shall provide Cbeyond with an initial feed via magnetic tape and daily update initially via magnetic tape and subsequently via an electronic gateway to be developed mutually by Cbeyond and BellSouth of end user address and number changes.  Directory Assistance Services must provide both the ported and Cbeyond telephone numbers to the extent available in BellSouth’s database assigned to a end user.  Privacy indicators must be properly identified to assure the non-published numbers are accurately identified.

## 14.8	Calling Name (CNAM) Database Service

The Agreement for Calling Name (CNAM) with standard pricing is included as Exhibit B to this Attachment.  Cbeyond must provide to its account manager a written request with a requested activation date to activate this service.  If Cbeyond is interested in requesting CNAM with volume and term pricing, Cbeyond must contact its account manager to request a separate CNAM volume and term Agreement.

## 14.9	SCPs/Databases shall be equal to or better than all of the requirements for SCPs/Databases set forth in the following technical references:

### 14.9.1	GR-246-CORE, Bell Communications Research Specification of  Signaling System Number 7, ISSUE 1 (Telcordia (formerly BellCore), December 199);

### 14.9.2	GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP). (Telcordia (formerly BellCore), March 1994);

### 14.9.3	GR-954-CORE, CCS Network Interface Specification (CCSNIS) Supporting Line Information Database (LIDB) Service 6, Issue 1, Rev. 1 (Telcordia (formerly BellCore), October 1995);

### 14.9.4	GR-1149-CORE, OSSGR Section 10: System Interfaces, Issue 1 (Telcordia (formerly BellCore), October 1995)  (Replaces TR-NWT-001149);

### 14.9.5	Telcordia (formerly BellCore) GR-1158-CORE, OSSGR Section 22.3: Line Information Database 6, Issue (Telcordia (formerly BellCore), October 1995);

### 14.9.6	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service (Telcordia (formerly BellCore), May 1995); and

### 14.9.7	BOC Notes on BellSouth Networks, SR-TSV-002275, ISSUE 2, (Telcordia (formerly BellCore), April 1994).

## 14.10	Service Creation Environment and Service Management System (SCE/SMS) Advanced Intelligent Network (AIN) Access.

### 14.10.1	BellSouth’s Service Creation Environment and Service Management System (SCE/SMS) Advanced Intelligent Network (AIN) Access shall provide Cbeyond the capability that will allow Cbeyond and other third parties to create service applications in a BellSouth Service Creation Environment and deploy those applications in a BellSouth SMS to a BellSouth SCP.  The third party service applications interact with AIN triggers provisioned on a BellSouth SSP.

### 14.10.2	BellSouth’s SCE/SMS AIN Access shall provide access to SCE hardware, software, testing and technical support (e.g., help desk, system administrator) resources available to Cbeyond.  Scheduling procedures shall provide Cbeyond equivalent priority to these resources

### 14.10.3	BellSouth SCP shall partition and protect Cbeyond service logic and data from unauthorized access, execution or other types of compromise.

### 14.10.4	When Cbeyond selects SCE/SMS AIN Access, BellSouth shall provide training, documentation, and technical support to enable Cbeyond to use BellSouth’s SCE/SMS AIN Access to create and administer applications.  Training, documentation, and technical support will address use of SCE and SMS access and administrative functions, but will not include support for the creation of a specific service application.

### 14.10.5	When Cbeyond selects SCE/SMS AIN Access, BellSouth shall provide for a secure, controlled access environment in association with its internal use of AIN components.  Cbeyond access will be provided via remote data connection (e.g., dial-in, ISDN).

### 14.10.6	When Cbeyond selects SCE/SMS AIN Access, BellSouth shall allow Cbeyond to download data forms and/or tables to BellSouth SCP via BellSouth SMS without intervention from BellSouth (e.g., service customization and end user subscription).

# 15.	Dark Fiber

### 15.1  		Definition

Dark Fiber is optical transmission facilities without attached multiplexing, aggregation or other electronics that connects two points within BellSouth’s network.  Dark Fiber also includes strands of optical fiber existing in aerial or underground cable which may have lightwave repeater (regenerator or optical amplifier) equipment interspliced to it at appropriate distances, but which has no line terminating elements terminated to such strands to operationalize its transmission capabilities.

## 15.2		Requirements

### 15.2.1	BellSouth shall make available Dark Fiber where it exists in BellSouth’s network and where, as a result of future building or deployment, it becomes available.  If BellSouth has plans to use the fiber within a two-year planning period, there is no requirement to provide said fiber to Cbeyond.

### 15.2.2	If the requested dark fiber has any lightwave repeater equipment interspliced to it, BellSouth will remove such equipment at Cbeyond’s request subject to time and materials charges.

BellSouth shall use its best efforts to provide to Cbeyond information regarding the location, availability and performance parameters of Dark Fiber within ten (10) business days, after receiving a request from Cbeyond ("Request").  Within such time period, BellSouth shall send written confirmation of availability of the Dark Fiber ("Confirmation").

### 15.2.4	BellSouth shall use its best efforts to make Dark Fiber available to Cbeyond within thirty (30) business days after it receives written confirmation from Cbeyond that the Dark Fiber previously deemed available by BellSouth is wanted for use by Cbeyond.  This includes identification of appropriate connection points (e.g., Light Guide Interconnection (LGX) or splice points) to enable Cbeyond to connect or splice Cbeyond provided transmission media (e.g., optical fiber) or equipment to the Dark Fiber.

# 16.	SS7 Network Interconnection

### 16.1	Definition

SS7 Network Interconnection is the interconnection of Cbeyond local Signaling Transfer Point Switches (STP) and Cbeyond local or tandem switching systems with BellSouth STPs.  This interconnection provides connectivity that enables the exchange of SS7 messages among BellSouth switching systems and databases (DBs), Cbeyond local or tandem switching systems, and other third-party switching systems directly connected to the BellSouth SS7 network.

### 16.2	Technical Requirements

#### 16.2.1	SS7 Network Interconnection shall provide connectivity to all components of the BellSouth SS7 network.  These include:

##### 16.2.1.1	BellSouth local or tandem switching systems;

##### 16.2.1.2	BellSouth DBs; and

##### 16.2.1.3	Other third-party local or tandem switching systems.

#### 16.2.2	The connectivity provided by SS7 Network Interconnection shall fully support the functions of BellSouth switching systems and DBs and Cbeyond or other third-party switching systems with A-link access to the BellSouth SS7 network.

If traffic is routed based on dialed or translated digits between an Cbeyond local switching system and a BellSouth or other third-party local switching system, either directly or via a BellSouth tandem switching system, then it is a requirement that the BellSouth SS7 network convey via SS7 Network Interconnection the TCAP messages that are necessary to provide Call Management services (Automatic Callback, Automatic Recall, and Screening List Editing) between the Cbeyond local STPs and BellSouth or other third-party local switch.

#### 16.2.3	When the capability to route messages based on Intermediate Signaling Network Identifier (ISNI) is generally available on BellSouth STPs,  the BellSouth SS7 Network shall also convey TCAP messages using SS7 Network Interconnection in similar circumstances where the BellSouth switch routes traffic based on a Carrier Identification Code (CIC).

#### 16.2.4	SS7 Network Interconnection shall provide all functions of the MTP as specified in ANSI T1.111. This includes:

##### 16.2.4.1	Signaling Data Link functions, as specified in ANSI T1.111.2;

##### 16.2.4.2	Signaling Link functions, as specified in ANSI T1.111.3; and

##### 16.2.4.3	Signaling Network Management functions, as specified in ANSI T1.111.4.

#### 16.2.5	SS7 Network Interconnection shall provide all functions of the SCCP necessary for Class 0 (basic connectionless) service, as specified in ANSI T1.112.  In particular, this includes Global Title Translation (GTT) and SCCP Management procedures, as specified in T1.112.4.  Where the destination signaling point is a BellSouth switching system or DB, or is another third-party local or tandem switching system directly connected to the BellSouth SS7 network, SS7 Network Interconnection shall include final GTT of messages to the destination and SCCP Subsystem Management of the destination.  Where the destination signaling point is a Cbeyond local or tandem switching system, SS7 Network Interconnection shall include intermediate GTT of messages to a gateway pair of Cbeyond local STPs, and shall not include SCCP Subsystem Management of the destination.

#### 16.2.6	SS7 Network Interconnection shall provide all functions of the Integrated Services Digital Network User Part (ISDNUP), as specified in ANSI T1.113.

#### 16.2.7	SS7 Network Interconnection shall provide all functions of the TCAP, as specified in ANSI T1.114.

#### 16.2.8	If and when Internetwork MTP Routing Verification Test (MRVT) and SCCP Routing Verification Test (SRVT) become approved ANSI standards and available capabilities of BellSouth STPs, SS7 Network Interconnection shall provide these functions of the OMAP.

#### 16.2.9	SS7 Network Interconnection shall be equal to or better than the following performance requirements:

##### 16.2.9.1	MTP Performance, as specified in ANSI T1.111.6;

##### 16.2.9.2	SCCP Performance, as specified in ANSI T1.112.5; and

##### 16.2.9.3	ISDNUP Performance, as specified in ANSI T1.113.5.

### 16.3	Interface Requirements

#### 16.3.1	BellSouth shall offer the following SS7 Network Interconnection options to connect Cbeyond or Cbeyond-designated local or tandem switching systems or STPs to the BellSouth SS7 network:

##### 16.3.1.1	A-link interface from Cbeyond local or tandem switching systems; and

##### 16.3.1.2	B-link interface from Cbeyond STPs.

#### 16.3.2	The Signaling Point of Interconnection (SPOI) for each link shall be located at a cross-connect element, such as a DSX-1, in the Central Office (CO) where the BellSouth STP is located.  There shall be a DS1 or higher rate transport interface at each of the SPOIs.  Each signaling link shall appear as a DS0 channel within the DS1 or higher rate interface.  BellSouth shall offer higher rate DS1 signaling links for interconnecting Cbeyond local switching systems or STPs with BellSouth STPs as soon as these become approved ANSI standards and available capabilities of BellSouth STPs.  BellSouth and Cbeyond will work jointly to establish mutually acceptable SPOI.

#### 16.3.3	BellSouth CO shall provide intraoffice diversity between the SPOIs and the BellSouth STP, so that no single failure of intraoffice facilities or equipment shall cause the failure of both B-links in a layer connecting to a BellSouth STP. BellSouth and Cbeyond will work jointly to establish mutually acceptable SPOI.

#### 16.3.4	The protocol interface requirements for SS7 Network Interconnection include the MTP, ISDNUP, SCCP, and TCAP. These protocol interfaces shall conform to the following specifications:

##### 16.3.4.1	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

##### 16.3.4.2	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service;

##### 16.3.4.3	Telcordia (formerly BellCore) GR-1429-CORE, CCS Network Interface Specification (CCSNIS) Supporting Call Management Services; and

##### 16.3.4.4	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

#### 16.3.5	BellSouth shall set message screening parameters to block accept messages from Cbeyond local or tandem switching systems destined to any signaling point in the BellSouth SS7 network with which the Cbeyond switching system has a legitimate signaling relation.

### 16.4	SS7 Network Interconnection shall be equal to or better than all of the requirements for SS7 Network Interconnection set forth in the following technical references:

#### 16.4.1	ANSI T1.110-1992 American National Standard Telecommunications - Signaling System Number 7 (SS7) - General Information;

#### 16.4.2	ANSI T1.111-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP);

#### 16.4.3	ANSI T1.111A-1994 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP) Supplement;

#### 16.4.4	ANSI T1.112-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Signaling Connection Control Part (SCCP);

#### 16.4.5	ANSI T1.113-1995 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Integrated Services Digital Network (ISDN) User Part;

#### 16.4.6	ANSI T1.114-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Transaction Capabilities Application Part (TCAP);

#### 16.4.7	ANSI T1.115-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Monitoring and Measurements for Networks;

#### 16.4.8	ANSI T1.116-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Operations, Maintenance and Administration Part (OMAP);

#### 16.4.9	ANSI T1.118-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Intermediate Signaling Network Identification (ISNI);

#### 16.4.10	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

#### 16.4.11	Telcordia (formerly BellCore) GR-954-CORE, CCS Network Interface Specification (CCSNIS) Supporting Line Information Database (LIDB) Service;

#### 16.4.12	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service;

#### 16.4.13	Telcordia (formerly BellCore) GR-1429-CORE, CCS Network Interface Specification (CCSNIS) Supporting Call Management Services; and,

#### 16.4.14	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

# 17.	Basic 911 and E911

If Cbeyond orders network elements and other services, then Cbeyond is also responsible for providing E911 to its end users.  BellSouth agrees to offer access to the 911/E911 network pursuant to the following terms and conditions set forth in this Attachment.

## 17.1	Definition

Basic 911 and E911 is an additional requirement that provides a caller access to the applicable emergency service bureau by dialing a 3-digit universal telephone number (911).

## 17.2	Requirements

### 17.2.1	Basic 911 Service Provisioning.  For Basic 911 service, BellSouth will provide to Cbeyond a list consisting of each municipality that subscribes to Basic 911 service.  The list will also provide, if known, the E911 conversion date for each municipality and, for network routing purposes, a ten-digit directory number representing the appropriate emergency answering position for each municipality subscribing to 911.  Cbeyond will be required to arrange to accept 911 calls from its end users in municipalities that subscribe to Basic 911 service and translate the 911 call to the appropriate 10-digit directory number as stated on the list provided by BellSouth.  Cbeyond will be required to route that call to BellSouth at the appropriate tandem or end office.  When a municipality converts to E911 service, Cbeyond will be required to discontinue the Basic 911 procedures and being using E911 procedures.

### 17.2.2	E911 Service Provisioning.  For E911 service, Cbeyond will be required to install a minimum of two dedicated trunks originating from the Cbeyond serving wire center and terminating to the appropriate E911 tandem.  The dedicated trunks shall be, at a minimum, DS-0 level trunks configured either as a 2-wire analog interface or as part of a digital (1.544 Mb/s) interface.  Either configuration shall use CAMA-type signaling with multifrequency (“MF”) pulsing that will deliver automatic number identification (“ANI”) with the voice portion of the call.  If the user interface is digital, MF pulses, as well as other AC signals, shall be encoded per the u-255 Law convention.  Cbeyond will be required to provide BellSouth daily updates to the E911 database.  Cbeyond will be required to forward 911 calls to the appropriate E911 tandem, along with ANI, based upon the current E911 end office to tandem homing arrangement as provided by BellSouth.  If the E911 tandem trunks are not available, Cbeyond will be required to route the call to a designated 7-digit local number residing in the appropriate Public Service Answering Point (“PSAP”).  This call will be transported over BellSouth’s interoffice network and will not carry the ANI of the calling party.  Cbeyond shall be responsible for providing BellSouth with complete and accurate data for submission to the 911/E911 database for the purpose of providing 911/E911 to its end users.

### 17.2.3	Rates.  Charges for 911/E911 service are borne by the municipality purchasing the service.  BellSouth will impose no charge on Cbeyond beyond applicable charges for BellSouth trunking arrangements.

### 17.2.4	Basic 911 and E911 functions provided to Cbeyond shall be at least at parity with the support and services that BellSouth provides to its end users for such similar functionality.

### Detailed Practices and Procedures.  The detailed practices and procedures contained in the E911 Local Exchange Carrier Guide For Facility-Based Providers as amended from time to time during the term of this Agreement will determine the appropriate practices and procedures for BellSouth and Cbeyond to follow in providing 911/E911 services.

# 18.	Rates

## 18.1.	General

The prices that Cbeyond shall pay to BellSouth for  Network Elements and Other Services are set forth in Exhibit C to this Attachment.  It is the intent of the parties that where applicable state commissions have approved rates for network elements and other services set forth in this Agreement as of the date hereof, such rates have been included in Exhibit C.

## 18.2.	Operational Support Systems (OSS)

BellSouth has developed and made available the following mechanized systems by which Cbeyond may submit LSRs electronically.

LENS		Local Exchange Navigation System

EDI		Electronic Data Interface

EDI-PC	Electronic Data Interface – Personal Computer

TAG		Telecommunications Access Gateway

LSRs submitted by means of one of these interactive interfaces will incur an OSS electronic ordering charge, as specified in the table below.  An individual LSR will be identified for billing purposes by its Purchase Order Number (PON).  LSRs submitted by means other than one of these interactive interfaces (mail, fax, courier, etc.) will incur a manual order charge as specified in the table below:

| ## OPERATIONAL SUPPORT SYSTEMS | AL, GA, LA, MS, SC | FL, KY, NC, TN |
| --- | --- | --- |
| OSS LSR charge, per LSR received from the CLEC by one of the OSS interactive interfaces | $3.50 SOMEC | $3.50 SOMEC |
| Incremental charge per LSR received from the CLEC by means other than one of the OSS interactive interfaces | See applicable rate element | $19.99 SOMAN |

Note:  In addition to the OSS charges, applicable discounted service order and related          discounted charges apply per the tariff.

Denial/Restoral OSS Charge

In the event Cbeyond provides a list of customers to be denied and restored, rather than an LSR, each location on the list will require a separate PON and, therefore will be billed as one LSR per location.

Cancellation OSS Charge

Cbeyond will incur an OSS charge for an accepted LSR that is later canceled by Cbeyond.

Note:  Supplements or clarifications to a previously billed LSR will not incur another OSS charge.

Network Elements and Other Services Manual Additive

The Commissions in Alabama, Georgia, Louisiana, Mississippi and South Carolina have ordered incremental manual non-recurring charges (NRC) for Network Elements and Other Services ordered by means other than one of the interactive interfaces.  These ordered Network Elements and Other Services manual additive NRCs will apply in these states, rather than the charge per LSR.

## 18.3	True-up

This section applies only to Tennessee and other interim rates listed in this attachment.

The interim prices for Network Elements and Other Services and Local Interconnection shall be subject to true-up according to the following procedures:

1.	The interim prices shall be trued-up, either up or down, based on final prices determined either by further agreement between the Parties, or by a final order (including any appeals) of the Commission which final order meets the criteria of (3) below.  The Parties shall implement the true-up by comparing the actual volumes and demand for each item, together with interim prices for each item, with the final prices determined for each item.  Each Party shall keep its own records upon which the true-up can be based, and any final payment from one Party to the other shall be in an amount agreed upon by the Parties based on such records.  In the event of any disagreement as between the records or the Parties regarding the amount of such true-up, the Parties agree that the body having jurisdiction over the matter shall be called upon to resolve such differences, or the Parties may mutually agree to submit the matter to the Dispute Resolution process in accordance with the provisions of Section 16 of the General Terms and Conditions and Attachment 1 of the Agreement.

2.	The Parties may continue to negotiate toward final prices, but in the event that no such Agreement is reached within nine (9) months, either Party may petition the Commission to resolve such disputes and to determine final prices for each item.  Alternatively, upon mutual agreement, the Parties may submit the matter to  the Dispute Resolution Process set forth in Section 16 of the General

Terms and Conditions and Attachment 1 of the Agreement, so long as they file the resulting Agreement with the Commission as a “negotiated Agreement” under Section 252(e) of the Act.

3.	A final order of this Commission that forms the basis of a true-up shall be the final order as to prices based on appropriate cost studies, or potentially may be a final order in any other Commission proceeding which meets the following criteria:

(a)	BellSouth and CLEC is entitled to be a full Party to the proceeding;

(b)	It shall apply the provisions of the federal Telecommunications Act of 1996, including but not limited to Section 252(d)(1) (which contains pricing standards) and all then-effective implementing rules and regulations; and,

(c)	It shall include as an issue the geographic deaveraging of network element and other services prices, which deaveraged prices, if any are required by said final order, shall form the basis of any true-up.

EXHIBIT A

LINE INFORMATION DATA BASE (LIDB)

STORAGE AGREEMENT

I.	SCOPE

A.	This Agreement sets forth the terms and conditions pursuant to which BST agrees to store in its LIDB certain information at the request of the Local Exchange Company and pursuant to which BST, its LIDB customers and Local Exchange Carrier shall have access to such information.  Local Exchange Carrier understands that BST provides access to information in its LIDB to various telecommunications service providers pursuant to applicable tariffs and agrees that information stored at the request of Local Exchange Carrier, pursuant to this Agreement, shall be available to those telecommunications service providers.  The terms and conditions contained in the attached Addendum(s)  are hereby made a part of this Agreement as if fully incorporated herein.

B.	LIDB is accessed for the following purposes:

1.	Billed Number Screening

2.	Calling Card Validation

3.	Fraud Control

C.	BST will provide seven days per week, 24-hours per day, fraud monitoring on Calling Cards, bill-to-third and collect calls made to numbers in BST’s LIDB, provided that such information is included in the LIDB query.  BST will establish fraud alert thresholds and will notify the Local Exchange Company of fraud alerts so that the Local Exchange Company may take action it deems appropriate. Local Exchange Company understands and agrees BST will administer all data stored in the LIDB, including the data provided by Local Exchange Company pursuant to this Agreement, in the same manner as BST’s data for BST’s end user customers.  BST shall not be responsible to Local Exchange Company for any lost revenue which may result from BST’s administration of the LIDB pursuant to its established practices and procedures as they exist and as they may be changed by BST in its sole discretion from time to time.

Local Exchange Company understands that BST currently has in effect numerous billing and collection agreements with various interexchange carriers and billing clearing houses.  Local Exchange Company further understands that these billing and collection customers of BST query BST’s LIDB to determine whether to accept various billing options from end users.  Additionally, Local Exchange Company understands that presently BST has no method to differentiate between BST’s own billing and line data in the LIDB and such data which it includes in the LIDB on Local Exchange Company’s behalf pursuant to this Agreement.  Therefore, until such time as BST can and does implement in its LIDB and its supporting systems the means to differentiate Local Exchange Company’s data from BST’s data and the Parties to this Agreement execute appropriate amendments hereto, the following terms and conditions shall apply:

(a)	The Local Exchange Company agrees that it will accept responsibility for telecommunications services billed by BST for its billing and collection customers for Local Exchange Customer’s end user accounts which are resident in LIDB pursuant to this Agreement.  Local Exchange Company authorizes BST to place such charges on Local Exchange Company’s bill from BST and agrees that it shall pay all such charges.  Charges for which Local Exchange Company hereby takes responsibility include, but are not limited to, collect and third number calls.

(b)	Charges for such services shall appear on a separate BST bill page identified with the name of the entity for which BST is billing the charge.

(c)	Local Exchange Company shall have the responsibility to render a billing statement to its end users for these charges, but Local Exchange Company’s obligation to pay BST for the charges billed shall be independent of whether Local Exchange Company is able or not to collect from the Local Exchange Company’s end users.

- BST shall not become involved in any disputes between Local Exchange Company and the entities for which BST performs billing and collection.  BellSouth will not issue adjustments for charges billed on behalf of an entity to Local Exchange Company.  It shall be the responsibility of the Local Exchange Company and the other entity to negotiate and arrange for any appropriate adjustments.

II.	TERM

This Agreement will be effective as of June 30, 2000 and will continue in effect for one year, and thereafter may be continued until terminated by either Party upon thirty (30) days written notice to the other Party.

III.	FEES FOR SERVICE AND TAXES

A.	The Local Exchange Company will not be charged a fee for storage services provided by BST to the Local Exchange Company, as described in Section I of this Agreement.

B.	Sales, use and all other taxes (excluding taxes on BST’s income) determined by BST or any taxing authority to be due to any federal, state or local taxing jurisdiction with respect to the provision of the service set forth herein will be paid by the Local Exchange Company.  The Local Exchange Company shall have the right to have BST contest with the imposing jurisdiction, at the Local Exchange Company’s expense, any such taxes that the Local Exchange Company deems are improperly levied.

IV.	INDEMNIFICATION

To the extent not prohibited by law, each Party will indemnify the other and hold the other harmless against any loss, cost, claim, injury, or liability relating to or arising out of negligence or willful misconduct by the indemnifying Party or its agents or contractors in connection with the indemnifying Party’s provision of services, provided, however, that any indemnity for any loss, cost, claim, injury or liability arising out of or relating to errors or omissions in the provision of services under this Agreement shall be limited as otherwise specified in this Agreement.  The indemnifying Party under this Section agrees to defend any suit brought against the other Party for any such loss, cost, claim, injury or liability.  The indemnified Party agrees to notify the other Party promptly, in writing, of any written claims, lawsuits, or demands for which the other Party is responsible under this Section and to cooperate in every reasonable way to facilitate defense or settlement of claims.  The indemnifying Party shall not be liable under this Section for settlement by the indemnified Party of any claim, lawsuit, or demand unless the defense of the claim, lawsuit, or demand has been tendered to it in writing and the indemnifying Party has unreasonably failed to assume such defense.

V.	LIMITATION OF LIABILITY

In the absence of gross negligence or willful misconduct, neither Party shall be liable to the other Party for any lost profits or revenues or for any indirect, incidental or consequential damages incurred by the other Party arising from this Agreement or the services performed or not performed hereunder, regardless of the cause of such loss or damage.

VI.	MISCELLANEOUS

A.  It is understood and agreed to by the Parties that BST may provide similar services to other companies.

B.	All terms, conditions and operations under this Agreement shall be performed in accordance with, and subject to, all applicable local, state or federal legal and regulatory tariffs, rulings, and other requirements of the federal courts, the U. S. Department of Justice and state and federal regulatory agencies.  Nothing in this Agreement shall be construed to cause either Party to violate any such legal or regulatory requirement and either Party’s obligation to perform shall be subject to all such requirements.

C.	The Local Exchange Company agrees to submit to BST all advertising, sales promotion, press releases, and other publicity matters relating to this Agreement wherein BST’s corporate or trade names, logos, trademarks or service marks or those of BST’s affiliated companies are mentioned or language from which the connection of said names or trademarks therewith may be inferred or implied; and the Local Exchange Company further agrees not to publish or use advertising, sales promotions, press releases, or publicity matters without BST’s prior written approval.

D.	This Agreement constitutes the entire Agreement between the Local Exchange Company and BST which supersedes all prior Agreements or contracts, oral or written representations, statements, negotiations, understandings, proposals and undertakings with respect to the subject matter hereof.

E.	Except as expressly provided in this Agreement, if any part of this Agreement is held or construed to be invalid or unenforceable, the validity of any other Section of this Agreement shall remain in full force and effect to the extent permissible or appropriate in furtherance of the intent of this Agreement.

F.	Neither Party shall be held liable for any delay or failure in performance of any part of this Agreement for any cause beyond its control and without its fault or negligence, such as acts of God, acts of civil or military authority, government regulations, embargoes, epidemics, war, terrorist acts, riots, insurrections, fires, explosions, earthquakes, nuclear accidents, floods, strikes, power blackouts, volcanic action, other major environmental disturbances, unusually severe weather conditions, inability to secure products or services of other persons or transportation facilities, or acts or omissions of transportation common carriers.

G.	This Agreement shall be deemed to be a contract made under the laws of the Cbeyond of Georgia, and the construction, interpretation and performance of this Agreement and all transactions hereunder shall be governed by the domestic law of such Cbeyond.

FACILITIES BASED ADDENDUM

TO LINE INFORMATION DATA BASE (LIDB)

STORAGE AGREEMENT

This is a Facilities Based Addendum to the Line Information Data Base Storage Agreement dated June 30, 2000, between BellSouth Telecommunications, Inc. (“BST”), and TriVergent Communications, Inc. (“Local Exchange Company”), effective the 30th day of June, 2000.

I.	GENERAL

This Addendum sets forth the terms and conditions for Local Exchange Company’s provision of billing number information to BST for inclusion in BST’s LIDB.  BST will store in its LIDB the billing number information provided by Local Exchange Company, and BST will provide responses to on-line, call-by-call queries to this information for purposes specified in Section I.B. of the Agreement.

II.	DEFINITIONS

A.	Billing number - a number that the Local Exchange Company creates for the purpose of identifying an account liable for charges.  This number may be a line or a special billing number.

B.	Line number - a ten digit number that identifies a telephone line administered by the Local Exchange Company.

C.	Special billing number - a ten digit number that identifies a billing account established by the Local Exchange Company.

D.	Calling Card number - a billing number plus PIN number.

E.	PIN number - a four digit security code assigned by the Local Exchange Company which is added to a billing number to compose a fourteen digit calling card number.

F.	Toll billing exception indicator - associated with a billing number to indicate that it is considered invalid for billing of collect calls or third number calls or both, by the Local Exchange Company.

G.	Billed Number Screening - refers to the activity of determining whether a toll billing exception indicator is present for a particular billing number.

H.	Calling Card Validation - refers to the activity of determining whether a particular calling card number exists as stated or otherwise provided by a caller.

I.	Billing number information - information about billing number, Calling Card number and toll billing exception indicator provided to BST by the Local Exchange Company.

III.	RESPONSIBILITIES OF PARTIES

A.	The Local Exchange Company will  provide its billing number information to BST’s LIDB each business day by a method that has been mutually agreed upon by both Parties.

B.	BST will store in its LIDB the billing number information provided by the Local Exchange Company.  Under normal operating conditions, BST shall include the Local Exchange Company’s billing number information in its LIDB no later than two business days following BST’s receipt of such billing number information, provided that BST shall not be held responsible for any delay or failure in performance to the extent such delay or failure is caused by circumstances or conditions beyond BST’s reasonable control.  BST will store in its LIDB an unlimited volume of the Local Exchange Company’s working telephone numbers.

C.	BST will provide responses to on-line, call-by-call queries to the stored information for the specific purposes listed in the next paragraph.

D.	BST is authorized to use the billing number information provided by the Local Exchange Company to perform the following functions for authorized users on an on-line basis:

1.	Validate a 14 digit Calling Card number where the first 10 digits are a line number or special billing number assigned by the Local Exchange Company, and where the last four digits (PIN) are a security code assigned by the Local Exchange Company.

2.	Determine whether the Local Exchange Company or the subscriber has identified the billing number as one which should not be billed for collect or third number calls, or both.

E.	The Local Exchange Company will provide its own billing number information to BST for storage and to be used for Billed Number Screening and Calling Card Validation.  The Local Exchange Company will arrange and pay for transport of updates to BST.

- COMPLIANCE

Unless expressly authorized in writing by the Local Exchange Company, all billing number information provided pursuant to this Addendum shall be used for no purposes other than those set forth in this Addendum.

EXHIBIT B

CALLING NAME DELIVERY (CNAM) DATABASE SERVICES

# 1.00  DEFINITIONS

For the purpose of this Attachment, the following terms shall be defined as:

CALLING NAME DELIVERY DATABASE SERVICE (CNAM) - The ability to associate a name with the calling party number, allowing the end user subscriber (to which a call is being terminated) to view the calling party's name before the call is answered.  This service also provides Cbeyond the opportunity to load and store its subscriber names in the BellSouth CNAM SCPs.

CALLING PARTY NUMBER (CPN) - The number of the calling party that is delivered to the terminating switch using common channel signaling system 7 (CCS7) technology, and that is contained in the Initial Address Message (IAM) portion of the CCS7 call setup.

COMMON CHANNEL SIGNALING SYSTEM 7 (CCS7) - A network signaling

technology in which all signaling information between two or more nodes is transmitted over high-speed data links, rather than over voice circuits.

SERVICE CONTROL POINTs (SCPs) - The real-time data base systems that contain the names to be provided in response to queries received from CNAM SSPs.

SERVICE MANAGEMENT SYSTEM (SMS) - The main operations support system of CNAM DATABASE SERVICE.  CNAM records are loaded into the SMS, which in turn downloads into the CNAM SCP.

SERVICE SWITCHING POINTs (SSPs) - Features of computerized switches in the telephone network that determine that a terminating line has subscribed to CNAM service, and then communicate with CNAM SCPs in order to provide the name associated with the calling party number.

SUBSYSTEM NUMBER (SSN) - The address used in the Signaling Connection Control Part (SCCP) layer of the SS7 protocol to designate an application at an end signaling point.  A SSN for CNAM at the end office designates the CNAM application within the end office.  BellSouth uses the CNAM SSN of 232.

# ATTACHMENT

2.01   This Attachment  contains  the terms and conditions where BellSouth will provide to the Cbeyond access to the BellSouth CNAM SCP for query or record storage purposes.

2.02	Cbeyond shall submit to BellSouth a notice of its intent to access and utilize BellSouth CNAM Database Services pursuant to the terms and conditions of this Attachment.  Said notice shall be in writing, no less than 60 days prior to Cbeyond’s access to BellSouth’s CNAM Database Services and shall be addressed to Cbeyond’s Account Manager.

# 3.00  PHYSICAL CONNECTION AND COMPENSATION

  - BellSouth's provision of CNAM Database Services to  Cbeyond requires interconnection from Cbeyond to BellSouth CNAM Service Control Points (SCPs).  Such interconnections shall be established pursuant to Attachment 3 of this Agreement .  The appropriate charge for access to and use of the BellSouth CNAM Database service shall be as set forth in this Attachment.

  - In order to formulate a CNAM query to be sent to the BellSouth CNAM SCP, Cbeyond shall  provide its own CNAM SSP. Cbeyond’s CNAM SSPs must be compliant with TR-NWT-001188, "CLASS Calling Name Delivery Generic Requirements".

  - If Cbeyond elects to access the BellSouth CNAM SCP via a third party CCS7 transport provider, the third party CCS7 provider shall interconnect with the BellSouth CCS7 network according to BellSouth's Common Channel Signaling Interconnection Guidelines and Telcordia (formerly BellCore)'s CCS Network Interface Specification document, TR-TSV-000905.  In addition, the third party provider shall establish CCS7 interconnection at the BellSouth Local Signal Transfer Points (LSTPs) serving the BellSouth CNAM SCPs that Cbeyond desires to query.

  - Out-Of-Region Customers

If the customer queries the BellSouth CNAM SCP via a third party national SS7 transport provider, the third party SS7 provider shall interconnect with the BellSouth CCS7 network according to BellSouth's Common Channel Signaling Interconnection Guidelines and Telcordia’s (formerly BellCore’s) CCS Network Interface Specification document, TR-TSV-000905. In addition, the third party provider shall establish SS7 interconnection at one or more of the BellSouth Gateway Signal Transfer Points (STPs).  The payment of all costs associated with the transport of SS7 signals via a third party will be established by mutual agreement of the Parties in writing and shall, by this reference become an integral part of this Agreement.

# 4.00  CNAM RECORD INITIAL LOAD AND UPDATES

4.01	The mechanism to be used by Cbeyond for initial CNAM record load and/or updates shall be determined by mutual agreement.    The initial load and all updates shall be provided by Cbeyond in the BellSouth specified format and shall contain records for every working telephone number that can originate phone calls.  It is the responsibility of Cbeyond to provide accurate information to BellSouth on a current basis.

  - Updates to the SMS shall occur no less than once a week, reflect service order activity affecting either name or telephone number, and involve only record additions, deletions or changes.

4.03	Cbeyond CNAM records provided for storage in the BellSouth CNAM SCP shall be available, on a SCP query basis only, to all Parties querying the BellSouth CNAM SCP.  Further, CNAM service shall be provided by each Party consistent with state and/or federal regulation.