Attachment 2

Network Elements and Other Services

TABLE OF CONTENTS

1.	Introduction	3

2.	Unbundled Loops	4

3.	Integrated Digital Loop Carriers	9 9

4.	Network Interface Device	9 9

5.	Unbundled Loop Concentration (ULC) System	10

6.	Sub-loop Elements	11

7.	Local Switching	14

8.	Transport	27

10.	Operator Systems	35

11.	Signaling	40

12.	Signaling Transfer Points (STPs)	42

13.	Service Control Points/DataBases	46

13.8	Calling Name (CNAM) Database Service	51

14.	DARK FIBER	53

15.	SS7 Network Interconnection	54

16.	Basic 911 and E911	57

17.1.	General	64

17.2.	Operational Support Systems (OSS)	64

1.00  DEFINITIONS	72

2.0	ATTACHMENT	72

3.00  PHYSICAL CONNECTION AND COMPENSATION	73

4.00  CNAM RECORD INITIAL LOAD AND UPDATES	73

EXHIBIT A – LIDB STORAGE AGREEMENT……………………………………EXHIBIT A

EXHIBIT B – CNAM DATABASE SERVICES……………………………………EXHIBIT B

EXHIBIT C – RATES .………….…………………………………………………..EXHIBIT C

ACCESS TO NETWORK ELEMENTS AND OTHER SERVICES

# Introduction

1.1	Network Element is defined to mean a facility or equipment used in the provision of a telecommunications service.  Such term may include, but is not limited to, features, functions, and capabilities that are provided by means of such facility or equipment, including but not limited to, subscriber numbers, databases, signaling systems, and information sufficient for billing and collection or used in the transmission, routing, or other provision of a telecommunications service.  BellSouth offers access to the Network Elements, unbundled loops; network interface device; sub-loop elements; local switching; transport; tandem switching; operator systems; signaling; access to call-related databases; dark fiber as set forth in this Attachment.

### 1.2	BellSouth shall, upon request of Premiere, and to the extent technically feasible, provide to Premiere access to its network elements for the provision of Premiere’s telecommunications service.  If no rate is identified in the contract, the rate for the specific service or function will be as set forth in applicable BellSouth tariff or as negotiated by the Parties upon request by either Party.

### 1.3	Premiere may purchase network elements and other services from BellSouth for the purpose of combining such network elements in any manner Premiere chooses to provide telecommunication services to its intended users, including recreating existing BellSouth services.  With the exception of the sub-loop elements which are located outside of the central office, BellSouth shall deliver the network elements purchased by Premiere  for combining to the designated Premiere collocation space.  The network elements shall be provided as set forth in this Attachment.

### 1.4	BellSouth will provide the following combined  network elements for purchase by Premiere.  The rate of the following combined  network elements is the sum of the individual element prices as set forth in this Attachment.  Order Coordination as defined in Section 2 of Attachment 2 of this Agreement is available for each of these combinations:

- SL2 loop and cross connect

- Port and cross connect

- Port and cross connect and common (shared) transport

- Port and vertical features

- SL2 Loop with loop concentration

- Port and common (shared) transport

- SL2 Loop and LNP

### 1.5	BellSouth shall comply with the requirements as set forth in the technical references within Attachment 2 to the extent that they are consistent with the greater of BellSouth’s actual performance or applicable industry standards.

1.6	In the event that any final and nonappealable legislative, regulatory, judicial or other legal action modifies or redefines the “Network Elements” in a manner which materially affects the terms of this Attachment or the Network Elements and/or prices set forth herein, either Party may, on thirty (30) days written notice, require renegotiation of such terms, and the Parties shall renegotiate in good faith such new terms in accordance with such legislative, regulatory, judicial or other legal action.  In the event such new terms are not renegotiated within ninety (90) days after the notice for renegotiation, either Party may petition the Commission for resolution of the dispute between the Parties.  Each Party reserves the right to seek judicial review of any Commission ruling concerning this Attachment.

1.7	Premiere  will adopt and adhere to the reasonable and nondiscriminatory standards contained in the applicable CLEC Work Center Operational Understanding Agreement regarding maintenance and installation of service.

1.8	BellSouth will provide reasonable and nondiscriminatory access to Network Elements on an unbundled basis, pursuant to the terms, conditions and rates set forth in this attachment, and in accordance with all effective rules and decisions of the FCC and the Commission.

# 2.	Unbundled Loops

### 2.1.1	BellSouth shall provide nondiscriminatory access, in accordance with FCC Rule 51.311 and Section 251(c)(3) of the Act, to the local loop on an unbundled basis pursuant to the following terms and conditions and at the rates approved by the Commission and set forth in this Attachment.

## 2.2	Definition

### 2.2.1	The local loop network element is defined as a transmission facility between a distribution frame (or its equivalent) in an incumbent LEC central office and the loop demarcation point at an end-user customer premises, including inside wire owned by the incumbent LEC.  The local loop network element includes all features, functions, and capabilities of such transmission facility.  Those features, functions, and capabilities include, but are not limited to, Dark Fiber as described in Section 14 of this Attachment, attached electronics (except those electronics used for the provision of advanced services, such as Digital Subscriber Line Access Multiplexers), and loop conditioning.  The local loop includes, but is not limited to, DSl, DS3, fiber, and other high capacity loops.  Unless otherwise requested, all loops will be provisioned with a Network Interface Device (“NID”).

### 2.2.2	The provisioning of service to a CLEC will require cross-office cabling and cross-connections within the central office to connect the loop to a local switch or to other transmission equipment in collocation space.  These cross-connects are a separate element and are not considered a part of the loop.

BellSouth Order Coordination referenced in Attachment 2 includes two types:  “Order Coordination” and “Order Coordination - Time Specific.”

“Order Coordination” refers to standard BellSouth service order coordination involving SL2 voice loops and all digital loops.  Order coordination for physical conversions will be scheduled at BellSouth’s discretion during normal working hours on the committed due date and Premiere advised.

“Order Coordination – Time Specific” refers to service order coordination in which Premiere  requests a specific time for a service order conversion to take place.  Loops on a single service order of 14 or more loops will be provisioned on a project basis.  This is a chargeable option for any coordinated order and is billed in addition to the OC charge.  Premiere may specify a time between 8:00 a.m. and 5:00 p.m. (location time) Monday through Friday (excluding holidays).  If Premiere specifies a time outside this window, or selects a time or quantity of loops that requires BellSouth technicians to work outside normal work hours, overtime charges will apply in addition to the OC and OC-TS charges.  Overtime charges will be applied according to actual costs based on type of force group required to perform the work, overtime hours worked and any special circumstances.

Where facilities are available, BellSouth will install  loops within a 5-7 business days interval.  For orders of 14 or more  loops, the installation will be handled on a project basis and the intervals will be set by the BellSouth project manager for that order.  Some  loops require a Service Inquiry (SI) to determine if facilities are available prior to issuing the order.  The interval for the SI process is 3-5 business days and is separate from the installation interval.  For expedite requests by Premiere, expedite charges will apply for intervals less than 5 days.  The charges outlined in BST’s FCC # 1 Tariff, Section 5.1.1, will apply.  If Premiere cancels an order for network elements and other services, any costs incurred by BellSouth in conjunction with the provisioning of that order will be recovered in accordance with FCC #1 Tariff, Section 5.4.

If Premiere modifies an order after being sent a Firm Order Confirmation (FOC) from BellSouth, any costs incurred by BellSouth to accommodate the modification will be reimbursed by Premiere.

2.2.3	Intervals for loop conversions shall be as follows:  (1) for single loop conversions per location, the conversion shall be completed within fifteen (15) minutes; (2) for up to ten (10) loop conversions per location, the conversion of all loops shall be completed within sixty (60) minutes, and each individual loop conversion shall be completed within fifteen (15) minutes; (3) for loop conversions not exceeding thirty (30) loops per location and not determined complex or exceptionally large, the conversion of all loops shall be completed within one hundred and twenty (120) minutes: and (4) all loops above a thirty loop quantity, or ten (10) loop quantity and determined as complex (a cut that requires more operation than a single cut point), will be negotiated by Premiere and BellSouth prior to the due date.

### 2.2.4	BellSouth will offer Unbundled Voice Loops (UVL) in two different service levels - Service Level One (SL1) and Service Level Two (SL2).  SL1 loops will be non-designed, will not have test points, and will not come with any Order Coordination (OC) or engineering information/circuit make-up data.  Upon issuance of an order in the service order system, SL1 loops will be activated on the due date in the same manner and time frames that BellSouth normally activates POTS-type loops for its customers.  If Premiere requests work to be done for SL1s that requires BellSouth technicians to work outside normal work hours, overtime charges will be applied according to actual costs based on type of force group required to perform the work, overtime hours worked and any special circumstances.

### SL2 loops shall have test points, will be designed with a Design Layout Record provided to Premiere, and will be provided with Order Coordination.  The OC feature will allow Premiere to coordinate the installation of the loop with the disconnect of an existing customer’s service and/or number portability service.  In these cases, BellSouth will perform the order conversion with standard order coordination at its discretion during normal work hours.

### 2.2.5	BellSouth will also offer Unbundled Digital Loops (UDL).  They will be designed, will be provisioned with test points (where appropriate), and will come standard with Order Coordination and a Design Layout Record (DLR).

### 2.2.6	As a chargeable option on all  loops except UVL-SL1, BellSouth will offer Order Coordination - Time Specific (OC-TS).  This will allow Premiere the ability to specify the time that the coordinated conversion takes place.  The OC-TS charge for orders due on the same day at the same location will be applied on a per Local Service Request (LSR) basis.

### 2.2.7	Premiere will be responsible for testing and isolating troubles on the  loops.  Once Premiere has isolated a trouble to the BellSouth provided loop, Premiere will issue a trouble to BellSouth on the loop.  BellSouth will take the actions necessary to repair the loop if a trouble actually exists.  BellSouth will repair these loops in the same time frames that BellSouth repairs similarly situated loops to its customers.

### 2.2.8	If Premiere reports a trouble on SL1 loops and no trouble actually exists, BellSouth will charge Premiere for any dispatching and testing (both inside and outside the CO) required by BellSouth in order to confirm the loop’s working status.

### 2.2.9	If Premiere reports a trouble on SL2 loops and no trouble actually exists, BellSouth will charge Premiere for any dispatching and testing, (outside the CO) required by BellSouth in order to confirm the loop’s working status.

## 2.4	Technical Requirements

### 2.4.1	To the extent available within BST’s Network at a particular location, BellSouth will offer loops capable of supporting telecommunications services such as: POTS, Centrex, basic rate ISDN, analog PBX, voice grade private line, ADSL, HDSL, DS1 and digital data (up to 64 kb/s).  Additional services may include digital PBXs, primary rate ISDN, xDSL, and Nx 64 kb/s.  If a requested loop type is not available, then the CLEC can use the Special Construction process to request that BellSouth place facilities or otherwise modify facilities in order to meet the CLEC’s request.

### 2.4.1.1	The loop will support the transmission, signaling, performance and interface requirements of the services described in 2.3.1 above.  It is recognized that the requirements of different services are different, and that a number of types or grades of loops are required to support these services.  Services provided over the loop by Premiere will be consistent with industry standards and BST’s TR73600.

#### 2.4.1.2	In some instances, Premiere will require access to a copper twisted pair loop unfettered by any intervening equipment (e.g., filters, load coils, range extenders, etc.), so that Premiere can use the loop for a variety of services by attaching appropriate terminal equipment at the ends.  Premiere will determine the type of service that will be provided over the loop.  In some cases, Premiere may be required to pay additional charges for the removal of certain types of equipment.  BellSouth’s Special Construction process will be used to determine the costs and feasibility of these activities.

In cases in which Premiere has requested that BellSouth remove equipment from the BellSouth loop, BellSouth will no longer be expected to maintain and repair the loop to the standards specified for that loop type in the TR73600 and other standards referenced in this Agreement.

Premiere, in performance of its obligations pursuant to the preceding Section, shall maintain records that will reflect that pursuant to Premiere’s request BellSouth has removed certain equipment from BellSouth provided loops and as such the loop may not perform within the technical specifications associated with that loop type.  Premiere will not report to BellSouth troubles on said loops where the loops are not performing within the technical specifications of that loop type.

In addition, Premiere recognizes there may be instances where a loop modified in this manner may be subjected to normal network configuration changes that may cause the circuit characteristics to be changed and may create an outage of the service that Premiere has placed on the loop.  If this occurs, BellSouth will work cooperatively with Premiere to restore the circuit to its previous modified status as quickly as possible.  Premiere will pay the Time and Materials costs associated with BellSouth’s work efforts needed to bring the loop back to its previous modified status.

### 2.4.2	The loop shall be provided to Premiere in accordance with the following Technical References:

BellSouth’s TR73600, Unbundled Local Loop Technical Specification

#### 2.4.2.1	Telcordia (formerly BellCore) TR-NWT-000057, Functional Criteria for Digital Loop Carrier Systems, Issue 2, January 1993.

#### 2.4.2.2	Telcordia (formerly BellCore) TR-NWT-000393, Generic Requirements for ISDN Basic Access Digital Subscriber Lines.

#### 2.4.2.3	ANSI T1.102 - 1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces.

#### 2.4.2.4	ANSI T1.403 - 1989, American National Standard for Telecommunications - Carrier to Customer Installation, DS1 Metallic Interface Specification.

2.5	Unbundled Loop Modifications (Line Conditioning)

2.5.1	Subject to applicable and effective FCC rules and orders, BellSouth shall condition loops, as requested by Premiere, whether or not BellSouth offers advanced services to the End User on that loop.

    - Loop conditioning is defined as the removal from the loop of any devices that may diminish the capability of the loop to deliver high-speed switched wireline telecommunications capability, including xDSL service.  Such devices include, but are not limited to, load coils, bridge taps, low pass filters, and range extenders.

    - The Unbundled Loop Modifications (ULM) offering provides the following elements: 1) removal of equipment on loops equal to or less than 18kft; 2) removal of equipment of loops longer than 18kft; and 3) removal of bridged-taps on loops of any length.

    - BellSouth shall recover the cost of line conditioning requested by Premiere through a recurring charge and/or nonrecurring charge(s) in accordance with the FCC’s forward-looking pricing principles promulgated pursuant to Section 252 (d) (1) of the Act and in compliance with FCC Rule 52.507 (e).

# 3.	Integrated Digital Loop Carriers

### 3.1.1	Where BellSouth uses Integrated Digital Loop Carrier (IDLC) systems to provide the local loop and BellSouth has a suitable alternate facility available, BellSouth will make arrangements to permit Premiere to order a contiguous  local loop.  To the extent it is technically feasible, these arrangements will provide Premiere with the capability to serve end users at a level that is at parity with the level of service BellSouth provides its customers.  If no alternate facility is available, BellSouth will utilize its Special Construction (SC)  process to determine the additional costs required to provision the loop facilities.  Premiere will then have the option of paying the one-time SC rates to place the loop facilities or Premiere may chose some other method of providing service to the end-user (e.g., Resale, private facilities, etc.).

# 4.	Network Interface Device

## 4.1	Definition

### 4.1.1	The NID is defined as any means of interconnection of end-user customer premises wiring to BellSouth’s distribution plant, such as a cross connect device used for that purpose.  BellSouth shall permit Premiere to connect its own loop facilities to on-premises wiring through BellSouth’s network interface device, or at any other technically feasible point.

## 4.2	Technical Requirements

### 4.2.1	The Network Interface Device shall provide a clean, accessible point of connection for the inside wiring and for the Distribution Media and shall maintain a connection to ground that meets the requirements set forth below.

### 4.2.2	The NID shall be capable of transferring electrical analog or digital signals between the customer’s inside wiring and the Distribution Media.

### 4.2.3	All NID posts or connecting points shall be in place, secure, usable and free of any rust or corrosion.  The protective ground connection shall exist and be properly installed.  The ground wire will also be free of rust or corrosion and have continuity relative to ground.

### 4.2.4	The NID shall be capable of withstanding all normal local environmental variations.

### 4.2.5	Where feasible, the NID shall be physically accessible to Premiere designated personnel.  In cases where entrance to the end user’s premises is required to give access to the NID, Premiere shall obtain entrance permission directly from the end user.

### 4.2.6	BellSouth shall offer the NID as a stand-alone component.  Additionally, Premiere may connect its loop to any spare capacity on the BellSouth NID. Where necessary to comply with an effective Commission order, BellSouth will allow  Premiere to disconnect the BellSouth loop from the BellSouth NID in order to connect Premiere’s loop to the BellSouth NID.  In these cases, Premiere accepts all liability associated with this process and it is Premiere’s responsibility to make sure the disconnected BellSouth loop is properly grounded.

## 4.3	Interface Requirements

### 4.3.1	The NID shall be equal to or better than all of the requirements for NIDs set forth in the following technical references:

#### 4.3.1.1	Telcordia (formerly BellCore) Technical Advisory TA-TSY-000120 “Customer Premises or Network Ground Wire”;

#### 4.3.1.2	Telcordia (formerly BellCore) Generic Requirement GR-49-CORE “Generic Requirements for Outdoor Telephone Network Interface Devices”;

#### 4.3.1.3	Telcordia (formerly BellCore) Technical Requirement TR-NWT-00239 “Indoor Telephone Network Interfaces”;

#### 4.3.1.4	Telcordia (formerly BellCore) Technical Requirement TR-NWT-000937 “Generic Requirements for Outdoor and Indoor Building Entrance”.

# 5.	Unbundled Loop Concentration (ULC) System

### 5.1.1	BellSouth will provide to Premiere  loop concentration (ULC).  Loop concentration systems in the central office concentrate the signals transmitted over local loops onto a digital loop carrier system.  The concentration device is placed inside a BellSouth central office. BellSouth will offer ULC with a TR008 interface or a TR303 interface.

### 5.1.2	ULC will be offered in two sizes.  System A will allow up to 96 BellSouth loops to be concentrated onto multiple DS1s.  The high speed connection from the concentrator will be at the electrical DS1 level and may connect to Premiere at Premiere’s collocation site.  System B will allow up to 192 BellSouth loops to be concentrated onto multiple DS1s.  System A may be upgraded to a System B.  A minimum of two DS1s is required for each system (i.e., System A requires two DS1s and System B would require an additional two DS1s or four in total ).  All DS1 interfaces will terminate to the CLEC’s collocation space.  ULC service is offered with or without concentration and with or without protection. A Line Interface element will be required for each  loop that is terminated onto the ULC system.  Rates for ULC are as set forth in this Attachment.

# 6.	Sub-loop Elements

## 6.1	Where facilities permit and subject to applicable and effective FCC rules and orders, BellSouth shall offer access to its Unbundled Sub Loop (USL), Unbundled Sub Loop Concentration (USLC) System and Unbundled Network Terminating Wire (UNTW) elements. BellSouth shall provide nondiscriminatory access, in accordance with § 51.311 and section 251(c)(3) of the Act, to the subloop on an unbundled basis pursuant to the following terms and conditions and at the rates set forth in this Attachment.

## 6.2	Unbundled Sub-Loop (USL)

### 6.2.1	Definition

#### 6.2.1.1	The subloop network element is defined as any portion of the loop that is technically feasible to access at terminals in BellSouth’s outside plant, including inside wire owned and controlled by BellSouth, if any.  An accessible terminal is any point on the loop where technicians can access the wire or fiber within the cable without removing a splice case to reach the wire or fiber within.  Such points may include, but are not limited to, the pole or pedestal, the network interface device, the minimum point of entry, the single point of interconnection, the main distribution frame, the remote terminal, and the feeder/distribution interface.

#### 6.2.1.2	Technical feasibility.  Subject to applicable and effective FCC rules and orders, if the Parties are unable to reach agreement, pursuant to voluntary negotiations, as to whether it is technically feasible, or whether sufficient space is available, to unbundle the subloop at the point where a carrier requests, BellSouth shall have the burden of demonstrating to the Commission, pursuant to state arbitration proceedings under section 252 of the Act, that there is not sufficient space available, or that it is not technically feasible, to unbundle the subloop at the point requested.

#### 6.2.1.3.	Best practices.  Once any state commission has determined that it is technically feasible to unbundle subloops at a designated point, BellSouth shall have the burden of demonstrating, pursuant to state arbitration proceedings under section 252 of the Act, that it is not technically feasible, or that sufficient space is not available, to unbundle its own loops at such a point.

#### 6.2.1.4.	Subloop access via collocation.  Where requested by Premiere, BellSouth shall provide access to the subloop in accordance with the FCC’s collocation rules, 47 C.F.R. §§ 51.321-323.

#### 6.2.1.5.	Single point of interconnection.  Subject to applicable and effective FCC rules and orders, BellSouth shall provide a single point of interconnection at multi-unit premises that is suitable for use by multiple carriers.  This obligation is in addition to BellSouth’s obligation to provide nondiscriminatory access to subloops at any technically feasible point.  If the Parties are unable to negotiate terms and conditions regarding a single point of interconnection, issues in dispute, including compensation due BellSouth under forward-looking pricing principles, shall be resolved under the dispute resolution processes set forth in this Agreement.

### 6.2.2	Requirements for All Unbundled Sub-Loops

#### 6.2.2.1	Unbundled Sub-Loops shall be capable of carrying all signaling messages or tones needed to provide telecommunications services.

### Unbundled Sub-Loop shall support functions associated with provisioning, maintenance and testing of the Unbundled Sub-Loop.  In these scenarios, Premiere would be required to place a cross-box, remote terminal (RT), or other similar device and deliver a cable to the BellSouth remote terminal or cross-box.  This cable would be connected, by a BST technician, to a cross-connect panel within the BellSouth RT/cross-box.  Premiere’s cable pairs can then be connected to BST’s USL within the BST cross-box by the BST technician.

### 6.2.3	Interface Requirements

#### 6.2.3.1	Unbundled Sub-Loop shall be equal to or better than each of the applicable interface requirements set forth in the following technical references:

#### 6.2.3.2	Telcordia (formerly BellCore) TR-NWT-000049, “Generic Requirements for Outdoor Telephone Network Interface Devices,” Issued December 1,1994.

## 6.3	Unbundled Sub-Loop Concentration System (USLC)

## 6.3.1	 BellSouth will provide Premiere with the ability to concentrate its sub-loops onto multiple DS1s back to the BellSouth Central Office.  The DS1s will then be terminated into Premiere’s collocation space. TR-008 and TR303 interface standards are available.

### 6.3.2	USLC, using the Lucent Series 5 equipment, will be offered in two different systems.  System A will allow up to 96 of Premiere’s sub-loops to be concentrated onto multiple DS1s.  System B will allow an additional 96 of Premiere’s sub-loops to be concentrated onto multiple DS1s.  One System A may be supplemented with one System B and they both must be physically located in a single Series 5 dual channel bank.  A minimum of two DS1s is required for each system (i.e., System A requires two DS1s and System B would require an additional two DS1s or four in total).  The DS1 level facility that connects the RT site with the serving wire center is known as a Feeder Interface.  All DS1 Feeder Interfaces will terminate to the CLEC’s collocation space within the SWC that serves the RT where the CLEC’s sub-loops are connected.  USLC service is offered with or without concentration and with or without a protection DS1.

### 6.3.3	In these scenarios Premiere would be required to place a cross-box, remote terminal (RT), or other similar device and deliver a cable to the BellSouth remote terminal.  This cable would be connected, by a BellSouth technician, to a cross-connect panel within the BellSouth RT/cross-box and would allow Premiere’s sub-loops to then be placed on the ULSC and transported to their collocation space at a DS1 level.

## 6.4	Unbundled Network Terminating Wire (UNTW)

### 6.4.1	BellSouth will provide Premiere with access to its Unbundled Network Terminating Wire (UNTW) pursuant to the following terms and conditions at rates as set forth in this Attachment, and in a manner consistent with applicable and effective FCC rules and decisions, including, but not limited to 47 C.F.R § 51.319.

## 6.5	Definition

### UNTW is twisted copper wire that extends from BellSouth’s point-of-entry into a multi-dwelling unit (MDU) complex or multi-tenant unit (MTU) complex to the point of demarcation at the end-users location.  The UNTW will not include a Network Interface Device (NID).

6.6		Requirements

6.6.1 	BellSouth will offer spare pairs that are available to an end-users premises to Premiere. Available spare pairs are defined as pairs that are not being utilized by BellSouth or by a third party to provide an End User with working service at the time of Premiere’s request for UNTW. If BellSouth has relinquished the first pair to Premiere and the End User decides to change local service providers to BellSouth, Premiere will relinquish the first pair back to BellSouth.

6.6.2 	Notwithstanding the foregoing, should BellSouth subsequently require the use of additional pair(s) to provide for the activation of additional lines in an End User’s premises in response to a request from such End User and no additional pairs are available, Premiere agrees to surrender its spare pair(s) upon request by BellSouth.

6.6.3	If an End User of Premiere desires to receive local exchange service from a service provider who is not a Party to this Agreement, and such third party service provider needs access to the BellSouth UNTW to provide local exchange service to the End User, then Premiere agrees to surrender the requisite number of its inactive spare pair(s) if no other spare pair is available and upon request by BellSouth.

6.6.4 	If Premiere has placed NTW at a location and an End User desires to receive local exchange service from BellSouth and BellSouth needs access to Premiere’s NTW to provide local exchange service to the End User, then Premiere agrees to surrender the requisite number of its spare pair(s) upon request by BellSouth.

6.6.5 	In new construction, where possible, both Parties may at their option and with the property owner’s agreement install their own NTW. In existing construction, BellSouth shall not be required to install new or additional NTW beyond existing NTW to provision the services of Premiere.

## 6.7	Technical Requirements

### 6.7.1	In these scenarios, BellSouth will connect the requested UNTW pairs to a cross-connect panel designed for CLEC access to BellSouth’s NTW.  Premiere will be required to place a cross-box, terminal, or other similar device and deliver a cable to this cross-connect panel.  Premiere will then connect their cable to the cross-connect panel to access the requested UNTW pairs.

## 7.0		Preordering Loop Makeup (LMU)

7.1		Description of Service

7.1.1	BellSouth shall make available to Premiere loop makeup (LMU) data for BellSouth's network facilities.  This section addresses LMU as a preordering transaction, distinct from Premiere ordering any other service(s).  Loop Makeup Service Inquiries (LMUSI) for preordering loop makeup are likewise unique from other preordering functions with associated service inquiries (SI) as described in this Agreement.

    - BellSouth will provide Premiere with loop makeup information consisting of the composition of the loop material (copper/fiber); the existence, location and type of equipment on the loop, including but not limited to digital loop carrier or other remote concentration devises, feeder/distribution interfaces, bridge taps, load coils, pair-gain devices; the loop length; and the wire gauge.  The LMUSI may be utilized by Premiere for the purpose of determining whether the loop requested is capable of supporting DSL service or other advanced data services.  The determination shall be made solely by Premiere and BellSouth shall not be liable in any way for the performance of the advanced data services provisioned over said loop.

    - BellSouth’s LMU information is provided to Premiere as it exists either in BellSouth's databases or in its hard copy facility records.  BellSouth does not guarantee accuracy or reliability of the LMU information provided.

    - BellSouth offers LMU information for the sole purpose of allowing Premiere to determine whether, in Premiere’s judgment, BellSouth's loops will support the specific services that Premiere wishes to provide over those loops. Premiere may choose to use equipment that it deems will enable it to provide a certain type and level of service over a particular BellSouth loop; however, such configurations may not match BellSouth's or the industry’s standards and specifications for the intended type and level of service.  Accordingly, Premiere shall be responsible for insuring that the specific loop type (ADSL, HDSL, or otherwise) ordered on the LSR matches the LMU of the facility requested.  Premiere bears full responsibility for being knowledgeable of BellSouth's technical standards and the specifications of BellSouth's loops.  Premiere bears full responsibility for making the appropriate ordering decisions of matching BellSouth loops with Premiere's equipment for accomplishing Premiere's end goal for the intended service it wishes to provide its end-user(s).   Premiere is fully responsible for any of its service configurations that may differ from BellSouth's technical standard for the loop type ordered.

  - Submitting Loop Makeup Service Inquiries

#### Premiere will be able to obtain LMU information by submitting a LMUSI mechanically or manually.  Mechanized LMUSIs should be submitted through BellSouth's Operational Support Systems interfaces.   After obtaining the resulting loop data from the mechanized LMUSI process, if Premiere determines that it needs further loop data information in order to make a determination of loop service capability, Premiere may initiate a separate manual SI for a separate nonrecurring charge as set forth in Section 7.2.3.  Mechanized LMU has been made available for limited deployment to those CLECs that have effective X-Digital Subscriber Line (xDSL) Beta Test Agreements in place with BellSouth.  CLECs will be notified once a successful Beta Test has been completed, and mechanized LMU shall then be available to Premiere.

    - Manual LMUSIs shall be submitted on the preordering manual LMUSI form by means of  fax or electronic-mail to BellSouth’s Complex Resale Support Group (CRSG)/Account Team utilizing the Preordering Loop Makeup Service Inquiry form.  The standard service interval for the return of a Loop Makeup Manual Service Inquiry is seven business days.  This service interval is distinct from the interval applied to the subsequent service order.  Manual LMUSIs are not subject to expedite requests.

    - LMUSI Types and Associated Charges

Premiere may request LMU information by submitting LMUSIs in accordance with the rate elements in Exhibit C.

      - Premiere will be assessed a nonrecurring charge for each facility queried as specified in Exhibit C.  Rates for all states are interim and subject to true-up pending approval of final rates by the respective State Commissions.  True-ups will be retroactive to the effective date of this Agreement. Premiere may reserve facilities for up to four (4) days in connection with a LMUSI.  Reserved facilities for which Premiere does not plan to place a UNE local service request (LSR) should be cancelled by Premiere.  Should Premiere wish to cancel a reservation on a spare facility, the cancellation will require a facility reservation number (RESID/FRN).

      - The reservation holding timeframe is a maximum of four days from the time that BellSouth's LMU data is returned to Premiere for the facility queried.  During this holding time and prior to Premiere’s placing an LSR, the reserved facilities are rendered unavailable to other customers, whether for CLEC(s) or for BellSouth.  Notwithstanding the foregoing, BellSouth does not guarantee that a reservation will assure Premiere’s ability to order the exact facility reserved.

      - If Premiere does not submit an LSR for a UNE service on a reserved facility within the four-day reservation timeframe, the reservation of that spare facility will become invalid and the facility will be released.

      - Charges for preordering LMUSI are separate from any charges associated with ordering other services from BellSouth.

    - Ordering of Other UNE Services

      - Whenever Premiere has reserved a facility through BellSouth's preordering LMU service, should Premiere seek to place a subsequent UNE LSR on a reserved facility, Premiere shall provide BellSouth the RESID/FRN of the single spare facility on the appropriate UNE LSR., Premiere will be billed the appropriate rate element for the specific type UNE loop ordered by Premiere as set forth in this Attachment. Premiere will not be billed any additional Loop Makeup charges for the loop so ordered.  Should Premiere choose to place a UNE LSR having previously submitted a request for preordering LMU without a reservation, Premiere will be billed the appropriate rate element for the specific UNE loop ordered as well as additional Loop Makeup charges as set forth in this Attachment.  Rates are provided in Exhibit C in this Attachment.

      - Where Premiere submits an LSR to order facilities reserved during the LMUSI process, BellSouth will use its best efforts to assign to Premiere the facility reserved as indicated on the return of the LMU.  Multi-facility reservations per single RESID/FRN as provided with the mechanized LMUSI process are less likely to result in the specific assignment requested by Premiere.  For those occasions when BellSouth cannot assign the specific facility reserved by Premiere during the LMU pre-ordering transaction, due to incomplete or incorrect information provided by Premiere during the ordering process, BellSouth will assign to Premiere, subject to availability, a facility that meets the BellSouth technical standards of the BellSouth type loop as ordered by Premiere.  If the ordered loop type is not available, Premiere may utilize the Unbundled Loop Modification process or the Special Construction process, as applicable, to obtain the loop type ordered.

7.3	Rates

The prices that Premiere shall pay to BellSouth for Network Elements and Other Services are set forth in Exhibit C to this Attachment.  If Premiere purchases a service(s) from a tariff, all terms and conditions and rates as set forth in such tariff shall apply.

## 7.4	Operational Support Systems (OSS)

The terms, conditions and rates for OSS are as set forth in Section 21.2 of this Attachment and the General Terms and Conditions of this Agreement.

# High Frequency Spectrum Network Element

  - General

    - BellSouth shall provide Premiere access to the high frequency portion of the local loop as an unbundled network element only where BellSouth is the voice service provider to the end user (“High Frequency Spectrum”) at the rates set forth in Exhibit C.  BellSouth shall provide Premiere with the High Frequency Spectrum irrespective of whether BellSouth chooses to offer xDSL services on the loop.

    - The High Frequency Spectrum is defined as the frequency range above the voiceband on a copper loop facility carrying analog circuit-switched voiceband transmissions. Access to the High Frequency Spectrum is intended to allow Premiere the ability to provide Digital Subscriber Line (“xDSL”) data services to the end user for which BellSouth provides voice services.  The High Frequency Spectrum shall be available for any version of xDSL presumed acceptable for deployment pursuant to 47 CFR Section 51.230, including, but not limited to, ADSL, HDSL, and any other xDSL technology that is presumed to be acceptable for deployment pursuant to FCC rules. BellSouth will continue to have access to the low frequency portion of the loop spectrum (from 300 Hertz to at least 3000 Hertz, and potentially up to 3400 Hertz, depending on equipment and facilities) for the purposes of providing voice service. Premiere shall only use xDSL technology that is within the PSD mask parameters set forth in T1.413 or other applicable industry standards.  Premiere shall provision xDSL service on the High Frequency Spectrum in accordance with the applicable Technical Specifications and Standards.

    - The following loop requirements are necessary for Premiere to be able to access the High Frequency Spectrum:  an unconditioned, 2-wire copper loop.  An unconditioned loop is a copper loop with no load coils, low-pass filters, range extenders, DAMLs, or similar devices and minimal bridged taps consistent with ANSI T1.413 and T1.601.  BellSouth will provide Premiere access to the Unbundled Loop Modification (Line Conditioning), in accordance with Section 2.5 of this Agreement.  BellSouth is not required to condition a loop for access to the high frequency spectrum if conditioning of that loop significantly degrades BellSouth’s voice service. If Premiere requests that BellSouth condition a loop longer than 18,000 ft. and such conditioning significantly degrades the voice services on the loop, Premiere shall pay for the loop to be restored to its original state.

    - Premiere’s termination point is the point of termination for Premiere on the toll main distributing frame in the central office (“Termination Point”).  BellSouth will use jumpers to connect Premiere’s connecting block to the splitter.  The splitter will route the High Frequency Spectrum on the circuit to Premiere’s xDSL equipment in Premiere’s collocation space.

    - Premiere shall have access to the splitter for test purposes, irrespective of where the splitter is placed in the BellSouth premises.

- 8.2		Provisioning of High Frequency Spectrum and Splitter Space

- 8.2.1	BellSouth will provide Premiere with access to the High Frequency Spectrum as follows:

- 8.2.1.1	BellSouth will install splitters within forty-two (42) calendar days of Premiere’s submission of such order to the BellSouth Complex Resale Support Group; provided, however, that in the event BellSouth did not have reasonable notice that a particular central office was to have a splitter installed therein, the forty-two (42) day interval shall not apply. Collocation itself or an application for collocation will serve as reasonable notice.

- 8.2.1.2	Once a splitter is installed on behalf of Premiere in a central office, Premiere shall be entitled to order the High Frequency Spectrum on lines served out of that central office.

- 8.2.1.3	BellSouth will bill and Premiere shall pay the SOMAN and SOMEC charges as described in the General Terms and Conditions of this Agreement when Premiere orders High Frequency Spectrum for end-user service.

- 8.2.1.4	BellSouth will select, purchase, install, and maintain a central office POTS splitter and provide Premiere access to data ports on the splitter. At least 30 days before making a change in splitter suppliers, BellSouth will provide Premiere with a carrier notification letter, informing Premiere of change.  Premiere shall purchase ports on the splitter as set forth more fully below.

- 8.2.1.5	BellSouth will install the splitter in (i) a common area close to the Premiere collocation area, if possible; or (ii) in a BellSouth relay rack as close to the Premiere DS0 termination point as possible. For purposes of this section, a common area is defined as an area in the central office in which both Parties have access to a common test access point.   BellSouth will cross-connect the splitter data ports to a specified Premiere DS0 at such time that a Premiere end user’s service is established.

- The High Frequency Spectrum shall only be available on loops on which BellSouth is also providing, and continues to provide, analog voice service directly to the end user.  In the event the end-user terminates its BellSouth provided voice service for any reason, and Premiere desires to continue providing xDSL service on such loop, Premiere shall be required to purchase a full stand-alone loop unbundled network element. In the event BellSouth disconnects the end-user’s voice service pursuant to its tariffs or applicable law, and Premiere desires to continue providing xDSL service on such loop, Premiere shall be permitted to continue using the line by purchasing the full stand-alone loop unbundled network element.  To the extent commercially practicable, BellSouth shall give Premiere notice in a reasonable time prior to disconnect, which notice shall give Premiere an adequate opportunity to notify BellSouth of its intent to purchase such loop.  In those cases in which BellSouth no longer provides voice service to the end user and Premiere purchases the full stand-alone loop, Premiere may elect the type of loop it will purchase.  Premiere will pay the appropriate recurring and non-recurring rates for such loop as set forth in Exhibit C to this Attachment.  In the event Premiere purchases a voice grade loop, Premiere acknowledges that such loop may not remain xDSL compatible.

- 8.2.1.6	Only one competitive local exchange carrier shall be permitted access to the High Frequency Spectrum of any particular loop.

8.3	Ordering

8.3.1	To order High Frequency Spectrum on a particular loop, Premiere must have a DSLAM collocated in the central office that serves the end-user of such loop. Premiere may order splitters in a central office once it has installed its Digital Subscriber Line Access Multiplexer (“DSLAM”) in that central office.   BellSouth will install these splitters within the interval provided in paragraph 8.2.1.1.

    - BellSouth will devise a splitter order form that allows Premiere to order splitter ports in increments of 24 ports.

      - BellSouth will provide Premiere the Local Service Request (“LSR”) format to be used when ordering the High Frequency Spectrum.

    - BellSouth will provide access to the High Frequency Spectrum within the following target intervals:  BellSouth will return a manual Firm Order Confirmation (“FOC”) in no more than two (2) business days after receipt of a valid, error free manual LSR.  When Premiere submits an electronic LSR for High Frequency Spectrum, BellSouth will return a FOC in four (4) hours ninety-five percent (95%) of the time, or, for orders that do not flow-through, in two (2) business days.  BellSouth will provide Premiere with access to the High Frequency Spectrum at the following target intervals:

      - For 1-5 lines at the same address within three (3) business days from BellSouth’s issuance of a FOC; 6-10 lines at same address within 5 business days from BellSouth’s issuance of a FOC; and more than 10 lines at the same address is to be negotiated.

    - BellSouth will provide to Premiere BellSouth’s Loop Qualification System that BellSouth uses to qualify loops for its own ADSL offering as described below.

    - BellSouth will provide Premiere access to the Preordering Loop Makeup (LMU), in accordance with Section 7.0 of this Agreement.  BellSouth shall bill and Premiere shall pay the rates for such services, as described in Exhibit C.

  - Maintenance and Repair

    - Premiere shall have access, for test, repair, and maintenance purposes, to any loop as to which it has access to the High Frequency Spectrum.  Premiere may access the loop at the point where the combined voice and data signal exits the central office splitter.

    - BellSouth will be responsible for repairing voice services and the physical line between the network interface device at the customer’s premises and the Termination Point of demarcation in the central office.  Premiere will be responsible for repairing data services.  Each Party will be responsible for maintaining its own equipment.

    - Premiere shall inform its end users to direct data problems to Premiere, unless both voice and data services are impaired, in which event the end users should call BellSouth.

    - Once a Party has isolated a trouble to the other Party’s portion of the loop, the Party isolating the trouble shall notify the end user that the trouble is on the other Party’s portion of the loop.

    - In the event Premiere’s deployment of xDSL on the High Frequency Spectrum significantly degrades the performance of other advanced services or of BellSouth’s voice service on the same loop, BellSouth shall notify Premiere and allow twenty-four (24) hours to cure the trouble.  If Premiere fails to resolve the trouble, BellSouth may discontinue Premiere’s access to the High Frequency Spectrum on such loop.

8.5	 Rates

The prices that Premiere shall pay to BellSouth for Network Elements and Other Services are set forth in Exhibit C to this Attachment.  If Premiere purchases a service(s) from a tariff, all terms and conditions and rates as set forth in such tariff shall apply.

## 8.6	Operational Support Systems (OSS)

The terms, conditions and rates for OSS are as set forth in Section 21.2 of this Attachment and the General Terms and Conditions of this Agreement.

# 9.	Switching

BellSouth agrees to offer access to local switching pursuant to the following terms and conditions and at the rates set forth in this Attachment.

## 9.1	Definition

## 9.1.1	Subject to applicable and effective FCC rules and orders, BellSouth shall provide nondiscriminatory access, in accordance with FCC Rule 51.311 and Section 251(c)(3) of the Act, to local circuit switching capability and local tandem switching capability on an unbundled basis, except as set forth in FCC Rule 51.319(c)(1)(B), to Premiere for the provision of a telecommunications service.  Subject to applicable and effective FCC rules and orders, BellSouth shall be required to provide nondiscriminatory access in accordance with FCC Rule 51.311 and Section 251(c)(3) of the Act to packet switching capability on an unbundled basis to Premiere for the provision of a telecommunications service as described in Rule 51.319(c)(3)(B).

## 9.1.1.1	Local Circuit Switching Capability, including Tandem Switching Capability.  The local circuit switching capability network element is defined as:

- Line-side facilities, which include, but are not limited to, the connection between a loop termination at a main distribution frame and a switch line card;

- Trunk-side facilities, which include, but are not limited to, the connection between trunk termination at a trunk-side cross-connect panel and a switch trunk card; and

- All features, functions and capabilities of the switch, which include, but are not limited to:

a.	The basic switching function of connecting lines to lines, lines to trunks, trunks to lines, and trunks to trunks, as well as the same basic capabilities made available to the incumbent LEC’s customers, such as a telephone number, white page listing and dial tone, and

b.	All other features that the switch is capable of providing, including but not limited to, customer calling, customer local area signaling service features, and Centrex, as well as any technically feasible customized routing functions provided by the switch.

9.1.1.2			Subject to applicable and effective FCC rules and orders, notwithstanding BellSouth’s general duty to unbundle local circuit switching, BellSouth shall not be required to unbundle local circuit switching for Premiere in cases where Premiere intends to use such facilities to serve End Users with four or more voice grade (DS0) equivalents or lines, BellSouth provides nondiscriminatory, unrestricted, cost-based access to the enhanced extended link (“EEL”) throughout Density Zone 1, and BellSouth’s local circuit switches are located in:

- The top 50 Metropolitan Statistical Areas as set forth in Appendix B of the Third Report and Order and Fourth Further Notice of Proposed Rulemaking in CC Docket No. 96-98, and

2.	Density Zone I, as defined in FCC Rule 69.123, as of January 1, 1999.

9.1.1.3			Local Tandem Switching Capability.  The tandem switching capability network element is defined as:

(A)	Trunk-connect facilities, which include, but are not limited to, the connection between trunk termination at a cross connect panel and switch trunk card;

(B)	The basic switch trunk function of connecting trunks to trunks; and

(C)	The functions that are centralized in tandem switches (as distinguished from separate end office switches), including but not limited, to call recording, the routing of calls to operator services, and signaling conversion features.

Tandem Switching is the function that establishes a communications path between two switching offices through a third switching office (the Tandem switch).

9.1.1.4	Packet Switching Capability.  The packet switching capability network element is defined as the basic packet switching function of routing or forwarding packets, frames, cells or other data units based on address or other routing information contained in the packets, frames, cells or other data units, and the functions that are performed by Digital Subscriber Line Access Multiplexers, including but not limited to:

- The ability to terminate copper customer loops (which includes both a low band voice channel and a high-band data channel, or solely a data channel);

- The ability to forward the voice channels, if present, to a circuit switch or multiple circuit switches;

- The ability to extract data units from the data channels on the loops, and

4.	The ability to combine data units from multiple loops onto one or more trunks connecting to a packet switch or packet switches.

9.1.1.5			Subject to applicable and effective FCC rules and orders, BellSouth shall provide nondiscriminatory access to unbundled packet switching capability only in cases where each of the following conditions are satisfied:

- BellSouth has deployed digital loop carrier systems, including but not limited to, integrated digital loop carrier or universal digital loop carrier systems; or has deployed any other system in which fiber optic facilities replace copper facilities in the distribution section (e.g., end office to remote terminal, pedestal or environmentally controlled vault);

- There are no spare copper loops capable of supporting the xDSL services the requesting carrier seeks to offer;

- BellSouth has not permitted a requesting carrier to deploy a Digital Subscriber Line Access Multiplexer at the remote terminal, pedestal or environmentally controlled vault or other interconnection point, nor has the requesting carrier obtained a virtual collocation arrangement at these subloop interconnection points as defined by FCC Rule 51.319(b); and

### 4.	BellSouth has deployed packet switching capability for its own use.

### 9.1.2	A featureless port is one that has a line port, switching functionality, and an interoffice port.  A featured port is a port that includes all features then capable or a number of then capable features specifically requested by Premiere.  Any features that are not currently then capable but are technically feasible through the switch can be requested through the BFR process.

### 9.1.3	Where required to do so in order to comply with an effective Commission order, BellSouth will provide to Premiere purchasing  local BellSouth switching and reselling BellSouth local exchange service under Attachment 1, selective routing of calls to a requested directory assistance services platform or operator services platform.  Premiere customers may use the same dialing arrangements as BellSouth customers, but obtain a Premiere branded service.

## 9.2	Technical Requirements

### 9.2.1	The requirements set forth in this Section apply to Local Switching, but not to the Packet Switching function of Local Switching.

#### 9.2.1.1	Local Switching shall be equal to or better than the requirements for Local Switching set forth in Telcordia (formerly BellCore)’s Local Switching Systems General Requirements (FR-NWT-000064).

#### 9.2.1.2	When applicable, BellSouth shall route calls to the appropriate trunk or lines for call origination or termination.

#### 9.2.1.3	Subject to this section, BellSouth shall route calls on a per line or per screening class basis to (1) BellSouth platforms providing Network Elements or additional requirements (2) Operator Services platforms, (3) Directory Assistance platforms, and (4) Repair Centers.  Any other routing requests by Premiere will be made pursuant to the Bona Fide Request/ New Business Request Process as set forth in General Terms and Conditions.

#### 9.2.1.4	BellSouth shall provide unbranded recorded announcements and call progress tones to alert callers of call progress and disposition.

#### 9.2.1.5	BellSouth shall activate service for an Premiere customer or network interconnection on any of the Local Switching interfaces.  This includes provisioning changes to change a customer from BellSouth’s services to Premiere’s services without loss of switch feature functionality as defined in this Agreement.

#### 9.2.1.6	BellSouth shall perform routine testing (e.g., Mechanized Loop Tests (MLT) and test calls such as 105, 107 and 108 type calls) and fault isolation on a mutually agreed upon schedule.

#### 9.2.1.7	BellSouth shall repair and restore any equipment or any other maintainable component that may adversely impact Local Switching.

#### 9.2.1.8	BellSouth shall control congestion points such as those caused by radio station call-ins, and network routing abnormalities.  All traffic shall be restricted in a non discriminatory manner.

#### 9.2.1.9	BellSouth shall perform manual call trace and permit customer originated call trace.

#### 9.2.1.10	Special Services provided by BellSouth will include the following:

##### 7.2.1.10.1	Telephone Service Prioritization;

##### 9.2.1.10.2	Related services for handicapped;

##### 9.2.1.10.3	Soft dial tone where required by law; and

##### 9.2.1.10.4	Any other service required by law.

#### 9.2.1.11	BellSouth shall provide Switching Service Point (SSP) capabilities and signaling software to interconnect the signaling links destined to the Signaling Transfer Point Switch (STP).  These capabilities shall adhere to Telcordia (formerly BellCore) specifications - TCAP (GR-1432-CORE), ISUP(GR-905-CORE), Call Management (GR-1429-CORE), Switched Fractional DS1 (GR-1357-CORE), Toll Free Service (GR-1428-CORE), Calling Name (GR-1597-CORE), Line Information Database (GR-954-CORE), and Advanced Intelligent Network (GR-2863-CORE).

#### 9.2.1.12	BellSouth shall provide interfaces to adjuncts through Telcordia (formerly BellCore) standard interfaces.  These adjuncts can include, but are not limited to, the Service Circuit Node and Automatic Call Distributors.

#### 9.2.1.13	BellSouth shall provide performance data regarding a customer line, traffic characteristics or other measurable elements to Premiere, upon a reasonable request from Premiere.  CLEC will pay BellSouth for all costs incurred to provide such performance data through the Business Opportunity Request process.

#### 9.2.1.14	BellSouth shall offer Local Switching that provides feature offerings at parity to those provided by BellSouth to itself or any other Party.  Such feature offerings shall include but are not limited to:

##### 9.2.1.14.1	Basic and primary rate ISDN;

##### 9.2.1.14.2	Residential features;

##### 9.2.1.14.3	Customer Local Area Signaling Services (CLASS/LASS);

##### 9.2.1.14.4	CENTREX (including equivalent administrative capabilities, such as customer accessible reconfiguration and detailed message recording); and

##### 9.2.1.14.5	Advanced intelligent network triggers supporting Premiere and BellSouth service applications.

BellSouth shall offer to Premiere all AIN triggers in connection with its SMS/SCE offering which are supported by BellSouth for offering AIN-based services. Triggers that are currently available are:

###### 9.2.1.14.5.1	Off-Hook Immediate

###### 9.2.1.14.5.2	Off-Hook Delay

###### 9.2.1.14.5.3	Termination Attempt

###### 9.2.1.14.5.4	6/10 Public Office Dialing Plan

###### 9.2.1.14.5.5	Feature Code Dialing

###### 9.2.1.14.5.6	Customer Dialing Plan

##### 9.2.1.14.6	When the following triggers are supported by BellSouth, BellSouth will make these triggers available to Premiere:

###### 9.2.1.14.6.1	Private EAMF Trunk

###### 9.2.1.14.6.2	Shared Interoffice Trunk (EAMF, SS7)

###### 9.2.1.14.6.3	N11

###### 9.2.1.14.6.4	Automatic Route Selection

#### 9.2.1.15	Where capacity exists, BellSouth shall assign each Premiere customer line the class of service designated by Premiere (e.g., using line class codes or other switch specific provisioning methods), and shall route directory assistance calls from Premiere customers to Premiere directory assistance operators at Premiere’s option.

#### 9.2.1.16	Where capacity exists, BellSouth shall assign each Premiere customer line the class of services designated by Premiere (e.g., using line class codes or other switch specific provisioning methods) and shall route operator calls from Premiere customers to Premiere operators at Premiere’s option.  For example, BellSouth may translate 0- and 0+ intraLATA traffic, and route the call through appropriate trunks to an Premiere Operator Services Position System (OSPS).  Calls from Local Switching must pass the ANI-II digits unchanged.

#### 9.2.1.17	Local Switching shall be offered in accordance with the requirements of the following technical references:

##### 9.2.1.17.1	Telcordia (formerly BellCore) GR-1298-CORE, AIN Switching System Generic Requirements, as implemented in BellSouth’s switching equipment;

##### 9.2.1.17.2	Telcordia (formerly BellCore) GR-1299-CORE, AIN Switch-Service Control Point (SCP)/Adjunct Interface Generic Requirements;

##### 9.2.1.17.3	Telcordia (formerly BellCore) TR-NWT-001284, AIN 0.1 Switching System Generic Requirements;

##### 9.2.1.17.4	Telcordia (formerly BellCore) SR-NWT-002247, AIN Release 1 Update.

### 9.2.2	Interface Requirements

#### 9.2.2.1	BellSouth shall provide the following interfaces to loops:

#### 9.2.2.2	Standard Tip/Ring interface including loopstart or groundstart, on-hook signaling (e.g., for calling number, calling name and message waiting lamp);

#### 9.2.2.3	Coin phone signaling;

#### 9.2.2.4	Basic Rate Interface ISDN adhering to appropriate Telcordia (formerly BellCore) Technical Requirements;

#### 9.2.2.5	Two-wire analog interface to PBX;

##### 9.2.2.5.1	Four-wire analog interface to PBX;

#### 9.2.2.6	Four-wire DS1 interface to PBX or customer provided equipment (e.g. computers and voice response systems);

#### 9.2.2.7	Primary Rate ISDN to PBX adhering to ANSI standards Q.931, Q.932 and appropriate Telcordia (formerly BellCore) Technical Requirements;

#### 9.2.2.8	Switched Fractional DS1 with capabilities to configure Nx64 channels (where N = 1 to 24); and

#### 9.2.2.9	Loops adhering to Telcordia (formerly BellCore) TR-NWT-08 and TR-NWT-303 specifications to interconnect Digital Loop Carriers.

#### 9.2.2.10	BellSouth shall provide access to the following but not limited to:

#### 9.2.2.11	SS7 Signaling Network or Multi-Frequency trunking if requested by Premiere;

#### 9.2.2.12	Interface to Premiere operator services systems or Operator Services through appropriate trunk interconnections for the system; and

#### 9.2.2.13	Interface to Premiere directory assistance services through the Premiere switched network or to Directory Assistance Services through the appropriate trunk interconnections for the system; and 950 access or other Premiere required access to interexchange carriers as requested through appropriate trunk interfaces.

10.		Interoffice Transmission Facilities

BellSouth shall provide nondiscriminatory access, in accordance with FCC Rule 51.311 and Section 251(c)(3) of the Act, to interoffice transmission facilities on an unbundled basis to Premiere for the provision of a telecommunications service.

10.1	Interoffice transmission facility network elements include:

- Dedicated transport, defined as BellSouth’s transmission facilities, including all technically feasible capacity-related services including, but not limited to, DS1, DS3 and OCn levels, dedicated to a particular customer or carrier, that provide telecommunications between wire centers or switches owned by BellSouth, or between wire centers and switches owned by BellSouth and Premiere;

- Dark Fiber transport, defined as BellSouth’s optical transmission facilities without attached multiplexing, aggregation or other electronics;

3. 	Shared transport, defined as transmission facilities shared by more than one carrier, including BellSouth, between end office switches, between end office switches and tandem switches, and between tandem switches, in BellSouth’s network..

10.1.1	BellSouth shall:

- Provide Premiere exclusive use of interoffice transmission facilities dedicated to a particular customer or carrier, or shared use of the features, functions, and capabilities of interoffice transmission facilities shared by more than one customer or carrier;

- Provide all technically feasible transmission facilities, features, functions, and capabilities that Premiere could use to provide telecommunications services;

- Permit, to the extent technically feasible, Premiere to connect such interoffice facilities to equipment designated by Premiere, including but not limited to, Premiere’s collocated facilities; and

- Permit, to the extent technically feasible, Premiere to obtain the functionality provided by BellSouth’s digital cross-connect systems in the same manner that BellSouth provides such functionality to interexchange carriers.

## 10.2	Technical Requirements of Common (Shared) Transport

### 10.2.1	Common (Shared) Transport provided on DS1 or VT1.5 circuits, shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for Central Office to Central Office (“CO to CO”) connections in the appropriate industry standards.

### 10.2.2	Common (Shared) Transport provided on DS3 circuits, STS-1 circuits, and higher transmission bit rate circuits, shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for CO to CO connections in the appropriate industry standards.

### 10.2.3	BellSouth shall be responsible for the engineering, provisioning, and maintenance of the underlying equipment and facilities that are used to provide Common (Shared) Transport.

### 10.2.4	At a minimum, Common (Shared) Transport shall meet all of the requirements set forth in the following technical references (as applicable for the transport technology being used):

#### 10.2.4.1	ANSI T1.101-1994, American National Standard for Telecommunications - Synchronization Interface Standard Performance and Availability;

#### 10.2.4.2	ANSI T1.102-1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces;

#### 10.2.4.3	ANSI T1.102.01-199x, American National Standard for Telecommunications - Digital Hierarchy - VT1.5;

#### 10.2.4.4	ANSI T1.105-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Basic Description including Multiplex Structure, Rates and Formats;

#### 10.2.4.5	ANSI T1.105.01-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Automatic Protection Switching;

#### 10.2.4.6	ANSI T1.105.02-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Payload Mappings;

#### 10.2.4.7	ANSI T1.105.03-1994, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Jitter at Network Interfaces;

#### 10.2.4.8	ANSI T1.105.03a-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET): Jitter at Network Interfaces - DS1 Supplement;

#### 10.2.4.9	ANSI T1.105.05-1994, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Tandem Connection;

#### 10.2.4.10	ANSI T1.105.06-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Physical Layer Specifications;

#### 10.2.4.11	ANSI T1.105.07-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Sub STS-1 Interface Rates and Formats;

#### 10.2.4.12	ANSI T1.105.09-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Network Element Timing and Synchronization;

#### 10.2.4.13	ANSI T1.106-1988, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (Single Mode);

#### 10.2.4.14	ANSI T1.107-1988, American National Standard for Telecommunications - Digital Hierarchy - Formats Specifications;

#### 10.2.4.15	ANSI T1.107a-1990 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications (DS3 Format Applications);

#### 10.2.4.16	ANSI T1.107b-1991 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications;

#### 10.2.4.17	ANSI T1.117-1991, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (SONET) (Single Mode - Short Reach);

#### 10.2.4.18	ANSI T1.403-1989, Carrier to Customer Installation, DS1 Metallic Interface Specification;

#### 10.2.4.19	ANSI T1.404-1994, Network-to-Customer Installation - DS3 Metallic Interface Specification;

#### 10.2.4.20	ITU Recommendation G.707, Network node interface for the synchronous digital hierarchy (SDH);

#### 10.2.4.21	ITU Recommendation G.704, Synchronous frame structures used at 1544, 6312, 2048, 8488 and 44736 kbit/s hierarchical levels;

#### 10.2.4.22	Telcordia (formerly BellCore) FR-440 and TR-NWT-000499, Transport Systems Generic Requirements (TSGR):  Common Requirements;

#### 10.2.4.23	Telcordia (formerly BellCore) GR-820-CORE, Generic Transmission Surveillance:  DS1 & DS3 Performance;

#### 10.2.4.24	Telcordia (formerly BellCore) GR-253-CORE, Synchronous Optical Network Systems (SONET);  Common Generic Criteria;

#### 10.2.4.25	Telcordia (formerly BellCore) TR-NWT 000507, Transmission, Section 7, Issue 5 (Telcordia (formerly BellCore), December 1993). (A module of LSSGR, FR-NWT-000064.);

#### 10.2.4.26	Telcordia (formerly BellCore) TR-NWT-000776, Network Interface Description for ISDN Customer Access;

#### 10.2.4.27	Telcordia (formerly BellCore) TR-INS-000342, High-Capacity Digital Special Access Service-Transmission Parameter Limits and lnterface Combinations, Issue 1 February 1991;

#### 10.2.4.28	Telcordia (formerly BellCore) ST-TEC 000052, Telecommunications Transmission Engineering Textbook, Volume 2: Facilities, Third Edition, Issue I May 1989;

#### 10.2.4.29	Telcordia (formerly BellCore) ST-TEC-000051, Telecommunications Transmission Engineering Textbook Volume 1:  Principles, Third Edition. Issue 1 August 1987.

## 10.3	Dedicated Transport

#### 10.3.1.	BellSouth shall offer Dedicated Transport in each of the following ways:

##### 10.3.1.1	As capacity on a shared facility.

##### 10.3.1.2	As a circuit (e.g., DS0, DS1 or DS3) dedicated to Premiere.

#### 10.3.2	When Dedicated Transport is provided as a system it shall include:

##### 10.3.2.1	Transmission equipment such as multiplexers, line terminating equipment, amplifiers, and regenerators;

##### 10.3.2.2	Inter-office transmission facilities such as optical fiber, copper twisted pair, and coaxial cable.

### 10.3.3	Unbundled Local Channel

10.3.3.1	The Unbundled Local Channel is the dedicated transmission path between Premiere’s Point of Presence and the BellSouth Serving Wire Center.

10.3.3.2	BellSouth currently offers Unbundled Local Channels for switched traffic.  Rates for these elements are listed in this Attachment.  For those states that do not contain rates in this Attachment for DS1 and DS3 switched Local Channels, the rates in the applicable State Access Tariff will apply as interim rates.  When final rates are developed, these interim rates will be subject to true-up, and the Parties will amend the Agreement to reflect the new rates.

10.3.3.3	BellSouth currently offers Unbundled Local Channels for non-switched traffic at DS1 and DS3 levels at rates as set forth in Exhibit C to this Attachment.

### 10.3.4	Technical Requirements

This Section sets forth technical requirements for all Dedicated Transport.

#### 10.3.4.1	When BellSouth provides Dedicated Transport as a circuit or a system, the entire designated transmission circuit or system (e.g., DS0, DS1,DS3) shall be dedicated to Premiere designated traffic.

#### 10.3.4.2	BellSouth shall offer Dedicated Transport in all technologies that become available including, but not limited to, DS1 and DS3 transport systems, SONET (or SDH) Bi-directional Line Switched Rings, SONET (or SDH) Unidirectional Path Switched Rings, and SONET (or SDH) point-to-point transport systems (including linear add-drop systems), at all available transmission bit rates.  While SONET Ring facilities are not available in every application, they are typically available in the major metropolitan areas.

#### 10.3.4.3	For DS1 or VT1.5 circuits, Dedicated Transport shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for Customer Interface to Central Office (“CI to CO”) connections in the appropriate industry standards.

#### 10.3.4.4	Where applicable, for DS3 circuits, Dedicated Transport shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for CI to CO connections in the appropriate industry standards.

#### 10.3.4.5	BellSouth shall offer the following interface transmission rates for Dedicated Transport:

##### 10.3.4.5.1	DS0 Equivalent;

##### 10.3.4.5.2	DS1 (Extended SuperFrame - ESF and D4 channel bank shall be provided);

##### 10.3.4.5.3	DS3 where applicable (M13 multiplexer shall be provided);

##### 10.3.4.5.4	SDH Standard interface rates in accordance with International Telecommunications Union (ITU) Recommendation G.707 and Plesiochronous Digital Hierarchy (PDH) rates per ITU Recommendation G.704.

#### 10.3.4.6	When Dedicated Transport is provided as a system, BellSouth shall design the system according to our network infrastructure to allow for the termination points specified by Premiere.

10.3.5	At a minimum, Dedicated Transport shall meet each of the requirements set forth in the following technical references:

#### 10.3.5.1	ANSI T1.231-1993 -American National Standard for Telecommunications - Digital Hierarchy - Layer 1 In-Service Digital Transmission performance monitoring.

##### 10.3.5.1.1	ANSI T1.102-1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces;

##### 10.3.5.1.2	ANSI T1.106-1988, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (Single Mode);

##### 10.3.5.1.3	ANSI T1.107-1988, American National Standard for Telecommunications - Digital Hierarchy - Formats Specifications;

##### 10.3.5.1.4	ANSI T1.107a-1990 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications (DS3 Format Applications);

##### 10.3.5.1.5	ANSI T1.107b-1991 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications;

##### 10.3.5.1.6	Telcordia (formerly BellCore) FR-440 and TR-NWT-000499, Transport Systems Generic Requirements (TSGR):  Common Requirements;

##### 10.3.5.1.7	Telcordia (formerly BellCore) GR-820-CORE, Generic Transmission Surveillance:  DS1 & DS3 Performance;

##### 10.3.5.1.8	Telcordia (formerly BellCore) TR-NWT 000507, Transmission, Section 7, Issue 5 (Telcordia (formerly BellCore), December 1993). (A module of LSSGR, FR-NWT-000064.);

##### 10.3.5.1.9	Telcordia (formerly BellCore) TR-INS-000342, High-Capacity Digital Special Access Service-Transmission Parameter Limits and lnterface Combinations, Issue 1 February 1991;

##### 10.3.5.1.10	Telcordia (formerly BellCore) ST-TEC 000052, Telecommunications Transmission Engineering Textbook, Volume 2: Facilities, Third Edition, Issue I May 1989;

##### 10.3.5.1.11	Telcordia (formerly BellCore) ST-TEC-000051, Telecommunications Transmission Engineering Textbook Volume 1:  Principles, Third Edition. Issue 1 August 1987;

#### 11.	Tandem Switching

#### 11.1	Tandem Switching is as defined in Section 9.1.1.3 of this Attachment.

#### 11.2 	Technical Requirements

11.2.1 	Tandem Switching shall have the same capabilities or equivalent capabilities as those described in Bell Communications Research TR-TSY-000540 Issue 2R2, Tandem Supplement, 6/1/90. The requirements for Tandem Switching include, but are not limited to the following:

##### 11.2.1.1	Tandem Switching shall provide signaling to establish a tandem connection;

##### 11.2.1.2	Tandem Switching will provide screening as jointly agreed to by Premiere and BellSouth;

##### 11.2.1.3	Tandem Switching shall provide Advanced Intelligent Network triggers supporting AIN features where such routing is not available from the originating end office switch, to the extent such Tandem switch has such capability;

##### 11.2.1.4	Tandem Switching shall provide access to Toll Free number portability database as designated by Premiere;

##### 11.2.1.5	Tandem Switching shall provide all trunk interconnections discussed under the “Network Interconnection” section (e.g., SS7, MF, DTMF, DialPulse, PRI-ISDN, DID, and CAMA-ANI (if appropriate for 911));

##### 11.2.1.6	Tandem Switching shall provide connectivity to PSAPs where 911 solutions are deployed and the tandem is used for 911; and

##### 11.2.1.7	Where appropriate, Tandem Switching shall provide connectivity to transit traffic to and from other carriers.

#### 11.2.2	Tandem Switching shall accept connections (including the necessary signaling and trunking interconnections) between end offices, other tandems, IXCs, ICOs, CAPs and CLEC switches.

#### 11.2.3	Tandem Switching shall provide local tandeming functionality between two end offices including two offices belonging to different CLEC's (e.g., between a CLEC end office and the end office of another CLEC).

#### 11.2.4	Tandem Switching shall preserve CLASS/LASS features and Caller ID as traffic is processed.

#### 11.2.5	Tandem Switching shall record billable events and send them to the area billing centers designated by Premiere.  Tandem Switching will provide recording of all billable events as jointly agreed to by Premiere and BellSouth.

#### 11.2.6	Upon a reasonable request from Premiere, BellSouth shall perform routine testing and fault isolation on the underlying switch that is providing Tandem Switching and all its interconnections.  The results and reports of the testing shall be made immediately available to Premiere.

#### 11.2.7	BellSouth shall maintain Premiere’s trunks and interconnections associated with Tandem Switching at least at parity to its own trunks and interconnections.

#### 11.2.8	BellSouth shall control congestion points and network abnormalities.  All traffic will be restricted in a non discriminatory manner.

#### 11.2.9	Selective Call Routing through the use of line class codes is not available through the use of tandem switching.  Selective Call Routing through the use of line class codes is an end office capability only.  Detailed primary and overflow routing plans for all interfaces available within BellSouth switching network shall be mutually agreed to by Premiere and BellSouth.

#### 11.2.10	Tandem Switching shall process originating toll-free traffic received from Premiere local switch.

#### 11.2.11	In support of AIN triggers and features, Tandem Switching shall provide SSP capabilities when these capabilities are not available from the Local Switching Network Element, to the extent such Tandem Switch has such capability.

### 11.3	Interface Requirements

#### 11.3.1	Tandem Switching shall provide interconnection to the E911 PSAP where the underlying Tandem is acting as the E911 Tandem.

#### 11.3.2	Tandem Switching shall interconnect, with direct trunks, to all carriers with which BellSouth interconnects.

#### 11.3.3	BellSouth shall provide all signaling necessary to provide Tandem Switching with no loss of feature functionality.

#### 11.3.4	Tandem Switching shall interconnect with Premiere's switch, using two-way trunks, for traffic that is transiting via BellSouth network to interLATA or intraLATA carriers.  At Premiere’s request, Tandem Switching shall record and keep records of traffic for billing.

#### 11.3.5	Tandem Switching shall provide an alternate final routing pattern for Premiere traffic overflowing from direct end office high usage trunk groups.

### 11.4	Tandem Switching shall meet or exceed (i.e., be more favorable to Premiere) each of the requirements for Tandem Switching set forth in the following technical references:

#### 11.4.1	Bell Communications Research TR-TSY-000540 Issue 2R2, Tandem Supplement, 6/1/90;

#### 11.4.2	GR-905-CORE covering CCSNIS;

#### 11.4.3	GR-1429-CORE for call management features; and 
GR-2863-CORE and Telcordia (formerly BellCore) GR-2902-CORE covering CCS AIN interconnection

# 12.	Operator Systems

BellSouth agrees to offer access to operator systems pursuant to the terms and conditions following and at the rates set forth in this Attachment.

## 12.1	Definition

Operator Systems is the Network Element that provides operator and automated call handling and billing, special services, end user telephone listings and optional call completion services.  The Operator Systems, Network Element provides two types of functions:  Operator Service functions and Directory Assistance Service functions, each of which are described in detail below.

## 12.2	Operator Service

### 12.2.1	Definition

Operator Service provides:  (1) operator handling for call completion (for example, collect, third number billing, and manual credit card calls), (2) operator or automated assistance for billing after the end user has dialed the called number (for example, credit card calls); and (3) special services including but not limited to Busy Line Verification and Emergency Line Interrupt (ELI), Emergency Agency Call, Operator-assisted Directory Assistance, and Rate Quotes.

### 12.2.2	Requirements

#### 12.2.2.1	When Premiere requests BellSouth to provide Operator Services, the following requirements apply:

##### 12.2.2.1.1	BellSouth shall complete 0+ and 0- dialed local calls.

##### 12.2.2.1.2	BellSouth shall complete 0+ intraLATA toll calls.

##### 12.2.2.1.3	BellSouth shall complete calls that are billed to Premiere end user's calling card that can be validated by BellSouth.

##### 12.2.2.1.4	BellSouth shall complete person-to-person calls.

##### 12.2.2.1.5	BellSouth shall complete collect calls.

##### 12.2.2.1.6	BellSouth shall provide the capability for callers to bill to a third party and complete such calls.

##### 12.2.2.1.7	BellSouth shall complete station-to-station calls.

##### 12.2.2.1.8	BellSouth shall process emergency calls.

##### 12.2.2.1.9	BellSouth shall process Busy Line Verify and Emergency Line Interrupt requests.

##### 12.2.2.1.10	BellSouth shall process emergency call trace, as they do for their End users prior to the Effective Date.  Call must originate from a 911 provider.

##### 12.2.2.1.11	BellSouth shall process operator-assisted directory assistance calls.

#### 12.2.2.2	BellSouth shall adhere to equal access requirements, providing Premiere local end users the same IXC access as provided to BellSouth end users.

#### 12.2.2.3	BellSouth shall exercise at least the same level of fraud control in providing Operator Service to Premiere that BellSouth provides for its own operator service.

#### 12.2.2.4	BellSouth shall perform Billed Number Screening when handling Collect, Person-to-Person, and Billed-to-Third-Party calls.

#### 12.2.2.5	BellSouth shall direct customer account and other similar inquiries to the customer service center designated by Premiere.

#### 12.2.2.6	BellSouth shall provide a feed of customer call records in “EMI” format to Premiere in accordance with CLECODUF standards specified in Attachment 7.

### 12.2.3	Interface Requirements

With respect to Operator Services for calls that originate on local switching capability provided by or on behalf of Premiere, the interface requirements shall conform to the then current established system interface specifications for the platform used to provide Operator Service and the interface shall conform to industry standards.

## 12.3	Directory Assistance Service

### 12.3.1	Definition

Directory Assistance Service provides local end user telephone number listings with the option to complete the call at the callers direction separate and distinct from local switching.

### 12.3.2	Requirements

#### 12.3.2.1	Directory Assistance Service shall provide up to two listing requests per call.  If available and if requested by Premiere’s end user, BellSouth shall provide caller-optional directory assistance call completion service at rates contained in this Attachment to one of the provided listings, equal to that which BellSouth provides its end users.  If not available, Premiere may request such requirement pursuant to the Bona Fide Request/New Business Process as set forth in General Terms and Conditions.

#### 12.3.2.2	Directory Assistance Service Updates

##### 12.3.2.2.1	BellSouth shall update end user listings changes daily.  These changes include:

###### 12.3.2.2.1.1	New end user connections:  BellSouth will provide service to Premiere that is equal to the service it provides to itself and its end users;

###### 12.3.2.2.1.2	End user disconnections:  BellSouth will provide service to Premiere that is equal to the service it provides to itself and its end users; and

###### 12.3.2.2.1.3	End user address changes:  BellSouth will provide service to Premiere that is equal to the service it provides to itself and its end users;

#### 12.3.2.3	These updates shall also be provided for non-listed and non-published numbers for use in emergencies.

## 12.4	Branding for Operator Call Processing and Directory Assistance

### 12.4.1	The BellSouth Operator Systems Branding Feature provides a definable announcement to Premiere end users using Directory Assistance (DA)/Operator Call Processing (OCP) prior to placing them in queue or connecting them to an available operator or automated operator system. This feature allows Premiere to have its calls custom branded with Premiere name on whose behalf BellSouth is providing Directory Assistance and/or Operator Call Processing.  Rates for Custom Branding, Operator Call Process and Directory Assistance are set forth in this Attachment.

### 12.4.2	BellSouth offers four service levels of branding to Premiere when ordering Directory Assistance and/or Operator Call Processing.

#### 12.4.2.1	Service Level 1 - BellSouth Branding

#### 12.4.2.2	Service Level 2 - Unbranded

#### 12.4.2.3	Service Level 3 - Custom Branding

#### 12.4.2.4	Service Level 4 - Self Branding (applicable only to Premiere for Resale or use with an Unbundled Port when routing to an operator service provider other than BellSouth).

### 12.4.3	For Resellers and Use with an Unbundled Port

#### 12.4.3.1	BellSouth Branding is the Default Service Level.

#### 12.4.3.2	Unbranding, Custom Branding, and Self Branding require Premiere to order selective routing for each originating BellSouth end office identified by Premiere.  Rates for Selective Routing are set forth in this Attachment.

#### 12.4.3.3	Customer Branding and Self Branding require Premiere to order dedicated trunking from each BellSouth end office identified by Premiere, to either the BellSouth Traffic Operator Position System (TOPS) or Premiere Operator Service Provider.  Rates for trunks are set forth in applicable BellSouth tariffs.

#### 12.4.3.4	Unbranding - Unbranded Directory Assistance and/or Operator Call Processing calls ride common trunk groups provisioned by BellSouth from those end offices identified by Premiere to the BellSouth TOPS.  These calls are routed to “No Announcement.”

### 12.4.4	For Facilities Based Carriers

#### 12.4.4.1	All Service Levels require Premiere to order dedicated trunking from their end office(s) point of interface to the BellSouth TOPS Switches. Rates for trunks are set forth in applicable BellSouth tariffs.

#### 12.4.4.2	Customized Branding includes charges for the recording of the branding announcement and the loading of the audio units in each TOPS Switch, IVS and NAV equipment for which Premiere requires service

Directory Assistance customized branding uses:

- the recording of the name;

- the front-end loading of the Digital Recorded Announcement Machine (DRAM) in each TOPS switch.

Operator Call Processing customized branding uses:

- the recording of the name;

- the front-end loading of the DRAM in the TOPS Switch;

- the back-end loading in the audio units in the Automated Alternate Billing System (AABS) in the Interactive Voice Subsystem (IVS);

- the 0- automation loading for the audio units in the Enhanced Billing and Access Service (EBAS) in the Network Applications Vehicle (NAV).

12.4.4.3	BellSouth will provide to Premiere purchasing  local BellSouth switching and reselling BellSouth local exchange service, selective routing of calls to a requested directory assistance services platform or operator services platform.  Premiere end users may use the same dialing arrangements as BellSouth end users, but obtain a Premiere branded service.

## 12.5	Directory Assistance Database Service (DADS)

### 12.5.1	BellSouth shall make its Directory Assistance Database Service (DADS) available solely for the expressed purpose of providing Directory Assistance type services to Premiere end users.  The term “end user” denotes any entity which obtains Directory Assistance type services for its own use from a DADS customer.  Directory Assistance type service is defined as Voice Directory Assistance (DA Operator assisted and Electronic Directory Assistance (Data System assisted)).  Premiere agrees that Directory Assistance Database Service (DADS) will not be used for any purpose which violates federal or state laws, statutes, regulatory orders or tariffs.  Except for the permitted users, Premiere agrees not to disclose DADS to others and shall provide due care in providing for the security and confidentiality of DADS. Further, Premiere authorizes the inclusion of Premiere Subscriber listings in the BellSouth Directory Assistance products.

### 12.5.2	BellSouth shall provide Premiere initially with a base file of subscriber listings which reflect all listing change activity occurring since Premiere’s most recent update via magnetic tape, and subsequently using electronic connectivity such as Network Data Mover to be developed mutually by Premiere and BellSouth.  Premiere agrees to assume the costs associated with CONNECT: Direct TM connectivity, which will vary depending upon volume and mileage.

### 12.5.3	BellSouth will require approximately one month after receiving an order to prepare the Base File.  BellSouth will provide daily updates which will reflect all listing change activity occurring since CLEC’s most recent update.  BellSouth shall provide updates to Premiere on a Business, Residence, or combined Business and Residence basis.  Premiere agrees that the updates shall be used solely to keep the information current.  Delivery of Daily Updates will commence the day after Premiere receives the Base File.

### 12.5.4	BellSouth is authorized to include Premiere Subscriber List Information in its Directory Assistance Database Service (DADS) and its Directory Publishers Database Service (DPDS).  Any other use by BellSouth of Premiere Subscriber List Information is not authorized and with the exception of a request for DADS or DPDS, BellSouth shall refer any request for such information to Premiere.

### 12.5.5	Rates for DADS are as set forth in this Attachment.

## 12.6	Direct Access to Directory Assistance Service

### 12.6.1	Direct Access to Directory Assistance Service (DADAS) will provide Premiere’s directory assistance operators with the ability to search all available BellSouth’s subscriber listings using the Directory Assistance search format.  Subscription to DADAS will allow Premiere to utilize its own switch, operator workstations and optional audio subsystems.

### 12.6.2	BellSouth will provide DADAS from its DA location.  Premiere will access the DADAS system via a telephone company provided point of availability.  Premiere has the responsibility of providing the physical links required to connect to the point of availability.  These facilities may be purchased from the telephone company as rates and charges billed separately from the charges associated with this offering.

### 12.6.3	A specified interface to each Premiere subsystem will be provided by BellSouth.  Interconnection between Premiere system and a specified BellSouth location will be pursuant to the use of Premiere owned or Premiere leased facilities and shall be appropriate sized based upon the volume of queries being generated by Premiere.

### 12.6.4	The specifications for the three interfaces necessary for interconnection are available in the following documents:

#### 12.6.4.1	DADAS to Subscriber Operator Position System—Northern Telecom Document CSI-2300-07; Universal Gateway/ Position Message Interface Format Specification

#### 12.6.4.2	DADAS to Subscriber Switch—Northern Telecom Document Q210-1 Version A107; NTDMS/CCIDAS System Application Protocol; and AT&T Document 250-900-535 Operator Services Position System Listing Service and Application Call Processing Data Link Interface Specification

#### 12.6.4.3	DADAS to Audio Subsystem (Optional)—Directory One Call Control to Audio Response Unit system interface specifications are available through Northern Telecom as a licensed access protocol—Northern Telecom Document 355-004424 and Gateway/Interactive Voice subsystem Protocol Specification

### 12.6.5	Rates for DADAS are as set forth in this Attachment.

# 13.	Signaling

BellSouth agrees to offer access to  signaling and access to BellSouth’s signaling databases subject to compatibility testing and at the rates set forth in this Attachment.  BellSouth may provide mediated access to BellSouth signaling systems and databases.  Available signaling elements include signaling links, signal transfer points and service control points.  Signaling functionality will be available with both A-link and B-link connectivity.

## 13.1	Definition of Signaling Link Transport

Signaling Link Transport is a set of two or four dedicated 56 Kbps. transmission paths between CLEC-designated Signaling Points of Interconnection (SPOI) that provides appropriate physical diversity.

## 13.2	Technical Requirements

### 13.2.1	Signaling Link Transport shall consist of full duplex mode 56 kbps transmission paths.

### 13.2.2	Of the various options available, Signaling Link Transport shall perform in the following two ways:

#### 13.2.2.1	As an “A-link” which is a connection between a switch or SCP and a home Signaling Transfer Point Switch (STP) pair; and

#### 13.2.2.2	As a “B-link” which is a connection between two STP pairs in different company networks (e.g., between two STP pairs for two Competitive Local Exchange Carriers (CLECs)).

### 13.2.3	Signaling Link Transport shall consist of two or more signaling link layers as follows:

#### 13.2.3.1	An A-link layer shall consist of two links.

#### 13.2.3.2	A B-link layer shall consist of four links.

### 13.2.4	A signaling link layer shall satisfy a performance objective such that:

#### 13.2.4.1	There shall be no more than two minutes down time per year for an A-link layer; and

#### 13.2.4.2	There shall be negligible (less than 2 seconds) down time per year for a B-link layer.

### 13.2.5	A signaling link layer shall satisfy interoffice and intraoffice diversity of facilities and equipment, such that:

#### 13.2.5.1	No single failure of facilities or equipment causes the failure of both links in an A-link layer (i.e., the links should be provided on a minimum of two separate physical paths end-to-end); and

#### 13.2.5.2	No two concurrent failures of facilities or equipment shall cause the failure of all four links in a B-link layer (i.e., the links should be provided on a minimum of three separate physical paths end-to-end).

## 13.3	Interface Requirements

### 13.3.1	There shall be a DS1 (1.544 Mbps) interface at the Premiere-designated SPOIs.  Each 56 kbps transmission path shall appear as a DS0 channel within the DS1 interface.

# 14.	Signaling Transfer Points (STPs)

## 14.1	Definition - Signaling Transfer Points is a signaling network function that includes all of the capabilities provided by the signaling transfer point switches (STPs) and their associated signaling links which enable the exchange of SS7 messages among and between switching elements, database elements and signaling transfer point switches

## 14.2	Technical Requirements

### 14.2.1	STPs shall provide access to Network Elements connected to BellSouth SS7 network. These include:

#### 14.2.1.1	BellSouth Local Switching or Tandem Switching;

#### 14.2.1.2	BellSouth Service Control Points/DataBases;

#### 14.2.1.3	Third-party local or tandem switching;

#### 14.2.1.4	Third-party-provided STPs.

### 14.2.2	The connectivity provided by STPs shall fully support the functions of all other Network Elements connected to BellSouth SS7 network.  This explicitly includes the use of BellSouth SS7 network to convey messages which neither originate nor terminate at a signaling end point directly connected to BellSouth SS7 network (i.e., transient messages).  When BellSouth SS7 network is used to convey transient messages, there shall be no alteration of the Integrated Services Digital Network User Part (ISDNUP) or Transaction Capabilities Application Part (TCAP) user data that constitutes the content of the message.

### 14.2.3	If a BellSouth tandem switch routes calling traffic, based on dialed or translated digits, on SS7 trunks between an Premiere local switch and third party local switch, BellSouth SS7 network shall convey the TCAP messages that are necessary to provide Call Management features (Automatic Callback, Automatic Recall, and Screening List Editing) between Premiere local STPs and the STPs that provide connectivity with the third party local switch, even if the third party local switch is not directly connected to BellSouth STPs.

### 14.2.4	STPs shall provide all functions of the MTP as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.  This includes:

#### 14.2.4.1	Signaling Data Link functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements,

#### 14.2.4.2	Signaling Link functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements, and

#### 14.2.4.3	Signaling Network Management functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.

### 14.2.5	STPs shall provide all functions of the SCCP necessary for Class 0 (basic connectionless) service, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.  In particular, this includes Global Title Translation (GTT) and SCCP Management procedures, as specified in T1.112.4.  In cases where the destination signaling point is a Premiere or third party local or tandem switching system directly connected to BellSouth SS7 network, BellSouth shall perform final GTT of messages to the destination and SCCP Subsystem Management of the destination.  In all other cases, BellSouth shall perform intermediate GTT of messages to a gateway pair of STPs in an SS7 network connected with BellSouth SS7 network, and shall not perform SCCP Subsystem Management of the destination.  If BellSouth performs final GTT to a Premiere database, then Premiere agrees to provide BellSouth with the Destination Point Code for the Premiere database.

### 14.2.6	STPs shall provide on a non-discriminatory basis all functions of the OMAP commonly provided by STPs, as specified in the reference in Section 12.4.5 of this Attachment.  All OMAP functions will be on a "where available" basis and can include:

#### 14.2.6.1	MTP Routing Verification Test (MRVT) and

#### 14.2.6.2	SCCP Routing Verification Test (SRVT).

### 14.2.7	In cases where the destination signaling point is a BellSouth local or tandem switching system or database, or is an Premiere or third party local or tandem switching system directly connected to the BellSouth SS7 network, STPs shall perform MRVT and SRVT to the destination signaling point.  In all other cases, STPs shall perform MRVT and SRVT to a gateway pair of STPs in an SS7 network connected with the BellSouth SS7 network.  This requirement shall be superseded by the specifications for Internetwork MRVT and SRVT if and when these become approved ANSI standards and available capabilities of BellSouth STPs, and if mutually agreed upon by Premiere and BellSouth.

### 14.2.8	STPs shall be on parity with BellSouth.

### 14.2.9	SS7 Advanced Intelligent Network (AIN) Access

#### 14.2.9.1	When technically feasible and upon request by Premiere, SS7 Access shall be made available in association with  switching.  SS7 AIN Access is the provisioning of AIN 0.1 triggers in an equipped BellSouth local switch and interconnection of the BellSouth SS7 network with the Premiere SS7 network to exchange TCAP queries and responses with an Premiere SCP.

#### 14.2.9.2	SS7 AIN Access shall provide Premiere SCP access to BellSouth local switch in association with  switching via interconnection of BellSouth SS7 and Premiere SS7 Networks.  BellSouth shall offer SS7 access through its STPs.  If BellSouth requires a mediation device on any part of its network specific to this form of access, BellSouth must route its messages in the same manner.  The interconnection arrangement shall result in the BellSouth local switch recognizing the Premiere SCP as at least at parity with BellSouth’s SCPs in terms of interfaces, performance and capabilities.

## 14.3	Interface Requirements

### 14.3.1	BellSouth shall provide the following STPs options to connect Premiere or Premiere-designated local switching systems or STPs to BellSouth SS7 network:

#### 14.3.1.1	An A-link interface from Premiere local switching systems; and,

#### 14.3.1.2	A B-link interface from Premiere local STPs.

### 14.3.2	Each type of interface shall be provided by one or more sets (layers) of signaling links.

### 14.3.3	The Signaling Point of Interconnection (SPOI) for each link shall be located at a cross-connect element, such as a DSX-1, in the Central Office (CO) where BellSouth STP is located.  There shall be a DS1 or higher rate transport interface at each of the SPOIs.  Each signaling link shall appear as a DS0 channel within the DS1 or higher rate interface. BellSouth shall offer higher rate DS1 signaling for interconnecting Premiere local switching systems or STPs with BellSouth STPs as soon as these become approved ANSI standards and available capabilities of BellSouth STPs.  BellSouth and Premiere will work jointly to establish mutually acceptable SPOIs.

### 14.3.4	BellSouth CO shall provide intraoffice diversity between the SPOIs and BellSouth STPs, so that no single failure of intraoffice facilities or equipment shall cause the failure of both B-links in a layer connecting to a BellSouth STP.  BellSouth and Premiere will work jointly to establish mutually acceptable SPOIs.

### 14.3.5	BellSouth shall provide MTP and SCCP protocol interfaces that shall conform to all sections relevant to the MTP or SCCP in the following specifications:

#### 14.3.5.1	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

#### 14.3.5.2	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

### 14.3.6	Message Screening

#### 14.3.6.1	BellSouth shall set message screening parameters so as to accept valid messages from Premiere local or tandem switching systems destined to any signaling point within BellSouth’s SS7 network where the Premiere switching system has a legitimate signaling relation.

#### 14.3.6.2	BellSouth shall set message screening parameters so as to pass valid messages from Premiere local or tandem switching systems destined to any signaling point or network accessed through BellSouth’s SS7 network where the Premiere switching system has a legitimate signaling relation.

#### 14.3.6.3	BellSouth shall set message screening parameters so as to accept and pass/send valid messages destined to and from Premiere from any signaling point or network interconnected through BellSouth’s SS7 network where the Premiere SCP has a legitimate signaling relation.

## 14.4	STPs shall be equal to or better than all of the requirements for STPs set forth in the following technical references:

### 14.4.1	ANSI T1.111-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP);

### 14.4.2	ANSI T1.111A-1994 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP) Supplement;

### 14.4.3	ANSI T1.112-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Signaling Connection Control Part (SCCP);

### 14.4.4	ANSI T1.115-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Monitoring and Measurements for Networks;

### 14.4.5	ANSI T1.116-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Operations, Maintenance and Administration Part (OMAP);

### 14.4.6	ANSI T1.118-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Intermediate Signaling Network Identification (ISNI);

### 14.4.7	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP); and

### 14.4.8	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

# 15.	Service Control Points/DataBases

## 15.1	Definition

### 15.1.1	Databases are the Network Elements that provide the functionality for storage of, access to, and manipulation of information required to offer a particular service and/or capability.  Databases include, but are not limited to:  Local Number Portability, LIDB, Toll Free Number Database, Automatic Location Identification/Data Management System, Calling Name Database, access to Service Creation Environment and Service Management System (SCE/SMS) application databases and Directory Assistance.

### 15.1.2	A Service Control Point (SCP) is a specific type of Database functionality deployed in a Signaling System 7 (SS7) network that executes service application logic in response to SS7 queries sent to it by a switching system also connected to the SS7 network.  Service Management Systems provide operational interfaces to allow for provisioning, administration and maintenance of subscriber data and service application data stored in SCPs.

## 15.2	Technical Requirements for SCPs/Databases

Requirements for SCPs/Databases within this section address storage of information, access to information (e.g. signaling protocols, response times), and administration of information (e.g., provisioning, administration, and maintenance).  All SCPs/Databases shall be provided to Premiere in accordance with the following requirements.

### 15.2.1	BellSouth shall provide physical access to SCPs through the SS7 network and protocols with TCAP as the application layer protocol.

### 15.2.2	BellSouth shall provide physical interconnection to databases via industry standard interfaces and protocols (e.g. SS7, ISDN and X.25).

### 15.2.3	The reliability of interconnection options shall be consistent with requirements for diversity and survivability.

### 15.2.4	Database Availability

Call processing databases shall have a maximum unscheduled availability of 30 minutes per year.  Unavailability due to software and hardware upgrades shall be scheduled during minimal usage periods and only be undertaken upon proper notification to providers which might be impacted.  Any downtime associated with the provision of call processing related databases will impact all service providers, including BellSouth, equally.

### 15.2.5	The operational interface provided by BellSouth shall complete Database transactions (i.e., add, modify, delete) for Premiere customer records stored in BellSouth databases within 3 days, or sooner where BellSouth provisions its own customer records within a shorter interval.

## 15.3	Local Number Portability Database

### 15.3.1	Definition

The Permanent Number Portability (PNP) database supplies routing numbers for calls involving numbers that have been ported from one local service provider to another.  PNP is currently being worked in industry forums. The results of these forums will dictate the industry direction of PNP.  BellSouth agrees to provide access to the PNP database at rates, terms and conditions as set forth by BellSouth and in accordance with an effective FCC or Commission directive.

## 15.4	Line Information Database (LIDB)

BellSouth will store in its LIDB only records relating to service in the BellSouth region.   The LIDB Storage Agreement is included in this Attachment.

### 15.4.1	Definition

The Line Information Database (LIDB) is a transaction-oriented database accessible through Common Channel Signaling (CCS) networks.  It contains records associated with end user Line Numbers and Special Billing Numbers.  LIDB accepts queries from other Network Elements and provides appropriate responses.  The query originator need not be the owner of LIDB data.  LIDB queries include functions such as screening billed numbers that provides the ability to accept Collect or Third Number Billing calls and validation of Telephone Line Number based non-proprietary calling cards.  The interface for the LIDB functionality is the interface between BellSouth CCS network and other CCS networks.  LIDB also interfaces to administrative systems.

### 15.4.2	Technical Requirements

BellSouth will offer to Premiere any additional capabilities that are developed for LIDB during  the life of this Agreement.

#### 15.4.2.1	BellSouth shall process Premiere’s Customer records in LIDB at least at parity with BellSouth customer records, with respect to other LIDB functions.  BellSouth shall indicate to Premiere what additional functions (if any) are performed by LIDB in the BellSouth network.

#### 15.4.2.2	Within two (2) weeks after a request by Premiere, BellSouth shall provide Premiere with a list of the customer data items which Premiere would have to provide in order to support each required LIDB function.  The list shall indicate which data items are essential to LIDB function, and which are required only to support certain services.  For each data item, the list shall show the data formats, the acceptable values of the data item and the meaning of those values.

#### 15.4.2.3	BellSouth shall provide LIDB systems for which operating deficiencies that would result in calls being blocked, shall not exceed 30 minutes per year.

#### 15.4.2.4	BellSouth shall provide LIDB systems for which operating deficiencies that would not result in calls being blocked shall not exceed 12 hours per year.

#### 15.4.2.5	BellSouth shall provide LIDB systems for which the LIDB function shall be in overload no more than 12 hours per year.

#### 15.4.2.6	All additions, updates and deletions of Premiere data to the LIDB shall be solely at the direction of Premiere.  Such direction from Premiere will not be required where the addition, update or deletion is necessary to perform standard fraud control measures (e.g., calling card auto-deactivation).

#### 15.4.2.7	BellSouth shall provide priority updates to LIDB for Premiere data upon Premiere’s request (e.g., to support fraud detection), via password-protected telephone card, facsimile, or electronic mail within one hour of notice from the established BellSouth contact.

#### 15.4.2.8	BellSouth shall provide LIDB systems such that no more than 0.01% of Premiere customer records will be missing from LIDB, as measured by Premiere audits.  BellSouth will audit Premiere records in LIDB against DBAS to identify record mismatches and provide this data to a designated Premiere contact person to resolve the status of the records and BellSouth will update system appropriately.  BellSouth will refer record of mis-matches to Premiere within one business day of audit.  Once reconciled records are received back from Premiere, BellSouth will update LIDB the same business day if less than 500 records are received before 1:00PM Central Time.  If more than 500 records are received, BellSouth will contact Premiere to negotiate a time frame for the updates, not to exceed three business days.

#### 15.4.2.9	BellSouth shall perform backup and recovery of all of Premiere’s data in LIDB including sending to LIDB all changes made since the date of the most recent backup copy, in at least the same time frame BellSouth performs backup and recovery of BellSouth data in LIDB for itself.  Currently, BellSouth performs backups of the LIDB for itself on a weekly basis and when a new software release is scheduled, a backup is performed prior to loading the new release.

#### 15.4.2.10	BellSouth shall provide Premiere with LIDB reports of data which are missing or contain errors, as well as any misrouted errors, within a reasonable time period as negotiated between Premiere and BellSouth.

#### 15.4.2.11	BellSouth shall prevent any access to or use of Premiere data in LIDB by BellSouth personnel that are outside of established administrative and fraud control personnel, or by any other Party that is not authorized by Premiere in writing.

#### 15.4.2.12	BellSouth shall provide Premiere performance of the LIDB Data Screening function, which allows a LIDB to completely or partially deny specific query originators access to LIDB data owned by specific data owners, for Customer Data that is part of an NPA-NXX or RAO-0/1XX wholly or partially owned by Premiere at least at parity with BellSouth Customer Data. BellSouth shall obtain from Premiere the screening information associated with LIDB Data Screening of Premiere data in accordance with this requirement.  BellSouth currently does not have LIDB Data Screening capabilities.  When such capability is available, BellSouth shall offer it to Premiere under the Bona Fide Request/New Business Process as set forth in General Terms and Conditions .

#### 15.4.2.13	BellSouth shall accept queries to LIDB associated with Premiere customer records, and shall return responses in accordance with industry standards.

#### 15.4.2.14	BellSouth shall provide mean processing time at the LIDB within 0.50 seconds under normal conditions as defined in industry standards.

#### 15.4.2.15	BellSouth shall provide processing time at the LIDB within 1 second for 99% of all messages under normal conditions as defined in industry standards.

### 15.4.3	Interface Requirements

BellSouth shall offer LIDB in accordance with the requirements of this subsection.

#### 15.4.3.1	The interface to LIDB shall be in accordance with the technical references contained within.

#### 15.4.3.2	The CCS interface to LIDB shall be the standard interface described herein.

#### 15.4.3.3	The LIDB Data Base interpretation of the ANSI-TCAP messages shall comply with the technical reference herein.  Global Title Translation shall be maintained in the signaling network in order to support signaling network routing to the LIDB.

## 15.5	Toll Free Number Database

The Toll Free Number Database is a SCP that provides functionality necessary for toll free (e.g., 800 and 888) number services by providing routing information and additional so-called vertical features during call set-up in response to queries from SSPs. BellSouth shall provide the Toll Free Number Database in accordance with the following:

### 15.5.1	Technical Requirements

#### 15.5.1.1	BellSouth shall make BellSouth Toll Free Number Database available for Premiere to query with a toll-free number and originating information.

#### 15.5.1.2	The Toll Free Number Database shall return carrier identification and, where applicable, the queried toll free number, translated numbers and instructions as it would in response to a query from a BellSouth switch.

#### 15.5.1.3	The SCP shall also provide, at Premiere's option, such additional feature as described in SR-TSV-002275 (BOC Notes on BellSouth Networks, SR-TSV-002275, Issue 2, (Telcordia (formerly BellCore), April 1994)) as are available to BellSouth.  These may include but are not limited to:

##### 15.5.1.3.1	Network Management;

##### 15.5.1.3.2	Customer Sample Collection; and

##### 15.5.1.3.3	Service Maintenance

## 15.6	Automatic Location Identification/Data Management System (ALI/DMS)

The ALI/DMS Database contains end user information (including name, address, telephone information, and sometimes special information from the local service provider or end user) used to determine to which Public Safety Answering Point  (PSAP) to route the call.  The ALI/DMS database is used to provide more routing flexibility for E911 calls than Basic 911.  BellSouth shall provide the Emergency Services Database in accordance with the following:

### 15.6.1	Technical Requirements

#### 15.6.1.1	BellSouth shall offer Premiere a data link to the ALI/DMS database or permit Premiere to provide its own data link to the ALI/DMS database. BellSouth shall provide error reports from the ALI/DMS database to Premiere immediately after Premiere inputs information into the ALI/DMS database.  Alternately, Premiere may utilize BellSouth, to enter end user information into the data base on a demand basis, and validate end user information on a demand basis.

#### 15.6.1.2	The ALI/DMS database shall contain the following end user information:

##### 15.6.1.2.1	Name;

##### 15.6.1.2.2	Address;

##### 15.6.1.2.3	Telephone number; and

##### 15.6.1.2.4	Other information as appropriate (e.g., whether a end user is blind or deaf or has another disability).

#### 15.6.1.3	When BellSouth is responsible for administering the ALI/DMS database in its entirety, ported number NXXs entries for the ported numbers should be maintained unless Premiere requests otherwise and shall be updated if Premiere requests, provided Premiere supplies BellSouth with the updates.

#### 15.6.1.4	When Remote Call Forwarding (RCF) is used to provide number portability to the local end user and a remark or other appropriate field information is available in the database, the shadow or “forwarded-to” number and an indication that the number is ported shall be added to the customer record.

#### 15.6.1.5	If BellSouth is responsible for configuring PSAP features (for cases when the PSAP or BellSouth supports an ISDN interface) it shall ensure that CLASS Automatic Recall (Call Return) is not used to call back to the ported number.  Although BellSouth currently does not have ISDN interface, BellSouth agrees to comply with this requirement once ISDN interfaces are in place.

### 15.6.2	Interface Requirements

The interface between the E911 Switch or Tandem and the ALI/DMS database for Premiere end users shall meet industry standards.

# 15.7	Calling Name (CNAM) Database Service.  The Agreement for Calling Name (CNAM) with standard pricing is included as Exhibit B to this Attachment.  Premiere must provide to its account manager a written request with a requested activation date to activate this service.  If Premiere is interested in requesting CNAM with volume and term pricing, Premiere must contact its account manager to request a separate CNAM volume and term Agreement.

## 15.8	SCPs/Databases shall be equal to or better than all of the requirements for SCPs/Databases set forth in the following technical references:

### 15.8.1	GR-246-CORE, Bell Communications Research Specification of  Signaling System Number 7, ISSUE 1 (Telcordia (formerly BellCore), December 199);

### 15.8.2	GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP). (Telcordia (formerly BellCore), March 1994);

### 15.8.3	GR-954-CORE, CCS Network Interface Specification (CCSNIS) Supporting Line Information Database (LIDB) Service 6, Issue 1, Rev. 1 (Telcordia (formerly BellCore), October 1995);

### 15.8.4	GR-1149-CORE, OSSGR Section 10: System Interfaces, Issue 1 (Telcordia (formerly BellCore), October 1995)  (Replaces TR-NWT-001149);

### 15.8.5	Telcordia (formerly BellCore) GR-1158-CORE, OSSGR Section 22.3: Line Information Database 6, Issue (Telcordia (formerly BellCore), October 1995);

### 15.8.6	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service (Telcordia (formerly BellCore), May 1995); and

### 15.8.7	BOC Notes on BellSouth Networks, SR-TSV-002275, ISSUE 2, (Telcordia (formerly BellCore), April 1994).

## 15.9	Service Creation Environment and Service Management System (SCE/SMS) Advanced Intelligent Network (AIN) Access.

### 15.9.1	BellSouth’s Service Creation Environment and Service Management System (SCE/SMS) Advanced Intelligent Network (AIN) Access shall provide Premiere the capability that will allow Premiere and other third parties to create service applications in a BellSouth Service Creation Environment and deploy those applications in a BellSouth SMS to a BellSouth SCP.  The third party service applications interact with AIN triggers provisioned on a BellSouth SSP.

### 15.9.2	BellSouth’s SCE/SMS AIN Access shall provide access to SCE hardware, software, testing and technical support (e.g., help desk, system administrator) resources available to Premiere.  Scheduling procedures shall provide Premiere equivalent priority to these resources

### 15.9.3	BellSouth SCP shall partition and protect Premiere service logic and data from unauthorized access, execution or other types of compromise.

### 15.9.4	When Premiere selects SCE/SMS AIN Access, BellSouth shall provide training, documentation, and technical support to enable Premiere to use BellSouth’s SCE/SMS AIN Access to create and administer applications.  Training, documentation, and technical support will address use of SCE and SMS access and administrative functions, but will not include support for the creation of a specific service application.

### 15.9.5	When Premiere selects SCE/SMS AIN Access, BellSouth shall provide for a secure, controlled access environment in association with its internal use of AIN components.  Premiere access will be provided via remote data connection (e.g., dial-in, ISDN).

### 15.9.6	When Premiere selects SCE/SMS AIN Access, BellSouth shall allow Premiere to download data forms and/or tables to BellSouth SCP via BellSouth SMS without intervention from BellSouth (e.g., service customization and end user subscription).

# 16.	DARK FIBER

BellSouth agrees to offer access to Dark Fiber pursuant to the terms and conditions following and at the rates set forth in this Attachment.

### 16.1.1	Dark Fiber is defined as BellSouth’s optical transmission facilities without attached multiplexing, aggregation or other electronics.  Premiere may utilize Dark Fiber for either loops or transport. Dark Fiber does not have electronics on either end of the Dark Fiber segment.   It may be strands of optical fiber existing in aerial or underground structure. No regeneration or optical amplification will be included with this element.

## 16.2	Requirements

### 16.2.1	BellSouth shall make available Dark Fiber where it exists in BellSouth’s network and where, as a result of future building or deployment, it becomes available.  BellSouth shall offer all Dark Fiber to Premiere pursuant to the prices set forth in this Attachment.

### 16.2.2	Premiere may test the quality of the Dark Fiber to confirm its usability and performance specifications.

### 16.2.3	BellSouth shall use its best efforts to provide to Premiere information regarding the location, availability and performance of Dark Fiber within ten (10) business days for a records based answer and twenty (20) business days for a field based answer, after receiving a request from Premiere ("Request").  Within such time period, BellSouth shall send written confirmation of availability of the Dark Fiber ("Confirmation").

### 16.2.4	BellSouth shall use its best efforts to make Dark Fiber available to Premiere within thirty (30) business days after it receives written confirmation from Premiere that the Dark Fiber previously deemed available by BellSouth is wanted for use by Premiere.  This includes identification of appropriate connection points (e.g., Light Guide Interconnection (LGX) or splice points) to enable Premiere to connect or splice Premiere provided transmission media (e.g., optical fiber) or equipment to the Dark Fiber.

# 17.	SS7 Network Interconnection

### 17.1.1	Definition

SS7 Network Interconnection is the interconnection of Premiere local Signaling Transfer Point Switches (STP) and Premiere local or tandem switching systems with BellSouth STPs.  This interconnection provides connectivity that enables the exchange of SS7 messages among BellSouth switching systems and databases (DBs), Premiere local or tandem switching systems, and other third-party switching systems directly connected to the BellSouth SS7 network.

### 17.1.2	Technical Requirements

#### 17.1.2.1	SS7 Network Interconnection shall provide connectivity to all components of the BellSouth SS7 network.  These include:

##### 17.1.2.1.1	BellSouth local or tandem switching systems;

##### 17.1.2.1.2	BellSouth DBs; and

##### 17.1.2.1.3	Other third-party local or tandem switching systems.

#### 17.1.2.2	The connectivity provided by SS7 Network Interconnection shall fully support the functions of BellSouth switching systems and DBs and Premiere or other third-party switching systems with A-link access to the BellSouth SS7 network.

If traffic is routed based on dialed or translated digits between an Premiere local switching system and a BellSouth or other third-party local switching system, either directly or via a BellSouth tandem switching system, then it is a requirement that the BellSouth SS7 network convey via SS7 Network Interconnection the TCAP messages that are necessary to provide Call Management services (Automatic Callback, Automatic Recall, and Screening List Editing) between the Premiere local STPs and BellSouth or other third-party local switch.

#### 17.1.2.3	When the capability to route messages based on Intermediate Signaling Network Identifier (ISNI) is generally available on BellSouth STPs,  the BellSouth SS7 Network shall also convey TCAP messages using SS7 Network Interconnection in similar circumstances where the BellSouth switch routes traffic based on a Carrier Identification Code (CIC).

#### 17.1.2.4	SS7 Network Interconnection shall provide all functions of the MTP as specified in ANSI T1.111. This includes:

##### 17.1.2.4.1	Signaling Data Link functions, as specified in ANSI T1.111.2;

##### 17.1.2.4.2	Signaling Link functions, as specified in ANSI T1.111.3; and

##### 17.1.2.4.3	Signaling Network Management functions, as specified in ANSI T1.111.4.

#### 17.1.2.5	SS7 Network Interconnection shall provide all functions of the SCCP necessary for Class 0 (basic connectionless) service, as specified in ANSI T1.112.  In particular, this includes Global Title Translation (GTT) and SCCP Management procedures, as specified in T1.112.4.  Where the destination signaling point is a BellSouth switching system or DB, or is another third-party local or tandem switching system directly connected to the BellSouth SS7 network, SS7 Network Interconnection shall include final GTT of messages to the destination and SCCP Subsystem Management of the destination.  Where the destination signaling point is an Premiere local or tandem switching system, SS7 Network Interconnection shall include intermediate GTT of messages to a gateway pair of Premiere local STPs, and shall not include SCCP Subsystem Management of the destination.

#### 17.1.2.6	SS7 Network Interconnection shall provide all functions of the Integrated Services Digital Network User Part (ISDNUP), as specified in ANSI T1.113.

#### 17.1.2.7	SS7 Network Interconnection shall provide all functions of the TCAP, as specified in ANSI T1.114.

#### 17.1.2.8	If and when Internetwork MTP Routing Verification Test (MRVT) and SCCP Routing Verification Test (SRVT) become approved ANSI standards and available capabilities of BellSouth STPs, SS7 Network Interconnection shall provide these functions of the OMAP.

#### 17.1.2.9	SS7 Network Interconnection shall be equal to or better than the following performance requirements:

##### 17.1.2.9.1	MTP Performance, as specified in ANSI T1.111.6;

##### 17.1.2.9.2	SCCP Performance, as specified in ANSI T1.112.5; and

##### 17.1.2.9.3	ISDNUP Performance, as specified in ANSI T1.113.5.

### 17.1.3	Interface Requirements

#### 17.1.3.1	BellSouth shall offer the following SS7 Network Interconnection options to connect Premiere or Premiere-designated local or tandem switching systems or STPs to the BellSouth SS7 network:

##### 17.1.3.1.1	A-link interface from Premiere local or tandem switching systems; and

##### 17.1.3.1.2	B-link interface from Premiere STPs.

#### 17.1.3.2	The Signaling Point of Interconnection (SPOI) for each link shall be located at a cross-connect element, such as a DSX-1, in the Central Office (CO) where the BellSouth STP is located.  There shall be a DS1 or higher rate transport interface at each of the SPOIs.  Each signaling link shall appear as a DS0 channel within the DS1 or higher rate interface.  BellSouth shall offer higher rate DS1 signaling links for interconnecting Premiere local switching systems or STPs with BellSouth STPs as soon as these become approved ANSI standards and available capabilities of BellSouth STPs.  BellSouth and Premiere will work jointly to establish mutually acceptable SPOI.

#### 17.1.3.3	BellSouth CO shall provide intraoffice diversity between the SPOIs and the BellSouth STP, so that no single failure of intraoffice facilities or equipment shall cause the failure of both B-links in a layer connecting to a BellSouth STP. BellSouth and Premiere will work jointly to establish mutually acceptable SPOI.

#### 17.1.3.4	The protocol interface requirements for SS7 Network Interconnection include the MTP, ISDNUP, SCCP, and TCAP. These protocol interfaces shall conform to the following specifications:

##### 17.1.3.4.1	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

##### 17.1.3.4.2	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service;

##### 17.1.3.4.3	Telcordia (formerly BellCore) GR-1429-CORE, CCS Network Interface Specification (CCSNIS) Supporting Call Management Services; and

##### 17.1.3.4.4	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

#### 17.1.3.5	BellSouth shall set message screening parameters to block accept messages from Premiere local or tandem switching systems destined to any signaling point in the BellSouth SS7 network with which the Premiere switching system has a legitimate signaling relation.

### 17.1.4	SS7 Network Interconnection shall be equal to or better than all of the requirements for SS7 Network Interconnection set forth in the following technical references:

#### 17.1.4.1	ANSI T1.110-1992 American National Standard Telecommunications - Signaling System Number 7 (SS7) - General Information;

#### 17.1.4.2	ANSI T1.111-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP);

#### 17.1.4.3	ANSI T1.111A-1994 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP) Supplement;

#### 17.1.4.4	ANSI T1.112-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Signaling Connection Control Part (SCCP);

#### 17.1.4.5	ANSI T1.113-1995 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Integrated Services Digital Network (ISDN) User Part;

#### 17.1.4.6	ANSI T1.114-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Transaction Capabilities Application Part (TCAP);

#### 17.1.4.7	ANSI T1.115-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Monitoring and Measurements for Networks;

#### 17.1.4.8	ANSI T1.116-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Operations, Maintenance and Administration Part (OMAP);

#### 17.1.4.9	ANSI T1.118-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Intermediate Signaling Network Identification (ISNI);

#### 17.1.4.10	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

#### 17.1.4.11	Telcordia (formerly BellCore) GR-954-CORE, CCS Network Interface Specification (CCSNIS) Supporting Line Information Database (LIDB) Service;

#### 17.1.4.12	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service;

#### 17.1.4.13	Telcordia (formerly BellCore) GR-1429-CORE, CCS Network Interface Specification (CCSNIS) Supporting Call Management Services; and,

#### 17.1.4.14	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

# 18.	Basic 911 and E911

If Premiere orders network elements and other services, then Premiere is also responsible for providing E911 to its end users.  BellSouth agrees to offer access to the 911/E911 network pursuant to the following terms and conditions set forth in this Attachment.

## 18.1	Definition

Basic 911 and E911 is an additional requirement that provides a caller access to the applicable emergency service bureau by dialing a 3-digit universal telephone number (911).

## 18.2	Requirements

### 18.2.1	Basic 911 Service Provisioning.  For Basic 911 service, BellSouth will provide to Premiere a list consisting of each municipality that subscribes to Basic 911 service.  The list will also provide, if known, the E911 conversion date for each municipality and, for network routing purposes, a ten-digit directory number representing the appropriate emergency answering position for each municipality subscribing to 911.  Premiere will be required to arrange to accept 911 calls from its end users in municipalities that subscribe to Basic 911 service and translate the 911 call to the appropriate 10-digit directory number as stated on the list provided by BellSouth.  Premiere will be required to route that call to BellSouth at the appropriate tandem or end office.  When a municipality converts to E911 service, Premiere will be required to discontinue the Basic 911 procedures and being using E911 procedures.

### 18.2.2	E911 Service Provisioning.  For E911 service, Premiere will be required to install a minimum of two dedicated trunks originating from the Premiere serving wire center and terminating to the appropriate E911 tandem.  The dedicated trunks shall be, at a minimum, DS-0 level trunks configured either as a 2-wire analog interface or as part of a digital (1.544 Mb/s) interface.  Either configuration shall use CAMA-type signaling with multifrequency (“MF”) pulsing that will deliver automatic number identification (“ANI”) with the voice portion of the call.  If the user interface is digital, MF pulses, as well as other AC signals, shall be encoded per the u-255 Law convention.  Premiere will be required to provide BellSouth daily updates to the E911 database.  Premiere will be required to forward 911 calls to the appropriate E911 tandem, along with ANI, based upon the current E911 end office to tandem homing arrangement as provided by BellSouth.  If the E911 tandem trunks are not available, Premiere will be required to route the call to a designated 7-digit local number residing in the appropriate Public Service Answering Point (“PSAP”).  This call will be transported over BellSouth’s interoffice network and will not carry the ANI of the calling party.  Premiere shall be responsible for providing BellSouth with complete and accurate data for submission to the 911/E911 database for the purpose of providing 911/E911 to its end users.

### 18.2.3	Rates.  Charges for 911/E911 service are borne by the municipality purchasing the service.  BellSouth will impose no charge on Premiere beyond applicable charges for BellSouth trunking arrangements.

### 18.2.4	Basic 911 and E911 functions provided to Premiere shall be at least at parity with the support and services that BellSouth provides to its end users for such similar functionality.

### Detailed Practices and Procedures.  The detailed practices and procedures contained in the E911 Local Exchange Carrier Guide For Facility-Based Providers as amended from time to time during the term of this Agreement will determine the appropriate practices and procedures for BellSouth and Premiere to follow in providing 911/E911 services.

# 19.	Combinations

19.1	For purposes of this Section, references to “Already Combined” network elements shall mean that such network elements are in fact already combined by BellSouth in the BellSouth network to provide service to a particular end user at a particular location.

19.2	EELs

19.2.1	Where necessary to comply with an effective FCC and/or State Commission order, or as otherwise mutually agreed by the Parties, BellSouth shall offer access to loop and transport combinations, also known as the Enhanced Extended Link (“EEL”) as defined in Section 19.3 below.

19.2.2	Subject to Section 19.2.3 below, BellSouth will provide access to the EEL in the  combinations set forth in 19.3 following.  This offering is intended to provide connectivity from an end user’s location through that end user’s SWC to Premiere’s POP serving wire center.  The circuit must be connected to Premiere’s switch for the purpose of provisioning telecommunications services, including telephone exchange service, to Premiere’s end-user customers.  Except as provided for in paragraph 22 of the FCC’s Supplemental Order Clarification, released June 2, 2000, in CC Docket No. 96-98 (“June 2, 2000 Order”), the EEL will be connected to Premiere’s facilities in Premiere’s collocation space at the POP SWC.  Premiere may purchase BellSouth’s access facilities between Premiere’s POP and Premiere’s collocation space at the POP SWC.

19.2.3	BellSouth shall provide EEL combinations to Premiere in the state of Georgia regardless of whether or not such EELs are Already Combined. In all other states, BellSouth shall make available to Premiere those EEL combinations described in Section 19.3 below only to the extent such combinations are Already Combined.

19.2.4	BellSouth will make available EEL combinations to Premiere in density Zone 1, as defined in 47 C.F.R. 69.123 as of January 1, 1999, in the  Miami, Orlando, Fort Lauderdale, Charlotte, New Orleans, Greensboro and Nashville MSAs, regardless of whether or not such EELs are Already Combined.

19.2.5	Additionally, BellSouth shall make available to Premiere a combination of an unbundled loop and tariffed special access interoffice facilities.  To the extent Premiere will require multiplexing functionality in connection with such combination, BellSouth will provide access to multiplexing within the central office pursuant to the terms, conditions and rates set forth in its Access Services Tariffs. The combination of an unbundled loop and tariffed special access interoffice facilities and any associated tariffed services, including but not limited to multiplexing, shall not be eligible for conversion to UNEs as described in Section 19.5 below.  Where multiplexing functionality is required in connection with loop and transport combinations, such multiplexing will be provided at the rates and on the terms set forth in this Agreement.

19.3	EEL Combinations

19.3.1	DS1 Interoffice Channel + DS1 Channelization + 2-wire VG Local Loop

19.3.2	DS1 Interoffice Channel + DS1 Channelization + 4-wire VG Local Loop

19.3.3	DS1 Interoffice Channel + DS1 Channelization + 2-wire ISDN Local Loop

19.3.4	DS1 Interoffice Channel + DS1 Channelization + 4-wire 56 kbps Local Loop

19.3.5	DS1 Interoffice Channel + DS1 Channelization + 4-wire 64 kbps Local Loop

19.3.6	DS1 Interoffice Channel + DS1 Local Loop

19.3.7	DS3 Interoffice Channel + DS3 Local Loop

19.3.8	STS-1 Interoffice Channel + STS-1 Local Loop

19.3.9	DS3 Interoffice Channel + DS3 Channelization + DS1 Local Loop

19.3.10	STS-1 Interoffice Channel + DS3 Channelization + DS1 Local Loop

19.3.11	2-wire VG Interoffice Channel + 2-wire VG Local Loop

19.3.12	4wire VG Interoffice Channel + 4-wire VG Local Loop

19.3.13	4-wire 56 kbps Interoffice Channel + 4-wire 56 kbps Local Loop

19.3.14	4-wire 64 kbps Interoffice Channel + 4-wire 64 kbps Local Loop

19.4	Other Network Element Combinations

In the state of Georgia, BellSouth shall make available to Premiere, in accordance with Section 19.6 below: (1) combinations of network elements other than EELs that are Already Combined; and (2) combinations of network elements other than EELs that are not Already Combined but that BellSouth ordinarily combines in its network.  In all other states, BellSouth shall make available to Premiere, in accordance with Section 19.6 below, combinations of network elements other than EELs only to the extent such combinations are Already Combined.

19.5	Special Access Service Conversions

19.5.1	Premiere may not convert special access services to combinations of loop and transport network elements, whether or not Premiere self-provides its entrance facilities (or obtains entrance facilities from a third party), unless Premiere uses the combination to provide a “significant amount of local exchange service” (as described in Section 19.5.2 below), in addition to exchange access service, to a particular customer.

19.5.2	For the purpose of special access conversions, a “significant amount of local exchange service” is as defined in the FCC’s June 2, 2000 Order.  The Parties agree to incorporate by reference paragraph 22 of the June 2, 2000 Order.  When  Premiere requests conversion of special access circuits, Premiere will self-certify to BellSouth in the manner specified in paragraph 29 of the June 2, 2000 Order that the circuits to be converted qualify for conversion.  In addition there may be extraordinary circumstances where  Premiere is providing a significant amount of local exchange service, but does not qualify under any of the three options set forth in paragraph 22 of June 2, 2000 Order.  In such case,  Premiere may petition the FCC for a waiver of the local usage options set forth in the June 2, 2000 Order.  If a waiver is granted, then upon Premiere’s request the Parties shall amend this Agreement to the extent necessary to incorporate the terms of such waiver for such extraordinary circumstance.

19.5.3 	Upon request for conversions of up to 15 circuits from special access to EELs, BellSouth shall perform such conversions within seven (7) days from BellSouth’s receipt of a valid, error free service order from Premiere.  Requests for conversions of fifteen (15) or more circuits from special access to EELs will be provisioned on a project basis.  Conversions should not require the special access circuit to be disconnected and reconnected because only the billing information or other administrative information associated with the circuit will change when Premiere requests a conversion.  The Access Service Request process will be used for conversion requests.

19.5.4	BellSouth may, at its sole expense, and upon thirty (30) days notice to Premiere, audit Premiere’s records not more than once in any twelve month period, unless an audit finds non-compliance with the local usage options referenced in the June 2, 2000 Order, in order to verify the type of traffic being transmitted over combinations of loop and transport network elements.  If, based on its audits, BellSouth concludes that Premiere is not providing a significant amount of local exchange traffic over the combinations of loop and transport network elements, BellSouth may file a complaint with the appropriate Commission, pursuant to the dispute resolution process set forth in this Agreement.  In the event that BellSouth prevails, BellSouth may convert such combinations of loop and transport network elements to special access services and may seek appropriate retroactive reimbursement from Premiere.

19.6	Rates

19.6.1	Georgia

19.6.1.1	The non-recurring and recurring rates for the EEL Combinations of network elements set forth in 19.3, whether Already Combined or new, are as set forth in this Attachment.

19.6.1.2	On an interim basis, for combinations of loop and transport network elements not set forth in Section 19.3, where the elements are not Already Combined but are ordinarily combined in BellSouth’s network, the non-recurring and recurring charges for such UNE combinations shall be the sum of the stand-alone non-recurring and recurring charges of the network elements which make up the combination.  These interim rates shall be subject to true-up based on the Commission’s review of BellSouth’s cost studies.

19.6.1.3	To the extent that Premiere seeks to obtain other combinations of network elements that BellSouth ordinarily combines in its network which have not been specifically priced by the Commission when purchased in combined form, Premiere, at its option, can request that such rates be determined pursuant to the Bona Fide Request/New Business Request (NBR) process set forth in this Agreement.

## 19.6.2	All Other States

## 19.6.2.1	Subject to Section 19.2.3 and 19.4 preceding, all other states, the  rates for (1) Already Combined EEL combinations set forth in Section 19.3, and (2) other combinations of network elements that are Already Combined in the network will be the sum of the recurring rates for the individual network elements plus a nonrecurring charge as specified in Exhibit C hereto.

## 19.6.2.2	Rates for new EEL combinations in Density Zone 1 in the Miami, Orlando, Fort Lauderdale, Charlotte, New Orleans, Greensboro and Nashville MSAs shall be as set forth in Exhibit C hereto; provided, however, that to the extent a rate is not established in Exhibit C, the rate shall be the sum of the recurring and nonrecurring charges for the individual network elements as set forth in Exhibit C to this Attachment, unless otherwise established by the Commission.

# 20.7	Port/Loop Combinations

## 20.7.1	At Premiere’s request, BellSouth shall provide access to combinations of port and loop network elements, as set forth in Section 20.7.4 below, that are Already Combined in BellSouth’s network except as specified in Sections 20.7.1.1 and 20.7.1.2 below, consistent with the requirements of 47 C.F.R. 315(b) and all applicable FCC and Commission rules and policies.

## 20.7.1.1	BellSouth shall not provide access to combinations of unbundled port and loop network elements in locations where, pursuant to FCC rules,  BellSouth is not required to provide circuit switching as an unbundled network element.

## 20.7.1.2	In accordance with effective and applicable FCC rules, BellSouth shall not provide unbundled circuit switching in density Zone 1, as defined in 47 C.F.R. 69.123 as of January 1, 1999, of the Atlanta, Miami, Orlando, Fort Lauderdale, Charlotte, New Orleans, Greensboro and Nashville MSAs to Premiere if Premiere’s customer has 4 or more DS0 equivalent lines.

## 20.7.2 	In Georgia, BellSouth shall provide combinations of port and loop network elements to Premiere regardless of whether or not such combinations are Already Combined except in those locations where BellSouth is not required to provide unbundled circuit switching, as set forth in Section 20.7.1.1 above. In all other states, and subject to Sections 20.7.1.1 and 20.7.1.2 above, BellSouth shall provide combinations of port and loop network elements to Premiere only to the extent such elements are Already Combined.

## 20.7.3	Rates for Combinations of Loop and Port Network Elements

## 20.7.3.1	Rates for combinations of loop and port network elements, as set forth in Section 20.7.4, are provided in Exhibit C of this Attachment.

## 20.7.3.2	Rates for Circuit Switching

## 20.7.3.2.1	Rates for circuit switching, where BellSouth is not required, pursuant to Sections 20.7.1.1 and 20.7.1.2, to provide circuit switching are as set forth in Exhibit C of this Attachment.

## 20.7.4	Combination Offerings

20.7.4.1	2-wire voice grade port, voice grade loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

20.7.4.2	2-wire voice grade DID port, voice grade loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

20.7.4.3	2-wire CENTREX port, voice grade loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

20.7.4.4.	2-wire ISDN Basic Rate Interface, voice grade loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

20.7.4.5	2-wire ISDN Primary Rate Interface, DS1 loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

20.7.4.6	4-wire DS1 Trunk port, DS1 Loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

21.		Rates

# 21.1.	General

The prices that Premiere shall pay to BellSouth for  Network Elements and Other Services are set forth in Exhibit C to this Attachment.

# 21.2.	Operations Support Systems (“OSS”)

BellSouth shall provide nondiscriminatory access in accordance with FCC Rule 51.311 and Section 251(c)(3) of the Act to OSS on an unbundled basis to Premiere for the provision of a telecommunications service. OSS functions consist of pre-ordering, ordering, provisioning, maintenance and repair, and billing functions supported by BellSouth’s databases and information.  Subject to applicable and effective FCC rules and orders, BellSouth, as part of its duty to provide access to the pre-ordering function, shall provide Premiere with nondiscriminatory access to the same detailed information about the loop that is available to its own retail unit.  The rate associated with BellSouth’s provision of loop qualification information shall be as set forth in this Attachment.

All Local Service Requests (“LSRs”) submitted for products and services under this Attachment will be subject to the OSS charges set forth in the General Terms and Conditions of this Agreement.

21.3 	True-up

This section applies only to Tennessee.

The interim prices for Network Elements and Other Services and Local Interconnection shall be subject to true-up according to the following procedures:

1.	The interim prices shall be trued-up, either up or down, based on final prices determined either by further agreement between the Parties, or by a final order (including any appeals) of the Commission which final order meets the criteria of (3) below.  The Parties shall implement the true-up by comparing the actual volumes and demand for each item, together with interim prices for each item, with the final prices determined for each item.  Each Party shall keep its own records upon which the true-up can be based, and any final payment from one Party to the other shall be in an amount agreed upon by the Parties based on such records.  In the event of any disagreement as between the records or the Parties regarding the amount of such true-up, the Parties agree that the body having jurisdiction over the matter shall be called upon to resolve such differences, or the Parties may mutually agree to submit the matter to the Dispute Resolution process in accordance with the provisions of Section 16 of the General Terms and Conditions and Attachment 1 of the Agreement.

2.	The Parties may continue to negotiate toward final prices, but in the event that no such Agreement is reached within nine (9) months, either Party may petition the Commission to resolve such disputes and to determine final prices for each item.  Alternatively, upon mutual agreement, the Parties may submit the matter to  the Dispute Resolution Process set forth in Section 16 of the General

Terms and Conditions and Attachment 1 of the Agreement, so long as they file the resulting Agreement with the Commission as a “negotiated Agreement” under Section 252(e) of the Act.

3.	A final order of this Commission that forms the basis of a true-up shall be the final order as to prices based on appropriate cost studies, or potentially may be a final order in any other Commission proceeding which meets the following criteria:

(a)	BellSouth and CLEC is entitled to be a full Party to the proceeding;

(b)	It shall apply the provisions of the federal Telecommunications Act of 1996, including but not limited to Section 252(d)(1) (which contains pricing standards) and all then-effective implementing rules and regulations; and,

(c)	It shall include as an issue the geographic deaveraging of network element and other services prices, which deaveraged prices, if any are required by said final order, shall form the basis of any true-up.

21.4	Geographic Deaveraging

The Parties acknowledge that this Agreement contains geographically

deaveraged rates consistent with 47 C.F.R. 51.507(f) for all states except

North Carolina.  Upon adoption of geographically deaveraged rates by the North Carolina Utilities Commission, the Parties shall amend this

Agreement to include such rates.  Prior thereto, rates for the state of

North Carolina shall be as set forth in this Agreement.

EXHIBIT A

LINE INFORMATION DATA BASE (LIDB)

STORAGE AGREEMENT

I.	SCOPE

A.	This Agreement sets forth the terms and conditions pursuant to which BST agrees to store in its LIDB certain information at the request of the Local Exchange Company and pursuant to which BST, its LIDB customers and Local Exchange Carrier shall have access to such information.  Local Exchange Carrier understands that BST provides access to information in its LIDB to various telecommunications service providers pursuant to applicable tariffs and agrees that information stored at the request of Local Exchange Carrier, pursuant to this Agreement, shall be available to those telecommunications service providers.  The terms and conditions contained in the attached Addendum(s)  are hereby made a part of this Agreement as if fully incorporated herein.

B.	LIDB is accessed for the following purposes:

1.	Billed Number Screening

2.	Calling Card Validation

3.	Fraud Control

C.	BST will provide seven days per week, 24-hours per day, fraud monitoring on Calling Cards, bill-to-third and collect calls made to numbers in BST’s LIDB, provided that such information is included in the LIDB query.  BST will establish fraud alert thresholds and will notify the Local Exchange Company of fraud alerts so that the Local Exchange Company may take action it deems appropriate.  Local Exchange Company understands and agrees BST will administer all data stored in the LIDB, including the data provided by Local Exchange Company pursuant to this Agreement, in the same manner as BST’s data for BST’s end user customers.  BST shall not be responsible to Local Exchange Company for any lost revenue which may result from BST’s administration of the LIDB pursuant to its established practices and procedures as they exist and as they may be changed by BST in its sole discretion from time to time.

Local Exchange Company understands that BST currently has in effect numerous billing and collection agreements with various interexchange carriers and billing clearing houses.  Local Exchange Company further understands that these billing and collection customers of BST query BST’s LIDB to determine whether to accept various billing options from end users.  Additionally, Local Exchange Company understands that presently BST has no method to differentiate between BST’s own billing and line data in the LIDB and such data which it includes in the LIDB on Local Exchange Company’s behalf pursuant to this Agreement.  Therefore, until such time as BST can and does implement in its LIDB and its supporting systems the means to differentiate Local Exchange Company’s data from BST’s data and the Parties to this Agreement execute appropriate amendments hereto, the following terms and conditions shall apply:

(a)	The Local Exchange Company agrees that it will accept responsibility for telecommunications services billed by BST for its billing and collection customers for Local Exchange Customer’s end user accounts which are resident in LIDB pursuant to this Agreement.  Local Exchange Company authorizes BST to place such charges on Local Exchange Company’s bill from BST and agrees that it shall pay all such charges.  Charges for which Local Exchange Company hereby takes responsibility include, but are not limited to, collect and third number calls.

(b)	Charges for such services shall appear on a separate BST bill page identified with the name of the entity for which BST is billing the charge.

(c)	Local Exchange Company shall have the responsibility to render a billing statement to its end users for these charges, but Local Exchange Company’s obligation to pay BST for the charges billed shall be independent of whether Local Exchange Company is able or not to collect from the Local Exchange Company’s end users.

- BST shall not become involved in any disputes between Local Exchange Company and the entities for which BST performs billing and collection.  BellSouth will not issue adjustments for charges billed on behalf of an entity to Local Exchange Company.  It shall be the responsibility of the Local Exchange Company and the other entity to negotiate and arrange for any appropriate adjustments.

II.	TERM

This Agreement will be effective as of ____________, 199__, and will continue in effect for one year, and thereafter may be continued until terminated by either Party upon thirty (30) days written notice to the other Party.

III.	FEES FOR SERVICE AND TAXES

A.	The Local Exchange Company will not be charged a fee for storage services provided by BST to the Local Exchange Company, as described in Section I of this Agreement.

B.	Sales, use and all other taxes (excluding taxes on BST’s income) determined by BST or any taxing authority to be due to any federal, state or local taxing jurisdiction with respect to the provision of the service set forth herein will be paid by the Local Exchange Company.  The Local Exchange Company shall have the right to have BST contest with the imposing jurisdiction, at the Local Exchange Company’s expense, any such taxes that the Local Exchange Company deems are improperly levied.

IV.	INDEMNIFICATION

To the extent not prohibited by law, each Party will indemnify the other and hold the other harmless against any loss, cost, claim, injury, or liability relating to or arising out of negligence or willful misconduct by the indemnifying Party or its agents or contractors in connection with the indemnifying Party’s provision of services, provided, however, that any indemnity for any loss, cost, claim, injury or liability arising out of or relating to errors or omissions in the provision of services under this Agreement shall be limited as otherwise specified in this Agreement.  The indemnifying Party under this Section agrees to defend any suit brought against the other Party for any such loss, cost, claim, injury or liability.  The indemnified Party agrees to notify the other Party promptly, in writing, of any written claims, lawsuits, or demands for which the other Party is responsible under this Section and to cooperate in every reasonable way to facilitate defense or settlement of claims.  The indemnifying Party shall not be liable under this Section for settlement by the indemnified Party of any claim, lawsuit, or demand unless the defense of the claim, lawsuit, or demand has been tendered to it in writing and the indemnifying Party has unreasonably failed to assume such defense.

V.	LIMITATION OF LIABILITY

Neither Party shall be liable to the other Party for any lost profits or revenues or for any indirect, incidental or consequential damages incurred by the other Party arising from this Agreement or the services performed or not performed hereunder, regardless of the cause of such loss or damage.

VI.	MISCELLANEOUS

A.  It is understood and agreed to by the Parties that BST may provide similar services to other companies.

B.	All terms, conditions and operations under this Agreement shall be performed in accordance with, and subject to, all applicable local, state or federal legal and regulatory tariffs, rulings, and other requirements of the federal courts, the U. S. Department of Justice and state and federal regulatory agencies.  Nothing in this Agreement shall be construed to cause either Party to violate any such legal or regulatory requirement and either Party’s obligation to perform shall be subject to all such requirements.

C.	The Local Exchange Company agrees to submit to BST all advertising, sales promotion, press releases, and other publicity matters relating to this Agreement wherein BST’s corporate or trade names, logos, trademarks or service marks or those of BST’s affiliated companies are mentioned or language from which the connection of said names or trademarks therewith may be inferred or implied; and the Local Exchange Company further agrees not to publish or use advertising, sales promotions, press releases, or publicity matters without BST’s prior written approval.

D.	This Agreement constitutes the entire Agreement between the Local Exchange Company and BST which supersedes all prior Agreements or contracts, oral or written representations, statements, negotiations, understandings, proposals and undertakings with respect to the subject matter hereof.

E.	Except as expressly provided in this Agreement, if any part of this Agreement is held or construed to be invalid or unenforceable, the validity of any other Section of this Agreement shall remain in full force and effect to the extent permissible or appropriate in furtherance of the intent of this Agreement.

F.	Neither Party shall be held liable for any delay or failure in performance of any part of this Agreement for any cause beyond its control and without its fault or negligence, such as acts of God, acts of civil or military authority, government regulations, embargoes, epidemics, war, terrorist acts, riots, insurrections, fires, explosions, earthquakes, nuclear accidents, floods, strikes, power blackouts, volcanic action, other major environmental disturbances, unusually severe weather conditions, inability to secure products or services of other persons or transportation facilities, or acts or omissions of transportation common carriers.

G.	This Agreement shall be deemed to be a contract made under the laws of the State of Georgia, and the construction, interpretation and performance of this Agreement and all transactions hereunder shall be governed by the domestic law of such State.

FACILITIES BASED ADDENDUM

TO LINE INFORMATION DATA BASE (LIDB)

STORAGE AGREEMENT

This is a Facilities Based Addendum to the Line Information Data Base Storage Agreement dated ________________________________, 199 __, between BellSouth Telecommunications, Inc. (“BST”), and _________________________________________(“Local Exchange Company”), effective the _____ day of _________________, 199 ___.

I.	GENERAL

This Addendum sets forth the terms and conditions for Local Exchange Company’s provision of billing number information to BST for inclusion in BST’s LIDB.  BST will store in its LIDB the billing number information provided by Local Exchange Company, and BST will provide responses to on-line, call-by-call queries to this information for purposes specified in Section I.B. of the Agreement.

II.	DEFINITIONS

A.	Billing number - a number that the Local Exchange Company creates for the purpose of identifying an account liable for charges.  This number may be a line or a special billing number.

B.	Line number - a ten digit number that identifies a telephone line administered by the Local Exchange Company.

C.	Special billing number - a ten digit number that identifies a billing account established by the Local Exchange Company.

D.	Calling Card number - a billing number plus PIN number.

E.	PIN number - a four digit security code assigned by the Local Exchange Company which is added to a billing number to compose a fourteen digit calling card number.

F.	Toll billing exception indicator - associated with a billing number to indicate that it is considered invalid for billing of collect calls or third number calls or both, by the Local Exchange Company.

G.	Billed Number Screening - refers to the activity of determining whether a toll billing exception indicator is present for a particular billing number.

H.	Calling Card Validation - refers to the activity of determining whether a particular calling card number exists as stated or otherwise provided by a caller.

I.	Billing number information - information about billing number, Calling Card number and toll billing exception indicator provided to BST by the Local Exchange Company.

III.	RESPONSIBILITIES OF PARTIES

A.	The Local Exchange Company will  provide its billing number information to BST’s LIDB each business day by a method that has been mutually agreed upon by both Parties.

B.	BST will store in its LIDB the billing number information provided by the Local Exchange Company.  Under normal operating conditions, BST shall include the Local Exchange Company’s billing number information in its LIDB no later than two business days following BST’s receipt of such billing number information, provided that BST shall not be held responsible for any delay or failure in performance to the extent such delay or failure is caused by circumstances or conditions beyond BST’s reasonable control.  BST will store in its LIDB an unlimited volume of the Local Exchange Company’s working telephone numbers.

C.	BST will provide responses to on-line, call-by-call queries to the stored information for the specific purposes listed in the next paragraph.

D.	BST is authorized to use the billing number information provided by the Local Exchange Company to perform the following functions for authorized users on an on-line basis:

1.	Validate a 14 digit Calling Card number where the first 10 digits are a line number or special billing number assigned by the Local Exchange Company, and where the last four digits (PIN) are a security code assigned by the Local Exchange Company.

2.	Determine whether the Local Exchange Company or the subscriber has identified the billing number as one which should not be billed for collect or third number calls, or both.

E.	The Local Exchange Company will provide its own billing number information to BST for storage and to be used for Billed Number Screening and Calling Card Validation.  The Local Exchange Company will arrange and pay for transport of updates to BST.

- COMPLIANCE

Unless expressly authorized in writing by the Local Exchange Company, all billing number information provided pursuant to this Addendum shall be used for no purposes other than those set forth in this Addendum.

EXHIBIT B

CALLING NAME DELIVERY (CNAM) DATABASE SERVICES

# 1.00  DEFINITIONS

For the purpose of this Attachment, the following terms shall be defined as:

CALLING NAME DELIVERY DATABASE SERVICE (CNAM) - The ability to associate a name with the calling party number, allowing the end user subscriber (to which a call is being terminated) to view the calling party's name before the call is answered.  This service also provides Premiere the opportunity to load and store its subscriber names in the BellSouth CNAM SCPs.

CALLING PARTY NUMBER (CPN) - The number of the calling party that is delivered to the terminating switch using common channel signaling system 7 (CCS7) technology, and that is contained in the Initial Address Message (IAM) portion of the CCS7 call setup.

COMMON CHANNEL SIGNALING SYSTEM 7 (CCS7) - A network signaling

technology in which all signaling information between two or more nodes is transmitted over high-speed data links, rather than over voice circuits.

SERVICE CONTROL POINTs (SCPs) - The real-time data base systems that contain the names to be provided in response to queries received from CNAM SSPs.

SERVICE MANAGEMENT SYSTEM (SMS) - The main operations support system of CNAM DATABASE SERVICE.  CNAM records are loaded into the SMS, which in turn downloads into the CNAM SCP.

SERVICE SWITCHING POINTs (SSPs) - Features of computerized switches in the telephone network that determine that a terminating line has subscribed to CNAM service, and then communicate with CNAM SCPs in order to provide the name associated with the calling party number.

SUBSYSTEM NUMBER (SSN) - The address used in the Signaling Connection Control Part (SCCP) layer of the SS7 protocol to designate an application at an end signaling point.  A SSN for CNAM at the end office designates the CNAM application within the end office.  BellSouth uses the CNAM SSN of 232.

# 2.0	ATTACHMENT

2.01   This Attachment  contains  the terms and conditions where BellSouth will provide to the Premiere access to the BellSouth CNAM SCP for query or record storage purposes.

2.02	Premiere shall submit to BellSouth a notice of its intent to access and utilize BellSouth CNAM Database Services pursuant to the terms and conditions of this Attachment.  Said notice shall be in writing, no less than 60 days prior to Premiere’s access to BellSouth’s CNAM Database Services and shall be addressed to Premiere’s Account Manager.

# 3.00  PHYSICAL CONNECTION AND COMPENSATION

3.01	BellSouth's provision of CNAM Database Services to  Premiere requires interconnection from Premiere to BellSouth CNAM Service Control Points (SCPs).  Such interconnections shall be established pursuant to Attachment 3 of this Agreement .  The appropriate charge for access to and use of the BellSouth CNAM Database service shall be as set forth in this Attachment.

3.02	In order to formulate a CNAM query to be sent to the BellSouth CNAM SCP, Premiere Premiere shall  provide its own CNAM SSP.  Premiere’s CNAM SSPs must be compliant with TR-NWT-001188, "CLASS Calling Name Delivery Generic Requirements".

3.03	If Premiere elects to access the BellSouth CNAM SCP via a third party CCS7 transport provider, the third party CCS7 provider shall interconnect with the BellSouth CCS7 network according to BellSouth's Common Channel Signaling Interconnection Guidelines and Telcordia (formerly BellCore)'s CCS Network Interface Specification document, TR-TSV-000905.  In addition, the third party provider shall establish CCS7 interconnection at the BellSouth Local Signal Transfer Points (LSTPs) serving the BellSouth CNAM SCPs that Premiere desires to query.

3.04	Out-Of-Region Customers

If the customer queries the BellSouth CNAM SCP via a third party national SS7 transport provider, the third party SS7 provider shall interconnect with the BellSouth CCS7 network according to BellSouth's Common Channel Signaling Interconnection Guidelines and Bellcore's CCS Network Interface Specification document, TR-TSV-000905. In addition, the third party provider shall establish SS7 interconnection at one or more of the BellSouth Gateway Signal Transfer Points (STPs).  The payment of all costs associated with the transport of SS7 signals via a third party will be established by mutual agreement of the Parties and writing shall, by this reference become an integral part of this Agreement.

# 4.00  CNAM RECORD INITIAL LOAD AND UPDATES

4.01	The mechanism to be used by Premiere for initial CNAM record load and/or updates shall be determined by mutual agreement.    The initial load and all updates shall be provided by Premiere in the BellSouth specified format and shall contain records for every working telephone number that can originate phone calls.  It is the responsibility of Premiere to provide accurate information to BellSouth on a current basis.

4.02	Updates to the SMS shall occur no less than once a week, reflect service order activity affecting either name or telephone number, and involve only record additions, deletions or changes.

4.03	Premiere CNAM records provided for storage in the BellSouth CNAM SCP shall be available, on a SCP query basis only, to all Parties querying the BellSouth CNAM SCP.  Further, CNAM service shall be provided by each Party consistent with state and/or federal regulation.