III.	Residential

Conservation Programs

III.	RESIDENTIAL CONSERVATION PROGRAMS

Progress Energy Florida's DSM Plan includes five (5) residential programs:

- Home Energy Check  -  residential energy audits

- Home Energy Improvement  -  "umbrella" program for existing homes

- New Construction  -  "umbrella" program for new residential construction, multi-family, and manufactured homes

- Low Income Weatherization Assistance Program  -  "umbrella" program for the weatherization of low income family homes

- Residential Energy Management  -  residential load control

Each program is described in detail in the following sections.

A.	Home Energy Check Program

Program Start Date:	▸ 1995

Policies and Procedures

The Home Energy Check is PEF’s residential energy audit program, which provides its customers with an analysis of their current energy use and recommendations on how they can save on their electricity bill through low-cost or no-cost energy-saving practices and measures.  It also serves as the foundation of the Home Energy Improvement program in that it serves as a prerequisite for participation in any of the retrofit-type components of the Home Energy Improvement program.  The exception is an emergency replacement of high efficient heat pump(s).  This requirement exists so that PEF can:  1) provide the customer with an overview of typical energy use, 2) verify that the action requested (e.g., additional attic insulation) will address the customer's problem, and 3) help to minimize free ridership in the Home Energy Improvement program.

The Home Energy Check program offers PEF customers the following types of audits:

Type 1: Free Walk-Through Audit (Home Energy Check)

Type 2: Customer-completed Mail In Audit (Do It Yourself Home Energy Check)

Type 3: Online Home Energy Check (Internet Option)-a customer-completed audit

Type 4: Phone Assisted Audit –A customer assisted survey of structure and appliance use

Type 5: Paid Walk-Through Audit (Computer Assisted Audit)

Type 6: Home Energy Rating Audit (Class I, II, III).

All residential customers of PEF are eligible to receive any of the above energy audits.  There is no charge for Type 1 through Type 4 audits, while there is a $15 customer charge for the Type 5 audit.  When a customer requests a Home Energy Check, they will be given the option of either receiving a Type 2 audit survey in the mail or scheduling a Type 1 or Type 5 walk-through audit.  A PEF auditor will usually conduct the audit, although PEF may also work with other agencies and/or utilities as an extension of PEF’s services, in which case an approved auditor from another organization may conduct the audit.  The Home Energy Rating as outlined in PEF’s "Florida Energy Gauge Ratings" rate tariff (Section II, sheet number 2.6) is available to all eligible PEF customers upon request.

Cumulative participation estimates for the program are shown in the following table.

| Year | Total Number of Customers [1] | Total Number of Eligible Customers | Annual Number of Program Participants [2] | Cumulative Penetration Level (%) Calculated |
| --- | --- | --- | --- | --- |
| 2005 | 1,382,699 | 1,382,699 | 27,500 | 2% |
| 2006 | 1,406,712 | 1,406,712 | 54,500 | 4% |
| 2007 | 1,431,102 | 1,431,102 | 81,500 | 6% |
| 2008 | 1,455,971 | 1,455,971 | 108,000 | 7% |
| 2009 | 1,481,124 | 1,481,124 | 134,000 | 9% |
| 2010 | 1,505,866 | 1,505,866 | 154,000 | 10% |
| 2011 | 1,529,665 | 1,529,665 | 174,000 | 11% |
| 2012 | 1,552,660 | 1,552,660 | 194,500 | 13% |
| 2013 | 1,575,153 | 1,575,153 | 216,000 | 14% |
| 2014 | 1,597,449 | 1,597,449 | 238,000 | 15% |

1.	Total Number of Customers is the forecast of all residential customers, from the November 2003 Forecast.

2.	Annual Number of Program Participants is the projected number of cumulative energy audits that will be conducted.

Savings Estimates

The total program savings were developed by estimating impacts for each audit level and for low-cost energy efficiency measures promoted through the program.  The total program savings are shown in the following table.

| At the Meter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Per Customer kWh | Per Customer Winter kW Reduction | Per Customer Summer kW Reduction | Total Annual kWh | Total Annual Winter kW Reduction | Total Annual Summer kW Reduction |
| Year | Reduction |  |  | Reduction |  |  |
| 2005 | 155 | 0.047 | 0.047 | 4,258,500 | 1,304 | 1,304 |
| 2006 | 156 | 0.048 | 0.048 | 8,475,500 | 2,595 | 2,595 |
| 2007 | 156 | 0.048 | 0.048 | 12,692,500 | 3,886 | 3,886 |
| 2008 | 155 | 0.047 | 0.047 | 16,744,500 | 5,126 | 5,126 |
| 2009 | 154 | 0.047 | 0.047 | 20,631,500 | 6,315 | 6,315 |
| 2010 | 155 | 0.048 | 0.048 | 23,897,000 | 7,315 | 7,315 |
| 2011 | 156 | 0.048 | 0.048 | 27,162,500 | 8,316 | 8,316 |
| 2012 | 156 | 0.048 | 0.048 | 30,346,000 | 9,290 | 9,290 |
| 2013 | 155 | 0.047 | 0.047 | 33,489,000 | 10,251 | 10,251 |
| 2014 | 154 | 0.047 | 0.047 | 36,550,000 | 11,186 | 11,186 |
| At the Generator |  |  |  |  |  |  |
|  | Per Customer kWh | Per Customer Winter kW Reduction | Per Customer Summer kW Reduction | Total Annual kWh | Total Annual Winter kW Reduction | Total Annual Summer kW Reduction |
| Year | Reduction |  |  | Reduction |  |  |
| 2005 | 165 | 0.051 | 0.051 | 4,550,509 | 1,393 | 1,393 |
| 2006 | 166 | 0.051 | 0.051 | 9,056,672 | 2,772 | 2,772 |
| 2007 | 166 | 0.051 | 0.051 | 13,562,835 | 4,152 | 4,152 |
| 2008 | 166 | 0.051 | 0.051 | 17,892,684 | 5,477 | 5,477 |
| 2009 | 165 | 0.051 | 0.051 | 22,046,219 | 6,747 | 6,747 |
| 2010 | 166 | 0.051 | 0.051 | 25,535,637 | 7,817 | 7,817 |
| 2011 | 167 | 0.051 | 0.051 | 29,025,055 | 8,886 | 8,886 |
| 2012 | 167 | 0.051 | 0.051 | 32,426,851 | 9,927 | 9,927 |
| 2013 | 166 | 0.051 | 0.051 | 35,785,369 | 10,954 | 10,954 |
| 2014 | 164 | 0.051 | 0.051 | 39,056,264 | 11,953 | 11,953 |

Per customer impacts vary from year to year because of the changing mix of audit participants in the various audit levels, as well as the mix of low-cost measures assumed to be installed in any given year.

Impact Evaluation Plan

The range of possible recommendations resulting from the audit, and the inclusion of both technological and behavioral recommendations suggests the need to survey Home Energy Check participants to determine what specific conservation actions have been implemented within each market segment due to the completed audit.  These impacts are currently being evaluated, and these survey results, combined with the participant-specific data gathered during the audit, will be used to determine the savings that can be directly attributable to the Home Energy Check program.

B.	Home Energy Improvement Program

Program Start Date:	▸ 1995

▸ Program modified in 2000

▸ Proposed modification for 2005

Policies and Procedures

The Home Energy Improvement program is the umbrella program to increase energy efficiency for existing residential homes.  It combines efficiency improvements to the thermal envelope with upgraded electric appliances.  The program seeks to meet the following overall goals:

- Improve customer comfort levels through efficient equipment and home thermal integrity upgrades.

- Obtain energy and demand impacts that are accurate, sustainable, and measurable.

- Enhance contractor awareness of new technologies.

- Educate customers about additional opportunities associated with an energy efficient home.

- Obtain cost effective resources from the marketplace.

- Minimize “lost opportunities” in the existing home market.

The program provides incentives for attic insulation upgrades, duct testing and repair, and high efficiency heat pumps.   The program eligibility requirements to qualify for participation are as follows:

- The home must be metered by Progress Energy Florida, Inc.

- The home is required to have a residential energy audit prior to participation for the attic insulation and duct test and repair.

- Duct repairs must be sealed with mastic meeting UL 181 specifications consistent with duct manufacturer’s requirements.

- New construction homes do not qualify under this program.

- High efficiency heat pump incentives will be paid for replacing existing electric heat pumps and/or electric resistance heat.

Incentive Levels and specific eligibility requirements for each measure promoted in this program will be presented in the “Program Participation Standards.”

Attic Insulation Upgrade

This portion of the program encourages customers to add insulation to the ceiling area by paying a portion of the installed cost.  The home must have an existing insulation level of less than R-12 to participate.  The customer must have either whole house electric cooling or whole house electric heating to be eligible for this program.  The maximum incentive available will be $100 per residence; the specific incentive is determined by the resulting insulation level.

Duct Test and Repair

This portion of the program is designed to encourage eligible customers to improve their central duct system by reducing the air leakage rate.  This is accomplished by performing a duct leakage test, then offering to repair the leakage that is discovered by the duct test.  The home must have centrally-ducted electric cooling and electric heat to participate in this measure.  For a duct test, PEF will pay up to a maximum of $30 for the first unit and $20 for each additional unit at the same address.  For the duct repair, PEF will pay an incentive of up to $100 per unit.  For multi-family rental units, PEF will pay all the costs up to $100 per unit (top floor only) and no test is required.

High Efficiency Electric Heat Pumps

For high efficiency electric heat pumps, PEF will provide an incentive up to $350 per unit.  The specific incentive available is dependent upon the efficiency level of the unit installed and the type of electric heat the new equipment is replacing.  In order to qualify for an incentive both the air handler and the outdoor condensing unit shall be replaced, and both units shall be new.  This program seeks to accommodate emergency replacement situations by allowing a participant to have a Home Energy Check conducted after the installation and still be eligible for the incentive.

High Efficiency Alternate Water Heating

Progress Energy has determined that it is no longer cost-effective under the RIM test to continue offering incentives for alternate water heating. These measures have relatively low rates of participation and have small overall impacts.

Supplemental Incentive Bonus

To maximize the implementation of energy efficiency measures per participant, an incentive bonus is provided to high efficiency electric heat pump participants who also implement ceiling insulation upgrade, duct leakage repair, or both, within 90 days, before or after, of the installation date of the high efficiency electric heat pump.  The purpose of this incentive is to offset some of the customer’s large capital outlay to install more than one energy efficiency measure.  The maximum incentive bonus a customer can receive is $50.

Financing

PEF is offering as an alternative to the incentives, a financing option.  The financing option is an interest free (12 Month) installment-billing plan.  As an alternative to receiving an incentive, the customer may choose to finance their energy efficient measure for up to one-year interest free.

Cumulative participation estimates for the program are shown in the following table.

| Year | Total Number of Customers [1] | Total Number of Eligible Customers [2] | Annual Number of Measure Participants [3] | Cumulative Penetration Level (%) |
| --- | --- | --- | --- | --- |
| 2005 | 1,382,699 | 69,135 | 14,530 | 21% |
| 2006 | 1,406,712 | 139,471 | 24,475 | 18% |
| 2007 | 1,431,102 | 211,026 | 34,746 | 16% |
| 2008 | 1,455,971 | 283,824 | 45,669 | 16% |
| 2009 | 1,481,124 | 357,880 | 59,985 | 17% |
| 2010 | 1,505,866 | 433,174 | 69,279 | 16% |
| 2011 | 1,529,665 | 509,657 | 83,203 | 16% |
| 2012 | 1,552,660 | 587,290 | 97,127 | 17% |
| 2013 | 1,575,153 | 666,048 | 110,531 | 17% |
| 2014 | 1,597,449 | 745,920 | 123,935 | 17% |

- Total Number of Customers is the forecast of all residential customers, from the November 2003 Forecast.

- Total number of Eligible Customers is based on an estimate of the cumulative number of central heat pumps and air conditioners that are replaced each year.

- Annual number of Measure Participants is the projected number of cumulative measure installations from all measures promoted through this program.  Because customers can install multiple measures, the actual number of participants will be less.

Savings Estimates

Total program savings were developed by first estimating the total savings for each individual measure based on each measure’s (1) per customer savings and, (2) annual projected participation.  The total program savings were then computed as the sum of the individual measure savings, and are shown in the following tables.

| At the Meter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Per Customer kWh | Per Customer Winter kW Reduction | Per Customer Summer kW Reduction | Total Annual kWh | Total Annual Winter kW Reduction | Total Annual Summer kW Reduction |
| Year | Reduction |  |  | Reduction |  |  |
| 2005 | 824 | 1.311 | 0.580 | 11,971,615 | 19,052 | 8,424 |
| 2006 | 656 | 1.234 | 0.422 | 16,048,091 | 30,211 | 10,335 |
| 2007 | 643 | 1.202 | 0.427 | 22,352,961 | 41,780 | 14,820 |
| 2008 | 640 | 1.194 | 0.428 | 29,227,729 | 54,522 | 19,531 |
| 2009 | 643 | 1.202 | 0.427 | 36,648,374 | 68,503 | 24,330 |
| 2010 | 648 | 1.216 | 0.425 | 44,923,866 | 84,243 | 29,468 |
| 2011 | 657 | 1.237 | 0.423 | 54,624,103 | 102,912 | 35,171 |
| 2012 | 662 | 1.252 | 0.421 | 64,324,340 | 121,582 | 40,875 |
| 2013 | 666 | 1.262 | 0.420 | 73,622,097 | 139,440 | 46,411 |
| 2014 | 669 | 1.269 | 0.419 | 82,919,854 | 157,298 | 51,948 |
| At the Generator |  |  |  |  |  |  |
|  | Per Customer kWh | Per Customer Winter kW Reduction | Per Customer Summer kW Reduction | Total Annual kWh | Total Annual Winter kW Reduction | Total Annual Summer kW Reduction |
| Year | Reduction |  |  | Reduction |  |  |
| 2005 | 880 | 1.4011 | 0.6195 | 12,792,519 | 20,358 | 9,002 |
| 2006 | 701 | 1.3190 | 0.4512 | 17,148,522 | 32,282 | 11,043 |
| 2007 | 687 | 1.2849 | 0.4558 | 23,885,722 | 44,645 | 15,836 |
| 2008 | 684 | 1.2757 | 0.4570 | 31,231,899 | 58,261 | 20,870 |
| 2009 | 687 | 1.2846 | 0.4562 | 39,161,384 | 73,201 | 25,998 |
| 2010 | 693 | 1.2994 | 0.4545 | 48,004,333 | 90,019 | 31,488 |
| 2011 | 702 | 1.3217 | 0.4517 | 58,369,723 | 109,969 | 37,583 |
| 2012 | 708 | 1.3376 | 0.4497 | 68,735,114 | 129,919 | 43,678 |
| 2013 | 712 | 1.3481 | 0.4487 | 78,670,426 | 149,002 | 49,594 |
| 2014 | 715 | 1.3562 | 0.4479 | 88,605,738 | 168,085 | 55,510 |

Impact Evaluation Plan

The impact evaluation plan for an “umbrella” program such as this requires a varied approach given the number and type of measures being promoted.  Some measures provide large per unit impacts while other yield relatively smaller impacts.  The total impact from all smaller-impact measures could be potentially less than the uncertainty around an impact estimate of just one large measure.  Consequently, the impact evaluation will place greater emphasis on the larger impact measures.  The method of impact evaluation may vary depending on the participation levels actually achieved for each measure.  Engineering analysis and statistical billing analysis represents the primary methods that will be used to estimate demand and energy impacts.  These analyses will be supported by residential end-use metering data.

Cost-Effectiveness

The economic results of the program are as follows:

| Cost-Effectiveness Test | NPV Benefits $(000) | NPV Costs $(000) | NPV Net Benefits $(000) | B/C Ratio |
| --- | --- | --- | --- | --- |
| Rate Impact Measure | 95,438 | 90,520 | 4,917 | 1.05 |
| Participant | 86,790 | 20,377 | 66,414 | 4.26 |
| Total Resource Cost | 95,438 | 24,107 | 71,331 | 3.96 |

C.	New Construction

Program Start Date:	▸ 1995

▸  Program modified in 2000

▸ Program modified in 2004

Policies and Procedures

The New Construction program is an "umbrella" program for the New Construction, Multi-family, and Manufactured Home building segments.

The New Construction program promotes energy efficient construction in order to provide customers with more efficient dwellings combined with improved environmental comfort.

The objectives of the program include the following goals:

- Educate builders and builder/owners and property managers about energy efficient construction design to increase the supply of energy efficient homes.

- Educate customers and realtors about energy efficient construction design to increase the demand for energy efficient homes.

- Obtain energy and demand impacts that are accurate, sustainable, and measurable.

- Enhance contractor awareness of new technologies.

- Obtain cost effective resources from the marketplace.

- Minimize “lost opportunities” in the new home market.

The program provides education and information to the design community on energy efficient equipment and construction.  The program provides the following:

- Financial incentives for energy efficient equipment.

- “Third party” endorsement/certification and PEF’s seal of approval.

- Cooperative advertising for the most energy efficient builders.

The program facilitates the design and construction of energy efficient homes by working directly with the builders to comply with program requirements.  Builders that express interest in participating in this program will be required to fulfill pre-qualification requirements.  Then, as builders inform  PEF regarding their plans to design and build additional homes, PEF representatives will provide assistance to ensure that the design and construction of the home(s) meet program requirements.  Home certification criteria include the following:

- The home must be metered by Progress Energy Florida, Inc. .

- The builder must meet requirements listed in the Program Participation Standards.

- The heating source must be an all electric heat pump.  No resistance heat is allowed except as back up heat.  Straight air with electric strip is not allowed to participate.

- Duct sealing integrity, insulation levels, and equipment efficiencies, sizing and installations must meet specific program requirements.

This program has three levels of participation with various options within each level.  The builder is offered a choice of energy efficiency measures that more closely meet the home’s design criteria.  Program details such as builder qualification criteria, home certification requirements and incentive levels for high efficient equipment promoted by this program will be presented in the Program Participation Standards.

High Efficiency Electric Heat Pumps

For electric heat pumps,  PEF will provide an incentive up to a maximum of $300 per unit.  The specific incentive amount is dependent on the energy efficiency of the equipment.  The Program Participation Standards will specify additional qualifying criteria for incentive eligibility.

High Efficiency Alternate Electric Water Heating

Progress Energy has determined that it is no longer cost-effective under the RIM test to continue offering incentives for alternate water heating. These measures have relatively low rates of participation and have small overall impacts.

Program Participation

Cumulative participation estimates for the program are shown in the following table.

| Year | Total Number of Customers [1] | Total Number of Eligible Customers [2] | Annual Number of Measure Participants [3] | Cumulative Penetration Level (%) |
| --- | --- | --- | --- | --- |
| 2005 | 1,382,699 | 27,654 | 11,718 | 42% |
| 2006 | 1,406,712 | 55,788 | 22,927 | 41% |
| 2007 | 1,431,102 | 84,410 | 34,286 | 41% |
| 2008 | 1,455,971 | 113,530 | 44,796 | 39% |
| 2009 | 1,481,124 | 143,152 | 55,311 | 39% |
| 2010 | 1,505,866 | 173,270 | 65,832 | 38% |
| 2011 | 1,529,665 | 203,863 | 76,358 | 37% |
| 2012 | 1,552,660 | 234,916 | 86,891 | 37% |
| 2013 | 1,575,153 | 266,419 | 97,430 | 37% |
| 2014 | 1,597,449 | 298,368 | 107,972 | 36% |

- Total Number of Customers is the forecast of all residential customers, from the November 2003 Forecast.

- Total number of eligible new homes constructed in  PEF's territory.

- Annual Number of Measure Participants is the projected number of cumulative measure applications from all measures promoted by this program.  Because customer can install multiple measures, the actual number of participants will be less.

Savings Estimates

Total program savings were developed by first estimating the total savings for each individual measure based on each measure’s (1) per customer savings and, (2) annual projected participation.  The total program savings were then computed as the sum of the individual measure savings, and are shown in the following tables.

| At the Meter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Per Customer kWh | Per Customer Winter kW Reduction | Per Customer Summer kW Reduction | Total Annual kWh | Total Annual Winter kW Reduction | Total Annual Summer kW Reduction |
| Year | Reduction |  |  | Reduction |  |  |
| 2005 | 401 | 0.963 | 0.283 | 4,704,138 | 11,287 | 3,314 |
| 2006 | 408 | 0.982 | 0.278 | 9,350,301 | 22,523 | 6,370 |
| 2007 | 410 | 0.986 | 0.279 | 14,067,143 | 33,796 | 9,579 |
| 2008 | 416 | 1.000 | 0.283 | 18,638,492 | 44,785 | 12,691 |
| 2009 | 419 | 1.007 | 0.285 | 23,172,852 | 55,676 | 15,777 |
| 2010 | 421 | 1.011 | 0.287 | 27,708,806 | 66,568 | 18,864 |
| 2011 | 423 | 1.017 | 0.288 | 32,307,809 | 77,626 | 21,995 |
| 2012 | 426 | 1.023 | 0.290 | 36,984,156 | 88,887 | 25,181 |
| 2013 | 428 | 1.029 | 0.292 | 41,718,930 | 100,300 | 28,407 |
| 2014 | 431 | 1.037 | 0.294 | 46,548,436 | 111,962 | 31,700 |
| At the Generator |  |  |  |  |  |  |
|  | Per Customer kWh | Per Customer Winter kW Reduction | Per Customer Summer kW Reduction | Total Annual kWh | Total Annual Winter kW Reduction | Total Annual Summer kW Reduction |
| Year | Reduction |  |  | Reduction |  |  |
| 2005 | 429 | 1.0292 | 0.3022 | 5,026,705 | 12,061 | 3,542 |
| 2006 | 436 | 1.0497 | 0.2969 | 9,991,459 | 24,067 | 6,807 |
| 2007 | 438 | 1.0533 | 0.2985 | 15,031,739 | 36,114 | 10,236 |
| 2008 | 445 | 1.0683 | 0.3027 | 19,916,549 | 47,856 | 13,561 |
| 2009 | 448 | 1.0756 | 0.3048 | 24,761,834 | 59,493 | 16,858 |
| 2010 | 450 | 1.0805 | 0.3062 | 29,608,822 | 71,133 | 20,157 |
| 2011 | 452 | 1.0863 | 0.3078 | 34,523,183 | 82,949 | 23,503 |
| 2012 | 455 | 1.0931 | 0.3097 | 39,520,191 | 94,982 | 26,907 |
| 2013 | 458 | 1.1000 | 0.3116 | 44,579,632 | 107,177 | 30,355 |
| 2014 | 461 | 1.1081 | 0.3137 | 49,740,301 | 119,640 | 33,874 |

Impact Evaluation Plan

The Residential New Construction program includes the installation of varied types of measures.  As such, the impact evaluation plan should address interactive effects of multiple measures.  In order to capture the impacts of these measures, engineering simulations and statistical billing analysis will represent the primary methods used to estimate demand and energy impacts, although the specific method may vary depending on measure-specific participation levels.  These analyses may be supported by residential end-use metering data, where feasible.

Cost-Effectiveness

The economic results of the program are as follows:

| Cost-Effectiveness Test | NPV Benefits $(000) | NPV Costs $(000) | NPV Net Benefits $(000) | B/C Ratio |
| --- | --- | --- | --- | --- |
| Rate Impact Measure | 33,885 | 26,410 | 7,475 | 1.28 |
| Participant | 22,269 | 10,891 | 11,379 | 2.04 |
| Total Resource Cost | 33,885 | 15,032 | 18,854 | 2.25 |

D.	Low Income Weatherization Assistance Program

Program Start Date:	▸ 2000

Policies and Procedures

The Low-Income Weatherization Assistance program (LIWAP) is the umbrella program to improve energy efficiency for low-income customers in existing residential housing.  It combines efficiency improvements to the thermal envelope with upgraded electric appliances.  The program seeks to meet the following goals:

- Integrate PEF's LIWAP procedures with the Department of Community Affairs (DCA) and local weatherization providers to deliver energy efficiency measures to low-income families.

- Identify and educate contractors and low income customers about energy saving opportunities to improve home energy efficiency.

- Increase low-income families’ participation in PEF's DSM programs.

- Minimize "lost opportunities" in the existing marketplace.

The program provides incentives for attic insulation upgrades, duct testing and repair, reduced air infiltration, water heater wrap, HVAC maintenance, high efficiency heat pumps, heat recovery units, and dedicated heat pump water heaters.  The program eligibility requirements to qualify for participation are as follows:

- The residence must be in PEF's service area and be a residential PEF metered customer.

- Must meet Florida's weatherization low-income criteria in addition to income requirements required by DCA.

- Homes must be greater than two years old.

- Homes having previously received PEF incentives for listed measures are not eligible for the same measure.  Attic insulation and duct repairs have special exemptions as outlined in the Home Energy Improvement Program.

- A DCA approved provider or their approved contractors must perform all work.  PEF approved contractors may be used.

Incentive levels and specific eligibility requirements for each measure promoted in this program will be presented in the Program Participation Standards.

Attic Insulation Upgrade

This portion of the program encourages customers to add insulation to the ceiling area by paying a portion of the installed cost.  The home must have an existing insulation level of less than R-12 to participate.  The customer must have either whole house electric cooling or electric heating to be eligible for this program.  The maximum incentive available will be $100 per residence, the specific incentive is determined by the resulting insulation level.

Duct Test and Repair

This portion of the program is designed to encourage eligible customers to improve their central duct system by reducing the air leakage rate.  This is accomplished by performing a duct leakage test, then offering to repair the leakage that is discovered by the duct test.  The home must have central ducted electric cooling and electric heat to participate in this measure.  For a duct test, PEF will pay up to a maximum of $30 for the first unit and $20 for each additional unit at the same address.  For the duct repair, PEF will pay an incentive of up to $100 per unit.

Reduced Air Infiltration

The weatherization provider must demonstrate a minimum reduction of air infiltration into the home of 1500 cfm at 50 pascals to receive a $75 incentive.  The home must not exceed ASHRAE Standard 92.2-1989 for acceptable indoor air quality.

Water Heater Wrap/Replacement

The weatherization provider will wrap the water heater with an insulation value of at least R-6 side and R-8 top and insulate the pipes a minimum of 3 feet extending from the tank.  The temperature will be set down to 120 degrees.  To defray the cost of purchasing a high efficiency water heater, in lieu of installing an insulating jacket, the same $25 incentive would apply.

High Efficiency Electric Heat Pumps

For high efficient electric heat pumps, PEF will provide an incentive up to $350 per unit.  The specific incentive available is dependent upon the efficiency level of the unit installed and the type of electric heat the new equipment is replacing.  In order to qualify for an incentive, both the air handler and the outdoor condensing unit shall be replaced, and both units shall be new.  This program seeks to accommodate emergency replacement situations by allowing a participant to have a home energy audit conducted after the installation and still be eligible for the incentive.

High Efficiency Alternate Water Heating

The high efficiency water heating portion of this program promotes technologies that heat water more efficiently than a standard electric water heater and save energy.  The incentive depends on the type of technology being installed.  For heat recovery units, PEF will provide an incentive of $100 per residence.  For dedicated heat pump water heaters, PEF will provide an incentive of $200 per unit.

Heating and Air Conditioning Maintenance

To maximize efficiency a $40 incentive will be provided for a Heating & Air Conditioning contractor to perform service/tune-up maintenance on existing electric central heating and air conditioning systems.

Program Participation

Cumulative participation estimates for the program are shown in the following table.

| Year | Total Number of Customers [1] | Total Number of Eligible Customers [2] | Annual Number of Measure Participants [3] | Cumulative Penetration Level (%) |
| --- | --- | --- | --- | --- |
| 2005 | 1,382,699 | 1500 | 416 | 28% |
| 2006 | 1,406,712 | 3026 | 832 | 27% |
| 2007 | 1,431,102 | 4579 | 1248 | 27% |
| 2008 | 1,455,971 | 6158 | 1664 | 27% |
| 2009 | 1,481,124 | 7765 | 2080 | 27% |
| 2010 | 1,505,866 | 9398 | 2496 | 27% |
| 2011 | 1,529,665 | 11058 | 2912 | 26% |
| 2012 | 1,552,660 | 12742 | 3328 | 26% |
| 2013 | 1,575,153 | 14451 | 3744 | 26% |
| 2014 | 1,597,449 | 16184 | 4160 | 26% |

- Total Number of Customers is the forecast of all residential customers, from the November 2003 Forecast.

- Total number of Eligible Customers that are weatherized by local weatherization assistance providers.

- Annual Number of Measure Participants is the projected number of cumulative measure installations from all measures promoted by this program.  Because customers can install multiple measures, the actual number of participants will be less.

Savings Estimates

Total program savings were developed by first estimating the total savings for each individual measure based on each measure’s (1) per customer savings and, (2) annual projected participation.  The total program savings were then computed as the sum of the individual measure savings, and are shown in the following tables.

| At the Meter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Per Customer kWh | Per Customer Winter kW Reduction | Per Customer Summer kW Reduction | Total Annual kWh | Total Annual Winter kW Reduction | Total Annual Summer kW Reduction |
| Year | Reduction |  |  | Reduction |  |  |
| 2005 | 473 | 0.677 | 0.248 | 196,669 | 281 | 103 |
| 2006 | 473 | 0.677 | 0.248 | 393,338 | 563 | 206 |
| 2007 | 473 | 0.677 | 0.248 | 590,007 | 844 | 310 |
| 2008 | 473 | 0.677 | 0.248 | 786,676 | 1,126 | 413 |
| 2009 | 473 | 0.677 | 0.248 | 983,345 | 1,407 | 516 |
| 2010 | 473 | 0.677 | 0.248 | 1,180,014 | 1,689 | 619 |
| 2011 | 473 | 0.677 | 0.248 | 1,376,683 | 1,970 | 723 |
| 2012 | 473 | 0.677 | 0.248 | 1,573,352 | 2,252 | 826 |
| 2013 | 473 | 0.677 | 0.248 | 1,770,021 | 2,533 | 929 |
| 2014 | 473 | 0.677 | 0.248 | 1,966,690 | 2,814 | 1,032 |
| At the Generator |  |  |  |  |  |  |
|  | Per Customer kWh | Per Customer Winter kW Reduction | Per Customer Summer kW Reduction | Total Annual kWh | Total Annual Winter kW Reduction | Total Annual Summer kW Reduction |
| Year | Reduction |  |  | Reduction |  |  |
| 2005 | 505 | 0.7229 | 0.2652 | 210,155 | 301 | 110 |
| 2006 | 505 | 0.7229 | 0.2652 | 420,310 | 601 | 221 |
| 2007 | 505 | 0.7229 | 0.2652 | 630,464 | 902 | 331 |
| 2008 | 505 | 0.7229 | 0.2652 | 840,619 | 1,203 | 441 |
| 2009 | 505 | 0.7229 | 0.2652 | 1,050,774 | 1,504 | 552 |
| 2010 | 505 | 0.7229 | 0.2652 | 1,260,929 | 1,804 | 662 |
| 2011 | 505 | 0.7229 | 0.2652 | 1,471,083 | 2,105 | 772 |
| 2012 | 505 | 0.7229 | 0.2652 | 1,681,238 | 2,406 | 883 |
| 2013 | 505 | 0.7229 | 0.2652 | 1,891,393 | 2,707 | 993 |
| 2014 | 505 | 0.7229 | 0.2652 | 2,101,548 | 3,007 | 1,103 |

Impact Evaluation Plan

The impact evaluation plan for an “umbrella” program such as this requires a varied approach given the number and type of measures being promoted.  Some measures provide large per unit impacts while other yield relatively smaller impacts.  The total impact from all smaller-impact measures could be potentially less than the uncertainty around an impact estimate of just one large measure.  Consequently, the impact evaluation will place greater emphasis on the larger impact measures.  The method of impact evaluation may vary depending on the participation levels actually achieved for each measure.  Engineering analysis and statistical billing analysis represents the primary methods that will be used to estimate demand and energy impacts.  These analyses will be supported by residential end-use metering data.

Cost-Effectiveness

The economic results of the program are as follows:

| Cost-Effectiveness Test | NPV Benefits $(000) | NPV Costs $(000) | NPV Net Benefits $(000) | B/C Ratio |
| --- | --- | --- | --- | --- |
| Rate Impact Measure | 1,484 | 1,472 | 12 | 1.01 |
| Participant | 1,292 | 0 | 1,292 | NA |
| Total Resource Cost | 1,484 | 1,180 | 1,304 | 8.24 |

E.	Residential Energy Management Program

Program Start Date:	▸ 1981

▸ Program modified in 1995

▸ Program modified in  2000

Policies and Procedures

Residential Energy Management is a voluntary customer program that allows PEF to reduce peak demand and defer generation construction.  Peak demand is reduced by interrupting service to selected electrical equipment with radio controlled switches installed on the customers’ premises.  These controlled interruptions are at PEF's option, during specified time periods, and coincident with hours of peak demand.

Winter-Only Energy Management

The Winter-Only Energy Management component of the program represents a modified, cost-effective version of the previous year-round Residential Energy Management program.  It provides for winter only (November through March) direct load control of customer’s electric water heating and central electric heating appliances.  Eligible participants must have both appliances on the program and will receive monthly credits during the potential control months.  The amount of the credits are identical to those under the previous year-round Residential Energy Management program (rate schedule RSL-1) except that they are payable only during the winter months.

The winter Only Energy Management component will enable PEF to continue to provide customers a cost-effective alternative to standard residential service that can help lower their electric bills as well as reduce PEF’s winter peak demand.

Program Participation

Cumulative program participation estimates beginning in the year 2000 are shown in the following table, and reflect new equipment installations under the Winter-Only Energy Management component of the program.  There are no new participants (i.e., new Energy Management installations) projected for the Year-Round Energy Management component.

| Year | Total Number of Customers [1] | Total Number of Eligible Customers [2] | Annual Number of Program Participants | Cumulative Penetration Level (%) |
| --- | --- | --- | --- | --- |
| 2005 | 1,382,699 | 1,030,016 | 5,000 | 0.49% |
| 2006 | 1,406,712 | 1,065,266 | 9,500 | 0.89% |
| 2007 | 1,431,102 | 1,098,861 | 14,000 | 1.27% |
| 2008 | 1,455,971 | 1,131,190 | 18,300 | 1.62% |
| 2009 | 1,481,124 | 1,162,301 | 22,600 | 1.94% |
| 2010 | 1,505,866 | 1,191,704 | 26,900 | 2.26% |
| 2011 | 1,529,665 | 1,219,697 | 31,200 | 2.56% |
| 2012 | 1,552,660 | 1,246,468 | 35,500 | 2.85% |
| 2013 | 1,575,153 | 1,272,359 | 39,800 | 3.13% |
| 2014 | 1,597,449 | 1,297,713 | 44,800 | 3.45% |

- Total Number of Customers is the forecast of all residential customers, from the November 2003 Forecast.

- Total numbers of eligible customers are all residential customers not already on the Residential Energy Management program.

Savings Estimates

The total program savings shown in the following tables reflect the demand and energy savings associated with the new program participants projected for the Winter-Only Energy Management Program.

| At the Meter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Per Customer kWh | Per Customer Winter kW Reduction | Per Customer Summer kW Reduction | Total Annual kWh | Total Annual Winter kW Reduction | Total Annual Summer kW Reduction |
| Year | Reduction |  |  | Reduction |  |  |
| 2005 | 2.14 | 0.000 | 0.000 | 10,700 | 0 |  |
| 2006 | 2.14 | 0.000 | 0.000 | 21,400 | 0 |  |
| 2007 | 2.14 | 0.000 | 0.000 | 31,030 | 0 |  |
| 2008 | 2.14 | 0.000 | 0.000 | 40,660 | 0 |  |
| 2009 | 2.14 | 0.000 | 0.000 | 49,862 | 0 |  |
| 2010 | 2.14 | 0.000 | 0.000 | 59,064 | 0 |  |
| 2011 | 2.14 | 0.000 | 0.000 | 68,266 | 0 |  |
| 2012 | 2.14 | 0.000 | 0.000 | 77,468 | 0 |  |
| 2013 | 2.14 | 0.000 | 0.000 | 86,670 | 0 |  |
| 2014 | 2.14 | 0.000 | 0.000 | 95,872 | 0 |  |
| At the Generator |  |  |  |  |  |  |
|  | Per Customer kWh | Per Customer Winter kW Reduction | Per Customer Summer kW Reduction | Total Annual kWh | Total Annual Winter kW Reduction | Total Annual Summer kW Reduction |
| Year | Reduction |  |  | Reduction |  |  |
| 2005 | 2.29 | 0.0000 | 0.0000 | 11,434 | 0 | 0 |
| 2006 | 2.29 | 0.0000 | 0.0000 | 22,867 | 0 | 0 |
| 2007 | 2.29 | 0.0000 | 0.0000 | 33,158 | 0 | 0 |
| 2008 | 2.29 | 0.0000 | 0.0000 | 43,448 | 0 | 0 |
| 2009 | 2.29 | 0.0000 | 0.0000 | 53,281 | 0 | 0 |
| 2010 | 2.29 | 0.0000 | 0.0000 | 63,114 | 0 | 0 |
| 2011 | 2.29 | 0.0000 | 0.0000 | 72,947 | 0 | 0 |
| 2012 | 2.29 | 0.0000 | 0.0000 | 82,780 | 0 | 0 |
| 2013 | 2.29 | 0.0000 | 0.0000 | 92,613 | 0 | 0 |
| 2014 | 2.29 | 0.0000 | 0.0000 | 102,446 | 0 | 0 |

Impact Evaluation Plan

PEF is in the process of conducting a residential end-use metering study that will be used to estimate the appliance level, and duty-cycle impacts of residential load control.  This end-use metering data will be used to perform engineering and statistical analysis to estimate the impacts of the program.

Cost-Effectiveness

The following economic results for the current winter-only Residential Energy Management program:

| Cost-Effectiveness Test | NPV Benefits $(000) | NPV Costs $(000) | NPV Net Benefits $(000) | B/C Ratio |
| --- | --- | --- | --- | --- |
| Rate Impact Measure | 31,320 | 20,728 | 10,591 | 1.51 |
| Participant | 10,050 | 360 | 9,690 | 28 |
| Total Resource Cost | 30,960 | 10,678 | 20,281 | 2.90 |

Contractors, builders, builder/owners, and property managers are synonymous.

Exception would be for multi-family housing above three stories in height.