Attachment 2

Network Elements and Other Services

TABLE OF CONTENTS

1. Introduction	3

2. UNBUNDLED LOOPS	4

3. INTEGRATED DIGITAL LOOP CARRIERS	8

4.  NETWORK INTERFACE DEVICE ........................................……...…................................. 8

5.  UNBUNDLED LOOP CONCENTRATION (ULC) SYSTEM ........………........................... 9

6. SUB-LOOP ELEMENTS......................................................................….....………...............10

7. LOCAL SWITCHING	13

8. TRANSPORT	18

9.  TANDEM SWITCHING …………………………………………………………..………   23

10. Operator Systems	25

11. SIGNALING	31

12. Signaling Transfer Points (STPs)	32

13. Service Control Points/Databases	37

14. DARK FIBER	44

15. SS7 NETWORK INTERCONNECTION ...	45

16. BASIC 911 AND E911	49

17.  RATES…………………………………………..……………………………………….….50

EXHIBIT A – LIDB STORAGE AGREEMENT……………………………………EXHIBIT A

EXHIBIT B – CNAM DATABASE SERVICES……………………………………EXHIBIT B

EXHIBIT C – RATES .………….…………………………………………………..EXHIBIT C

ACCESS TO NETWORK ELEMENTS AND OTHER SERVICES

The rates, terms and conditions contained within this Attachment were negotiated as a whole and each rate, term and condition within the Attachment is interdependent upon the other rates, terms and conditions.

# Introduction

1.1	Network Element is defined to mean a facility or equipment used in the provision of a telecommunications service.  Such term may include, but is not limited to, features, functions, and capabilities that are provided by means of such facility or equipment, including but not limited to, subscriber numbers, databases, signaling systems, and information sufficient for billing and collection or used in the transmission, routing, or other provision of a telecommunications service.  BellSouth offers access to the Network Elements, unbundled loops; network interface device; sub-loop elements; local switching; transport; tandem switching; operator systems; signaling; access to call-related databases; dark fiber as set forth in this Attachment.

### 1.2	BellSouth shall, upon request of CLEC-1, and to the extent technically feasible, provide to CLEC-1 access to its  network elements for the provision of CLEC-1’s telecommunications service.  If no rate is identified in the contract, the rate for the specific service or function will be as set forth in applicable BellSouth tariff or as negotiated by the Parties upon request by either Party.

### 1.3	CLEC-1 may purchase network elements and other services from BellSouth for the purpose of combining such network elements in any manner CLEC-1 chooses to provide telecommunication services to its intended users, including recreating existing BellSouth services.  With the exception of the sub-loop elements which are located outside of the central office, BellSouth shall deliver the  network elements purchased by CLEC-1 for combining to the designated CLEC-1 collocation space.  The network elements shall be provided as set forth in this Attachment.

### 1.4	BellSouth will provide the following combined  network elements for purchase by CLEC-1.  The rate of the following combined  network elements is the sum of the individual element prices as set forth in this Attachment.  Order Coordination as defined in Section 2 of Attachment 2 of this Agreement is available for each of these combinations:

- SL2 loop and cross connect

- Port and cross connect

- Port and cross connect and common (shared) transport

- Port and vertical features

- SL2 Loop with loop concentration

- Port and common (shared) transport

- SL2 Loop and LNP

### 1.5	BellSouth shall comply with the requirements as set forth in the technical references within Attachment 2 to the extent that they are consistent with the greater of BellSouth’s actual performance or applicable industry standards.

1.6	In the event that any final and nonappealable legislative, regulatory, judicial or other legal action modifies or redefines the “Network Elements” in a manner which materially affects the terms of this Attachment or the Network Elements and/or prices set forth herein, either Party may, on thirty (30) days written notice, require renegotiation of such terms, and the Parties shall renegotiate in good faith such new terms in accordance with such legislative, regulatory, judicial or other legal action.  In the event such new terms are not renegotiated within ninety (90) days after the notice for renegotiation, either Party may petition the Commission for resolution of the dispute between the Parties.  Each Party reserves the right to seek judicial review of any Commission ruling concerning this Attachment.

1.7	CLEC-1 will adopt and adhere to the standards contained in the applicable CLEC Work Center Operational Understanding Agreement regarding maintenance and installation of service.

# 2.	Unbundled Loops

### 2.1.1	BellSouth agrees to offer access to  loops pursuant to the following terms and conditions and at the rates set forth in this Attachment.

## 2.2	Definition

### The loop is the physical medium or functional path on which a subscriber’s traffic is carried from the MDF or similar terminating device in a central office up to the termination at the NID at the customer’s premise.  Each  loop will be provisioned with a NID.

### The provisioning of service to a CLEC will require cross-office cabling and cross-connections within the central office to connect the loop to a local switch or to other transmission equipment in collocation space.  These cross-connects are a separate element and are not considered a part of the loop.

BellSouth Order Coordination referenced in Attachment 2 includes two types:  “Order Coordination” and “Order Coordination - Time Specific.”

“Order Coordination” refers to standard BellSouth service order coordination involving SL2 voice loops and all digital loops.  Order coordination for physical conversions will be scheduled at BellSouth’s discretion during normal working hours on the committed due date and CLEC-1 advised.

“Order Coordination – Time Specific” refers to service order coordination in which CLEC-1 requests a specific time for a service order conversion to take place.  Loops on a single service order of 14 or more loops will be provisioned on a project basis.  This is a chargeable option for any coordinated order and is billed in addition to the OC charge.  CLEC-1 may specify a time between 8:00 a.m. and 5:00 p.m. (location time) Monday through Friday (excluding holidays).  If CLEC-1 specifies a time outside this window, or selects a time or quantity of loops that requires BellSouth technicians to work outside normal work hours, overtime charges will apply in addition to the OC and OC-TS charges.  Overtime charges will be applied according to actual costs based on type of force group required to perform the work, overtime hours worked and any special circumstances.

Where facilities are available, BellSouth will install  loops within a 5-7 business days interval.  For orders of 14 or more  loops, the installation will be handled on a project basis and the intervals will be set by the BellSouth project manager for that order.  Some  loops require a Service Inquiry (SI) to determine if facilities are available prior to issuing the order.  The interval for the SI process is separate from the installation interval.  For expedite requests by CLEC-1, expedite charges will apply for intervals less than 5 days.  The charges outlined in BST’s FCC # 1 Tariff, Section 5.1.1, will apply.  If CLEC-1 cancels an order for network elements and other services, any costs incurred by BellSouth in conjunction with the provisioning of that order will be recovered in accordance with FCC #1 Tariff, Section 5.4.”

If CLEC-1 modifies an order after being sent a Firm Order Confirmation (FOC) from BellSouth, any costs incurred by BellSouth to accommodate the modification will be reimbursed by CLEC-1.

### BellSouth will offer Unbundled Voice Loops (UVL) in two different service levels - Service Level One (SL1) and Service Level Two (SL2).  SL1 loops will be non-designed, will not have test points, and will not come with any Order Coordination (OC) or engineering information/circuit make-up data.  Upon issuance of an order in the service order system, SL1 loops will be activated on the due date in the same manner and time frames that BellSouth normally activates POTS-type loops for its customers.  If CLEC-1 requests work to be done for SL1s that requires BellSouth technicians to work outside normal work hours, overtime charges will be applied according to actual costs based on type of force group required to perform the work, overtime hours worked and any special circumstances.

### SL2 loops shall have test points, will be designed with a Design Layout Record provided to CLEC-1, and will be provided with Order Coordination.  The OC feature will allow CLEC-1 to coordinate the installation of the loop with the disconnect of an existing customer’s service and/or number portability service.  In these cases, BellSouth will perform the order conversion with standard order coordination at its discretion during normal work hours.

### 2.2.4	BellSouth will also offer Unbundled Digital Loops (UDL).  They will be designed, will be provisioned with test points (where appropriate), and will come standard with Order Coordination and a Design Layout Record (DLR).

### 2.2.5	As a chargeable option on all  loops except UVL-SL1, BellSouth will offer Order Coordination - Time Specific (OC-TS).  This will allow CLEC-1 the ability to specify the time that the coordinated conversion takes place.  The OC-TS charge for orders due on the same day at the same location will be applied on a per Local Service Request (LSR) basis.

### 2.2.6	CLEC-1 will be responsible for testing and isolating troubles on the  loops.  Once CLEC-1 has isolated a trouble to the BellSouth provided loop, CLEC-1 will issue a trouble to BellSouth on the loop.  BellSouth will take the actions necessary to repair the loop if a trouble actually exists.  BellSouth will repair these loops in the same time frames that BellSouth repairs similarly situated loops to its customers.

### 2.2.7	If CLEC-1 reports a trouble on SL1 loops and no trouble actually exists, BellSouth will charge CLEC-1 for any dispatching and testing (both inside and outside the CO) required by BellSouth in order to confirm the loop’s working status.

### 2.2.8	If CLEC-1 reports a trouble on SL2 loops and no trouble actually exists, BellSouth will charge CLEC-1 for any dispatching and testing, (outside the CO) required by BellSouth in order to confirm the loop’s working status.

## 2.3	Technical Requirements

### To the extent available within BST’s Network at a particular location, BellSouth will offer loops capable of supporting telecommunications services such as: POTS, Centrex, basic rate ISDN, analog PBX, voice grade private line, ADSL, HDSL, DS1 and digital data (up to 64 kb/s).  Additional services may include digital PBXs, primary rate ISDN, xDSL, and Nx 64 kb/s.  If a requested loop type is not available, then the CLEC can use the Special Construction process to request that BellSouth place facilities or otherwise modify facilities in order to meet the CLEC’s request.

### 2.3.1.1	The loop will support the transmission, signaling, performance and interface requirements of the services described in 2.3.1 above.  It is recognized that the requirements of different services are different, and that a number of types or grades of loops are required to support these services.  Services provided over the loop by CLEC-1 will be consistent with industry standards and BST’s TR73600.

#### 2.3.1.2	In some instances, CLEC-1 will require access to a copper twisted pair loop unfettered by any intervening equipment (e.g., filters, load coils, range extenders, etc.), so that CLEC-1 can use the loop for a variety of services by attaching appropriate terminal equipment at the ends.  CLEC-1 will determine the type of service that will be provided over the loop.  In some cases, CLEC-1 may be required to pay additional charges for the removal of certain types of equipment.  BellSouth’s Special Construction process will be used to determine the costs and feasibility of these activities.

In cases in which CLEC-1 has requested that BellSouth remove equipment from the BellSouth loop, BellSouth will no longer be expected to maintain and repair the loop to the standards specified for that loop type in the TR73600 and other standards referenced in this Agreement.

CLEC-1, in performance of its obligations pursuant to the preceding Section, shall maintain records that will reflect that pursuant to CLEC-1’s request BellSouth has removed certain equipment from BellSouth provided loops and as such the loop may not perform within the technical specifications associated with that loop type.  CLEC-1 will not report to BellSouth troubles on said loops where the loops are not performing within the technical specifications of that loop type.

In addition, CLEC-1 recognizes there may be instances where a loop modified in this manner may be subjected to normal network configuration changes that may cause the circuit characteristics to be changed and may create an outage of the service that CLEC-1 has placed on the loop.  If this occurs, BellSouth will work cooperatively with CLEC-1 to restore the circuit to its previous modified status as quickly as possible.  CLEC-1 will pay the Time and Materials costs associated with BellSouth’s work efforts needed to bring the loop back to its previous modified status.

### 2.3.2	The loop shall be provided to CLEC-1 in accordance with the following Technical References:

BellSouth’s TR73600, Unbundled Local Loop Technical Specification

#### 2.3.2.1	Telcordia (formerly BellCore) TR-NWT-000057, Functional Criteria for Digital Loop Carrier Systems, Issue 2, January 1993.

#### 2.3.2.2	Telcordia (formerly BellCore) TR-NWT-000393, Generic Requirements for ISDN Basic Access Digital Subscriber Lines.

#### 2.3.2.3	ANSI T1.102 - 1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces.

#### 2.3.2.4	ANSI T1.403 - 1989, American National Standard for Telecommunications - Carrier to Customer Installation, DS1 Metallic Interface Specification.

# 3.	Integrated Digital Loop Carriers

### 3.1.1	Where BellSouth uses Integrated Digital Loop Carrier (IDLC) systems to provide the local loop and BellSouth has a suitable alternate facility available, BellSouth will make arrangements to permit CLEC-1 to order a contiguous  local loop.  To the extent it is technically feasible, these arrangements will provide CLEC-1 with the capability to serve end users at a level that is at parity with the level of service BellSouth provides its customers.  If no alternate facility is available, BellSouth will utilize its Special Construction (SC)  process to determine the additional costs required to provision the loop facilities.  CLEC-1 will then have the option of paying the one-time SC rates to place the loop facilities or CLEC-1 may chose some other method of providing service to the end-user (e.g., Resale, private facilities, etc.)

# 4.	Network Interface Device

## 4.1	Definition

### 4.1.1	The Network Interface Device (NID) is a single-line termination device or that portion of a multiple-line termination device required to terminate a single line or circuit at the end user customer’s premises.  The fundamental function of the NID is to establish the official network demarcation point between a carrier and its end-user customer.  The NID features two independent chambers or divisions which separate the service provider’s network from the customer’s inside wiring.  Each chamber or division contains the appropriate connection points or posts to which the service provider, and the end-user customer each make their connections.  The NID provides a protective ground connection, and is capable of terminating cables such as twisted pair cable.

## 4.2	Technical Requirements

### 4.2.1	The Network Interface Device shall provide a clean, accessible point of connection for the inside wiring and for the Distribution Media and shall maintain a connection to ground that meets the requirements set forth below.

### 4.2.2	The NID shall be capable of transferring electrical analog or digital signals between the customer’s inside wiring and the Distribution Media.

### 4.2.3	All NID posts or connecting points shall be in place, secure, usable and free of any rust or corrosion.  The protective ground connection shall exist and be properly installed.  The ground wire will also be free of rust or corrosion and have continuity relative to ground.

### 4.2.4	The NID shall be capable of withstanding all normal local environmental variations.

### 4.2.5	Where feasible, the NID shall be physically accessible to CLEC-1 designated personnel.  In cases where entrance to the end user’s premises is required to give access to the NID, CLEC-1 shall obtain entrance permission directly from the end user.

### 4.2.6	BellSouth shall offer the NID as a stand-alone component.  Additionally, CLEC-1 may connect its loop to any spare capacity on the BellSouth NID. Where necessary to comply with an effective Commission order, BellSouth will allow  CLEC-1 to disconnect the BellSouth loop from the BellSouth NID in order to connect CLEC-1’s loop to the BellSouth NID.  In these cases, CLEC-1 accepts all liability associated with this process and it is CLEC-1’s responsibility to make sure the disconnected BellSouth loop is properly grounded.

## 4.3	Interface Requirements

### 4.3.1	The NID shall be equal to or better than all of the requirements for NIDs set forth in the following technical references:

#### 4.3.1.1	Telcordia (formerly BellCore) Technical Advisory TA-TSY-000120 “Customer Premises or Network Ground Wire”;

#### 4.3.1.2	Telcordia (formerly BellCore) Generic Requirement GR-49-CORE “Generic Requirements for Outdoor Telephone Network Interface Devices”;

#### 4.3.1.3	Telcordia (formerly BellCore) Technical Requirement TR-NWT-00239 “Indoor Telephone Network Interfaces”;

#### 4.3.1.4	Telcordia (formerly BellCore) Technical Requirement TR-NWT-000937 “Generic Requirements for Outdoor and Indoor Building Entrance”

# 5.	Unbundled Loop Concentration (ULC) System

### 5.1.1	BellSouth will provide to CLEC-1  loop concentration (ULC).  Loop concentration systems in the central office concentrate the signals transmitted over local loops onto a digital loop carrier system.  The concentration device is placed inside a BellSouth central office. BellSouth will offer ULC with a TR008 interface or a TR303 interface.

### 5.1.2	ULC will be offered in two sizes.  System A will allow up to 96 BellSouth loops to be concentrated onto multiple DS1s.  The high speed connection from the concentrator will be at the electrical DS1 level and may connect to CLEC-1 at CLEC-1’s collocation site.  System B will allow up to 192 BellSouth loops to be concentrated onto multiple DS1s.  System A may be upgraded to a System B.  A minimum of two DS1s is required for each system (i.e., System A requires two DS1s and System B would require an additional two DS1s or four in total ).  All DS1 interfaces will terminate to the CLEC’s collocation space.  ULC service is offered with or without concentration and with or without protection. A Line Interface element will be required for each  loop that is terminated onto the ULC system.  Rates for ULC are as set forth in this Attachment.

# 6.	Sub-loop Elements

## 6.1	Where facilities permit and where necessary to comply with an effective Commission order, BellSouth shall offer access to its Unbundled Sub-Loop (USL), Unbundled Sub-Loop Concentration (USLC) System and Unbundled Network Terminating Wire (UNTW) elements.

## 6.2	Unbundled Sub-Loop (USL)

### 6.2.1	Definition

#### 6.2.1.1	Unbundled Sub-Loop provides connectivity between the NID component of the  sub-loop and the terminal block on the customer-side of a Feeder Distribution Interface (FDI). This termination and cross-connect field may be in the form of an outside plant distribution closure or remote terminal.  Riser Cable that extends from BellSouth’s point-of-entry into a building (e.g., equipment closet, terminal room, etc.) to the NID on a particular floor or office space in a multi-tenant building is also classified as a USL. Unbundled Sub-Loops will be provisioned as 2-wire or 4-wire circuits and will include a NID.

#### 6.2.1.2	The Unbundled Sub-Loop will be copper twisted pair.  If CLEC-1 requires a copper twisted pair Unbundled Sub-Loop in instances where the Unbundled Sub-Loop for services that BellSouth offers is other than a copper facility, BellSouth will provide that media if those facilities exist.  If there are no copper facilities available, BellSouth will use its Special Construction process to determine if facilities can be provided to CLEC-1.

### 6.2.2	Requirements for All Unbundled Sub-Loops

#### 6.2.2.1	Unbundled Sub-Loops shall be capable of carrying all signaling messages or tones needed to provide telecommunications services.

### Unbundled Sub-Loop shall support functions associated with provisioning, maintenance and testing of the Unbundled Sub-Loop.  In these scenarios, CLEC-1 would be required to place a cross-box, remote terminal (RT), or other similar device and deliver a cable to the BellSouth remote terminal or cross-box.  This cable would be connected, by a BST technician, to a cross-connect panel within the BellSouth RT/cross-box.  CLEC-1’s cable pairs can then be connected to BST’s USL within the BST cross-box by the BST technician.

### 6.2.3	Interface Requirements

#### 6.2.3.1	Unbundled Sub-Loop shall be equal to or better than each of the applicable interface requirements set forth in the following technical references:

#### 6.2.3.2	Telcordia (formerly BellCore) TR-NWT-000049, “Generic Requirements for Outdoor Telephone Network Interface Devices,” Issued December 1,1994;

## 6.3	Unbundled Sub-Loop Concentration System (USLC)

### 6.3.1	Where facilities permit and where necessary to comply with an effective Commission order, BellSouth will provide to CLEC-1 with the ability to concentrate its sub-loops onto multiple DS1s back to the BellSouth Central Office.  The DS1s will then be terminated into CLEC-1’s collocation space. TR-008 and TR303 interface standards are available.

### 6.3.2	USLC, using the Lucent Series 5 equipment, will be offered in two different systems.  System A will allow up to 96 of CLEC-1’s sub-loops to be concentrated onto multiple DS1s.  System B will allow an additional 96 of CLEC-1’s sub-loops to be concentrated onto multiple DS1s.  One System A may be supplemented with one System B and they both must be physically located in a single Series 5 dual channel bank.  A minimum of two DS1s is required for each system (i.e., System A requires two DS1s and System B would require an additional two DS1s or four in total).  The DS1 level facility that connects the RT site with the serving wire center is known as a Feeder Interface.  All DS1 Feeder Interfaces will terminate to the CLEC’s collocation space within the SWC that serves the RT where the CLEC’s sub-loops are connected.  USLC service is offered with or without concentration and with or without a protection DS1.

### 6.3.3	In these scenarios CLEC-1 would be required to place a cross-box, remote terminal (RT), or other similar device and deliver a cable to the BellSouth remote terminal.  This cable would be connected, by a BellSouth technician, to a cross-connect panel within the BellSouth RT/cross-box and would allow CLEC-1’s sub-loops to then be placed on the ULSC and transported to their collocation space at a DS1 level.

## 6.4	Unbundled Network Terminating Wire (UNTW)

### 6.4.1	BellSouth agrees to offer its Unbundled Network Terminating Wire (UNTW) to CLEC-1 pursuant to the following terms and conditions at rates as set forth in this Attachment.

## 6.5	Definition

### UNTW is twisted copper wire that extends from BellSouth’s point-of-entry into a multi-dwelling unit (MDU) complex or multi-tenant unit (MTU) complex to the point of demarcation at the end-users location.  The UNTW will not include a Network Interface Device (NID).

## 6.6	Requirements

### 6.6.1	BellSouth will retain the first pair of NTW going into each end-user premises.  BellSouth will offer spare pairs that are available to an end-users premises to CLEC-1.  Available spare pairs are defined as pairs that are not being utilized by BellSouth or by a third party to provide an end-user with working service at the time of CLEC-1’s request for UNTW.  If no spare pairs are available and the end-user is no longer using BellSouth’s local service, BellSouth will relinquish the first pair to CLEC-1.  If after BellSouth has relinquished the first pair to CLEC-1 and the end-user decides to change local service providers to BellSouth, CLEC-1 will relinquish the first pair back to BellSouth.

    - Notwithstanding the foregoing, should BellSouth subsequently require the use of additional pair(s) to provide for the activation of additional lines in an end-users premises in response to a request from such end-user, CLEC-1 agrees to surrender their spare pair(s) upon request by BellSouth.

    - If an end-user of CLEC-1 desires to receive local exchange service from a service provider who is not a Party to this Agreement, and such third party service provider needs access to the BellSouth UNTW to provide local exchange service to the end user, then CLEC-1 agrees to surrender the requisite number of its inactive spare pair(s) if no other spare pair is available and upon request by BellSouth.

    - If CLEC-1 has placed NTW at a location and an end-user desires to receive local exchange service from BellSouth and BellSouth needs access to CLEC-1’s NTW to provide local exchange service to the end-user, then CLEC-1 agrees to surrender the requisite number of its spare pair(s) upon request by BellSouth.

6.6.5	In new construction, where possible, both Parties may at their option and with the property owner’s agreement install their own NTW.  In existing construction, BellSouth shall not be required to install new or additional NTW beyond existing NTW to provision the services of the CLEC.

## 6.7	Technical Requirements

### 6.7.1	In these scenarios, BellSouth will connect the requested UNTW pairs to a cross-connect panel designed for CLEC access to BellSouth’s NTW.   CLEC-1 will be required to place a cross-box, terminal, or other similar device and deliver a cable to this cross-connect panel.  CLEC-1 will then connect their cable to the cross-connect panel to access the requested UNTW pairs.

# 7.	Local Switching

BellSouth agrees to offer access to local switching pursuant to the following terms and conditions and at the rates set forth in this Attachment.

## 7.1	Definition

### 7.1.1	Local Switching is the Network Element that provides the functionality required to connect the appropriate originating lines or trunks wired to the Main Distributing Frame (MDF) or Digital Cross Connect (DSX) panel to a desired terminating line or trunk. Such functionality shall include access to all of the features, functions, and capabilities that the underlying BellSouth switch that is providing such Local Switching function is then capable of providing, including but not limited to: line signaling and signaling software, digit reception, dialed number translations, call screening, routing, recording, call supervision, dial tone, switching, telephone number provisioning, announcements, calling features and capabilities (including call processing), CENTREX, Automatic Call Distributor (ACD), Carrier pre-subscription (e.g. long distance carrier, intraLATA toll), Carrier Identification Code (CIC) portability capabilities, testing and other operational features inherent to the switch and switch software.  It also provides access to transport, signaling (ISDN User Part (ISUP)) and Transaction Capabilities Application Part (TCAP), and platforms such as adjuncts, Public Safety Systems (911), operator services, Directory Assistance Services and Advanced Intelligent Network (AIN).  Remote Switching Module functionality is included in the Local Switching function.  The switching capabilities used will be based on the line side features they support.  Local Switching will also be capable of routing local, intraLATA, interLATA, and calls to international customer’s preferred carrier; call features (e.g., call forwarding) and CENTREX capabilities.  Where required to do so in order to comply with an effective Commission order, Local Switching, including the ability to route to CLEC-1’s transport facilities, dedicated facilities and systems, shall be unbundled from all other Network Elements and other services, i.e., Operator Systems, Common (Shared) Transport, and Dedicated Transport.  BellSouth and CLEC-1 shall continue to work with the appropriate industry groups to develop a long-term solution for selective routing.

### 7.1.2	A featureless port is one that has a line port, switching functionality, and an interoffice port.  A featured port is a port that includes all features then capable or a number of then capable features specifically requested by CLEC-1.  Any features that are not currently then capable but are technically feasible through the switch can be requested through the BFR process.

### 7.1.3	Where required to do so in order to comply with an effective Commission order, BellSouth will provide to CLEC-1 purchasing  local BellSouth switching and reselling BellSouth local exchange service under Attachment 1, selective routing of calls to a requested directory assistance services platform or operator services platform.  CLEC-1 customers may use the same dialing arrangements as BellSouth customers, but obtain a CLEC-1 branded service.

## 7.2	Technical Requirements

### 7.2.1	The requirements set forth in this Section apply to Local Switching, but not to the Data Switching function of Local Switching.

#### 7.2.1.1	Local Switching shall be equal to or better than the requirements for Local Switching set forth in Telcordia (formerly BellCore)’s Local Switching Systems General Requirements (FR-NWT-000064).

#### 7.2.1.2	When applicable, BellSouth shall route calls to the appropriate trunk or lines for call origination or termination.

#### 7.2.1.3	Subject to this section, BellSouth shall route calls on a per line or per screening class basis to (1) BellSouth platforms providing Network Elements or additional requirements (2) Operator Services platforms, (3) Directory Assistance platforms, and (4) Repair Centers.  Any other routing requests by CLEC-1 will be made pursuant to the Bona Fide Request/ New Business Request Process as set forth in General Terms and Conditions.

#### 7.2.1.4	BellSouth shall provide unbranded recorded announcements and call progress tones to alert callers of call progress and disposition.

#### 7.2.1.5	BellSouth shall activate service for an CLEC-1 customer or network interconnection on any of the Local Switching interfaces.  This includes provisioning changes to change a customer from BellSouth’s services to CLEC-1’s services without loss of switch feature functionality as defined in this Agreement.

#### 7.2.1.6	BellSouth shall perform routine testing (e.g., Mechanized Loop Tests (MLT) and test calls such as 105, 107 and 108 type calls) and fault isolation on a mutually agreed upon schedule.

#### 7.2.1.7	BellSouth shall repair and restore any equipment or any other maintainable component that may adversely impact Local Switching.

#### 7.2.1.8	BellSouth shall control congestion points such as those caused by radio station call-ins, and network routing abnormalities.  All traffic shall be restricted in a non discriminatory manner.

#### 7.2.1.9	BellSouth shall perform manual call trace and permit customer originated call trace.

#### 7.2.1.10	Special Services provided by BellSouth will include the following:

##### 7.2.1.10.1	Telephone Service Prioritization;

##### 7.2.1.10.2	Related services for handicapped;

##### 7.2.1.10.3	Soft dial tone where required by law; and

##### 7.2.1.10.4	Any other service required by law.

#### 7.2.1.11	BellSouth shall provide Switching Service Point (SSP) capabilities and signaling software to interconnect the signaling links destined to the Signaling Transfer Point Switch (STP).  These capabilities shall adhere to Telcordia (formerly BellCore) specifications - TCAP (GR-1432-CORE), ISUP(GR-905-CORE), Call Management (GR-1429-CORE), Switched Fractional DS1 (GR-1357-CORE), Toll Free Service (GR-1428-CORE), Calling Name (GR-1597-CORE), Line Information Database (GR-954-CORE), and Advanced Intelligent Network (GR-2863-CORE).

#### 7.2.1.12	BellSouth shall provide interfaces to adjuncts through Telcordia (formerly BellCore) standard interfaces.  These adjuncts can include, but are not limited to, the Service Circuit Node and Automatic Call Distributors.

#### 7.2.1.13	BellSouth shall provide performance data regarding a customer line, traffic characteristics or other measurable elements to CLEC-1, upon a reasonable request from CLEC-1.  CLEC will pay BellSouth for all costs incurred to provide such performance data through the Business Opportunity Request process.

#### 7.2.1.14	BellSouth shall offer Local Switching that provides feature offerings at parity to those provided by BellSouth to itself or any other Party.  Such feature offerings shall include but are not limited to:

##### 7.2.1.14.1	Basic and primary rate ISDN;

##### 7.2.1.14.2	Residential features;

##### 7.2.1.14.3	Customer Local Area Signaling Services (CLASS/LASS);

##### 7.2.1.14.4	CENTREX (including equivalent administrative capabilities, such as customer accessible reconfiguration and detailed message recording); and

##### 7.2.1.14.5	Advanced intelligent network triggers supporting CLEC-1 and BellSouth service applications.

BellSouth shall offer to CLEC-1 all AIN triggers in connection with its SMS/SCE offering which are supported by BellSouth for offering AIN-based services. Triggers that are currently available are:

###### 7.2.1.14.5.1	Off-Hook Immediate

###### 7.2.1.14.5.2	Off-Hook Delay

###### 7.2.1.14.5.3	Termination Attempt

###### 7.2.1.14.5.4	6/10 Public Office Dialing Plan

###### 7.2.1.14.5.5	Feature Code Dialing

###### 7.2.1.14.5.6	Customer Dialing Plan

##### 7.2.1.14.6	When the following triggers are supported by BellSouth, BellSouth will make these triggers available to CLEC-1:

###### 7.2.1.14.6.1	Private EAMF Trunk

###### 7.2.1.14.6.2	Shared Interoffice Trunk (EAMF, SS7)

###### 7.2.1.14.6.3	N11

###### 7.2.1.14.6.4	Automatic Route Selection

#### 7.2.1.15	Where capacity exists, BellSouth shall assign each CLEC-1 customer line the class of service designated by CLEC-1 (e.g., using line class codes or other switch specific provisioning methods), and shall route directory assistance calls from CLEC-1 customers to CLEC-1 directory assistance operators at CLEC-1’s option.

#### 7.2.1.16	Where capacity exists, BellSouth shall assign each CLEC-1 customer line the class of services designated by CLEC-1 (e.g., using line class codes or other switch specific provisioning methods) and shall route operator calls from CLEC-1 customers to CLEC-1 operators at CLEC-1’s option.  For example, BellSouth may translate 0- and 0+ intraLATA traffic, and route the call through appropriate trunks to an CLEC-1 Operator Services Position System (OSPS).  Calls from Local Switching must pass the ANI-II digits unchanged.

#### 7.2.1.17	Local Switching shall be offered in accordance with the requirements of the following technical references:

##### 7.2.1.17.1	Telcordia (formerly BellCore) GR-1298-CORE, AIN Switching System Generic Requirements, as implemented in BellSouth’s switching equipment;

##### 7.2.1.17.2	Telcordia (formerly BellCore) GR-1299-CORE, AIN Switch-Service Control Point (SCP)/Adjunct Interface Generic Requirements;

##### 7.2.1.17.3	Telcordia (formerly BellCore) TR-NWT-001284, AIN 0.1 Switching System Generic Requirements;

##### 7.2.1.17.4	Telcordia (formerly BellCore) SR-NWT-002247, AIN Release 1 Update.

### 7.2.2	Interface Requirements

#### 7.2.2.1	BellSouth shall provide the following interfaces to loops:

#### 7.2.2.2	Standard Tip/Ring interface including loopstart or groundstart, on-hook signaling (e.g., for calling number, calling name and message waiting lamp);

#### 7.2.2.3	Coin phone signaling;

#### 7.2.2.4	Basic Rate Interface ISDN adhering to appropriate Telcordia (formerly BellCore) Technical Requirements;

#### 7.2.2.5	Two-wire analog interface to PBX;

##### 7.2.2.5.1	Four-wire analog interface to PBX;

#### 7.2.2.6	Four-wire DS1 interface to PBX or customer provided equipment (e.g. computers and voice response systems);

#### 7.2.2.7	Primary Rate ISDN to PBX adhering to ANSI standards Q.931, Q.932 and appropriate Telcordia (formerly BellCore) Technical Requirements;

#### 7.2.2.8	Switched Fractional DS1 with capabilities to configure Nx64 channels (where N = 1 to 24); and

#### 7.2.2.9	Loops adhering to Telcordia (formerly BellCore) TR-NWT-08 and TR-NWT-303 specifications to interconnect Digital Loop Carriers.

#### 7.2.2.10	BellSouth shall provide access to the following but not limited to:

#### 7.2.2.11	SS7 Signaling Network or Multi-Frequency trunking if requested by  CLEC-1;

#### 7.2.2.12	Interface to CLEC-1 operator services systems or Operator Services through appropriate trunk interconnections for the system; and

#### 7.2.2.13	Interface to CLEC-1 directory assistance services through the CLEC-1 switched network or to Directory Assistance Services through the appropriate trunk interconnections for the system; and 950 access or other CLEC-1 required access to interexchange carriers as requested through appropriate trunk interfaces.

# 8.	Transport

BellSouth agrees to offer access to  transport including Common (Shared) Transport, Dedicated Transport and Tandem Switching pursuant to following terms and conditions and at the rates set forth in this Attachment.

## 8.1	Definition of Common (Shared) Transport

Common (Shared) Transport is an interoffice transmission path between two BellSouth end-offices, BellSouth end-office and a local tandem, or between two local tandems.  Where BellSouth Network Elements are connected by intra-office wiring, such wiring is provided as a part of the Network Elements and is not Common (Shared) Transport.  Common (Shared) Transport consists of BellSouth inter-office transport facilities and is unbundled from local switching.

## 8.2	Technical Requirements of Common (Shared) Transport

### 8.2.1	Common (Shared) Transport provided on DS1 or VT1.5 circuits, shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for Central Office to Central Office (“CO to CO”) connections in the appropriate industry standards.

### 8.2.2	Common (Shared) Transport provided on DS3 circuits, STS-1 circuits, and higher transmission bit rate circuits, shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for CO to CO connections in the appropriate industry standards.

### 8.2.3	BellSouth shall be responsible for the engineering, provisioning, and maintenance of the underlying equipment and facilities that are used to provide Common (Shared) Transport.

### 8.2.4	At a minimum, Common (Shared) Transport shall meet all of the requirements set forth in the following technical references (as applicable for the transport technology being used):

#### 8.2.4.1	ANSI T1.101-1994, American National Standard for Telecommunications - Synchronization Interface Standard Performance and Availability;

#### 8.2.4.2	ANSI T1.102-1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces;

#### 8.2.4.3	ANSI T1.102.01-199x, American National Standard for Telecommunications - Digital Hierarchy - VT1.5;

#### 8.2.4.4	ANSI T1.105-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Basic Description including Multiplex Structure, Rates and Formats;

#### 8.2.4.5	ANSI T1.105.01-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Automatic Protection Switching;

#### 8.2.4.6	ANSI T1.105.02-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Payload Mappings;

#### 8.2.4.7	ANSI T1.105.03-1994, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Jitter at Network Interfaces;

#### 8.2.4.8	ANSI T1.105.03a-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET): Jitter at Network Interfaces - DS1 Supplement;

#### 8.2.4.9	ANSI T1.105.05-1994, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Tandem Connection;

#### 8.2.4.10	ANSI T1.105.06-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Physical Layer Specifications;

#### 8.2.4.11	ANSI T1.105.07-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Sub STS-1 Interface Rates and Formats;

#### 8.2.4.12	ANSI T1.105.09-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Network Element Timing and Synchronization;

#### 8.2.4.13	ANSI T1.106-1988, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (Single Mode);

#### 8.2.4.14	ANSI T1.107-1988, American National Standard for Telecommunications - Digital Hierarchy - Formats Specifications;

#### 8.2.4.15	ANSI T1.107a-1990 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications (DS3 Format Applications);

#### 8.2.4.16	ANSI T1.107b-1991 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications;

#### 8.2.4.17	ANSI T1.117-1991, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (SONET) (Single Mode - Short Reach);

#### 8.2.4.18	ANSI T1.403-1989, Carrier to Customer Installation, DS1 Metallic Interface Specification;

#### 8.2.4.19	ANSI T1.404-1994, Network-to-Customer Installation - DS3 Metallic Interface Specification;

#### 8.2.4.20	ITU Recommendation G.707, Network node interface for the synchronous digital hierarchy (SDH);

#### 8.2.4.21	ITU Recommendation G.704, Synchronous frame structures used at 1544, 6312, 2048, 8488 and 44736 kbit/s hierarchical levels;

#### 8.2.4.22	Telcordia (formerly BellCore) FR-440 and TR-NWT-000499, Transport Systems Generic Requirements (TSGR):  Common Requirements;

#### 8.2.4.23	Telcordia (formerly BellCore) GR-820-CORE, Generic Transmission Surveillance:  DS1 & DS3 Performance;

#### 8.2.4.24	Telcordia (formerly BellCore) GR-253-CORE, Synchronous Optical Network Systems (SONET);  Common Generic Criteria;

#### 8.2.4.25	Telcordia (formerly BellCore) TR-NWT 000507, Transmission, Section 7, Issue 5 (Telcordia (formerly BellCore), December 1993). (A module of LSSGR, FR-NWT-000064.);

#### 8.2.4.26	Telcordia (formerly BellCore) TR-NWT-000776, Network Interface Description for ISDN Customer Access;

#### 8.2.4.27	Telcordia (formerly BellCore) TR-INS-000342, High-Capacity Digital Special Access Service-Transmission Parameter Limits and lnterface Combinations, Issue 1 February 1991;

#### 8.2.4.28	Telcordia (formerly BellCore) ST-TEC 000052, Telecommunications Transmission Engineering Textbook, Volume 2: Facilities, Third Edition, Issue I May 1989;

#### 8.2.4.29	Telcordia (formerly BellCore) ST-TEC-000051, Telecommunications Transmission Engineering Textbook Volume 1:  Principles, Third Edition. Issue 1 August 1987.

## 8.3	Dedicated Transport

### 8.3.1	Definition

#### 8.3.1.1	Dedicated Transport is defined as BellSouth transmission facilities dedicated to a particular customer or carrier that provide telecommunications between wire centers owned by BellSouth or requesting telecommunications carriers, or between switches owned by BellSouth or requesting telecommunications carriers.

#### 8.3.1.2	BellSouth shall offer Dedicated Transport in each of the following ways:

##### 8.3.1.2.1	As capacity on a shared facility.

##### 8.3.1.2.2	As a circuit (e.g., DS0, DS1 or DS3) dedicated to CLEC-1.

#### 8.3.1.3	When Dedicated Transport is provided as a system it shall include:

##### 8.3.1.3.1	Transmission equipment such as multiplexers, line terminating equipment, amplifiers, and regenerators;

##### 8.3.1.3.2	Inter-office transmission facilities such as optical fiber, copper twisted pair, and coaxial cable.

### 8.3.2	Unbundled Local Channel

      - The Unbundled Local Channel is the dedicated transmission path between CLEC-1’s Point of Presence and the BellSouth Serving Wire Center.

      - BellSouth currently offers Unbundled Local Channels for switched traffic.  Rates for these elements are listed in this Attachment.  For those states that do not contain rates in this Attachment for DS1 and DS3 switched Local Channels, the rates in the applicable State Access Tariff will apply as interim rates.  When final rates are developed, these interim rates will be subject to true-up, and the Parties will amend the Agreement to reflect the new rates.

8.3.2.3	BellSouth currently offers Unbundled Local Channels for non-switched traffic at DS1 and DS3 levels at rates as set forth in Exhibit C to this Attachment.

### 8.3.3	Technical Requirements

This Section sets forth technical requirements for all Dedicated Transport.

#### 8.3.3.1	When BellSouth provides Dedicated Transport as a circuit or a system, the entire designated transmission circuit or system (e.g., DS0, DS1,DS3) shall be dedicated to CLEC-1 designated traffic.

#### 8.3.3.2	BellSouth shall offer Dedicated Transport in all technologies that become available including, but not limited to, DS1 and DS3 transport systems, SONET (or SDH) Bi-directional Line Switched Rings, SONET (or SDH) Unidirectional Path Switched Rings, and SONET (or SDH) point-to-point transport systems (including linear add-drop systems), at all available transmission bit rates.  While SONET Ring facilities are not available in every application, they are typically available in the major metropolitan areas.

#### 8.3.3.3	For DS1 or VT1.5 circuits, Dedicated Transport shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for Customer Interface to Central Office (“CI to CO”) connections in the appropriate industry standards.

#### 8.3.3.4	Where applicable, for DS3 circuits, Dedicated Transport shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for CI to CO connections in the appropriate industry standards.

#### 8.3.3.5	BellSouth shall offer the following interface transmission rates for Dedicated Transport:

##### 8.3.3.5.1	DS0 Equivalent;

##### 8.3.3.5.2	DS1 (Extended SuperFrame - ESF and D4 channel bank shall be provided);

##### 8.3.3.5.3	DS3 where applicable (M13 multiplexer shall be provided);

##### 8.3.3.5.4	SDH Standard interface rates in accordance with International Telecommunications Union (ITU) Recommendation G.707 and Plesiochronous Digital Hierarchy (PDH) rates per ITU Recommendation G.704.

#### 8.3.3.6	When Dedicated Transport is provided as a system, BellSouth shall design the system according to our network infrastructure to allow for the termination points specified by CLEC-1.

### 8.3.4	At a minimum, Dedicated Transport shall meet each of the requirements set forth in the following technical references:

#### 8.3.4.1	ANSI T1.231-1993 -American National Standard for Telecommunications - Digital Hierarchy - Layer 1 In-Service Digital Transmission performance monitoring.

##### 8.3.4.1.1	ANSI T1.102-1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces;

##### 8.3.4.1.2	ANSI T1.106-1988, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (Single Mode);

##### 8.3.4.1.3	ANSI T1.107-1988, American National Standard for Telecommunications - Digital Hierarchy - Formats Specifications;

##### 8.3.4.1.4	ANSI T1.107a-1990 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications (DS3 Format Applications);

##### 8.3.4.1.5	ANSI T1.107b-1991 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications;

##### 8.3.4.1.6	Telcordia (formerly BellCore) FR-440 and TR-NWT-000499, Transport Systems Generic Requirements (TSGR):  Common Requirements;

##### 8.3.4.1.7	Telcordia (formerly BellCore) GR-820-CORE, Generic Transmission Surveillance:  DS1 & DS3 Performance;

##### 8.3.4.1.8	Telcordia (formerly BellCore) TR-NWT 000507, Transmission, Section 7, Issue 5 (Telcordia (formerly BellCore), December 1993). (A module of LSSGR, FR-NWT-000064.);

##### 8.3.4.1.9	Telcordia (formerly BellCore) TR-INS-000342, High-Capacity Digital Special Access Service-Transmission Parameter Limits and lnterface Combinations, Issue 1 February 1991;

##### 8.3.4.1.10	Telcordia (formerly BellCore) ST-TEC 000052, Telecommunications Transmission Engineering Textbook, Volume 2: Facilities, Third Edition, Issue I May 1989;

##### 8.3.4.1.11	Telcordia (formerly BellCore) ST-TEC-000051, Telecommunications Transmission Engineering Textbook Volume 1:  Principles, Third Edition. Issue 1 August 1987;

## 9.	Tandem Switching

### 9.1	Definition

Tandem Switching is the function that establishes a communications path between two switching offices through a third switching office (the Tandem switch).

### 9.2	Technical Requirements

#### 9.2.1	Tandem Switching shall have the same capabilities or equivalent capabilities as those described in Bell Communications Research TR-TSY-000540 Issue 2R2, Tandem Supplement, 6/1/90.  The requirements for Tandem Switching include, but are not limited to the following:

##### 9.4.2.1.1	Tandem Switching shall provide signaling to establish a tandem connection;

##### 9.4.2.1.2	Tandem Switching will provide screening as jointly agreed to by CLEC-1 and BellSouth;

##### 9.4.2.1.3	Tandem Switching shall provide Advanced Intelligent Network triggers supporting AIN features where such routing is not available from the originating end office switch, to the extent such Tandem switch has such capability;

##### 9.4.2.1.4	Tandem Switching shall provide access to Toll Free number portability database as designated by CLEC-1;

##### 9.4.2.1.5	Tandem Switching shall provide all trunk interconnections discussed under the “Network Interconnection” section (e.g., SS7, MF, DTMF, DialPulse, PRI-ISDN, DID, and CAMA-ANI (if appropriate for 911));

##### 9.4.2.1.6	Tandem Switching shall provide connectivity to PSAPs where 911 solutions are deployed and the tandem is used for 911; and

##### 9.4.2.1.7	Where appropriate, Tandem Switching shall provide connectivity to transit traffic to and from other carriers.

#### 9.4.2.2	Tandem Switching shall accept connections (including the necessary signaling and trunking interconnections) between end offices, other tandems, IXCs, ICOs, CAPs and CLEC switches.

#### 9.4.2.3	Tandem Switching shall provide local tandeming functionality between two end offices including two offices belonging to different CLEC's (e.g., between a CLEC end office and the end office of another CLEC).

#### 9.4.2.4	Tandem Switching shall preserve CLASS/LASS features and Caller ID as traffic is processed.

#### 9.4.2.5	Tandem Switching shall record billable events and send them to the area billing centers designated by CLEC-1.  Tandem Switching will provide recording of all billable events as jointly agreed to by CLEC-1 and BellSouth.

#### 9.4.2.6	Upon a reasonable request from CLEC-1, BellSouth shall perform routine testing and fault isolation on the underlying switch that is providing Tandem Switching and all its interconnections.  The results and reports of the testing shall be made immediately available to CLEC-1.

#### 9.4.2.7	BellSouth shall maintain CLEC-1’s trunks and interconnections associated with Tandem Switching at least at parity to its own trunks and interconnections.

#### 9.4.2.8	BellSouth shall control congestion points and network abnormalities.  All traffic will be restricted in a non discriminatory manner.

#### 9.4.2.9	Selective Call Routing through the use of line class codes is not available through the use of tandem switching.  Selective Call Routing through the use of line class codes is an end office capability only.  Detailed primary and overflow routing plans for all interfaces available within BellSouth switching network shall be mutually agreed to by CLEC-1 and BellSouth.

#### 9.4.2.10	Tandem Switching shall process originating toll-free traffic received from CLEC-1 local switch.

#### 9.4.2.11	In support of AIN triggers and features, Tandem Switching shall provide SSP capabilities when these capabilities are not available from the Local Switching Network Element, to the extent such Tandem Switch has such capability.

### 9.4.3	Interface Requirements

#### 9.4.3.1	Tandem Switching shall provide interconnection to the E911 PSAP where the underlying Tandem is acting as the E911 Tandem.

#### 9.4.3.2	Tandem Switching shall interconnect, with direct trunks, to all carriers with which BellSouth interconnects.

#### 9.4.3.3	BellSouth shall provide all signaling necessary to provide Tandem Switching with no loss of feature functionality.

#### 9.4.3.4	Tandem Switching shall interconnect with CLEC-1's switch, using two-way trunks, for traffic that is transiting via BellSouth network to interLATA or intraLATA carriers.  At CLEC-1’s request, Tandem Switching shall record and keep records of traffic for billing.

#### 9.4.3.5	Tandem Switching shall provide an alternate final routing pattern for CLEC-1 traffic overflowing from direct end office high usage trunk groups.

### 9.4.4	Tandem Switching shall meet or exceed (i.e., be more favorable to CLEC-1) each of the requirements for Tandem Switching set forth in the following technical references:

#### 9.4.4.1	Bell Communications Research TR-TSY-000540 Issue 2R2, Tandem Supplement, 6/1/90;

#### 9.4.4.2	GR-905-CORE covering CCSNIS;

#### GR-1429-CORE for call management features; and 
GR-2863-CORE and Telcordia (formerly BellCore) GR-2902-CORE covering CCS AIN interconnection

# 10.	Operator Systems

BellSouth agrees to offer access to operator systems pursuant to the terms and conditions following and at the rates set forth in this Attachment.

## 10.1	Definition

Operator Systems is the Network Element that provides operator and automated call handling and billing, special services, end user telephone listings and optional call completion services.  The Operator Systems, Network Element provides two types of functions:  Operator Service functions and Directory Assistance Service functions, each of which are described in detail below.

## 10.2	Operator Service

### 10.2.1	Definition

Operator Service provides:  (1) operator handling for call completion (for example, collect, third number billing, and manual credit card calls), (2) operator or automated assistance for billing after the end user has dialed the called number (for example, credit card calls); and (3) special services including but not limited to Busy Line Verification and Emergency Line Interrupt (ELI), Emergency Agency Call, Operator-assisted Directory Assistance, and Rate Quotes.

### 11.2.2	Requirements

#### 10.2.2.1	When CLEC-1 requests BellSouth to provide Operator Services, the following requirements apply:

##### 10.2.2.1.1	BellSouth shall complete 0+ and 0- dialed local calls.

##### 10.2.2.1.2	BellSouth shall complete 0+ intraLATA toll calls.

##### 10.2.2.1.3	BellSouth shall complete calls that are billed to CLEC-1 end user's calling card that can be validated by BellSouth.

##### 10.2.2.1.4	BellSouth shall complete person-to-person calls.

##### 10.2.2.1.5	BellSouth shall complete collect calls.

##### 10.2.2.1.6	BellSouth shall provide the capability for callers to bill to a third party and complete such calls.

##### 10.2.2.1.7	BellSouth shall complete station-to-station calls.

##### 10.2.2.1.8	BellSouth shall process emergency calls.

##### 10.2.2.1.9	BellSouth shall process Busy Line Verify and Emergency Line Interrupt requests.

##### 10.2.2.1.10	BellSouth shall process emergency call trace, as they do for their End users prior to the Effective Date.  Call must originate from a 911 provider.

##### 10.2.2.1.11	BellSouth shall process operator-assisted directory assistance calls.

#### 10.2.2.2	BellSouth shall adhere to equal access requirements, providing CLEC-1 local end users the same IXC access as provided to BellSouth end users.

#### 10.2.2.3	BellSouth shall exercise at least the same level of fraud control in providing Operator Service to CLEC-1 that BellSouth provides for its own operator service.

#### 10.2.2.4	BellSouth shall perform Billed Number Screening when handling Collect, Person-to-Person, and Billed-to-Third-Party calls.

#### 10.2.2.5	BellSouth shall direct customer account and other similar inquiries to the customer service center designated by CLEC-1.

#### 10.2.2.6	BellSouth shall provide a feed of customer call records in “EMI” format to CLEC-1 in accordance with CLECODUF standards specified in Attachment 7.

### 10.2.3	Interface Requirements

With respect to Operator Services for calls that originate on local switching capability provided by or on behalf of CLEC-1, the interface requirements shall conform to the then current established system interface specifications for the platform used to provide Operator Service and the interface shall conform to industry standards.

## 10.3	Directory Assistance Service

### 10.3.1	Definition

Directory Assistance Service provides local end user telephone number listings with the option to complete the call at the callers direction separate and distinct from local switching.

### 10.3.2	Requirements

#### 10.3.2.1	Directory Assistance Service shall provide up to two listing requests per call.  If available and if requested by CLEC-1’s end user, BellSouth shall provide caller-optional directory assistance call completion service at rates contained in this Attachment to one of the provided listings, equal to that which BellSouth provides its end users.  If not available, CLEC-1 may request such requirement pursuant to the Bona Fide Request/New Business Process as set forth in General Terms and Conditions.

#### 10.3.2.2	Directory Assistance Service Updates

##### 10.3.2.2.1	BellSouth shall update end user listings changes daily.  These changes include:

###### 10.3.2.2.1.1	New end user connections:  BellSouth will provide service to CLEC-1 that is equal to the service it provides to itself and its end users;

###### 10.3.2.2.1.2	End user disconnections:  BellSouth will provide service to CLEC-1 that is equal to the service it provides to itself and its end users; and

###### 10.3.2.2.1.3	End user address changes:  BellSouth will provide service to CLEC-1 that is equal to the service it provides to itself and its end users;

#### 10.3.2.3	These updates shall also be provided for non-listed and non-published numbers for use in emergencies.

## 10.4	Branding for Operator Call Processing and Directory Assistance

### 10.4.1	The BellSouth Operator Systems Branding Feature provides a definable announcement to CLEC-1 end users using Directory Assistance (DA)/Operator Call Processing (OCP) prior to placing them in queue or connecting them to an available operator or automated operator system. This feature allows CLEC-1 to have its calls custom branded with CLEC-1 name on whose behalf BellSouth is providing Directory Assistance and/or Operator Call Processing.  Rates for Custom Branding, Operator Call Process and Directory Assistance are set forth in this Attachment.

### 10.4.2	BellSouth offers four service levels of branding to CLEC-1 when ordering Directory Assistance and/or Operator Call Processing.

#### 10.4.2.1	Service Level 1 - BellSouth Branding

#### 10.4.2.2	Service Level 2 - Unbranded

#### 10.4.2.3	Service Level 3 - Custom Branding

#### 10.4.2.4	Service Level 4 - Self Branding (applicable only to CLEC-1 for Resale or use with an Unbundled Port when routing to an operator service provider other than BellSouth).

### 10.4.3	For Resellers and Use with an Unbundled Port

#### 10.4.3.1	BellSouth Branding is the Default Service Level.

#### 10.4.3.2	Unbranding, Custom Branding, and Self Branding require CLEC-1 to order selective routing for each originating BellSouth end office identified by CLEC-1.  Rates for Selective Routing are set forth in this Attachment.

#### 10.4.3.3	Customer Branding and Self Branding require CLEC-1 to order dedicated trunking from each BellSouth end office identified by CLEC-1, to either the BellSouth Traffic Operator Position System (TOPS) or CLEC-1 Operator Service Provider.  Rates for trunks are set forth in applicable BellSouth tariffs.

#### 10.4.3.4	Unbranding - Unbranded Directory Assistance and/or Operator Call Processing calls ride common trunk groups provisioned by BellSouth from those end offices identified by CLEC-1 to the BellSouth TOPS.  These calls are routed to “No Announcement.”

### 10.4.4	For Facilities Based Carriers

#### 10.4.4.1	All Service Levels require CLEC-1 to order dedicated trunking from their end office(s) point of interface to the BellSouth TOPS Switches. Rates for trunks are set forth in applicable BellSouth tariffs.

#### Customized Branding includes charges for the recording of the branding announcement and the loading of the audio units in each TOPS Switch, IVS and NAV equipment for which CLEC-1 requires service

Directory Assistance customized branding uses:

- the recording of the name;

- the front-end loading of the Digital Recorded Announcement Machine (DRAM) in each TOPS switch.

Operator Call Processing customized branding uses:

- the recording of the name;

- the front-end loading of the DRAM in the TOPS Switch;

- the back-end loading in the audio units in the Automated Alternate Billing System (AABS) in the Interactive Voice Subsystem (IVS);

- the 0- automation loading for the audio units in the Enhanced Billing and Access Service (EBAS) in the Network Applications Vehicle (NAV).

10.4.4.3	BellSouth will provide to CLEC-1 purchasing  local BellSouth switching and reselling BellSouth local exchange service, selective routing of calls to a requested directory assistance services platform or operator services platform.  CLEC-1 end users may use the same dialing arrangements as BellSouth end users, but obtain a CLEC-1 branded service.

## 10.5	Directory Assistance Database Service (DADS)

### 10.5.1	BellSouth shall make its Directory Assistance Database Service (DADS) available solely for the expressed purpose of providing Directory Assistance type services to CLEC-1 end users.  The term “end user” denotes any entity which obtains Directory Assistance type services for its own use from a DADS customer.  Directory Assistance type service is defined as Voice Directory Assistance (DA Operator assisted and Electronic Directory Assistance (Data System assisted)).  CLEC-1 agrees that Directory Assistance Database Service (DADS) will not be used for any purpose which violates federal or state laws, statutes, regulatory orders or tariffs.  Except for the permitted users, CLEC-1 agrees not to disclose DADS to others and shall provide due care in providing for the security and confidentiality of DADS. Further, CLEC-1 authorizes the inclusion of CLEC-1 Subscriber listings in the BellSouth Directory Assistance products.

### 10.5.2	BellSouth shall provide CLEC-1 initially with a base file of subscriber listings which reflect all listing change activity occurring since CLEC-1’s most recent update via magnetic tape, and subsequently using electronic connectivity such as Network Data Mover to be developed mutually by CLEC-1 and BellSouth.  CLEC-1 agrees to assume the costs associated with CONNECT: Direct TM connectivity, which will vary depending upon volume and mileage.

### 10.5.3	BellSouth will require approximately one month after receiving an order to prepare the Base File.  BellSouth will provide daily updates which will reflect all listing change activity occurring since CLEC’s most recent update.  BellSouth shall provide updates to CLEC-1 on a Business, Residence, or combined Business and Residence basis.  CLEC-1 agrees that the updates shall be used solely to keep the information current.  Delivery of Daily Updates will commence the day after CLEC-1 receives the Base File.

### 10.5.4	BellSouth is authorized to include CLEC-1 Subscriber List Information in its Directory Assistance Database Service (DADS) and its Directory Publishers Database Service (DPDS).  Any other use by BellSouth of CLEC-1 Subscriber List Information is not authorized and with the exception of a request for DADS or DPDS, BellSouth shall refer any request for such information to CLEC-1.

### 10.5.5	Rates for DADS are as set forth in this Attachment.

## 10.6	Direct Access to Directory Assistance Service

### 10.6.1	Direct Access to Directory Assistance Service (DADAS) will provide CLEC-1’s directory assistance operators with the ability to search all available BellSouth’s subscriber listings using the Directory Assistance search format.  Subscription to DADAS will allow CLEC-1 to utilize its own switch, operator workstations and optional audio subsystems.

### 10.6.2	BellSouth will provide DADAS from its DA location.  CLEC-1 will access the DADAS system via a telephone company provided point of availability.  CLEC-1 has the responsibility of providing the physical links required to connect to the point of availability.  These facilities may be purchased from the telephone company as rates and charges billed separately from the charges associated with this offering.

### 10.6.3	A specified interface to each CLEC-1 subsystem will be provided by BellSouth.  Interconnection between CLEC-1 system and a specified BellSouth location will be pursuant to the use of CLEC-1 owned or CLEC-1 leased facilities and shall be appropriate sized based upon the volume of queries being generated by CLEC-1.

### 10.6.4	The specifications for the three interfaces necessary for interconnection are available in the following documents:

#### 10.6.4.1	DADAS to Subscriber Operator Position System—Northern Telecom Document CSI-2300-07; Universal Gateway/ Position Message Interface Format Specification

#### 10.6.4.2	DADAS to Subscriber Switch—Northern Telecom Document Q210-1 Version A107; NTDMS/CCIDAS System Application Protocol; and AT&T Document 250-900-535 Operator Services Position System Listing Service and Application Call Processing Data Link Interface Specification

#### 10.6.4.3	DADAS to Audio Subsystem (Optional)—Directory One Call Control to Audio Response Unit system interface specifications are available through Northern Telecom as a licensed access protocol—Northern Telecom Document 355-004424 and Gateway/Interactive Voice subsystem Protocol Specification

### 10.6.5	Rates for DADAS are as set forth in this Attachment.

# 11.	Signaling

BellSouth agrees to offer access to  signaling and access to BellSouth’s signaling databases subject to compatibility testing and at the rates set forth in this Attachment.  BellSouth may provide mediated access to BellSouth signaling systems and databases.  Available signaling elements include signaling links, signal transfer points and service control points.  Signaling functionality will be available with both A-link and B-link connectivity.

## 11.1	Definition of Signaling Link Transport

Signaling Link Transport is a set of two or four dedicated 56 Kbps. transmission paths between CLEC-designated Signaling Points of Interconnection (SPOI) that provides appropriate physical diversity.

## 11.2	Technical Requirements

### 11.2.1	Signaling Link Transport shall consist of full duplex mode 56 kbps transmission paths.

### 11.2.2	Of the various options available, Signaling Link Transport shall perform in the following two ways:

#### 11.2.2.1	As an “A-link” which is a connection between a switch or SCP and a home Signaling Transfer Point Switch (STP) pair; and

#### 11.2.2.2	As a “B-link” which is a connection between two STP pairs in different company networks (e.g., between two STP pairs for two Competitive Local Exchange Carriers (CLECs)).

### 11.2.3	Signaling Link Transport shall consist of two or more signaling link layers as follows:

#### 11.2.3.1	An A-link layer shall consist of two links.

#### 11.2.3.2	A B-link layer shall consist of four links.

### 11.2.4	A signaling link layer shall satisfy a performance objective such that:

#### 11.2.4.1	There shall be no more than two minutes down time per year for an A-link layer; and

#### 11.2.4.2	There shall be negligible (less than 2 seconds) down time per year for a B-link layer.

### 11.2.5	A signaling link layer shall satisfy interoffice and intraoffice diversity of facilities and equipment, such that:

#### 11.2.5.1	No single failure of facilities or equipment causes the failure of both links in an A-link layer (i.e., the links should be provided on a minimum of two separate physical paths end-to-end); and

#### 11.2.5.2	No two concurrent failures of facilities or equipment shall cause the failure of all four links in a B-link layer (i.e., the links should be provided on a minimum of three separate physical paths end-to-end).

## 11.3	Interface Requirements

### 11.3.1	There shall be a DS1 (1.544 Mbps) interface at the CLEC-1-designated SPOIs.  Each 56 kbps transmission path shall appear as a DS0 channel within the DS1 interface.

# 12.	Signaling Transfer Points (STPs)

## 12.1	Definition - Signaling Transfer Points is a signaling network function that includes all of the capabilities provided by the signaling transfer point switches (STPs) and their associated signaling links which enable the exchange of SS7 messages among and between switching elements, database elements and signaling transfer point switches

## 12.2	Technical Requirements

### 12.2.1	STPs shall provide access to Network Elements connected to BellSouth SS7 network. These include:

#### 12.2.1.1	BellSouth Local Switching or Tandem Switching;

#### 12.2.1.2	BellSouth Service Control Points/DataBases;

#### 12.2.1.3	Third-party local or tandem switching;

#### 12.2.1.4	Third-party-provided STPs.

### 12.2.2	The connectivity provided by STPs shall fully support the functions of all other Network Elements connected to BellSouth SS7 network.  This explicitly includes the use of BellSouth SS7 network to convey messages which neither originate nor terminate at a signaling end point directly connected to BellSouth SS7 network (i.e., transient messages).  When BellSouth SS7 network is used to convey transient messages, there shall be no alteration of the Integrated Services Digital Network User Part (ISDNUP) or Transaction Capabilities Application Part (TCAP) user data that constitutes the content of the message.

### 12.2.3	If a BellSouth tandem switch routes calling traffic, based on dialed or translated digits, on SS7 trunks between an CLEC-1 local switch and third party local switch, BellSouth SS7 network shall convey the TCAP messages that are necessary to provide Call Management features (Automatic Callback, Automatic Recall, and Screening List Editing) between CLEC-1 local STPs and the STPs that provide connectivity with the third party local switch, even if the third party local switch is not directly connected to BellSouth STPs.

### 12.2.4	STPs shall provide all functions of the MTP as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.  This includes:

#### 12.2.4.1	Signaling Data Link functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements,

#### 12.2.4.2	Signaling Link functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements, and

#### 12.2.4.3	Signaling Network Management functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.

### 12.2.5	STPs shall provide all functions of the SCCP necessary for Class 0 (basic connectionless) service, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.  In particular, this includes Global Title Translation (GTT) and SCCP Management procedures, as specified in T1.112.4.  In cases where the destination signaling point is a CLEC-1 or third party local or tandem switching system directly connected to BellSouth SS7 network, BellSouth shall perform final GTT of messages to the destination and SCCP Subsystem Management of the destination.  In all other cases, BellSouth shall perform intermediate GTT of messages to a gateway pair of STPs in an SS7 network connected with BellSouth SS7 network, and shall not perform SCCP Subsystem Management of the destination.  If BellSouth performs final GTT to a CLEC-1 database, then CLEC-1 agrees to provide BellSouth with the Destination Point Code for the CLEC-1 database.

### 12.2.6	STPs shall provide on a non-discriminatory basis all functions of the OMAP commonly provided by STPs, as specified in the reference in Section 12.4.5 of this Attachment.  All OMAP functions will be on a "where available" basis and can include:

#### 12.2.6.1	MTP Routing Verification Test (MRVT) and

#### 12.2.6.2	SCCP Routing Verification Test (SRVT).

### 12.2.7	In cases where the destination signaling point is a BellSouth local or tandem switching system or database, or is an CLEC-1 or third party local or tandem switching system directly connected to the BellSouth SS7 network, STPs shall perform MRVT and SRVT to the destination signaling point.  In all other cases, STPs shall perform MRVT and SRVT to a gateway pair of STPs in an SS7 network connected with the BellSouth SS7 network.  This requirement shall be superseded by the specifications for Internetwork MRVT and SRVT if and when these become approved ANSI standards and available capabilities of BellSouth STPs, and if mutually agreed upon by CLEC-1 and BellSouth.

### 12.2.8	STPs shall be on parity with BellSouth.

### 12.2.9	SS7 Advanced Intelligent Network (AIN) Access

#### 12.2.9.1	When technically feasible and upon request by CLEC-1, SS7 Access shall be made available in association with  switching.  SS7 AIN Access is the provisioning of AIN 0.1 triggers in an equipped BellSouth local switch and interconnection of the BellSouth SS7 network with the CLEC-1 SS7 network to exchange TCAP queries and responses with an CLEC-1 SCP.

#### 12.2.9.2	SS7 AIN Access shall provide CLEC-1 SCP access to BellSouth local switch in association with  switching via interconnection of BellSouth SS7 and CLEC-1 SS7 Networks.  BellSouth shall offer SS7 access through its STPs.  If BellSouth requires a mediation device on any part of its network specific to this form of access, BellSouth must route its messages in the same manner.  The interconnection arrangement shall result in the BellSouth local switch recognizing the CLEC-1 SCP as at least at parity with BellSouth’s SCP’s in terms of interfaces, performance and capabilities.

## 12.3	Interface Requirements

### 12.3.1	BellSouth shall provide the following STPs options to connect CLEC-1 or CLEC-1-designated local switching systems or STPs to BellSouth SS7 network:

#### 12.3.1.1	An A-link interface from CLEC-1 local switching systems; and,

#### 12.3.1.2	A B-link interface from CLEC-1 local STPs.

### 12.3.2	Each type of interface shall be provided by one or more sets (layers) of signaling links.

### 12.3.3	The Signaling Point of Interconnection (SPOI) for each link shall be located at a cross-connect element, such as a DSX-1, in the Central Office (CO) where BellSouth STP is located.  There shall be a DS1 or higher rate transport interface at each of the SPOIs.  Each signaling link shall appear as a DS0 channel within the DS1 or higher rate interface. BellSouth shall offer higher rate DS1 signaling for interconnecting CLEC-1 local switching systems or STPs with BellSouth STPs as soon as these become approved ANSI standards and available capabilities of BellSouth STPs.  BellSouth and CLEC-1 will work jointly to establish mutually acceptable SPOIs.

### 12.3.4	BellSouth CO shall provide intraoffice diversity between the SPOIs and BellSouth STPs, so that no single failure of intraoffice facilities or equipment shall cause the failure of both B-links in a layer connecting to a BellSouth STP.  BellSouth and CLEC-1 will work jointly to establish mutually acceptable SPOIs.

### 12.3.5	BellSouth shall provide MTP and SCCP protocol interfaces that shall conform to all sections relevant to the MTP or SCCP in the following specifications:

#### 12.3.5.1	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

#### 12.3.5.2	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

### 12.3.6	Message Screening

#### 12.3.6.1	BellSouth shall set message screening parameters so as to accept valid messages from CLEC-1 local or tandem switching systems destined to any signaling point within BellSouth’s SS7 network where the CLEC-1 switching system has a legitimate signaling relation.

#### 12.3.6.2	BellSouth shall set message screening parameters so as to pass valid messages from CLEC-1 local or tandem switching systems destined to any signaling point or network accessed through BellSouth’s SS7 network where the CLEC-1 switching system has a legitimate signaling relation.

#### 12.3.6.3	BellSouth shall set message screening parameters so as to accept and pass/send valid messages destined to and from CLEC-1 from any signaling point or network interconnected through BellSouth’s SS7 network where the CLEC-1 SCP has a legitimate signaling relation.

## 12.4	STPs shall be equal to or better than all of the requirements for STPs set forth in the following technical references:

### 12.4.1	ANSI T1.111-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP);

### 12.4.2	ANSI T1.111A-1994 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP) Supplement;

### 12.4.3	ANSI T1.112-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Signaling Connection Control Part (SCCP);

### 12.4.4	ANSI T1.115-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Monitoring and Measurements for Networks;

### 12.4.5	ANSI T1.116-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Operations, Maintenance and Administration Part (OMAP);

### 12.4.6	ANSI T1.118-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Intermediate Signaling Network Identification (ISNI);

### 12.4.7	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP); and

### 12.4.8	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

# 13.	Service Control Points/DataBases

## 13.1	Definition

### 13.1.1	Databases are the Network Elements that provide the functionality for storage of, access to, and manipulation of information required to offer a particular service and/or capability.  Databases include, but are not limited to:  Local Number Portability, LIDB, Toll Free Number Database, Automatic Location Identification/Data Management System, Calling Name Database, access to Service Creation Environment and Service Management System (SCE/SMS) application databases and Directory Assistance.

### 13.1.2	A Service Control Point (SCP) is a specific type of Database functionality deployed in a Signaling System 7 (SS7) network that executes service application logic in response to SS7 queries sent to it by a switching system also connected to the SS7 network.  Service Management Systems provide operational interfaces to allow for provisioning, administration and maintenance of subscriber data and service application data stored in SCPs.

## 13.2	Technical Requirements for SCPs/Databases

Requirements for SCPs/Databases within this section address storage of information, access to information (e.g. signaling protocols, response times), and administration of information (e.g., provisioning, administration, and maintenance).  All SCPs/Databases shall be provided to CLEC-1 in accordance with the following requirements.

### 13.2.1	BellSouth shall provide physical access to SCPs through the SS7 network and protocols with TCAP as the application layer protocol.

### 13.2.2	BellSouth shall provide physical interconnection to databases via industry standard interfaces and protocols (e.g. SS7, ISDN and X.25).

### 13.2.3	The reliability of interconnection options shall be consistent with requirements for diversity and survivability.

### 13.2.4	Database Availability

Call processing databases shall have a maximum unscheduled availability of 30 minutes per year.  Unavailability due to software and hardware upgrades shall be scheduled during minimal usage periods and only be undertaken upon proper notification to providers which might be impacted.  Any downtime associated with the provision of call processing related databases will impact all service providers, including BellSouth, equally.

### 13.2.5	The operational interface provided by BellSouth shall complete Database transactions (i.e., add, modify, delete) for CLEC-1 customer records stored in BellSouth databases within 3 days, or sooner where BellSouth provisions its own customer records within a shorter interval.

## 13.3	Local Number Portability Database

### 13.3.1	Definition

The Permanent Number Portability (PNP) database supplies routing numbers for calls involving numbers that have been ported from one local service provider to another.  PNP is currently being worked in industry forums. The results of these forums will dictate the industry direction of PNP.  BellSouth agrees to provide access to the PNP database at rates, terms and conditions as set forth by BellSouth and in accordance with an effective FCC or Commission directive.

## 13.4	Line Information Database (LIDB)

BellSouth will store in its LIDB only records relating to service in the BellSouth region.   The LIDB Storage Agreement is included in this Attachment.

### 13.4.1	Definition

The Line Information Database (LIDB) is a transaction-oriented database accessible through Common Channel Signaling (CCS) networks.  It contains records associated with end user Line Numbers and Special Billing Numbers.  LIDB accepts queries from other Network Elements and provides appropriate responses.  The query originator need not be the owner of LIDB data.  LIDB queries include functions such as screening billed numbers that provides the ability to accept Collect or Third Number Billing calls and validation of Telephone Line Number based non-proprietary calling cards.  The interface for the LIDB functionality is the interface between BellSouth CCS network and other CCS networks.  LIDB also interfaces to administrative systems.

### 13.4.2	Technical Requirements

BellSouth will offer to CLEC-1 any additional capabilities that are developed for LIDB during  the life of this Agreement.

#### 13.4.2.1	BellSouth shall process CLEC-1’s Customer records in LIDB at least at parity with BellSouth customer records, with respect to other LIDB functions.  BellSouth shall indicate to CLEC-1 what additional functions (if any) are performed by LIDB in the BellSouth network.

#### 13.4.2.2	Within two (2) weeks after a request by CLEC-1, BellSouth shall provide CLEC-1 with a list of the customer data items which CLEC-1 would have to provide in order to support each required LIDB function.  The list shall indicate which data items are essential to LIDB function, and which are required only to support certain services.  For each data item, the list shall show the data formats, the acceptable values of the data item and the meaning of those values.

#### 13.4.2.3	BellSouth shall provide LIDB systems for which operating deficiencies that would result in calls being blocked, shall not exceed 30 minutes per year.

#### 13.4.2.4	BellSouth shall provide LIDB systems for which operating deficiencies that would not result in calls being blocked shall not exceed 12 hours per year.

#### 13.4.2.5	BellSouth shall provide LIDB systems for which the LIDB function shall be in overload no more than 12 hours per year.

#### 13.4.2.6	All additions, updates and deletions of CLEC-1 data to the LIDB shall be solely at the direction of CLEC-1.  Such direction from CLEC-1 will not be required where the addition, update or deletion is necessary to perform standard fraud control measures (e.g., calling card auto-deactivation).

#### 13.4.2.7	BellSouth shall provide priority updates to LIDB for CLEC-1 data upon CLEC-1’s request (e.g., to support fraud detection), via password-protected telephone card, facsimile, or electronic mail within one hour of notice from the established BellSouth contact.

#### 13.4.2.8	BellSouth shall provide LIDB systems such that no more than 0.01% of CLEC-1 customer records will be missing from LIDB, as measured by CLEC-1 audits.  BellSouth will audit CLEC-1 records in LIDB against DBAS to identify record mismatches and provide this data to a designated CLEC-1 contact person to resolve the status of the records and BellSouth will update system appropriately.  BellSouth will refer record of mis-matches to CLEC-1 within one business day of audit.  Once reconciled records are received back from CLEC-1, BellSouth will update LIDB the same business day if less than 500 records are received before 1:00PM Central Time.  If more than 500 records are received, BellSouth will contact CLEC-1 to negotiate a time frame for the updates, not to exceed three business days.

#### 13.4.2.9	BellSouth shall perform backup and recovery of all of CLEC-1’s data in LIDB including sending to LIDB all changes made since the date of the most recent backup copy, in at least the same time frame BellSouth performs backup and recovery of BellSouth data in LIDB for itself.  Currently, BellSouth performs backups of the LIDB for itself on a weekly basis and when a new software release is scheduled, a backup is performed prior to loading the new release.

#### 13.4.2.10	BellSouth shall provide CLEC-1 with LIDB reports of data which are missing or contain errors, as well as any misrouted errors, within a reasonable time period as negotiated between CLEC-1 and BellSouth.

#### 13.4.2.11	BellSouth shall prevent any access to or use of CLEC-1 data in LIDB by BellSouth personnel that are outside of established administrative and fraud control personnel, or by any other Party that is not authorized by CLEC-1 in writing.

#### 13.4.2.12	BellSouth shall provide CLEC-1 performance of the LIDB Data Screening function, which allows a LIDB to completely or partially deny specific query originators access to LIDB data owned by specific data owners, for Customer Data that is part of an NPA-NXX or RAO-0/1XX wholly or partially owned by CLEC-1 at least at parity with BellSouth Customer Data. BellSouth shall obtain from CLEC-1 the screening information associated with LIDB Data Screening of CLEC-1 data in accordance with this requirement.  BellSouth currently does not have LIDB Data Screening capabilities.  When such capability is available, BellSouth shall offer it to CLEC-1 under the Bona Fide Request/New Business Process as set forth in General Terms and Conditions .

#### 13.4.2.13	BellSouth shall accept queries to LIDB associated with CLEC-1 customer records, and shall return responses in accordance with industry standards.

#### 13.4.2.14	BellSouth shall provide mean processing time at the LIDB within 0.50 seconds under normal conditions as defined in industry standards.

#### 13.4.2.15	BellSouth shall provide processing time at the LIDB within 1 second for 99% of all messages under normal conditions as defined in industry standards.

### 13.4.3	Interface Requirements

BellSouth shall offer LIDB in accordance with the requirements of this subsection.

#### 13.4.3.1	The interface to LIDB shall be in accordance with the technical references contained within.

#### 13.4.3.2	The CCS interface to LIDB shall be the standard interface described herein.

#### 13.4.3.3	The LIDB Data Base interpretation of the ANSI-TCAP messages shall comply with the technical reference herein.  Global Title Translation shall be maintained in the signaling network in order to support signaling network routing to the LIDB.

## 13.5	Toll Free Number Database

The Toll Free Number Database is a SCP that provides functionality necessary for toll free (e.g., 800 and 888) number services by providing routing information and additional so-called vertical features during call set-up in response to queries from SSPs. BellSouth shall provide the Toll Free Number Database in accordance with the following:

### 13.5.1	Technical Requirements

#### 13.5.1.1	BellSouth shall make BellSouth Toll Free Number Database available for CLEC-1 to query with a toll-free number and originating information.

#### The Toll Free Number Database shall return carrier identification and, where applicable, the queried toll free number, translated numbers and instructions as it would in response to a query from a BellSouth switch.

#### 13.5.1.3	The SCP shall also provide, at CLEC-1's option, such additional feature as described in SR-TSV-002275 (BOC Notes on BellSouth Networks, SR-TSV-002275, Issue 2, (Telcordia (formerly BellCore), April 1994)) as are available to BellSouth.  These may include but are not limited to:

##### 13.5.1.3.1	Network Management;

##### 13.5.1.3.2	Customer Sample Collection; and

##### 13.5.1.3.3	Service Maintenance

## 13.6	Automatic Location Identification/Data Management System (ALI/DMS)

The ALI/DMS Database contains end user information (including name, address, telephone information, and sometimes special information from the local service provider or end user) used to determine to which Public Safety Answering Point  (PSAP) to route the call.  The ALI/DMS database is used to provide more routing flexibility for E911 calls than Basic 911.  BellSouth shall provide the Emergency Services Database in accordance with the following:

### 13.6.1	Technical Requirements

#### 13.6.1.1	BellSouth shall offer CLEC-1 a data link to the ALI/DMS database or permit CLEC-1 to provide its own data link to the ALI/DMS database. BellSouth shall provide error reports from the ALI/DMS database to CLEC-1 immediately after CLEC-1 inputs information into the ALI/DMS database.  Alternately, CLEC-1 may utilize BellSouth, to enter end user information into the data base on a demand basis, and validate end user information on a demand basis.

#### 13.6.1.2	The ALI/DMS database shall contain the following end user information:

##### 13.6.1.2.1	Name;

##### 13.6.1.2.2	Address;

##### 13.6.1.2.3	Telephone number; and

##### 13.6.1.2.4	Other information as appropriate (e.g., whether a end user is blind or deaf or has another disability).

#### 13.6.1.3	When BellSouth is responsible for administering the ALI/DMS database in its entirety, ported number NXXs entries for the ported numbers should be maintained unless CLEC-1 requests otherwise and shall be updated if CLEC-1 requests, provided CLEC-1 supplies BellSouth with the updates.

#### 13.6.1.4	When Remote Call Forwarding (RCF) is used to provide number portability to the local end user and a remark or other appropriate field information is available in the database, the shadow or “forwarded-to” number and an indication that the number is ported shall be added to the customer record.

#### 13.6.1.5	If BellSouth is responsible for configuring PSAP features (for cases when the PSAP or BellSouth supports an ISDN interface) it shall ensure that CLASS Automatic Recall (Call Return) is not used to call back to the ported number.  Although BellSouth currently does not have ISDN interface, BellSouth agrees to comply with this requirement once ISDN interfaces are in place.

### 13.6.2	Interface Requirements

The interface between the E911 Switch or Tandem and the ALI/DMS database for CLEC-1 end users shall meet industry standards.

## 13.7	Directory Assistance Database

BellSouth shall make its directory assistance database available to CLEC-1 in order to allow CLEC-1 to provide its end users with the same directory assistance telecommunications services BellSouth provides to BellSouth end users.  BellSouth shall provide CLEC-1 with an initial feed via magnetic tape and daily update initially via magnetic tape and subsequently via an electronic gateway to be developed mutually by CLEC-1 and BellSouth of end user address and number changes.  Directory Assistance Services must provide both the ported and CLEC-1 telephone numbers to the extent available in BellSouth’s database assigned to a end user.  Privacy indicators must be properly identified to assure the non-published numbers are accurately identified.

# 13.8	Calling Name (CNAM) Database Service.  The Agreement for Calling Name (CNAM) with standard pricing is included as Exhibit B to this Attachment.  CLEC-1 must provide to its account manager a written request with a requested activation date to activate this service.  If CLEC-1 is interested in requesting CNAM with volume and term pricing, CLEC-1 must contact its account manager to request a separate CNAM volume and term Agreement.

## 13.9	SCPs/Databases shall be equal to or better than all of the requirements for SCPs/Databases set forth in the following technical references:

### 13.9.1	GR-246-CORE, Bell Communications Research Specification of  Signaling System Number 7, ISSUE 1 (Telcordia (formerly BellCore), December 199);

### 13.9.2	GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP). (Telcordia (formerly BellCore), March 1994);

### 13.9.3	GR-954-CORE, CCS Network Interface Specification (CCSNIS) Supporting Line Information Database (LIDB) Service 6, Issue 1, Rev. 1 (Telcordia (formerly BellCore), October 1995);

### 13.9.4	GR-1149-CORE, OSSGR Section 10: System Interfaces, Issue 1 (Telcordia (formerly BellCore), October 1995)  (Replaces TR-NWT-001149);

### 13.9.5	Telcordia (formerly BellCore) GR-1158-CORE, OSSGR Section 22.3: Line Information Database 6, Issue (Telcordia (formerly BellCore), October 1995);

### 13.9.6	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service (Telcordia (formerly BellCore), May 1995); and

### 13.9.7	BOC Notes on BellSouth Networks, SR-TSV-002275, ISSUE 2, (Telcordia (formerly BellCore), April 1994).

## 13.10	Service Creation Environment and Service Management System (SCE/SMS) Advanced Intelligent Network (AIN) Access.

### 13.10.1	BellSouth’s Service Creation Environment and Service Management System (SCE/SMS) Advanced Intelligent Network (AIN) Access shall provide CLEC-1 the capability that will allow CLEC-1 and other third parties to create service applications in a BellSouth Service Creation Environment and deploy those applications in a BellSouth SMS to a BellSouth SCP.  The third party service applications interact with AIN triggers provisioned on a BellSouth SSP.

### 13.10.2	BellSouth’s SCE/SMS AIN Access shall provide access to SCE hardware, software, testing and technical support (e.g., help desk, system administrator) resources available to CLEC-1.  Scheduling procedures shall provide CLEC-1 equivalent priority to these resources

### 13.10.3	BellSouth SCP shall partition and protect CLEC-1 service logic and data from unauthorized access, execution or other types of compromise.

### 13.10.4	When CLEC-1 selects SCE/SMS AIN Access, BellSouth shall provide training, documentation, and technical support to enable CLEC-1 to use BellSouth’s SCE/SMS AIN Access to create and administer applications.  Training, documentation, and technical support will address use of SCE and SMS access and administrative functions, but will not include support for the creation of a specific service application.

### 13.10.5	When CLEC-1 selects SCE/SMS AIN Access, BellSouth shall provide for a secure, controlled access environment in association with its internal use of AIN components.  CLEC-1 access will be provided via remote data connection (e.g., dial-in, ISDN).

### 13.10.6	When CLEC-1 selects SCE/SMS AIN Access, BellSouth shall allow CLEC-1 to download data forms and/or tables to BellSouth SCP via BellSouth SMS without intervention from BellSouth (e.g., service customization and end user subscription).

# 14.	DARK FIBER

BellSouth agrees to offer access to Dark Fiber pursuant to the terms and conditions following and at the rates set forth in this Attachment.  In Georgia, BellSouth is not required to construct the fiber if it is not available.  In Kentucky, if BellSouth has plans to use the fiber in a three year planning period, there is no requirement to provide it.  In all other states, BellSouth is not required to place the fibers if there are no fibers available.  The Parties agree that Dark Fiber will be used in the provisioning of local service.

### 14.1.1	Dark Fiber is unused strands of optical fiber.  It may be strands of optical fiber existing in aerial or underground structure.  No line terminating elements terminated to such strands to operationalize its transmission capabilities will be available.  No regeneration or optical amplification will be included with this element.

## 14.2	Requirements

### 14.2.1	BellSouth shall make available Dark Fiber where it exists in BellSouth’s network and where, as a result of future building or deployment, it becomes available.  BellSouth shall offer all Dark Fiber to CLEC-1 pursuant to the prices set forth in this Attachment.

### 14.2.2	CLEC-1 may test the quality of the Dark Fiber to confirm its usability and performance specifications.

### 14.2.3	BellSouth shall use its best efforts to provide to CLEC-1 information regarding the location, availability and performance of Dark Fiber within ten (10) business days for a records based answer and twenty (20) business days for a field based answer, after receiving a request from CLEC-1 ("Request").  Within such time period, BellSouth shall send written confirmation of availability of the Dark Fiber ("Confirmation").

### 14.2.4	BellSouth shall use its best efforts to make Dark Fiber available to CLEC-1 within thirty (30) business days after it receives written confirmation from CLEC-1 that the Dark Fiber previously deemed available by BellSouth is wanted for use by CLEC-1.  This includes identification of appropriate connection points (e.g., Light Guide Interconnection (LGX) or splice points) to enable CLEC-1 to connect or splice CLEC-1 provided transmission media (e.g., optical fiber) or equipment to the Dark Fiber.

# 15.	SS7 Network Interconnection

### 15.1.1	Definition

SS7 Network Interconnection is the interconnection of CLEC-1 local Signaling Transfer Point Switches (STP) and CLEC-1 local or tandem switching systems with BellSouth STPs.  This interconnection provides connectivity that enables the exchange of SS7 messages among BellSouth switching systems and databases (DBs), CLEC-1 local or tandem switching systems, and other third-party switching systems directly connected to the BellSouth SS7 network.

### 15.1.2	Technical Requirements

#### 15.1.2.1	SS7 Network Interconnection shall provide connectivity to all components of the BellSouth SS7 network.  These include:

##### 15.1.2.1.1	BellSouth local or tandem switching systems;

##### 15.1.2.1.2	BellSouth DBs; and

##### 15.1.2.1.3	Other third-party local or tandem switching systems.

#### 15.1.2.2	The connectivity provided by SS7 Network Interconnection shall fully support the functions of BellSouth switching systems and DBs and CLEC-1 or other third-party switching systems with A-link access to the BellSouth SS7 network.

If traffic is routed based on dialed or translated digits between an CLEC-1 local switching system and a BellSouth or other third-party local switching system, either directly or via a BellSouth tandem switching system, then it is a requirement that the BellSouth SS7 network convey via SS7 Network Interconnection the TCAP messages that are necessary to provide Call Management services (Automatic Callback, Automatic Recall, and Screening List Editing) between the CLEC-1 local STPs and BellSouth or other third-party local switch.

#### 15.1.2.3	When the capability to route messages based on Intermediate Signaling Network Identifier (ISNI) is generally available on BellSouth STPs,  the BellSouth SS7 Network shall also convey TCAP messages using SS7 Network Interconnection in similar circumstances where the BellSouth switch routes traffic based on a Carrier Identification Code (CIC).

#### 15.1.2.4	SS7 Network Interconnection shall provide all functions of the MTP as specified in ANSI T1.111. This includes:

##### 15.1.2.4.1	Signaling Data Link functions, as specified in ANSI T1.111.2;

##### 15.1.2.4.2	Signaling Link functions, as specified in ANSI T1.111.3; and

##### 15.1.2.4.3	Signaling Network Management functions, as specified in ANSI T1.111.4.

#### 15.1.2.5	SS7 Network Interconnection shall provide all functions of the SCCP necessary for Class 0 (basic connectionless) service, as specified in ANSI T1.112.  In particular, this includes Global Title Translation (GTT) and SCCP Management procedures, as specified in T1.112.4.  Where the destination signaling point is a BellSouth switching system or DB, or is another third-party local or tandem switching system directly connected to the BellSouth SS7 network, SS7 Network Interconnection shall include final GTT of messages to the destination and SCCP Subsystem Management of the destination.  Where the destination signaling point is an CLEC-1 local or tandem switching system, SS7 Network Interconnection shall include intermediate GTT of messages to a gateway pair of CLEC-1 local STPs, and shall not include SCCP Subsystem Management of the destination.

#### 15.1.2.6	SS7 Network Interconnection shall provide all functions of the Integrated Services Digital Network User Part (ISDNUP), as specified in ANSI T1.113.

#### 15.1.2.7	SS7 Network Interconnection shall provide all functions of the TCAP, as specified in ANSI T1.114.

#### 15.1.2.8	If and when Internetwork MTP Routing Verification Test (MRVT) and SCCP Routing Verification Test (SRVT) become approved ANSI standards and available capabilities of BellSouth STPs, SS7 Network Interconnection shall provide these functions of the OMAP.

#### 15.1.2.9	SS7 Network Interconnection shall be equal to or better than the following performance requirements:

##### 15.1.2.9.1	MTP Performance, as specified in ANSI T1.111.6;

##### 15.1.2.9.2	SCCP Performance, as specified in ANSI T1.112.5; and

##### 15.1.2.9.3	ISDNUP Performance, as specified in ANSI T1.113.5.

### 15.1.3	Interface Requirements

#### 15.1.3.1	BellSouth shall offer the following SS7 Network Interconnection options to connect CLEC-1 or CLEC-1-designated local or tandem switching systems or STPs to the BellSouth SS7 network:

##### 15.1.3.1.1	A-link interface from CLEC-1 local or tandem switching systems; and

##### 15.1.3.1.2	B-link interface from CLEC-1 STPs.

#### 15.1.3.2	The Signaling Point of Interconnection (SPOI) for each link shall be located at a cross-connect element, such as a DSX-1, in the Central Office (CO) where the BellSouth STP is located.  There shall be a DS1 or higher rate transport interface at each of the SPOIs.  Each signaling link shall appear as a DS0 channel within the DS1 or higher rate interface.  BellSouth shall offer higher rate DS1 signaling links for interconnecting CLEC-1 local switching systems or STPs with BellSouth STPs as soon as these become approved ANSI standards and available capabilities of BellSouth STPs.  BellSouth and CLEC-1 will work jointly to establish mutually acceptable SPOI.

#### 15.1.3.3	BellSouth CO shall provide intraoffice diversity between the SPOIs and the BellSouth STP, so that no single failure of intraoffice facilities or equipment shall cause the failure of both B-links in a layer connecting to a BellSouth STP. BellSouth and CLEC-1 will work jointly to establish mutually acceptable SPOI.

#### 15.1.3.4	The protocol interface requirements for SS7 Network Interconnection include the MTP, ISDNUP, SCCP, and TCAP. These protocol interfaces shall conform to the following specifications:

##### 15.1.3.4.1	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

##### 15.1.3.4.2	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service;

##### 15.1.3.4.3	Telcordia (formerly BellCore) GR-1429-CORE, CCS Network Interface Specification (CCSNIS) Supporting Call Management Services; and

##### 15.1.3.4.4	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

#### 15.1.3.5	BellSouth shall set message screening parameters to block accept messages from CLEC-1 local or tandem switching systems destined to any signaling point in the BellSouth SS7 network with which the CLEC-1 switching system has a legitimate signaling relation.

### 15.1.4	SS7 Network Interconnection shall be equal to or better than all of the requirements for SS7 Network Interconnection set forth in the following technical references:

#### 15.1.4.1	ANSI T1.110-1992 American National Standard Telecommunications - Signaling System Number 7 (SS7) - General Information;

#### 15.1.4.2	ANSI T1.111-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP);

#### 15.1.4.3	ANSI T1.111A-1994 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP) Supplement;

#### 15.1.4.4	ANSI T1.112-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Signaling Connection Control Part (SCCP);

#### 15.1.4.5	ANSI T1.113-1995 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Integrated Services Digital Network (ISDN) User Part;

#### 15.1.4.6	ANSI T1.114-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Transaction Capabilities Application Part (TCAP);

#### 15.1.4.7	ANSI T1.115-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Monitoring and Measurements for Networks;

#### 15.1.4.8	ANSI T1.116-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Operations, Maintenance and Administration Part (OMAP);

#### 15.1.4.9	ANSI T1.118-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Intermediate Signaling Network Identification (ISNI);

#### 15.1.4.10	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

#### 15.1.4.11	Telcordia (formerly BellCore) GR-954-CORE, CCS Network Interface Specification (CCSNIS) Supporting Line Information Database (LIDB) Service;

#### 15.1.4.12	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service;

#### 15.1.4.13	Telcordia (formerly BellCore) GR-1429-CORE, CCS Network Interface Specification (CCSNIS) Supporting Call Management Services; and,

#### 15.1.4.14	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

# 16.	Basic 911 and E911

If CLEC-1 orders network elements and other services, then CLEC-1 is also responsible for providing E911 to its end users.  BellSouth agrees to offer access to the 911/E911 network pursuant to the following terms and conditions set forth in this Attachment.

## 16.1	Definition

Basic 911 and E911 is an additional requirement that provides a caller access to the applicable emergency service bureau by dialing a 3-digit universal telephone number (911).

## 16.2	Requirements

### 16.2.1	Basic 911 Service Provisioning.  For Basic 911 service, BellSouth will provide to CLEC-1 a list consisting of each municipality that subscribes to Basic 911 service.  The list will also provide, if known, the E911 conversion date for each municipality and, for network routing purposes, a ten-digit directory number representing the appropriate emergency answering position for each municipality subscribing to 911.  CLEC-1 will be required to arrange to accept 911 calls from its end users in municipalities that subscribe to Basic 911 service and translate the 911 call to the appropriate 10-digit directory number as stated on the list provided by BellSouth.  CLEC-1 will be required to route that call to BellSouth at the appropriate tandem or end office.  When a municipality converts to E911 service, CLEC-1 will be required to discontinue the Basic 911 procedures and being using E911 procedures.

### 16.2.2	E911 Service Provisioning.  For E911 service, CLEC-1 will be required to install a minimum of two dedicated trunks originating from the CLEC-1 serving wire center and terminating to the appropriate E911 tandem.  The dedicated trunks shall be, at a minimum, DS-0 level trunks configured either as a 2-wire analog interface or as part of a digital (1.544 Mb/s) interface.  Either configuration shall use CAMA-type signaling with multifrequency (“MF”) pulsing that will deliver automatic number identification (“ANI”) with the voice portion of the call.  If the user interface is digital, MF pulses, as well as other AC signals, shall be encoded per the u-255 Law convention.  CLEC-1 will be required to provide BellSouth daily updates to the E911 database.  CLEC-1 will be required to forward 911 calls to the appropriate E911 tandem, along with ANI, based upon the current E911 end office to tandem homing arrangement as provided by BellSouth.  If the E911 tandem trunks are not available, CLEC-1 will be required to route the call to a designated 7-digit local number residing in the appropriate Public Service Answering Point (“PSAP”).  This call will be transported over BellSouth’s interoffice network and will not carry the ANI of the calling party.  CLEC-1 shall be responsible for providing BellSouth with complete and accurate data for submission to the 911/E911 database for the purpose of providing 911/E911 to its end users.

### 16.2.3	Rates.  Charges for 911/E911 service are borne by the municipality purchasing the service.  BellSouth will impose no charge on CLEC-1 beyond applicable charges for BellSouth trunking arrangements.

### 16.2.4	Basic 911 and E911 functions provided to CLEC-1 shall be at least at parity with the support and services that BellSouth provides to its end users for such similar functionality.

### Detailed Practices and Procedures.  The detailed practices and procedures contained in the E911 Local Exchange Carrier Guide For Facility-Based Providers as amended from time to time during the term of this Agreement will determine the appropriate practices and procedures for BellSouth and CLEC-1 to follow in providing 911/E911 services.

17.		Rates

# 17.1.	General

The prices that CLEC-1 shall pay to BellSouth for  Network Elements and Other Services are set forth in Exhibit C to this Attachment.

# 17.2.	Operational Support Systems (OSS)

BellSouth has developed and made available the following mechanized systems by which CLEC-1 may submit LSRs electronically.

LENS		Local Exchange Navigation System

EDI		Electronic Data Interface

EDI-PC	Electronic Data Interface – Personal Computer

TAG		Telecommunications Access Gateway

LSRs submitted by means of one of these interactive interfaces will incur an OSS electronic ordering charge as specified in the table below.  An individual LSR will be identified for billing purposes by its Purchase Order Number (PON).  LSRs submitted by means other than one of these interactive interfaces (mail, fax, courier, etc.) will incur a manual order charge as specified in the table below:

| ## OPERATIONAL SUPPORT SYSTEMS | AL, GA, LA, MS, SC | FL, KY, NC, TN |
| --- | --- | --- |
| OSS LSR charge, per LSR received from the CLEC by one of the OSS interactive interfaces | $3.50 SOMEC | $3.50 SOMEC |
| Incremental charge per LSR received from the CLEC by means other than one of the OSS interactive interfaces | See applicable rate element | $19.99 SOMAN |

Note:  In addition to the OSS charges, applicable discounted service order and related          discounted charges apply per the tariff.

Denial/Restoral OSS Charge

In the event CLEC-1 provides a list of customers to be denied and restored, rather than an LSR, each location on the list will require a separate PON and, therefore will be billed as one LSR per location.

Cancellation OSS Charge

CLEC-1 will incur an OSS charge for an accepted LSR that is later canceled by CLEC-1.

Note:  Supplements or clarifications to a previously billed LSR will not incur another OSS charge.

Network Elements and Other Services Manual Additive

The Commissions in Alabama, Georgia, Louisiana, Mississippi and South Carolina have ordered incremental manual non-recurring charges (NRC) for Network Elements and Other Services ordered by means other than one of the interactive interfaces.  These ordered Network Elements and Other Services manual additive NRCs will apply in these states, rather than the charge per LSR.

Threshold Billing Plan

The Parties agree that CLEC-1 will incur the mechanized rate for all LSRs, both mechanized and manual, if the percentage of mechanized LSRs to total LSRs meets or exceeds the threshold percentages shown below:

Year		Ratio: Mechanized/Total LSRs

- 70%

- 80%

- 90%

The threshold plan will be discontinued in 2002.

BellSouth will track the total LSR volume for each CLEC for each quarter.  At the end of that time period, a Percent Electronic LSR calculation will be made for that quarter based on the LSR data tracked in the LCSC.  If this percentage exceeds the threshold volume, all of that CLECs’ future manual LSRs will be billed at the mechanized LSR rate.  To allow time for obtaining and analyzing the data and updating the billing system, this billing change will take place on the first day of the second month following the end of the quarter (e.g. May 1 for 1Q, Aug 1 for 2Q, etc.).  There will be no adjustments to the amount billed for previously billed LSRs.

The Parties agree that any charges BellSouth is unable to bill on April 15, 1999 will be trued up on or about July 1, 1999.

17.3 	 True-up

This section applies only to North Carolina and Tennessee.

The interim prices for Network Elements and Other Services and Local Interconnection shall be subject to true-up according to the following procedures:

1.	The interim prices shall be trued-up, either up or down, based on final prices determined either by further agreement between the Parties, or by a final order (including any appeals) of the Commission which final order meets the criteria of (3) below.  The Parties shall implement the true-up by comparing the actual volumes and demand for each item, together with interim prices for each item, with the final prices determined for each item.  Each Party shall keep its own records upon which the true-up can be based, and any final payment from one Party to the other shall be in an amount agreed upon by the Parties based on such records.  In the event of any disagreement as between the records or the Parties regarding the amount of such true-up, the Parties agree that the body having jurisdiction over the matter shall be called upon to resolve such differences, or the Parties may mutually agree to submit the matter to the Dispute Resolution process in accordance with the provisions of Section 16 of the General Terms and Conditions and Attachment 1 of the Agreement.

2.	The Parties may continue to negotiate toward final prices, but in the event that no such Agreement is reached within nine (9) months, either Party may petition the Commission to resolve such disputes and to determine final prices for each item.  Alternatively, upon mutual agreement, the Parties may submit the matter to  the Dispute Resolution Process set forth in Section 16 of the General

Terms and Conditions and Attachment 1 of the Agreement, so long as they file the resulting Agreement with the Commission as a “negotiated Agreement” under Section 252(e) of the Act.

3.	A final order of this Commission that forms the basis of a true-up shall be the final order as to prices based on appropriate cost studies, or potentially may be a final order in any other Commission proceeding which meets the following criteria:

(a)	BellSouth and CLEC is entitled to be a full Party to the proceeding;

(b)	It shall apply the provisions of the federal Telecommunications Act of 1996, including but not limited to Section 252(d)(1) (which contains pricing standards) and all then-effective implementing rules and regulations; and,

(c)	It shall include as an issue the geographic deaveraging of network element and other services prices, which deaveraged prices, if any are required by said final order, shall form the basis of any true-up.

EXHIBIT A

LINE INFORMATION DATA BASE (LIDB)

STORAGE AGREEMENT

I.	SCOPE

A.	This Agreement sets forth the terms and conditions pursuant to which BST agrees to store in its LIDB certain information at the request of the Local Exchange Company and pursuant to which BST, its LIDB customers and Local Exchange Carrier shall have access to such information.  Local Exchange Carrier understands that BST provides access to information in its LIDB to various telecommunications service providers pursuant to applicable tariffs and agrees that information stored at the request of Local Exchange Carrier, pursuant to this Agreement, shall be available to those telecommunications service providers.  The terms and conditions contained in the attached Addendum(s)  are hereby made a part of this Agreement as if fully incorporated herein.

B.	LIDB is accessed for the following purposes:

1.	Billed Number Screening

2.	Calling Card Validation

3.	Fraud Control

C.	BST will provide seven days per week, 24-hours per day, fraud monitoring on Calling Cards, bill-to-third and collect calls made to numbers in BST’s LIDB, provided that such information is included in the LIDB query.  BST will establish fraud alert thresholds and will notify the Local Exchange Company of fraud alerts so that the Local Exchange Company may take action it deems appropriate.  Local Exchange Company understands and agrees BST will administer all data stored in the LIDB, including the data provided by Local Exchange Company pursuant to this Agreement, in the same manner as BST’s data for BST’s end user customers.  BST shall not be responsible to Local Exchange Company for any lost revenue which may result from BST’s administration of the LIDB pursuant to its established practices and procedures as they exist and as they may be changed by BST in its sole discretion from time to time.

Local Exchange Company understands that BST currently has in effect numerous billing and collection agreements with various interexchange carriers and billing clearing houses.  Local Exchange Company further understands that these billing and collection customers of BST query BST’s LIDB to determine whether to accept various billing options from end users.  Additionally, Local Exchange Company understands that presently BST has no method to differentiate between BST’s own billing and line data in the LIDB and such data which it includes in the LIDB on Local Exchange Company’s behalf pursuant to this Agreement.  Therefore, until such time as BST can and does implement in its LIDB and its supporting systems the means to differentiate Local Exchange Company’s data from BST’s data and the Parties to this Agreement execute appropriate amendments hereto, the following terms and conditions shall apply:

(a)	The Local Exchange Company agrees that it will accept responsibility for telecommunications services billed by BST for its billing and collection customers for Local Exchange Customer’s end user accounts which are resident in LIDB pursuant to this Agreement.  Local Exchange Company authorizes BST to place such charges on Local Exchange Company’s bill from BST and agrees that it shall pay all such charges.  Charges for which Local Exchange Company hereby takes responsibility include, but are not limited to, collect and third number calls.

(b)	Charges for such services shall appear on a separate BST bill page identified with the name of the entity for which BST is billing the charge.

(c)	Local Exchange Company shall have the responsibility to render a billing statement to its end users for these charges, but Local Exchange Company’s obligation to pay BST for the charges billed shall be independent of whether Local Exchange Company is able or not to collect from the Local Exchange Company’s end users.

- BST shall not become involved in any disputes between Local Exchange Company and the entities for which BST performs billing and collection.  BellSouth will not issue adjustments for charges billed on behalf of an entity to Local Exchange Company.  It shall be the responsibility of the Local Exchange Company and the other entity to negotiate and arrange for any appropriate adjustments.

II.	TERM

This Agreement will be effective as of ____________, 199__, and will continue in effect for one year, and thereafter may be continued until terminated by either Party upon thirty (30) days written notice to the other Party.

III.	FEES FOR SERVICE AND TAXES

A.	The Local Exchange Company will not be charged a fee for storage services provided by BST to the Local Exchange Company, as described in Section I of this Agreement.

B.	Sales, use and all other taxes (excluding taxes on BST’s income) determined by BST or any taxing authority to be due to any federal, state or local taxing jurisdiction with respect to the provision of the service set forth herein will be paid by the Local Exchange Company.  The Local Exchange Company shall have the right to have BST contest with the imposing jurisdiction, at the Local Exchange Company’s expense, any such taxes that the Local Exchange Company deems are improperly levied.

IV.	INDEMNIFICATION

To the extent not prohibited by law, each Party will indemnify the other and hold the other harmless against any loss, cost, claim, injury, or liability relating to or arising out of negligence or willful misconduct by the indemnifying Party or its agents or contractors in connection with the indemnifying Party’s provision of services, provided, however, that any indemnity for any loss, cost, claim, injury or liability arising out of or relating to errors or omissions in the provision of services under this Agreement shall be limited as otherwise specified in this Agreement.  The indemnifying Party under this Section agrees to defend any suit brought against the other Party for any such loss, cost, claim, injury or liability.  The indemnified Party agrees to notify the other Party promptly, in writing, of any written claims, lawsuits, or demands for which the other Party is responsible under this Section and to cooperate in every reasonable way to facilitate defense or settlement of claims.  The indemnifying Party shall not be liable under this Section for settlement by the indemnified Party of any claim, lawsuit, or demand unless the defense of the claim, lawsuit, or demand has been tendered to it in writing and the indemnifying Party has unreasonably failed to assume such defense.

V.	LIMITATION OF LIABILITY

Neither Party shall be liable to the other Party for any lost profits or revenues or for any indirect, incidental or consequential damages incurred by the other Party arising from this Agreement or the services performed or not performed hereunder, regardless of the cause of such loss or damage.

VI.	MISCELLANEOUS

A.  It is understood and agreed to by the Parties that BST may provide similar services to other companies.

B.	All terms, conditions and operations under this Agreement shall be performed in accordance with, and subject to, all applicable local, state or federal legal and regulatory tariffs, rulings, and other requirements of the federal courts, the U. S. Department of Justice and state and federal regulatory agencies.  Nothing in this Agreement shall be construed to cause either Party to violate any such legal or regulatory requirement and either Party’s obligation to perform shall be subject to all such requirements.

C.	The Local Exchange Company agrees to submit to BST all advertising, sales promotion, press releases, and other publicity matters relating to this Agreement wherein BST’s corporate or trade names, logos, trademarks or service marks or those of BST’s affiliated companies are mentioned or language from which the connection of said names or trademarks therewith may be inferred or implied; and the Local Exchange Company further agrees not to publish or use advertising, sales promotions, press releases, or publicity matters without BST’s prior written approval.

D.	This Agreement constitutes the entire Agreement between the Local Exchange Company and BST which supersedes all prior Agreements or contracts, oral or written representations, statements, negotiations, understandings, proposals and undertakings with respect to the subject matter hereof.

E.	Except as expressly provided in this Agreement, if any part of this Agreement is held or construed to be invalid or unenforceable, the validity of any other Section of this Agreement shall remain in full force and effect to the extent permissible or appropriate in furtherance of the intent of this Agreement.

F.	Neither Party shall be held liable for any delay or failure in performance of any part of this Agreement for any cause beyond its control and without its fault or negligence, such as acts of God, acts of civil or military authority, government regulations, embargoes, epidemics, war, terrorist acts, riots, insurrections, fires, explosions, earthquakes, nuclear accidents, floods, strikes, power blackouts, volcanic action, other major environmental disturbances, unusually severe weather conditions, inability to secure products or services of other persons or transportation facilities, or acts or omissions of transportation common carriers.

G.	This Agreement shall be deemed to be a contract made under the laws of the State of Georgia, and the construction, interpretation and performance of this Agreement and all transactions hereunder shall be governed by the domestic law of such State.

FACILITIES BASED ADDENDUM

TO LINE INFORMATION DATA BASE (LIDB)

STORAGE AGREEMENT

This is a Facilities Based Addendum to the Line Information Data Base Storage Agreement dated ________________________________, 199 __, between BellSouth Telecommunications, Inc. (“BST”), and _________________________________________(“Local Exchange Company”), effective the _____ day of _________________, 199 ___.

I.	GENERAL

This Addendum sets forth the terms and conditions for Local Exchange Company’s provision of billing number information to BST for inclusion in BST’s LIDB.  BST will store in its LIDB the billing number information provided by Local Exchange Company, and BST will provide responses to on-line, call-by-call queries to this information for purposes specified in Section I.B. of the Agreement.

II.	DEFINITIONS

A.	Billing number - a number that the Local Exchange Company creates for the purpose of identifying an account liable for charges.  This number may be a line or a special billing number.

B.	Line number - a ten digit number that identifies a telephone line administered by the Local Exchange Company.

C.	Special billing number - a ten digit number that identifies a billing account established by the Local Exchange Company.

D.	Calling Card number - a billing number plus PIN number.

E.	PIN number - a four digit security code assigned by the Local Exchange Company which is added to a billing number to compose a fourteen digit calling card number.

F.	Toll billing exception indicator - associated with a billing number to indicate that it is considered invalid for billing of collect calls or third number calls or both, by the Local Exchange Company.

G.	Billed Number Screening - refers to the activity of determining whether a toll billing exception indicator is present for a particular billing number.

H.	Calling Card Validation - refers to the activity of determining whether a particular calling card number exists as stated or otherwise provided by a caller.

I.	Billing number information - information about billing number, Calling Card number and toll billing exception indicator provided to BST by the Local Exchange Company.

III.	RESPONSIBILITIES OF PARTIES

A.	The Local Exchange Company will  provide its billing number information to BST’s LIDB each business day by a method that has been mutually agreed upon by both Parties.

B.	BST will store in its LIDB the billing number information provided by the Local Exchange Company.  Under normal operating conditions, BST shall include the Local Exchange Company’s billing number information in its LIDB no later than two business days following BST’s receipt of such billing number information, provided that BST shall not be held responsible for any delay or failure in performance to the extent such delay or failure is caused by circumstances or conditions beyond BST’s reasonable control.  BST will store in its LIDB an unlimited volume of the Local Exchange Company’s working telephone numbers.

C.	BST will provide responses to on-line, call-by-call queries to the stored information for the specific purposes listed in the next paragraph.

D.	BST is authorized to use the billing number information provided by the Local Exchange Company to perform the following functions for authorized users on an on-line basis:

1.	Validate a 14 digit Calling Card number where the first 10 digits are a line number or special billing number assigned by the Local Exchange Company, and where the last four digits (PIN) are a security code assigned by the Local Exchange Company.

2.	Determine whether the Local Exchange Company or the subscriber has identified the billing number as one which should not be billed for collect or third number calls, or both.

E.	The Local Exchange Company will provide its own billing number information to BST for storage and to be used for Billed Number Screening and Calling Card Validation.  The Local Exchange Company will arrange and pay for transport of updates to BST.

- COMPLIANCE

Unless expressly authorized in writing by the Local Exchange Company, all billing number information provided pursuant to this Addendum shall be used for no purposes other than those set forth in this Addendum.

EXHIBIT B

CALLING NAME DELIVERY (CNAM) DATABASE SERVICES

# 1.00  DEFINITIONS

For the purpose of this Attachment, the following terms shall be defined as:

CALLING NAME DELIVERY DATABASE SERVICE (CNAM) - The ability to associate a name with the calling party number, allowing the end user subscriber (to which a call is being terminated) to view the calling party's name before the call is answered.  This service also provides CLEC-1CLEC-1 the opportunity to load and store its subscriber names in the BellSouth CNAM SCPs.

CALLING PARTY NUMBER (CPN) - The number of the calling party that is delivered to the terminating switch using common channel signaling system 7 (CCS7) technology, and that is contained in the Initial Address Message (IAM) portion of the CCS7 call setup.

COMMON CHANNEL SIGNALING SYSTEM 7 (CCS7) - A network signaling

technology in which all signaling information between two or more nodes is transmitted over high-speed data links, rather than over voice circuits.

SERVICE CONTROL POINTs (SCPs) - The real-time data base systems that contain the names to be provided in response to queries received from CNAM SSPs.

SERVICE MANAGEMENT SYSTEM (SMS) - The main operations support system of CNAM DATABASE SERVICE.  CNAM records are loaded into the SMS, which in turn downloads into the CNAM SCP.

SERVICE SWITCHING POINTs (SSPs) - Features of computerized switches in the telephone network that determine that a terminating line has subscribed to CNAM service, and then communicate with CNAM SCPs in order to provide the name associated with the calling party number.

SUBSYSTEM NUMBER (SSN) - The address used in the Signaling Connection Control Part (SCCP) layer of the SS7 protocol to designate an application at an end signaling point.  A SSN for CNAM at the end office designates the CNAM application within the end office.  BellSouth uses the CNAM SSN of 232.

# ATTACHMENT

2.01   This Attachment  contains  the terms and conditions where BellSouth will provide to the CLEC-1 access to the BellSouth CNAM SCP for query or record storage purposes.

2.02	CLEC-1 shall submit to BellSouth a notice of its intent to access and utilize BellSouth CNAM Database Services pursuant to the terms and conditions of this Attachment.  Said notice shall be in writing, no less than 60 days prior to CLEC-1’s access to BellSouth’s CNAM Database Services and shall be addressed to CLEC-1’s Account Manager.

# 3.00  PHYSICAL CONNECTION AND COMPENSATION

  - BellSouth's provision of CNAM Database Services to  CLEC-1 requires interconnection from CLEC-1 CLEC-1to BellSouth CNAM Service Control Points (SCPs).  Such interconnections shall be established pursuant to Attachment 3 of this Agreement .  The appropriate charge for access to and use of the BellSouth CNAM Database service shall be as set forth in this Attachment.

  - In order to formulate a CNAM query to be sent to the BellSouth CNAM SCP, CLEC-1 CLEC-1shall  provide its own CNAM SSP. CLEC-1’s CLEC-1CNAM SSPs must be compliant with TR-NWT-001188, "CLASS Calling Name Delivery Generic Requirements".

  - If CLEC-1 elects to access the BellSouth CNAM SCP via a third party CCS7 transport provider, the third party CCS7 provider shall interconnect with the BellSouth CCS7 network according to BellSouth's Common Channel Signaling Interconnection Guidelines and Telcordia (formerly BellCore)'s CCS Network Interface Specification document, TR-TSV-000905.  In addition, the third party provider shall establish CCS7 interconnection at the BellSouth Local Signal Transfer Points (LSTPs) serving the BellSouth CNAM SCPs that CLEC-1 desires to query.

  - Out-Of-Region Customers

If the customer queries the BellSouth CNAM SCP via a third party national SS7 transport provider, the third party SS7 provider shall interconnect with the BellSouth CCS7 network according to BellSouth's Common Channel Signaling Interconnection Guidelines and Bellcore's CCS Network Interface Specification document, TR-TSV-000905. In addition, the third party provider shall establish SS7 interconnection at one or more of the BellSouth Gateway Signal Transfer Points (STPs).  The payment of all costs associated with the transport of SS7 signals via a third party will be established by mutual agreement of the Parties and writing shall, by this reference become an integral part of this Agreement.

# 4.00  CNAM RECORD INITIAL LOAD AND UPDATES

4.01	The mechanism to be used by CLEC-1 for initial CNAM record load and/or updates shall be determined by mutual agreement.    The initial load and all updates shall be provided by CLEC-1 in the BellSouth specified format and shall contain records for every working telephone number that can originate phone calls.  It is the responsibility of CLEC-1 to provide accurate information to BellSouth on a current basis.

  - Updates to the SMS shall occur no less than once a week, reflect service order activity affecting either name or telephone number, and involve only record additions, deletions or changes.

4.03	CLEC-1 CNAM records provided for storage in the BellSouth CNAM SCP shall be available, on a SCP query basis only, to all Parties querying the BellSouth CNAM SCP.  Further, CNAM service shall be provided by each Party consistent with state and/or federal regulation.