Attachment 3

Local Interconnection

TABLE OF CONTENTS

1. Network interconnection 	3

2.  interconnection trunking and routing	9

3.  Network Design and Management for Interconnection	10

4.  Parity in Ordering and Provisioning	12

5. Local Dialing Parity	12

6. Interconnection Compensation	13

7. FRAME RELAY SERVICE	19

8. Operational Support Systems (OSS) Rates…………….……………………22

EXHIBIT A -- RATES	Exhibit A

Local Interconnection: Call Transport and Termination

The rates, terms and conditions contained within this Attachment were negotiated as a whole and each rate, term and condition within the Attachment is interdependent upon the other rates, terms and conditions.

The Parties shall provide interconnection with each other’s networks for the transmission and routing of telephone exchange service (local) and exchange access (intraLATA toll and switched access) on the following terms:

- Network Interconnection

  - Interconnection is available to both Parties through: (1) delivery of a Party’s facilities to a collocation arrangement or Fiber Meet arrangement as defined in this Agreement; or (2) interconnection via purchase of facilities from the other Party.  Interconnection may be provided by the Parties at any other technically feasible point.  Requests to BellSouth for interconnection at other points may be made through the Bona Fide Request/New Business Request process set out in General Terms and Conditions.

  - CLEC-1 must establish, at a minimum, a single Point of Presence, Interface, and Interconnection with BellSouth within the LATA for the delivery of CLEC-1’s originated local, intraLATA toll and transit traffic.  If CLEC-1 chooses to interconnect at a single Point of Interconnection within a LATA, the interconnection must be at a BellSouth Access Tandem. Furthermore, CLEC-1 must establish Points of Interconnection at all BellSouth access and local tandems where CLEC-1 NXXs are “homed.”  A “Homing” arrangement is defined by a “Final” Trunk Group between the BellSouth Tandem and CLEC-1 End Office switch.  A “Final” Trunk Group is the last choice telecommunications path between the Tandem and End Office switch.  It is CLEC-1’s responsibility to enter its own NPA/NXX access and/or local tandem “homing” arrangements into the national Local Exchange Routing Guide (LERG).

    - In order for CLEC-1 to home its NPA/NXX(s) on a BellSouth Tandem, CLEC-1’s NPA/NXX(s) must be assigned to an Exchange Rate Center Area served by that BellSouth Tandem and as specified by BellSouth.  The specified association between BellSouth Tandems and Exchange Rate Center Areas is defined in the BellSouth Call Transport & Termination Service For Facility Based CLECs section of the Facility Based CLEC Activation Requirements Customer Guide as it is revised from time to time.

  - A Point of Presence (POP) is the physical location (a structure where the environmental, power, air conditioning, etc. specifications for a Party’s terminating equipment can be met) at which a Party establishes itself for obtaining access to the other Party’s network.  The POP is the physical location within which the Point of Interfaces occur.

  - A Point of Interface is the physical telecommunications interface between BellSouth and CLEC-1’s interconnection functions.  It establishes the technical interface and point of operational responsibility.  The primary function of the Point of Interface is to serve as the terminus for the interconnection service.  The Point of Interface has the following main characteristics:

1.  	It is a cross-connect point to allow connection, disconnection, transfer or restoration of service.

2.	It is a point where BellSouth and CLEC-1 can verify and maintain specific performance objectives.

3.	It is specified according to the interface offered in the tariff or local interconnection agreement (for example:  for DS1 service the FCC # 1 tariff specifies that the interface meets the technical specifications detailed in Generic Requirements GR-342-CORE, Issue 1, December 1995.)

- The Parties provide their own equipment (CPE) to interface with the DS0, DS1, DS3, STS1 and/or OCn circuits on the customer premises.

  - The Point of Interconnection is the point at which the originating Party delivers its originated traffic to the terminating Party’s first point of switching on the terminating Party’s common (shared) network for call transport and termination.  Points of Interconnection are available at either Access Tandems, Local Tandems, or End Offices as described in this Agreement.  CLEC-1’s requested Point of Interconnection will also be used for the receipt and delivery of transit traffic at BellSouth Access and Local Tandems.  Points of Interconnection established at the BellSouth Local Tandem apply only to CLEC-1-originated local and local originating and terminating transit traffic.

  - CLEC-1, at its option, shall establish Points of Presence and Points of Interface for the delivery of its originated local and intraLATA toll traffic to BellSouth.  The Point of Interface may not necessarily be established at the Point of Interconnection.

  - BellSouth shall designate the Points of Presence and Points of Interface for the delivery of its originated local and intraLATA toll traffic to CLEC-1 for call transport and termination by CLEC-1.

  - Interconnection via Purchase of Facilities

    - Either Party may purchase Local Channel facilities from the Party’s specified Point of Interface to its designated serving wire center. The Parties agree that charges for such Local Channel facilities are as set forth in Exhibit A to this Attachment. If a nonrecurring or recurring rate is not identified in Exhibit A for a Local Channel, the rate shall be as set forth in the appropriate BellSouth intrastate or interstate tariff for switched access services.

- Additionally, either Party may purchase Dedicated Transport facilities from its designated serving wire center to the other Party’s first point of switching. The Parties agree that charges for such Dedicated Transport facilities are as set forth in Exhibit A to this Attachment. If a nonrecurring or recurring rate is not identified in Exhibit A for Dedicated Transport, the rate shall be as set forth in the appropriate BellSouth intrastate or interstate tariff for switched access services.

    - For the purposes of this Attachment, Local Channel is defined as a switch transport facility between a Party’s Point of Presence and its designated serving wire center.

    - For the purposes of this Attachment, Serving Wire Center is defined as the wire center owned by one Party from which the other Party would normally obtain dial tone for its Point of Presence.

    - For the purposes of this Attachment, Dedicated Transport is defined as a switch transport facility between a Party’s designated serving wire center and the first point of switching on the other Party’s common (shared) network.

  - BellSouth Multiple Tandem Access (MTA) provides for LATA wide BellSouth transport and termination of CLEC-1-originated local and BellSouth transported intraLATA toll traffic by establishing a Point of Interconnection at a BellSouth access tandem with  routing through multiple BellSouth access tandems as required.  However, CLEC-1 must still establish Points of Interconnection at all BellSouth access tandems where CLEC-1 NXXs are “homed”.   If CLEC-1 does not have NXXs homed at a BellSouth access tandem within a LATA and elects not to establish Points of Interconnection at such BellSouth access tandem, CLEC-1 can order MTA in each BellSouth access tandem within the LATA where it does have a Point of Interconnection and BellSouth will terminate traffic to end-users served through those BellSouth access tandems where CLEC-1 does not have a Point of Interconnection.  MTA shall be provisioned in accordance with BellSouth’s Ordering Guidelines.

    - MTA does not include switched access traffic that transits the BellSouth network to an Interexchange Carrier (IXC).  Switched Access traffic will be delivered to and by IXCs based on CLEC-1’s NXX Access Tandem homing arrangement as specified by CLEC-1 in the national Local Exchange Routing Guide (LERG).

    - For CLEC-1-originated local and intraLATA toll traffic that BellSouth transports but is destined for termination by a  third Party network (transit traffic), BellSouth MTA is required if multiple BellSouth access tandems are necessary to deliver the call to the third Party network.

    - With MTA, compensation for the termination of CLEC-1’s local traffic will be  billed on an elemental basis at the rates specified in Exhibit A to this Attachment on a statewide basis.

    - To the extent CLEC-1 does not purchase MTA in a calling area that has multiple access tandems serving the calling area as defined by BellSouth, CLEC-1 must establish Points of Interconnection to every access tandem in the calling area in order to serve the entire calling area. To the extent CLEC-1 does not purchase MTA and provides intraLATA toll service to its customers, it may be necessary for it to establish a Point of Interconnection to additional BellSouth access tandems that serve end offices outside the local calling area.

  - Local Tandem Interconnection.  This interconnection arrangement allows CLEC-1 to establish a Point of Interconnection at BellSouth local tandems for: (1) the delivery of CLEC-1-originated local traffic transported and terminated by BellSouth to BellSouth end offices within the local calling area as defined in BellSouth’s GSST, section A3 served by those BellSouth local tandems, and (2) for local transit traffic transported by BellSouth for third party network providers who have also established Points of Interconnection at those BellSouth local tandems.

    - When a specified local calling area is served by more than one BellSouth local tandem, CLEC-1 must designate a “home” local tandem for each of its assigned NPA/NXXs and establish trunk connections to such local tandems. Additionally, CLEC-1 may choose to establish a Point of Interconnection at the BellSouth local tandems where it has no codes homing but is not required to do so.  CLEC-1 may deliver local traffic to a “home” BellSouth local tandem that is destined for other BellSouth or third party network provider end offices subtending other BellSouth local tandems in the same local calling area where CLEC-1 does not choose to establish a Point of Interconnection.  It is CLEC-1’s responsibility to enter its own NPA/NXX local tandem homing arrangements into the Local Exchange Routing Guide (LERG) either directly or via a vendor in order for other third party network providers to determine appropriate traffic routing to CLEC-1’s codes.  Likewise, CLEC-1 shall obtain its routing information from the LERG.

    - Notwithstanding establishing Points of Interconnection to BellSouth’s local tandems, CLEC-1 must also establish Points of Interconnection to BellSouth access tandems within the LATA on which CLEC-1 has NPA/NXX’s homed for the delivery of Interexchange Carrier Switched Access (SWA) and toll traffic, and traffic to Type 2A CMRS connections located at the access tandems.  BellSouth cannot switch SWA traffic through more than one BellSouth access tandem.  SWA, Type 2A CMRS or toll traffic routed to the local tandem in error will not be backhauled to the BellSouth access tandem for completion. (Type 2A CMRS interconnection is defined in BellSouth’s A35 General Subscriber Services Tariff.)

    - BellSouth’s provisioning of local tandem interconnection assumes that CLEC-1 has executed the necessary local interconnection agreements with the other third party network providers subtending those local tandems as required by the Act.

  - Fiber Meet

    - "Fiber-Meet" is an interconnection arrangement whereby the Parties physically interconnect their networks via an optical fiber interface (as opposed to an electrical interface) at a mutually agreed upon location, at which one Party's facilities, provisioning, and maintenance responsibility begins and the other Party's responsibility ends (i.e. Point Of Interface).

    - If CLEC-1 elects to establish a Point of Interconnection with BellSouth pursuant to a Fiber Meet, CLEC-1 and BellSouth shall jointly engineer and operate a Synchronous Optical Network ("SONET") transmission system by which they shall interconnect their transmission and routing of local traffic via a Local Channel facility at either the DS0, DS1, or DS3 level and shall be ordered via an Access Services Request (“ASR”) in the initial phase of this offering.  The Parties shall work jointly to determine the specific transmission system.  However, CLEC-1’s SONET transmission must be compatible with BellSouth’s equipment in the serving wire center.  The same vendor’s equipment and software version must be used, and the Data Communications Channel (DCC) must be turned off.

    - BellSouth shall, wholly at its own expense, procure, install and maintain the agreed upon SONET equipment in the BellSouth Interconnection Wire Center ("BIWC").

    - CLEC-1 shall, wholly at its own expense, procure, install and maintain the agreed upon SONET equipment in the CLEC-1 Interconnection Wire Center ("CLEC-1 Wire Center").

    - BellSouth shall designate a Point of Interface outside the BIWC as a Fiber Meet point, and shall make all necessary preparations to receive, and to allow and enable CLEC-1 to deliver, fiber optic facilities into the Point of Interface with sufficient spare length to reach the fusion splice point at the Point of Interface.  BellSouth shall, wholly at its own expense, procure, install, and maintain the fusion splicing point in the Point of Interface.  A Common Language Location Identification ("CLLI") code will be established for each Point of Interface.  The code established must be a building type code.  All orders shall originate from the Point of Interface (i.e., Point of Interface to CLEC-1, Point of Interface to BellSouth).

    - CLEC-1 shall deliver and maintain such strands wholly at its own expense.  Upon verbal request by CLEC-1, BellSouth shall allow CLEC-1 access to the Fiber Meet entry point for maintenance purposes as promptly as possible.

    - The Parties shall jointly coordinate and undertake maintenance of the SONET transmission system.  Each Party shall be responsible for maintaining the components of their own SONET transmission system.

    - Each Party will be responsible for (i) providing its own transport facilities to the Fiber Meet, and (ii) the cost to build-out its facilities to such Fiber Meet.

    - Neither Party shall charge the other for its portion of the Fiber Meet facility used exclusively for non-transit local traffic (i.e. the Local Channel).  Charges incurred for other services including dedicated transport facilities to the Point of Interconnection if applicable will apply.   Charges for Switched and Special Access Services shall be billed in accordance with the applicable Access Service tariff (i.e. the BellSouth Interstate or Intrastate Access Services Tariff).

- Interconnection Trunking and Routing

  - BellSouth and CLEC-1 shall establish interconnecting trunk groups and trunking configurations between networks including the establishment of one-way or two-way trunks in accordance with the BellSouth Call Transport & Termination Service For Facility Based CLECs section of the Facility Based CLEC Activation Requirements Customer Guide as it is revised from time to time.

  - Any CLEC-1 interconnection request that deviates from the standard trunking configurations as described in the BellSouth Call Transport & Termination Service For Facility Based CLECs section of the Facility Based CLEC Activation Requirements Customer Guide that affects traffic delivered to CLEC-1 from a BellSouth switch that requires special BellSouth switch translations and other network modifications will require CLEC-1 to submit a Bona Fide Request/New Business Request via the Bona Fide Request/New Business Request Process set forth in General Terms and Conditions.

  - All terms and conditions, as well as charges, both non-recurring and recurring, associated with interconnecting trunk groups between BellSouth and CLEC-1 not addressed in Exhibit A shall be as set forth in the appropriate BellSouth intrastate or interstate tariff for switched access services.  For two-way trunking that carries the Parties’ local and intraLATA toll traffic, excluding transit traffic, the Parties shall be compensated for the nonrecurring and recurring charges for trunks and DS1 facilities at 50% of the applicable contractual or tariff rates for the services provided by each Party.  CLEC-1 shall be responsible for ordering and paying for any two-way trunks carrying transit traffic.

  - The Parties shall utilize direct end office trunking under the following conditions:

(1) Tandem Exhaust - If a tandem through which the Parties are interconnected is unable to, or is forecasted to be unable to support additional traffic loads for any period of time, the Parties will mutually agree on an end office trunking plan that will alleviate the tandem capacity shortage and ensure completion of traffic between CLEC-1 and BellSouth’s subscribers.

(2) Traffic Volume –To the extent either Party has the capability to measure the amount of traffic between a CLEC-1 switching center and a BellSouth end office, either Party shall install and retain direct end office trunking sufficient to handle actual or reasonably forecasted traffic volumes, whichever is greater, between a CLEC-1 switching center and a BellSouth end office where the traffic exceeds or is forecasted to exceed a single DS1 of local traffic per month.  Either Party will install additional capacity between such points when overflow traffic between CLEC-1’s switching center and BellSouth’s end office exceeds or is forecasted to exceed a single DS1 of local traffic per month.  In the case of one way trunking, additional trunking shall only be required by the Party whose trunking has achieved the preceding usage threshold.

Mutual Agreement - The Parties may install direct end office trunking upon mutual agreement in the absence of the conditions (1) or (2) above and agreement will not unreasonably be withheld.

  - Switched Access traffic will be delivered to and by IXCs based on CLEC-1’s NXX Access Tandem homing arrangement as specified by CLEC-1 in the national Local Exchange Routing Guide (LERG).

  - All trunk groups will be provisioned as Signaling System 7 (SS7) capable where technically feasible.

- Network Design and Management for Interconnection

  - Network Management and Changes. Both Parties will work cooperatively with each other to install and maintain the most effective and reliable interconnected telecommunications networks, including but not limited to, the exchange of toll-free maintenance contact numbers and escalation procedures.  Both Parties agree to provide public notice of changes in the information necessary for the transmission and routing of services using its local exchange facilities or networks, as well as of any other changes that would affect the interoperability of those facilities and networks.

  - Interconnection Technical Standards. The interconnection of all networks will be based upon accepted industry/national guidelines for transmission standards and traffic blocking criteria.  Interconnecting facilities shall conform, at a minimum, to the telecommunications industry standard of DS-1 pursuant to Bellcore Standard No. TR-NWT-00499.  Signal transfer point, Signaling System 7 (“SS7”) connectivity is required at each interconnection point.  BellSouth will provide out-of-band signaling using Common Channel Signaling Access Capability where technically and economically feasible, in accordance with the technical specifications set forth in the BellSouth Guidelines to Technical Publication, TR-TSV-000905.  Facilities of each Party shall provide the necessary on-hook, off-hook answer and disconnect supervision and shall hand off calling number ID (Calling Party Number) when technically feasible.

  - Quality of Interconnection.  The local interconnection for the transmission and routing of telephone exchange service and exchange access that each Party provides to each other will be at least equal in quality to what it provides to itself and any subsidiary or affiliate, where technically feasible, or to any other Party to which each Party provides local interconnection.

  - Network Management Controls.	Both Parties will work cooperatively with each other to apply sound network management principles by invoking appropriate network management controls, e.g., call gapping, to alleviate or prevent network congestion.

  - Common Channel Signaling.	Both Parties will provide LEC-to-LEC Common Channel Signaling (“CCS”) to each other, where available, in conjunction with all traffic in order to enable full interoperability of CLASS features and functions except for call return.  All CCS signaling parameters will be provided, including automatic number identification (“ANI”), originating line information (“OLI”) calling company category, charge number, etc.  All privacy indicators will be honored, and each Party will cooperate with each other on the exchange of Transactional Capabilities Application Part (“TCAP”) messages to facilitate full interoperability of CCS-based features between the respective networks.

  - Forecasting Requirements.

    - The Parties shall exchange technical descriptions and forecasts of their interconnection and traffic requirements in sufficient detail necessary to establish the interconnections required to assure traffic completion to and from all customers in their respective designated service areas. In order for BellSouth to provide as accurate reciprocal trunking forecasts as possible to CLEC-1, CLEC-1 must timely inform BellSouth of any known or anticipated events that may affect BellSouth reciprocal trunking requirements.  If CLEC-1 refuses to provide such information, BellSouth shall provide reciprocal trunking forecasts based only on existing trunk group growth and BellSouth’s annual estimated percentage of BellSouth subscriber line growth.

    - Both Parties shall meet every six months or at otherwise mutually agreeable intervals for the purpose of exchanging non-binding forecast of its traffic and volume requirements for the interconnection and network elements provided under this Agreement, in the form and in such detail as agreed by the Parties. The Parties agree that each forecast provided under this Section shall be deemed “Confidential Information” in the General Terms and Conditions – Part A of this Agreement.

    - The trunk forecast should include trunk requirements for all of the interconnecting trunk groups for the current year plus the next two future years.  The forecast meeting between the two companies may be a face-to-face meeting, video conference or audio conference.  It may be held regionally or geographically.  Ideally, these forecast meetings should be held at least semi-annually, or more often if the forecast is no longer usable.  Updates to a forecast or portions thereof should be made whenever the Party providing the forecast deems that the latest trunk requirements exceed the original quantities by 24 trunks or 10%, whichever is greater.  Either Party should notify the other Party if they have measurements indicating that a trunk group is exceeding its designed call carrying capacity and is impacting other trunk groups in the network.  Also, either Party should notify the other Party if they know of situations in which the traffic load is expected to increase significantly and thus affect the interconnecting trunk requirements as well as the trunk requirements within the other Party’s network.  The Parties agree that the forecast information provided under this Section shall be deemed “Confidential Information” as set forth in the General Terms and Conditions of this Agreement.

    - For a non-binding trunk forecast, agreement between the two Parties on the trunk quantities and the timeframe of those trunks does not imply any liability for failure to perform if the trunks are not available for use at the required time.

    - Signaling Call Information.  BellSouth and CLEC-1 will send and receive 10 digits for local traffic.  Additionally, BellSouth and CLEC-1 will exchange the proper call information, i.e. originated call company number and destination call company number, CIC, and OZZ, including all proper translations for routing between networks and any information necessary for billing.

- Parity in Ordering and Provisioning

Each Party shall provide interconnection ordering and provisioning services to the other Party that are equal to the ordering and provisioning services the Parties provide themselves.  Detailed procedures for ordering and provisioning BellSouth interconnection services are set forth in the BellSouth Call Transport & Termination Service For Facility Based CLECs section of the Facility Based CLEC Activation Requirements Customer Guide.

- Local Dialing Parity

Each Party shall provide local dialing parity, meaning that each Party’s customers will not have to dial any greater number of digits than the other Party’s customers to complete the same call.  In addition, under equivalent interconnection arrangements, CLEC-1 local service customers will experience at least the same quality as BellSouth local service customers regarding post-dial delay, call completion rate and transmission quality.

- Interconnection Compensation

  - Compensation for Local Traffic

    - Local Traffic is defined as any telephone call that originates in one exchange and terminates in either the same exchange, or other local calling area associated with the originating exchange as  defined and specified in Section A3 of BellSouth’s General Subscriber Service Tariff.  As clarification of this definition and for reciprocal compensation, Local Traffic does not include traffic that originates from or terminates to or through an enhanced service provider or information service provider.  As further clarification, Local Traffic does not include calls that do not transmit information of the user’s choosing.  In any event, neither Party will pay reciprocal compensation to the other if the “traffic” to which such reciprocal compensation would otherwise apply was generated, in whole or in part, for the purpose of creating an obligation on the part of the originating carrier to pay reciprocal compensation for such traffic.

    - The Parties shall provide for the mutual and reciprocal recovery of the costs for the elemental functions performed in transporting and terminating local traffic on each other’s network.  The Parties agree that charges for transport and termination of calls on its respective networks are as set forth in Exhibit A to this Attachment.

    - For the purposes of this Attachment, Common (Shared) Transport is defined as the transport of one Party’s traffic by the other Party over the other Party’s common (shared) facilities between the other Party’s tandem switch and end office switch and/or between the other Party’s tandem switches.

    - For the purposes of this Attachment, Tandem Switching is defined as the function that establishes a communications path between two switching offices through a third switching office (the Tandem switch).

    - For the purposes of this Attachment, End Office Switching is defined as the function that establishes a communications path between the trunk side and line side of the End Office switch.

    - If CLEC-1 utilizes a switch outside the LATA and BellSouth chooses to purchase dedicated or common (shared) transport from CLEC-1 for transport and termination of BellSouth originated traffic, BellSouth will pay CLEC-1 no more than the airline miles between the V & H coordinates of the Point of Interface within the LATA where CLEC-1 receives the BellSouth-originated traffic and the V & H coordinates of the BellSouth Exchange Rate Center Area that the CLEC-1 terminating NPA/NXX is associated in the same LATA.  For these situations, BellSouth will compensate CLEC-1 at either dedicated or common (shared) transport rates specified in Exhibit A and based upon the functions provided by CLEC-1 as defined in this Attachment.

    - Neither Party shall represent access services traffic (e.g. Internet Protocol (IP) Telephony, FGA, FGB, etc.) as Local Traffic for purposes of payment of reciprocal compensation.

  - Unidentifiable traffic.  CLEC-1 shall utilize its NPA/NXXs in such a way and will provide the necessary information so that BellSouth shall be able to distinguish Local from IntraLATA Toll traffic for BellSouth originated traffic.  CLEC-1 end users’ assigned NPA/NXX line numbers shall be physically located in the BellSouth rate center with which the NPA/NXX has been associated.  Whenever BellSouth delivers traffic to CLEC-1 for termination on the CLEC-1’s network, if BellSouth cannot determine, because of the manner in which CLEC-1 has utilized its NXX codes whether the traffic is local or toll, BellSouth will charge the applicable rates for originating intrastate network access service as reflected in BellSouth’s Intrastate Access Service Tariff.  BellSouth will make appropriate billing adjustments if CLEC-1 can provide sufficient information for BellSouth to determine whether said traffic is local or toll.

  - Percent Local Use. Each Party will report to the other a Percentage Local Usage (“PLU”). The application of the PLU will determine the amount of local minutes to be billed to the other Party.  For purposes of developing the PLU, each Party shall consider every local call and every long distance call, excluding intermediary traffic.  By the first of January, April, July and October of each year, BellSouth and CLEC-1 shall provide a positive report updating the PLU. Detailed requirements associated with PLU reporting shall be as set forth in BellSouth’s Standard Percent Local Use Reporting Platform for Interconnection Purchasers, as it is amended from time to time during this Agreement.  Notwithstanding the foregoing, where the terminating company has message recording technology that identifies the traffic terminated, such information, in lieu of the PLU factor, shall at the company’s option be utilized to determine the appropriate local usage compensation to be paid.

  - Percentage Interstate Usage.	For combined interstate and intrastate CLEC-1 traffic terminated by BellSouth over the same facilities, CLEC-1 will be required to provide a projected Percentage Interstate Usage (“PIU”) to BellSouth.  All jurisdictional report requirements, rules and regulations for Interexchange Carriers specified in BellSouth’s Intrastate Access Services Tariff will apply to CLEC-1.  After interstate and intrastate traffic percentages have been determined by use of PIU procedures, the PLU factor will be used for application and billing of local interconnection. Notwithstanding the foregoing, where the terminating company has message recording technology that identifies the traffic terminated, such information, in lieu of the PLU factor, shall at the company’s option be utilized to determine the appropriate local usage compensation to be paid.

  - Audits. - On thirty (30) days written notice, each Party must provide the other the ability and opportunity to conduct an annual audit to ensure the proper billing of traffic.  BellSouth and CLEC-1 shall retain records of call detail for a minimum of nine months from which a PLU and/or PIU can be ascertained.  The audit shall be accomplished during normal business hours at an office designated by the Party being audited.  Audit requests shall not be submitted more frequently than one (1) time per calendar year.  Audits shall be performed by a mutually acceptable independent auditory paid for by the Party requesting the audit.  The PLU and/or PIU shall be adjusted based upon the audit results and shall apply to the usage for the quarter the audit was completed, to the usage for the quarter prior to the completion of the audit, and to the usage for the two quarters following the completion of the audit.  If, as a result of an audit, either Party is found to have overstated the PLU and/or PIU by twenty percentage points (20%) or more, that Party shall reimburse the auditing Party for the cost of the audit.

  - Rate True-up

This section applies only to North Carolina and Tennessee.

    - The interim prices for Unbundled Network Elements and Other Services and  Local Interconnection shall be subject to true-up according to the following procedures:

    - The interim prices shall be trued-up, either up or down, based on final prices determined either by further agreement between the Parties, or by a final order (including any appeals) of the Commission which final order meets the criteria of (3) below.  The Parties shall implement the true-up by comparing the actual volumes and demand for each item, together with interim prices for each item, with the final prices determined for each item.  Each Party shall keep its own records upon which the true-up can be based, and any final payment from one Party to the other shall be in an amount agreed upon by the Parties based on such records.  In the event of any disagreement as between the records or the Parties regarding the amount of such true-up, the Parties agree that the body having jurisdiction over the matter shall be called upon to resolve such differences, or the Parties may mutually agree to submit the matter to the Dispute Resolution process in accordance with the provisions of Section 16 of the General Terms and Conditions and Attachment 1 of the Agreement.

    - The Parties may continue to negotiate toward final prices, but in the event that no such agreement is reached within nine (9) months, either Party may petition the Commission to resolve such disputes and to determine final prices for each item.  Alternatively, upon mutual agreement, the Parties may submit the matter to the Dispute Resolution Process set forth in the General Terms and Conditions of the Agreement, so long as they file the resulting Agreement with the Commission as a “negotiated agreement” under Section 252(e) of the Act.

    - A final order of this Commission that forms the basis of a true-up shall be the final order as to prices based on appropriate cost studies, or potentially may be a final order in any other Commission proceeding which meets the following criteria:

(a)	BellSouth and CLEC is entitled to be a full Party to the proceeding;

(b)	It shall apply the provisions of the federal Telecommunications Act of 1996, including but not limited to Section 252(d)(1) (which contains pricing standards) and all then-effective implementing rules and regulations; and,

- It shall include as an issue the geographic deaveraging of unbundled element prices, which deaveraged prices, if any are required by said final order, shall form the basis of any true-up.

  - Compensation for IntraLATA Toll Traffic

    - IntraLATA Toll Traffic. IntraLATA Toll Traffic is defined as any telephone call that is not local or switched access per this Agreement.

    - Compensation for  intraLATA toll traffic.  For terminating its intraLATA toll traffic on the other company’s network, the originating Party will pay the terminating Party BellSouth’s current intrastate or interstate, whichever is appropriate, terminating switched access tariff rates as set forth in BellSouth’s Intrastate or Interstate Access Services Tariff.  The appropriate charges will be determined by the routing of the call. If CLEC-1 is the BellSouth end user’s presubscribed interexchange carrier or if the BellSouth end user uses CLEC-1 as an interexchange carrier on a 101XXXX basis, BellSouth will charge CLEC-1 the appropriate BellSouth tariff charges for originating switched  access services.

    - Compensation for 800 Traffic.  Each Party shall compensate the other pursuant to the appropriate switched access charges, including the database query charge as set forth in the BellSouth intrastate or interstate switched access tariffs.

    - Records for 800 Billing.  Each Party will provide to the other the appropriate records necessary for billing  intraLATA 800 customers.  The records provided will be in a standard EMI format for a fee of $0.013 per record.

    - 800 Access Screening.  Should CLEC-1 require 800 Access Ten Digit Screening Service from BellSouth, it shall have signaling transfer points connecting directly to BellSouth’s local or regional signaling transfer point for service control point database query information. CLEC-1 shall utilize SS7 signaling links, ports and usage as set forth in Attachment 2. CLEC-1 will not utilize switched access FGD service.  800 Access Ten Digit Screening Service is an originating service that is provided via 800 Switched Access Service trunk groups from BellSouth’s SS7 equipped end office or access tandem providing an IXC identification function and delivery of a call to the IXC based on the dialed ten digit number.  The terms and conditions for this service are set out in BellSouth’s Intrastate Access Services Tariff as amended.

  - Mutual Provision of Switched Access Service

    - Switched Access Traffic. Switched Access Traffic is as defined in the BellSouth Access Tariff. Additionally, IP Telephony traffic will be considered switched access traffic.

    - When  BellSouth and CLEC-1 provide an access service connection between an interexchange carrier (“IXC”) and each other, each Party will provide its own access services to the IXC on a multi-bill, multi-tariff meet-point basis. Each Party will bill its own access services rates to the IXC with the exception of the interconnection charge.  The interconnection charge will be billed by the Party providing the end office function.  BellSouth will use the Multiple Exchange Carrier Access Billing (MECAB) system to establish meet point billing for all applicable traffic.  Thirty (30)-day billing periods will be employed for these arrangements.  The recording Party agrees to provide to the initial billing company, at no charge, the switched access detailed usage data within no more than sixty (60) days after the recording date.   The initial billing company will provide the switched access summary usage data to all subsequent billing companies within 10 days of rendering the initial bill to the IXC.   Each company will notify the other when it is not feasible to meet these requirements so that the customers may be notified for any necessary revenue accrual associated with the significantly delayed recording or billing.  As business requirements change data reporting requirements may be modified as necessary.

    - Where either company has been notified that the other company has a Billing Guarantee Practice, each company so notified (the Initial Billing Company or the recording company) will be held liable for any access revenues which it has caused to be determined unbillable under the guidelines of such Billing Guarantee Practice of the other company.  Each company will provide complete documentation to the other to substantiate any claim of unbillable access revenues.  A negotiated settlement will be agreed upon between the companies.

    - Each company will retain for a minimum period of sixty (60) days, access message detail sufficient to recreate any data which is lost or damaged by their company or any third party involved in processing or transporting data.

    - Each company agrees to recreate the lost or damaged data within forty-eight (48) hours of notification by the other or by an authorized third party handling the data.

    - Each company also agrees to process the recreated data within forty-eight (48) hours of receipt at its data processing center.

    - All claims should be filed with the other company within 120 days of the receipt of the date of the unbillable usage.

    - The Initial Billing Company shall keep records of its billing activities relating to jointly-provided Intrastate and Interstate access services in sufficient detail to permit the Subsequent Billing Company to, by formal or informal review or audit, to verify the accuracy and reasonableness of the jointly-provided access billing data provided by the Initial billing Company. Each company agrees to cooperate in such formal or informal reviews or audits and further agrees to jointly review the findings of such reviews or audits in order to resolve any differences concerning the findings thereof.

  - Transit Traffic Service.  Each Party shall provide tandem switching and transport services for the other’s transit traffic. Transit traffic is traffic originating on one Party’s network that is switched and transported by the other Party and terminates on a third party’s network.  Rates for local transit traffic shall be the applicable call transport and termination charges as set forth in Exhibit A to this Attachment.  Rates for intraLATA toll and Switched Access transit traffic shall be the applicable call transport and termination as set forth in BellSouth Interstate or Intrastate Switched Access tariffs. Billing associated with all transit traffic shall be pursuant to MECAB procedures. Wireless Type 1 traffic shall not be treated as transit traffic from a routing or billing perspective.  Wireless Type 2A traffic shall not be treated as transit traffic from a routing or billing perspective until BellSouth and the Wireless carrier have the capability to properly meet-point-bill in accordance with MECAB guidelines.

    - The delivery of traffic which transits the BellSouth network and is transported to another carrier’s network is excluded from any BellSouth billing guarantees and will be delivered at the rates as set forth in Exhibit A to this Attachment. CLEC-1 is responsible for and shall negotiate the necessary agreements or the placement of valid orders with the terminating carrier for the receipt of this traffic through the BellSouth network.  BellSouth will not be liable for any compensation to the terminating carrier as a result of providing the transit function.  An agreement or valid order with the terminating carrier will be established prior to the delivery of any transit traffic to BellSouth destined for the particular carrier’s network.   Further, CLEC-1 agrees to compensate BellSouth for any charges or costs for the delivery of transit traffic to a connecting carrier on behalf of CLEC-1 for which a valid contract or order has not been established.  Additionally, the Parties agree that any billing to a third party or other telecommunications carrier under this section shall be pursuant to MECAB procedures.

  - Interconnection with Enhanced Service Providers (ESPs)/Information Service Providers (ISPs). ESP/ISP traffic shall not be included in the  interconnection compensation arrangements of this Agreement.

- Frame Relay Service

  - In addition to the Local Interconnection services set forth above, BellSouth will offer a network to network Interconnection arrangement between BellSouth’s and CLEC-1’s frame relay switches as set forth below.  The following provisions will apply only to Frame Relay Service and Exchange Access Frame Relay Service in those states in which CLEC-1 is certified and providing Frame Relay Service as a Local Exchange Carrier and where traffic is being exchanged between CLEC-1 and BellSouth Frame Relay Switches in the same LATA.

  - The Parties agree to establish two-way Frame Relay facilities between their respective Frame Relay Switches to the mutually agreed upon Frame Relay Service point(s) of interconnection (“POI(s)”) within the LATA.  All POIs shall be within the same Frame Relay Network Serving Areas as defined in Section A40 of BellSouth’s General Subscriber Service Tariff except as set forth in this Attachment.

  - Upon the request of either Party, such interconnection will be established where BellSouth and CLEC-1 have Frame Relay Switches in the same LATA.  Where there are multiple Frame Relay switches in one central office, an interconnection with any one of the switches will be considered an interconnection with all of the switches at that central office for purposes of routing packet traffic.

  - The Parties agree to provision local and IntraLATA Frame Relay Service and Exchange Access Frame Relay Service (both intrastate and interstate) over Frame Relay interconnection facilities between the respective Frame Relay switches and the POIs.

  - The Parties agree to assess each other reciprocal charges for the facilities that each provides to the other according to the Percent Local Circuit Use Factor (PLCU), determined as follows:

    - If the data packets originate and terminate in locations in the same LATA, and consistent with the local definitions of the Agreement, the traffic is considered local.  Frame Relay framed packet data is transported within Virtual Circuits (VC).  For the purposes of this Agreement, if all the data packets transported within a VC remain within the LATA, then consistent with the local definitions in this Agreement, the traffic on that VC is local (“Local VC”).

    - If the originating and terminating locations of the two way packet data traffic are not in the same LATA, the traffic on that VC is interLATA (“InterLATA VC”).

    - The PLCU is determined by dividing the total number of Local VCs, by the total number of VCs on each Frame Relay facility.  To facilitate implementation, CLEC-1 may determine its PLCU in aggregate, by dividing the total number of Local VCs in a given LATA by the total number VCs in that LATA.  The Parties agree to renegotiate the method for determining PLCU, at BellSouth’s request, and within 90 days, if BellSouth notifies CLEC-1 that it has found that this method does not adequately represent the PLCU.

    - If there are no VCs on a facility when it is billed, the PLCU will be zero.

    - BellSouth will provide the circuit between the Parties’ respective Frame Relay Switches.  The Parties will be compensated as follows:  BellSouth will invoice, and CLEC-1 will pay, the total non-recurring and recurring charges for the circuit based upon the rates set forth in BellSouth’s Interstate Access Tariff, FCC No. 1.  CLEC-1 will then invoice, and BellSouth will pay, an amount calculated by multiplying the BellSouth billed charges for the circuit by one-half of CLEC-1’s PLCU.

  - The Parties agree to compensate each other for Frame Relay network-to-network interface (NNI) ports based upon the NNI rates set forth in BellSouth's Interstate Access Tariff, FCC No. 1 .  Compensation for each pair of NNI ports will be calculated as follows:  BellSouth will invoice, and CLEC-1 will pay, the total non-recurring and recurring charges for the NNI port.  CLEC-1 will then invoice, and BellSouth will pay, an amount calculated by multiplying the BellSouth billed non-recurring and recurring charges for the NNI port by CLEC-1’s PLCU.

  - Each Party agrees that there will be no charges to the other Party for its own subscriber’s Permanent Virtual Circuit (PVC) rate elements for the local PVC segment from its Frame Relay switch to its own subscriber’s premises. PVC rate elements include the Data Link Connection Identifier (DLCI) and Committed Information Rate (CIR).

  - For the PVC segment between the CLEC-1 and BellSouth Frame Relay switches, compensation for the PVC charges is based upon the rates in BellSouth's Interstate Access Tariff, FCC No. 1.

  - Compensation for PVC rate elements will be calculated as follows:

    - If CLEC-1 orders a VC connection between a BellSouth subscriber’s PVC segment and a PVC segment from the BellSouth Frame Relay switch to the CLEC-1 Frame Relay switch, BellSouth will invoice, and CLEC-1 will pay, the total non-recurring and recurring PVC charges for the PVC segment between the BellSouth and CLEC-1 Frame Relay switches.  If the VC is a Local VC, CLEC-1 will then invoice and BellSouth will pay, the total nonrecurring and recurring PVC charges billed for that segment.  If the VC is not local, no compensation will be paid to CLEC-1 for the PVC segment.

    - If BellSouth orders a Local VC connection between a CLEC-1 subscriber’s PVC segment and a PVC segment from the CLEC-1 Frame Relay switch to the BellSouth Frame Relay switch, BellSouth will invoice, and CLEC-1 will pay, the total non-recurring and recurring PVC and CIR charges for the PVC segment between the BellSouth and CLEC-1 Frame Relay switches.  If the VC is a Local VC, CLEC-1 will then invoice and BellSouth will pay the total non-recurring and recurring PVC and CIR charges billed for that segment.  If the VC is not local, no compensation will be paid to CLEC-1 for the PVC segment.

    - The Parties agree to compensate each other for requests to change a PVC segment or PVC service order record, according to the Feature Change charge as set forth in the BellSouth access tariff BellSouth Tariff FCC No, 1.

    - If CLEC-1 requests a change, BellSouth will invoice and CLEC-1 will pay a Feature Change charge for each affected PVC segment.

      - If BellSouth requests a change to a Local VC, CLEC-1 will invoice and BellSouth will pay a Feature Change charge for each affected PVC segment.

    - The Parties agree to limit the sum of the CIR for the VCs on a DS1 NNI port to not more than three times the port speed, or not more than six times the port speed on a DS3 NNI port.

    - Except as expressly provided herein, this Agreement does not address or alter in any way either Party’s provision of Exchange Access Frame Relay Service or interLATA Frame Relay Service.  All charges by each Party to the other for carriage of Exchange Access Frame Relay Service or interLATA Frame Relay Service are included in the BellSouth access tariff BellSouth Tariff FCC No, 1.

  - CLEC-1 will identify and report quarterly to BellSouth the PLCU of the Frame Relay facilities it uses, per section 8.5.3 above.

  - Either Party may request a review or audit of the various service components, consistent with the provisions of section E2 of the BellSouth State Access Services tariffs or Section 2 of the BellSouth FCC No.1 Tariff.

  - If during the term of this Agreement, BellSouth obtains authority to provide interLATA Frame Relay in any State, the Parties agree to renegotiate this arrangement for the exchange of Frame Relay Service Traffic within one hundred eighty (180) days of the date BellSouth receives interLATA authority.  In the event the Parties fail to renegotiate this Section 8 within the one hundred eighty day period, they will submit this matter to the appropriate State commission(s) for resolution.

- Operational Support Systems (OSS) Rates

BellSouth has developed and made available the following mechanized systems by which CLEC-1 may submit LSRs electronically.

LENS		Local Exchange Navigation System

EDI		Electronic Data Interface

EDI-PC	Electronic Data Interface – Personal Computer

TAG		Telecommunications Access Gateway

LSRs submitted by means of one of these interactive interfaces will incur an OSS electronic ordering charge as specified in the table below.  An individual LSR will be identified for billing purposes by its Purchase Order Number (PON).  LSRs submitted by means other than one of these interactive interfaces (mail, fax, courier, etc.) will incur a manual order charge as specified in the table below:

| ## OPERATIONAL SUPPORT SYSTEMS | AL, GA, LA, MS, SC | FL, KY, NC, TN |
| --- | --- | --- |
| OSS LSR charge, per LSR received from the CLEC by one of the OSS interactive interfaces | $3.50 SOMEC | $3.50 SOMEC |
| Incremental charge per LSR received from the CLEC by means other than one of the OSS interactive interfaces | See applicable rate element | $19.99 SOMAN |

Note:  In addition to the OSS charges, applicable discounted service order and related          discounted charges apply per the tariff.

Denial/Restoral OSS Charge

In the event CLEC-1 provides a list of customers to be denied and restored, rather than an LSR, each location on the list will require a separate PON and, therefore will be billed as one LSR per location.

Cancellation OSS Charge

CLEC-1 will incur an OSS charge for an accepted LSR that is later canceled by CLEC-1.

Note:  Supplements or clarifications to a previously billed LSR will not incur another OSS charge.

Network Elements and Other Services Manual Additive

The Commissions in Alabama, Georgia, Louisiana, Mississippi and South Carolina have ordered incremental manual non-recurring charges (NRC) for Network Elements and Other Services ordered by means other than one of the interactive interfaces.  These ordered Network Elements and Other Services manual additive NRCs will apply in these states, rather than the charge per LSR.

Threshold Billing Plan

The Parties agree that CLEC-1 will incur the mechanized rate for all LSRs, both mechanized and manual, if the percentage of mechanized LSRs to total LSRs meets or exceeds the threshold percentages shown below:

Year		Ratio: Mechanized/Total LSRs

- 70%

- 80%

- 90%

The threshold plan will be discontinued in 2002.

BellSouth will track the total LSR volume for each CLEC for each quarter.  At the end of that time period, a Percent Electronic LSR calculation will be made for that quarter based on the LSR data tracked in the LCSC.  If this percentage exceeds the threshold volume, all of that CLECs’ future manual LSRs will be billed at the mechanized LSR rate.  To allow time for obtaining and analyzing the data and updating the billing system, this billing change will take place on the first day of the second month following the end of the quarter (e.g. May 1 for 1Q, Aug 1 for 2Q, etc.).  There will be no adjustments to the amount billed for previously billed LSRs.

The Parties agree that any charges BellSouth is unable to bill on April 15, 1999 will be trued up on or about July 1, 1999.