Attachment 3

Network Interconnection

TABLE OF CONTENTS

1.	Network Interconnection	3

2.	Interconnection Trunking Architectures	7

3.	Network Design And Management For Interconnection	13

4.	Local Dialing Parity	15

5.	Interconnection Compensation	15

6.	Frame Relay Service	22

7.	 Remote Access Server (RAS) Network Interconnection	25

8.	Operational Support Systems (OSS) Rates	28

Rates...........................................................................................................…..............Exhibit A

Basic Architecture………………………………………………….………………..Exhibit B

One-Way Trunking Architecture………………………………….……………….Exhibit C

Two-Way Trunking Architecture……………………………….………………….Exhibit D

Supergroup Architecture……………………………………………………………Exhibit E

The Parties shall provide interconnection with each other’s networks for the transmission and routing of telephone exchange service (local) and exchange access (intraLATA toll and switched access) on the following terms:

# Network Interconnection

All negotiated rates, terms and conditions set forth in this Attachment pertain to the provision of network interconnection.

  - Interconnection is available to both Parties through: (1) delivery of a Party’s facilities to a collocation arrangement or Fiber Meet arrangement as defined in this Agreement; or (2) interconnection via purchase of facilities from the other Party.  Interconnection may be provided by the Parties at any other technically feasible point.  Requests to BellSouth for interconnection at other points may be made through the Bona Fide Request/New Business Request process set out in General Terms and Conditions.

  - NorthPoint must establish, at a minimum, a single Point of Presence, Interface, and Interconnection with BellSouth within the LATA for the delivery of NorthPoint’s originated local and intraLATA toll traffic and for the receipt and delivery of transit traffic.  If NorthPoint chooses to interconnect at a single Point of Interconnection within a LATA, the interconnection must be at a BellSouth Access Tandem. Furthermore, NorthPoint must establish Points of Interconnection at all BellSouth access and local tandems where NorthPoint NXXs are “homed.”  A “Homing” arrangement is defined by a “Final” Trunk Group between the BellSouth Tandem and NorthPoint End Office switch.  A “Final” Trunk Group is the last choice telecommunications path between the Tandem and End Office switch.  It is NorthPoint’s responsibility to enter its own NPA/NXX access and/or local tandem “homing” arrangements into the national Local Exchange Routing Guide (LERG).

    - In order for NorthPoint to home its NPA/NXX(s) on a BellSouth Tandem, NorthPoint’s NPA/NXX(s) must be assigned to an Exchange Rate Center Area served by that BellSouth Tandem and as specified by BellSouth.  The specified association between BellSouth Tandems and Exchange Rate Center Areas is defined in the Local Exchange Routing Guide (LERG) as it is revised from time to time.

  - A Point of Presence (POP) is the physical location (a structure where the environmental, power, air conditioning, etc. specifications for a Party’s terminating equipment can be met) at which a Party establishes itself for obtaining access to the other Party’s network.  The POP is the physical location within which the Point of Interfaces occur.

  - A Point of Interface is the physical telecommunications interface between BellSouth and NorthPoint’s interconnection functions.  It establishes the technical interface and point of operational responsibility.  The primary function of the Point of Interface is to serve as the terminus for the interconnection service.  The Point of Interface has the following main characteristics:

1.  	It is a cross-connect point to allow connection, disconnection, transfer or restoration of service.

2.	It is a point where BellSouth and NorthPoint can verify and maintain specific performance objectives.

3.	It is specified according to the interface offered in the tariff or local interconnection agreement (for example:  for DS1 service the FCC # 1 tariff specifies that the interface meets the technical specifications detailed in Generic Requirements GR-342-CORE, Issue 1, December 1995.)

- The Parties provide their own equipment (CPE) to interface with the DS0, DS1, DS3, STS1 and/or OCn circuits on the customer premises.

  - The Point of Interconnection is the point at which the originating Party delivers its originated traffic to the terminating Party’s first point of switching on the terminating Party’s common (shared) network for call transport and termination.  Points of Interconnection are available at either Access Tandems, Local Tandems, or End Offices as described in this Agreement.  NorthPoint’s requested Point of Interconnection will also be used for the receipt and delivery of transit traffic at BellSouth Access and Local Tandems.  Points of Interconnection established at the BellSouth Local Tandem apply only to NorthPoint-originated local and local originating and terminating transit traffic.

  - NorthPoint, at its option, shall establish Points of Presence and Points of Interface for the delivery of its originated local and intraLATA toll traffic to BellSouth.  The Point of Interface may not necessarily be established at the Point of Interconnection.

  - BellSouth, at its option, shall designate the Points of Presence and Points of Interface for the delivery of its originated local and intraLATA toll traffic to NorthPoint for call transport and termination by NorthPoint.   The Point of Interface may not necessarily be established at the Point of Interconnection.

  - Interconnection via Leased Dedicated Transport  Facilities

    - The originating Party may purchase Local Channel facilities from the terminating Party from the originating Party’s specified Point of Interface to its serving wire center. The Parties agree that charges for such Local Channel facilities are as set forth in Exhibit A to this Attachment. If a nonrecurring or recurring rate is not identified in Exhibit A for a Local Channel, the rate shall be as set forth in the appropriate BellSouth intrastate or interstate tariff for switched access services.

    - Additionally, either Party may purchase Dedicated Interoffice Transport facilities from its designated serving wire center to the other Party’s first point of switching. The Parties agree that charges for such Dedicated Transport facilities are as set forth in Exhibit A to this Attachment. If a nonrecurring or recurring rate is not identified in Exhibit A for Dedicated Transport, the rate shall be as set forth in the appropriate BellSouth intrastate or interstate tariff for switched access services.

    - For the purposes of this Attachment, Local Channel is defined as a switch transport facility between a Party’s Point of Presence and its designated serving wire center.

    - For the purposes of this Attachment, Serving Wire Center is defined as the wire center owned by one Party from which the other Party would normally obtain dial tone for its Point of Presence.

    - For the purposes of this Attachment, Dedicated Interoffice Transport is defined as a switch transport facility between a Party’s designated serving wire center and the first point of switching on the other Party’s common (shared) network.

  - Fiber Meet

    - Fiber Meet is an interconnection arrangement whereby the Parties physically interconnect their networks via an optical fiber interface (as opposed to an electrical interface) at which one Party's facilities, provisioning, and maintenance responsibility begins and the other Party's responsibility ends (i.e. Point of Interface).

    - If NorthPoint elects to interconnect with BellSouth pursuant to a Fiber Meet, NorthPoint and BellSouth shall jointly engineer and operate a Synchronous Optical Network ("SONET") transmission system by which they shall interconnect their transmission and routing of local traffic via a Local Channel facility at either the DS0, DS1, or DS3 level.  The Parties shall work jointly to determine the specific transmission system.  However, NorthPoint’s SONET transmission must be compatible with BellSouth’s equipment in the BellSouth Interconnection Wire Center.  The same vendor’s equipment and software version must be used, and the Data Communications Channel (DCC) must be turned off.

    - BellSouth shall, wholly at its own expense, procure, install and maintain the agreed upon SONET equipment in the BellSouth Interconnection Wire Center ("BIWC").

    - NorthPoint shall, wholly at its own expense, procure, install and maintain the agreed upon SONET equipment in the NorthPoint Interconnection Wire Center ("NorthPoint Wire Center").

    - BellSouth shall designate a Point of Interface outside the BIWC as a Fiber Meet point, and shall make all necessary preparations to receive, and to allow and enable NorthPoint to deliver, fiber optic facilities into the Point of Interface with sufficient spare length to reach the fusion splice point at the Point of Interface.  BellSouth shall, wholly at its own expense, procure, install, and maintain the fusion splicing point in the Point of Interface.  A Common Language Location Identification ("CLLI") code will be established for each Point of Interface.  The code established must be a building type code.  All orders shall originate from the Point of Interface (i.e., Point of Interface to NorthPoint, Point of Interface to BellSouth).

    - NorthPoint shall deliver and maintain such strands wholly at its own expense.  Upon verbal request by NorthPoint, BellSouth shall allow NorthPoint access to the Fiber Meet entry point for maintenance purposes as promptly as possible.

    - The Parties shall jointly coordinate and undertake maintenance of the SONET transmission system.  Each Party shall be responsible for maintaining the components of their own SONET transmission system.

    - Each Party will be responsible for (i) providing its own transport facilities to the Fiber Meet, and (ii) the cost to build-out its facilities to such Fiber Meet.

    - Neither Party shall charge the other for its portion of the Fiber Meet facility used exclusively for non-transit local traffic (i.e. the Local Channel).  Charges incurred for other services including dedicated transport facilities to the Point of Interconnection if applicable will apply.   Charges for Switched and Special Access Services shall be billed in accordance with the applicable Access Service tariff (i.e. the BellSouth Interstate or Intrastate Access Services Tariff).

# Interconnection Trunking Architectures

  - BellSouth and NorthPoint shall establish interconnecting trunk groups and trunking configurations between networks including the establishment of one-way or two-way trunks in accordance with the following provisions set forth in this Agreement.

  - Any NorthPoint interconnection request that deviates from the standard trunking  architectures as described in  this Agreement  that affects traffic delivered to NorthPoint from a BellSouth switch that requires special BellSouth switch translations and other network modifications will require NorthPoint to submit a Bona Fide Request/New Business Request via the Bona Fide Request/New Business Request Process set forth in General Terms and Conditions.

  - All terms and conditions, as well as charges, both non-recurring and recurring, associated with interconnecting trunk groups between BellSouth and NorthPoint not addressed in Exhibit A shall be as set forth in the appropriate BellSouth intrastate or interstate tariff for switched access services.  For two-way trunking that carries the Parties’ local and intraLATA toll traffic only, excluding trunking that carries Transit Traffic, the Parties shall be compensated for the nonrecurring and recurring charges for trunks and DS1 facilities at 50% of the applicable contractual or tariff rates for the services provided by each Party.  NorthPoint shall be responsible for ordering and paying for any two-way trunks carrying transit traffic.  Furthermore, NorthPoint shall be responsible for the compensation for two-way trunking that it orders for its local and intraLATA toll but utilizes unidirectionally.

  - Switched Access traffic will be delivered to and by IXCs based on NorthPoint’s NXX Access Tandem homing arrangement as specified by NorthPoint in the national Local Exchange Routing Guide (LERG).

  - All trunk groups will be provisioned as Signaling System 7 (SS7) capable where technically feasible.  If SS7 is not technically feasible multi-frequency (MF) protocol signaling shall be used.

  - In cases where NorthPoint is also an IXC, the IXC’s Feature Group D (FG D) trunking must remain separate from the local interconnection trunking.

2.7		Two-Way Trunking Requirements:

The following requirements apply to two-way trunking that carries the Parties  local and intraLATA toll.

- NorthPoint will initiate two-way trunk request.  The use of and quantity of two way trunking shall be mutually agreed upon and shall be jointly provisioned.

- The Point of Interface will be located at a mutually agreed location or point designated by BellSouth.  If an agreement cannot be reached on the location of the Point of Interface, each company will establish its own Point of Interface and order one-way trunks.

- BellSouth and NorthPoint will jointly review the trunk forecast, as needed, on a periodic basis, or at least every six (6) months.

- NorthPoint will order trunks using access service request (ASR) process in place for Local Interconnection after the joint planning meeting takes place between BellSouth and NorthPoint.

- BellSouth and NorthPoint must agree on traffic engineering parameters that will be used in the engineering of the trunk groups.

- BellSouth and NorthPoint must agree to meet and resolve service-affecting situations in a timely manner.  This contact will normally be made through the Account Team.

- Establishing a two-way trunk group does not preclude BellSouth or NorthPoint from adding one-way trunk groups within the same Local Calling Area.

- For technical reasons, two-way trunk groups may not be ordered to a BellSouth DMS100 Local Tandem or DMS100 End Office.

- BellSouth will be responsible for the installation and maintenance of its trunks and facilities to the mutually agreed Point of Interface, and NorthPoint will be responsible for the installation and maintenance of its trunks and facilities to the mutually agreed to Point of Interface.

2.8		BellSouth Access Tandem Interconnection Architectures

2.8.1	BellSouth Access Tandem Interconnection provides intratandem access to subtending end offices.  BellSouth Multiple Tandem Access (MTA), described later in this Agreement, may be ordered using any of the following access tandem architectures.

    - Basic Architecture

      - In this architecture, NorthPoint’s originating Local and IntraLATA Toll and originating and terminating Transit Traffic is transported on a single two-way trunk group between NorthPoint and BellSouth access tandem(s) within a LATA.  This group carries intratandem Transit Traffic between NorthPoint and Independent Companies, Interexchange Carriers, other CLECs and other network providers with which NorthPoint desires interconnection and has the proper contractual arrangements.  This group also carries NorthPoint originated intertandem traffic transiting a single BellSouth access tandem destined to third party tandems such as an Independent Company tandem or other CLEC tandem.  BellSouth originated Local and IntraLATA Toll traffic is transported on a single one-way trunk group terminating to NorthPoint.  The Two-way Trunking Requirements described in this Attachment do not apply to this architecture.  Other trunk groups for operator services, directory assistance, emergency services and intercept may be established if required. The LERG should be referenced for current routing and tandem serving arrangements. The Basic Architecture is illustrated in Exhibit B.

    - One-Way Trunking Architecture

2.8.3.1	In this architecture, the Parties interconnect using two one-way trunk groups.  One one-way trunk group carries NorthPoint-originated local and intraLATA toll traffic destined for BellSouth end-users.  The other one-way trunk group carries BellSouth-originated local and intraLATA toll traffic destined for NorthPoint end-users.  A third two-way trunk group is established for NorthPoint’s originating and terminating Transit Traffic.  This group carries intratandem Transit Traffic between NorthPoint and Independent Companies, Interexchange Carriers, other CLECs and other network providers with which NorthPoint desires interconnection and has the proper contractual arrangements.  This group also carries NorthPoint originated intertandem traffic transiting a single BellSouth access tandem destined to third party tandems such as an Independent Company tandem or other CLEC tandem.  Other trunk groups for operator services, directory assistance, emergency services and intercept may be established if required.   The LERG should be referenced for current routing and tandem serving arrangements.  The One-Way Trunking Architecture is illustrated in Exhibit C.

    - Two-Way Trunking Architecture

      - The Two-Way Trunking Architecture establishes one two-way trunk group to carry local and intraLATA toll traffic between NorthPoint and BellSouth.  To establish this architecture, NorthPoint and BellSouth must meet the Two-way Trunking Requirements described in this Attachment.  In addition, a two-way transit trunk group must be established for NorthPoint’s originating and terminating Transit Traffic. This group carries intratandem Transit Traffic between NorthPoint and Independent Companies, Interexchange Carriers, other CLECs and other network providers with which NorthPoint desires interconnection and has the proper contractual arrangements.  This group also carries NorthPoint originated intertandem traffic transiting a single BellSouth access tandem destined to third party tandems such as an Independent Company tandem or other CLEC tandem.  Other trunk groups for operator services, directory assistance, emergency services and intercept may be established if required. The LERG should be referenced for current routing and tandem serving arrangements. The Two-Way Trunking Architecture is illustrated in Exhibit D.

    - Supergroup Architecture

      - In the Supergroup Architecture, the Parties Local and  IntraLATA Toll and NorthPoint’s Transit Traffic is  exchanged on a single two-way trunk group between NorthPoint  and  BellSouth. To establish this architecture, NorthPoint and BellSouth must meet the Two-way Trunking Requirements described in this Attachment.  This group carries intratandem Transit Traffic between NorthPoint and Independent Companies, Interexchange Carriers, other CLECs and other network providers with which NorthPoint desires interconnection and has the proper contractual arrangements.  This group also carries NorthPoint originated intertandem traffic transiting a single BellSouth access tandem destined to third party tandems such as an Independent Company tandem or other CLEC tandem.  Other trunk groups for operator services, directory assistance, emergency services and intercept may be established if required. The LERG should be referenced for current routing and tandem serving arrangements. The Supergroup Architecture is illustrated in Exhibit E.

  - BellSouth Multiple Tandem Access (MTA) provides for LATA wide BellSouth transport and termination of NorthPoint-originated intraLATA toll and local traffic, that is transported by BellSouth, by establishing a Point of Interconnection at a BellSouth access tandem with routing through multiple BellSouth access tandems as required.  However, NorthPoint must still establish Points of Interconnection at all BellSouth access tandems where NorthPoint NXXs are “homed”.   If NorthPoint does not have NXXs homed at a BellSouth access tandem within a LATA and elects not to establish Points of Interconnection at such BellSouth access tandem, NorthPoint can order MTA in each BellSouth access tandem within the LATA where it does have a Point of Interconnection and BellSouth will terminate traffic to end-users served through those BellSouth access tandems where NorthPoint does not have a Point of Interconnection.  MTA shall be provisioned in accordance with BellSouth’s Ordering Guidelines.

  - MTA does not include switched access traffic that transits the BellSouth network to an Interexchange Carrier (IXC).  Switched Access traffic will be delivered to and by IXCs based on NorthPoint’s NXX Access Tandem homing arrangement as specified by NorthPoint in the national Local Exchange Routing Guide (LERG).

  - For NorthPoint-originated local and intraLATA toll traffic that BellSouth transports but is destined for termination by a third Party network (transit traffic), BellSouth MTA is required if multiple BellSouth access tandems are necessary to deliver the call to the third Party network.

  - The Parties agree that compensation for the BellSouth transport and/or termination of NorthPoint’s local and intraLATA toll traffic will be billed on a statewide basis at the applicable rates specified in Exhibit A to this Attachment for local traffic and at the BellSouth intrastate switched access tariff rates for intraLATA toll traffic.

  - To the extent NorthPoint does not purchase MTA in a calling area that has multiple access tandems serving the calling area as defined by BellSouth, NorthPoint must establish Points of Interconnection to every access tandem in the calling area in order to serve the entire calling area. To the extent NorthPoint does not purchase MTA and provides intraLATA toll service to its customers, it may be necessary for it to establish a Point of Interconnection to additional BellSouth access tandems that serve end offices outside the local calling area.  To the extent NorthPoint routes its traffic in such a way that utilizes BellSouth’s MTA service without properly ordering MTA service, NorthPoint agrees to pay BellSouth the associated transport and termination charges.

  - BellSouth End Office Interconnection

    - NorthPoint may establish interconnection at BellSouth end offices for the delivery of NorthPoint originated local and intralata toll traffic destined for BellSouth end-users served by that end-office.

    - When end office trunking is ordered by BellSouth to deliver BellSouth originated traffic to NorthPoint, BellSouth will provide overflow routing through BellSouth tandems consistent with how BellSouth overflows it’s traffic.  The overflow will be based on the homing arrangements NorthPoint displays in the LERG.  Likewise, if NorthPoint interconnects to a BellSouth end office for delivery of NorthPoint originated traffic, NorthPoint will overflow the traffic through the BellSouth tandems based on the BellSouth homing arrangements shown in the LERG.

    - The Parties shall utilize direct end office trunking under the following conditions:

(1) Tandem Exhaust - If a tandem through which the Parties are interconnected is unable to, or is forecasted to be unable to support additional traffic loads for any period of time, the Parties will mutually agree on an end office trunking plan that will alleviate the tandem capacity shortage and ensure completion of traffic between NorthPoint and BellSouth’s subscribers.

(2) Traffic Volume –To the extent either Party has the capability to measure the amount of traffic between a NorthPoint switching center and a BellSouth end office, either Party shall install and retain direct end office trunking sufficient to handle actual or reasonably forecasted traffic volumes, whichever is greater, between a NorthPoint switching center and a BellSouth end office where the traffic exceeds or is forecasted to exceed a single DS1 of local traffic per month.  Either Party will install additional capacity between such points when overflow traffic between NorthPoint’s switching center and BellSouth’s end office exceeds or is forecasted to exceed a single DS1 of local traffic per month.  In the case of one way trunking, additional trunking shall only be required by the Party whose trunking has achieved the preceding usage threshold.

- Mutual Agreement - The Parties may install direct end office trunking upon mutual agreement in the absence of conditions (1) or (2) above and agreement will not unreasonably be withheld.

  - Local Tandem Interconnection.

    - This interconnection arrangement allows NorthPoint to establish a Point of Interconnection at BellSouth local tandems for: (1) the delivery of NorthPoint-originated local traffic transported and terminated by BellSouth to BellSouth end offices within the local calling area as defined in BellSouth’s General Subscriber Services Tariff (GSST), section A3 served by those BellSouth local tandems, and (2) for local transit traffic transported by BellSouth for third party network providers who have also established Points of Interconnection at those BellSouth local tandems.

    - When a specified local calling area is served by more than one BellSouth local tandem, NorthPoint must designate a “home” local tandem for each of its assigned NPA/NXXs and establish trunk connections to such local tandems. Additionally, NorthPoint may choose to establish a Point of Interconnection at the BellSouth local tandems where it has no codes homing but is not required to do so.  NorthPoint may deliver local traffic to a “home” BellSouth local tandem that is destined for other BellSouth or third party network provider end offices subtending other BellSouth local tandems in the same local calling area where NorthPoint does not choose to establish a Point of Interconnection.  It is NorthPoint’s responsibility to enter its own NPA/NXX local tandem homing arrangements into the Local Exchange Routing Guide (LERG) either directly or via a vendor in order for other third party network providers to determine appropriate traffic routing to NorthPoint’s codes.  Likewise, NorthPoint shall obtain its routing information from the LERG.

    - Notwithstanding establishing Points of Interconnection to BellSouth’s local tandems, NorthPoint must also establish Points of Interconnection to BellSouth access tandems within the LATA on which NorthPoint has NPA/NXXs homed for the delivery of Interexchange Carrier Switched Access (SWA) and toll traffic, and traffic to Type 2A CMRS connections located at the access tandems.  BellSouth shall not switch SWA traffic through more than one BellSouth access tandem.  SWA, Type 2A CMRS or toll traffic routed to the local tandem in error will not be backhauled to the BellSouth access tandem for completion. (Type 2A CMRS interconnection is defined in BellSouth’s A35 General Subscriber Services Tariff.)

    - BellSouth’s provisioning of local tandem interconnection assumes that NorthPoint has executed the necessary local interconnection agreements with the other third party network providers subtending those local tandems as required by the Act.

# Network Design And Management For Interconnection

  - Network Management and Changes. Both Parties will work cooperatively with each other to install and maintain the most effective and reliable interconnected telecommunications networks, including but not limited to, the exchange of toll-free maintenance contact numbers and escalation procedures.  Both Parties agree to provide public notice of changes in the information necessary for the transmission and routing of services using its local exchange facilities or networks, as well as of any other changes that would affect the interoperability of those facilities and networks.

  - Interconnection Technical Standards. The interconnection of all networks will be based upon accepted industry/national guidelines for transmission standards and traffic blocking criteria.  Interconnecting facilities shall conform, at a minimum, to the telecommunications industry standard of DS-1 pursuant to Bellcore Standard No. TR-NWT-00499.  Signal transfer point, Signaling System 7 (“SS7”) connectivity is required at each interconnection point.  BellSouth will provide out-of-band signaling using Common Channel Signaling Access Capability where technically and economically feasible, in accordance with the technical specifications set forth in the BellSouth Guidelines to Technical Publication, TR-TSV-000905.  Facilities of each Party shall provide the necessary on-hook, off-hook answer and disconnect supervision and shall hand off calling number ID (Calling Party Number) when technically feasible.

  - Quality of Interconnection.  The local interconnection for the transmission and routing of telephone exchange service and exchange access that each Party provides to each other will be at least equal in quality to what it provides to itself and any subsidiary or affiliate, where technically feasible, or to any other Party to which each Party provides local interconnection.

  - Network Management Controls.  Both Parties will work cooperatively with each other to apply sound network management principles by invoking appropriate network management controls (e.g., call gapping) to alleviate or prevent network congestion.

  - Common Channel Signaling. 	Both Parties will provide LEC-to-LEC Common Channel Signaling (“CCS”) to each other, where available, in conjunction with all traffic in order to enable full interoperability of CLASS features and functions except for call return.  All CCS signaling parameters will be provided, including automatic number identification (“ANI”), originating line information (“OLI”) calling company category, charge number, etc.  All privacy indicators will be honored, and each Party will cooperate with each other on the exchange of Transactional Capabilities Application Part (“TCAP”) messages to facilitate full interoperability of CCS-based features between the respective networks.  Neither Party shall alter the CCS parameters, or be a party to altering such parameters, or knowingly pass CCS parameters that have been altered in order to circumvent appropriate interconnection charges.

  - Signaling Call Information.  BellSouth and NorthPoint will send and receive 10 digits for local traffic.  Additionally, BellSouth and NorthPoint will exchange the proper call information, i.e. originated call company number and destination call company number, CIC, and OZZ, including all proper translations for routing between networks and any information necessary for billing.

  - Forecasting Requirements.  The Parties shall exchange technical descriptions and forecasts of their interconnection and traffic requirements in sufficient detail necessary to establish the interconnections required to assure traffic completion to and from all customers in their respective designated service areas. In order for the Parties to provide as accurate reciprocal trunking forecasts as possible to each other,  each Party must timely inform  the other Party of any known or anticipated events that may affect  reciprocal trunking requirements.  If either Party  is unable to provide such information, The Parties shall provide reciprocal trunking forecasts based only on existing trunk group growth and  annual estimated percentage of  subscriber line growth.

    - Both Parties shall meet every six months or at otherwise mutually agreeable intervals for the purpose of exchanging non-binding forecasts of its traffic and volume requirements for the interconnection and network elements provided under this Agreement, in the form and in such detail as agreed by the Parties. The Parties agree that each forecast provided under this Section shall be deemed “Confidential Information” in the General Terms and Conditions – Part A of this Agreement.

    - The trunk forecast should include trunk requirements for all of the interconnecting trunk groups for the current year plus the next two future years.  The forecast meeting between the two companies may be a face-to-face meeting, video conference or audio conference.  It may be held regionally or geographically.  Ideally, these forecast meetings should be held at least semi-annually, or more often if the forecast is no longer usable.  Updates to a forecast or portions thereof should be made whenever the Party providing the forecast deems that the latest trunk requirements exceed the original quantities by 24 trunks or 10%, whichever is greater.  Either Party should notify the other Party if they have measurements indicating that a trunk group is exceeding its designed call carrying capacity and is impacting other trunk groups in the network.  Also, either Party should notify the other Party if they know of situations in which the traffic load is expected to increase significantly and thus affect the interconnecting trunk requirements as well as the trunk requirements within the other Party’s network.  The Parties agree that the forecast information provided under this Section shall be deemed “Confidential Information” as set forth in the General Terms and Conditions of this Agreement.

    - For a non-binding trunk forecast, agreement between the two Parties on the trunk quantities and the timeframe of those trunks does not imply any liability for failure to perform if the trunks are not available for use at the required time.

# Local Dialing Parity

4.1	BellSouth and NorthPoint shall provide local and toll dialing parity to each other with no unreasonable dialing delays.  Dialing parity shall be provided for all originating telecommunications services that require dialing to route a call.  BellSouth and NorthPoint shall permit similarly situated telephone exchange service end users to dial the same number of digits to make a local telephone call notwithstanding the identity of the end user’s or the called party’s telecommunications service provider.

# Interconnection Compensation

  - Compensation for Call Transportation and Termination for Local Traffic

    - Local Traffic is defined as any telephone call that originates in one exchange and terminates in either the same exchange, or other local calling area associated with the originating exchange as defined and specified in Section A3 of BellSouth’s General Subscriber Service Tariff.  As clarification of this definition and for reciprocal transport and termination compensation, Local Traffic does not include traffic that originates from or is directed to or through an enhanced service provider or information service provider.  As further clarification, Local Traffic does not include calls that do not transmit information of the user’s choosing.  In any event, neither Party will pay reciprocal compensation to the other if the “traffic” to which such reciprocal compensation would otherwise apply was generated, in whole or in part, for the purpose of creating an obligation on the part of the originating carrier to pay reciprocal compensation for such traffic.

    - The Parties shall provide for the mutual and reciprocal recovery of the costs for the elemental functions performed in transporting and terminating local traffic on each other’s network.  The Parties agree that charges for transport and termination of calls on its respective networks are as set forth in Exhibit A to this Attachment.

    - For the purposes of this Attachment, Common (Shared) Transport is defined as the transport of the originating  Party’s traffic by the terminating Party over the terminating Party’s common (shared) facilities between the terminating Party’s tandem switch and end office switch and/or between the terminating Party’s tandem switches.

    - For the purposes of this Attachment, Tandem Switching is defined as the function that establishes a communications path between two switching offices through a third switching office (the Tandem switch).

    - For the purposes of this Attachment, End Office Switching is defined as the function that establishes a communications path between the trunk side and line side of the End Office switch.

    - If NorthPoint utilizes a switch outside the LATA and BellSouth chooses to purchase dedicated or common (shared) transport from NorthPoint for transport and termination of BellSouth originated traffic, BellSouth will pay NorthPoint no more than the airline miles between the V & H coordinates of the Point of Interface within the LATA where NorthPoint receives the BellSouth-originated traffic and the V & H coordinates of the BellSouth Exchange Rate Center Area that the NorthPoint terminating NPA/NXX is associated in the same LATA.  For these situations, BellSouth will compensate NorthPoint at either dedicated or common (shared) transport rates specified in Exhibit A and based upon the functions provided by NorthPoint as defined in this Attachment.

    - Neither Party shall represent access services traffic (e.g. Internet Protocol (IP) Telephony, FGA, FGB, etc.) as Local Traffic for purposes of payment of reciprocal compensation.

    - The Parties agree that the jurisdiction of a call is determined by its originating and terminating (end-to-end) points.  For the purpose of delivery of BellSouth originating traffic to NorthPoint, BellSouth will pay to NorthPoint reciprocal compensation for Local Traffic terminating to NorthPoint end users physically located in the BellSouth rate center to which the NorthPoint end user’s NPA/NXX is assigned.  If NorthPoint assigns NPA/NXXs to specific BellSouth rate centers and assigns numbers from those NPA/NXXs to NorthPoint end users physically located outside of the rate center to which the NPA/NXX is assigned, BellSouth traffic originating from within the BellSouth rate center where the NPA/NXX is assigned and terminating to a NorthPoint customer physically located outside of such rate center, and at a location toll to the BellSouth originating rate center, shall not be deemed Local Traffic, and no compensation from BellSouth to NorthPoint shall be due therefor.  Further, NorthPoint agrees to identify such traffic to BellSouth and to compensate BellSouth for originating and transporting such traffic to NorthPoint at BellSouth’s tariffed intrastate switched access rates.  In addition, NorthPoint should not use NPA/NXXs to collect BellSouth originated local or intraLATA toll traffic and for delivery to a point outside the LATA from where the originating NPA/NXX rate center resides.

    - If NorthPoint does not identify such traffic to BellSouth, to the best of BellSouth’s ability BellSouth will determine which whole NorthPoint NPA/NXXs on which to charge the applicable rates for originating intrastate network access service as reflected in BellSouth’s Intrastate Access Service Tariff.  BellSouth shall make appropriate billing adjustments if NorthPoint can provide sufficient information for BellSouth to determine whether said traffic is local or toll.

  - Percent Local Use. Each Party will report to the other a Percentage Local Usage (“PLU”). The application of the PLU will determine the amount of local minutes to be billed to the other Party.  For purposes of developing the PLU, each Party shall consider every local call and every long distance call, excluding intermediary traffic.  By the first of January, April, July and October of each year, each Party shall provide a positive report updating the PLU.  Requirements associated with PLU calculation and reporting shall be as set forth in BellSouth’s Percent Local Use Reporting Guidebook, as it is amended from time to time.  Notwithstanding the foregoing, where the terminating Party has message recording technology that identifies the jurisdiction of traffic terminated as defined in this Agreement, such information, in lieu of the PLU factor, shall at the terminating Party’s option be utilized to determine the appropriate local usage compensation to be paid.

  - Percentage Interstate Usage.	In the case where NorthPoint desires to terminate its local traffic over or co-mingled on its switched access Feature Group D trunks, NorthPoint will be required to provide a projected Percentage Interstate Usage (“PIU”) to BellSouth.  All jurisdictional report requirements, rules and regulations for Interexchange Carriers specified in BellSouth’s Intrastate Access Services Tariff will apply to NorthPoint.  After interstate and intrastate traffic percentages have been determined by use of PIU procedures, the PLU factor will be used for application and billing of local interconnection. Notwithstanding the foregoing, where the terminating Party has message recording technology that identifies the jurisdiction of traffic terminated as defined in this Agreement, such information, in lieu of the PIU and PLU factor, shall at the terminating Party’s option be utilized to determine the appropriate local usage compensation to be paid.

  - Audits.  On thirty (30) days written notice, each Party must provide the other the ability and opportunity to conduct an annual audit to ensure the proper billing of traffic.  BellSouth and NorthPoint shall retain records of call detail for a minimum of nine months from which a PLU and/or PIU can be ascertained.  The audit shall be accomplished during normal business hours at an office designated by the Party being audited.  Audit requests shall not be submitted more frequently than one (1) time per calendar year.  Audits shall be performed by a mutually acceptable independent auditory paid for by the Party requesting the audit.  The PLU and/or PIU shall be adjusted based upon the audit results and shall apply to the usage for the quarter the audit was completed, to the usage for the quarter prior to the completion of the audit, and to the usage for the two quarters following the completion of the audit.  If, as a result of an audit, either Party is found to have overstated the PLU and/or PIU by twenty percentage points (20%) or more, that Party shall reimburse the auditing Party for the cost of the audit.

  - Rate True-up

This section applies only to North Carolina and Tennessee and other rates that are interim or expressly subject to true-up under this attachment.

    - The interim prices for Unbundled Network Elements and Other Services and  Local Interconnection shall be subject to true-up according to the following procedures:

    - The interim prices shall be trued-up, either up or down, based on final prices determined either by further agreement between the Parties, or by a final order (including any appeals) of the Commission which final order meets the criteria of (3) below.  The Parties shall implement the true-up by comparing the actual volumes and demand for each item, together with interim prices for each item, with the final prices determined for each item.  Each Party shall keep its own records upon which the true-up can be based, and any final payment from one Party to the other shall be in an amount agreed upon by the Parties based on such records.  In the event of any disagreement as between the records or the Parties regarding the amount of such true-up, the Parties agree that the body having jurisdiction over the matter shall be called upon to resolve such differences, or the Parties may mutually agree to submit the matter to the Dispute Resolution process in accordance with the provisions of Section 12 of the General Terms and Conditions and Attachment 1 of the Agreement.

    - The Parties may continue to negotiate toward final prices, but in the event that no such agreement is reached within nine (9) months, either Party may petition the Commission to resolve such disputes and to determine final prices for each item.  Alternatively, upon mutual agreement, the Parties may submit the matter to the Dispute Resolution Process set forth in the General Terms and Conditions of the Agreement, so long as they file the resulting Agreement with the Commission as a “negotiated agreement” under Section 252(e) of the Act.

    - A final order of this Commission that forms the basis of a true-up shall be the final order as to prices based on appropriate cost studies, or potentially may be a final order in any other Commission proceeding which meets the following criteria:

(a)	BellSouth and CLEC is entitled to be a full Party to the proceeding;

(b)	It shall apply the provisions of the federal Telecommunications Act of 1996, including but not limited to Section 252(d)(1) (which contains pricing standards) and all then-effective implementing rules and regulations; and,

- It shall include as an issue the geographic deaveraging of unbundled element prices, which deaveraged prices, if any are required by said final order, shall form the basis of any true-up.

  - Compensation for IntraLATA Toll Traffic

    - IntraLATA Toll Traffic. IntraLATA Toll Traffic is defined as any telephone call that is not local or switched access per this Agreement.

    - Compensation for  intraLATA toll traffic.  For terminating its intraLATA toll traffic on the other company’s network, the originating Party will pay the terminating Party BellSouth’s current intrastate or interstate, whichever is appropriate, terminating switched access tariff rates as set forth in BellSouth’s Intrastate or Interstate Access Services Tariff.  The appropriate charges will be determined by the routing of the call. If NorthPoint is the BellSouth end user’s presubscribed interexchange carrier or if the BellSouth end user uses NorthPoint as an interexchange carrier on a 101XXXX basis, BellSouth will charge NorthPoint the appropriate BellSouth tariff charges for originating switched access services.

    - Compensation for 8XX Traffic.  Each Party shall compensate the other pursuant to the appropriate switched access charges, including the database query charge as set forth in the BellSouth intrastate or interstate switched access tariffs.

    - Records for 8XX Billing.  Each Party will provide to the other the appropriate records necessary for billing intraLATA 8XX customers.  The records provided will be in a standard EMI format for a fee of $0.013 per record.

    - 8XX Access Screening.  BellSouth’s provision of 8XX TFD to NorthPoint requires interconnection from NorthPoint to BellSouth 8XX SCP.  Such interconnections shall be established pursuant to BellSouth’s Common Channel Signaling Interconnection Guidelines and Bellcore’s CCS Network Interface Specification document, TR-TSV-000905.  NorthPoint shall establish CCS7 interconnection at the BellSouth Local Signal Transfer Points serving the BellSouth 8XX SCPs that NorthPoint desires to query.  The terms and conditions for 8XX TFD are set out in BellSouth’s Intrastate Access Services Tariff as amended.

  - Mutual Provision of Switched Access Service

    - Switched Access Traffic. Switched Access Traffic is as defined in the BellSouth Access Tariff. Additionally, IP Telephony traffic will be considered switched access traffic.

    - When NorthPoint’s end office switch, subtending the BellSouth Access Tandem switch for receipt or delivery of switched access traffic, provides an access service connection to or from an interexchange carrier (“IXC”) by either a direct trunk group to the IXC utilizing BellSouth facilities, or via BellSouth’s tandem switch, each Party will provide its own access services to the IXC on a multi-bill, multi-tariff meet-point basis. Each Party will bill its own access services rates to the IXC with the exception of the interconnection charge.  The interconnection charge will be billed by the Party providing the end office function.  Each party will use the Multiple Exchange Carrier Access Billing (MECAB) system to establish meet point billing for all applicable traffic.  Thirty (30)-day billing periods will be employed for these arrangements.  The recording Party agrees to provide to the initial billing Party, at no charge, the switched access detailed usage data within no more than sixty (60) days after the recording date.   The initial billing Party will provide the switched access summary usage data to all subsequent billing Parties within 10 days of rendering the initial bill to the IXC.   Each Party will notify the other when it is not feasible to meet these requirements so that the customers may be notified for any necessary revenue accrual associated with the significantly delayed recording or billing.  As business requirements change data reporting requirements may be modified as necessary.

    - Where either Party has been notified that the other Party has a Billing Guarantee Practice, each Party so notified (the Initial Billing Party or the recording Party) will be held liable for any access revenues which it has caused to be determined unbillable under the guidelines of such Billing Guarantee Practice of the other Party.  Each Party will provide complete documentation to the other to substantiate any claim of unbillable access revenues.  A negotiated settlement will be agreed upon between the Parties.

    - Each Party will retain for a minimum period of sixty (60) days, access message detail sufficient to recreate any data which is lost or damaged by their company or any third party involved in processing or transporting data.

    - Each Party agrees to recreate the lost or damaged data within forty-eight (48) hours of notification by the other or by an authorized third party handling the data.

    - Each Party also agrees to process the recreated data within forty-eight (48) hours of receipt at its data processing center.

    - All claims should be filed with the other Party within 120 days of the receipt of the date of the unbillable usage.

    - The Initial Billing Party shall keep records of its billing activities relating to jointly-provided Intrastate and Interstate access services in sufficient detail to permit the Subsequent Billing Party to, by formal or informal review or audit, to verify the accuracy and reasonableness of the jointly-provided access billing data provided by the Initial billing Party. Each Party agrees to cooperate in such formal or informal reviews or audits and further agrees to jointly review the findings of such reviews or audits in order to resolve any differences concerning the findings thereof.

    - NorthPoint agrees not to deliver switched access traffic to BellSouth for termination except over NorthPoint ordered switched access trunks and facilities.

  - Transit Traffic Service. BellSouth shall provide tandem switching and transport services for NorthPoint’s transit traffic. Transit traffic is traffic originating on NorthPoint’s network that is switched and/or transported by BellSouth and delivered to a third party’s network, or traffic originating on a third Party’s network that is switched and/or transported by BellSouth and delivered to NorthPoint’s network .  Rates for local transit traffic shall be the applicable call transport and termination charges as set forth in Exhibit A to this Attachment.  Rates for intraLATA toll and Switched Access transit traffic shall be the applicable call transport and termination charges as set forth in BellSouth Interstate or Intrastate Switched Access tariffs.  Switched Access transit traffic presumes that NorthPoint’s end office is subtending the BellSouth Access Tandem for switched access traffic to and from NorthPoint’s end users utilizing BellSouth facilities, either by direct trunks with the IXC, or via the BellSouth Access Tandem.  Billing associated with all transit traffic shall be pursuant to Multiple Exchange Carrier Access Billing (MECAB) procedures. Wireless Type 1 traffic shall not be treated as transit traffic from a routing or billing perspective.  Wireless Type 2A traffic shall not be treated as transit traffic from a routing or billing perspective until BellSouth and the Wireless carrier have the capability to properly meet-point-bill in accordance with MECAB guidelines.

    - The delivery of traffic which transits the BellSouth network and is transported to another carrier’s network is excluded from any BellSouth billing guarantees and will be delivered at the rates stipulated in this Agreement to a terminating carrier.  BellSouth agrees to deliver this traffic to the terminating carrier; provided, however, that NorthPoint is solely responsible for negotiating and executing any appropriate contractual agreements with the terminating carrier for the receipt of this traffic through the BellSouth network.  BellSouth will not be liable for any compensation to the terminating carrier or to NorthPoint.  NorthPoint agrees to compensate BellSouth for any charges or costs for the delivery of transit traffic to a connecting carrier on behalf of NorthPoint.  Additionally, the Parties agree that any billing to a third party or other telecommunications carrier under this section shall be pursuant to MECAB procedures.

  - Interconnection with Enhanced Service Providers (ESPs)/Information Service Providers (ISPs). ESP/ISP traffic shall not be included in the interconnection compensation arrangements of this Agreement.

# Frame Relay Service

  - In addition to the Local Interconnection services set forth above, BellSouth will offer a network to network Interconnection arrangement between BellSouth’s and NorthPoint’s frame relay switches as set forth below.  The following provisions will apply only to Frame Relay Service and Exchange Access Frame Relay Service in those states in which NorthPoint is certified and providing Frame Relay Service as a Local Exchange Carrier and where traffic is being exchanged between NorthPoint and BellSouth Frame Relay Switches in the same LATA.

  - The Parties agree to establish two-way Frame Relay facilities between their respective Frame Relay Switches to the mutually agreed upon Frame Relay Service point(s) of interconnection (“POI(s)”) within the LATA.  All POIs shall be within the same Frame Relay Network Serving Areas as defined in Section A40 of BellSouth’s General Subscriber Service Tariff except as set forth in this Attachment.

  - Upon the request of either Party, such interconnection will be established where BellSouth and NorthPoint have Frame Relay Switches in the same LATA.  Where there are multiple Frame Relay switches in one central office, an interconnection with any one of the switches will be considered an interconnection with all of the switches at that central office for purposes of routing packet traffic.

  - The Parties agree to provision local and IntraLATA Frame Relay Service and Exchange Access Frame Relay Service (both intrastate and interstate) over Frame Relay interconnection facilities between the respective Frame Relay switches and the POIs.

  - The Parties agree to assess each other reciprocal charges for the facilities that each provides to the other according to the Percent Local Circuit Use Factor (PLCU), determined as follows:

    - If the data packets originate and terminate in locations in the same LATA, and consistent with the local definitions of the Agreement, the traffic is considered local.  Frame Relay framed packet data is transported within Virtual Circuits (VC).  For the purposes of this Agreement, if all the data packets transported within a VC remain within the LATA, then consistent with the local definitions in this Agreement, the traffic on that VC is local (“Local VC”).

    - If the originating and terminating locations of the two way packet data traffic are not in the same LATA, the traffic on that VC is interLATA (“InterLATA VC”).

    - The PLCU is determined by dividing the total number of Local VCs, by the total number of VCs on each Frame Relay facility.  To facilitate implementation, NorthPoint may determine its PLCU in aggregate, by dividing the total number of Local VCs in a given LATA by the total number VCs in that LATA.  The Parties agree to renegotiate the method for determining PLCU, at BellSouth’s request, and within 90 days, if BellSouth notifies NorthPoint that it has found that this method does not adequately represent the PLCU.

    - If there are no VCs on a facility when it is billed, the PLCU will be zero.

    - BellSouth will provide the circuit between the Parties’ respective Frame Relay Switches.  The Parties will be compensated as follows:  BellSouth will invoice, and NorthPoint will pay, the total non-recurring and recurring charges for the circuit based upon the rates set forth in BellSouth’s Interstate Access Tariff, FCC No. 1.  NorthPoint will then invoice, and BellSouth will pay, an amount calculated by multiplying the BellSouth billed charges for the circuit by one-half of NorthPoint’s PLCU.

  - The Parties agree to compensate each other for Frame Relay network-to-network interface (NNI) ports based upon the NNI rates set forth in BellSouth's Interstate Access Tariff, FCC No. 1 .  Compensation for each pair of NNI ports will be calculated as follows:  BellSouth will invoice, and NorthPoint will pay, the total non-recurring and recurring charges for the NNI port.  NorthPoint will then invoice, and BellSouth will pay, an amount calculated by multiplying the BellSouth billed non-recurring and recurring charges for the NNI port by NorthPoint’s PLCU.

  - Each Party agrees that there will be no charges to the other Party for its own subscriber’s Permanent Virtual Circuit (PVC) rate elements for the local PVC segment from its Frame Relay switch to its own subscriber’s premises. PVC rate elements include the Data Link Connection Identifier (DLCI) and Committed Information Rate (CIR).

  - For the PVC segment between the NorthPoint and BellSouth Frame Relay switches, compensation for the PVC charges is based upon the rates in BellSouth's Interstate Access Tariff, FCC No. 1.

  - Compensation for PVC rate elements will be calculated as follows:

    - If NorthPoint orders a VC connection between a BellSouth subscriber’s PVC segment and a PVC segment from the BellSouth Frame Relay switch to the NorthPoint Frame Relay switch, BellSouth will invoice, and NorthPoint will pay, the total non-recurring and recurring PVC charges for the PVC segment between the BellSouth and NorthPoint Frame Relay switches.  If the VC is a Local VC, NorthPoint will then invoice and BellSouth will pay, the total nonrecurring and recurring PVC charges billed for that segment.  If the VC is not local, no compensation will be paid to NorthPoint for the PVC segment.

    - If BellSouth orders a Local VC connection between a NorthPoint subscriber’s PVC segment and a PVC segment from the NorthPoint Frame Relay switch to the BellSouth Frame Relay switch, BellSouth will invoice, and NorthPoint will pay, the total non-recurring and recurring PVC and CIR charges for the PVC segment between the BellSouth and NorthPoint Frame Relay switches.  If the VC is a Local VC, NorthPoint will then invoice and BellSouth will pay the total non-recurring and recurring PVC and CIR charges billed for that segment.  If the VC is not local, no compensation will be paid to NorthPoint for the PVC segment.

    - The Parties agree to compensate each other for requests to change a PVC segment or PVC service order record, according to the Feature Change charge as set forth in the BellSouth access tariff BellSouth Tariff FCC No, 1.

    - If NorthPoint requests a change, BellSouth will invoice and NorthPoint will pay a Feature Change charge for each affected PVC segment.

      - If BellSouth requests a change to a Local VC, NorthPoint will invoice and BellSouth will pay a Feature Change charge for each affected PVC segment.

    - The Parties agree to limit the sum of the CIR for the VCs on a DS1 NNI port to not more than three times the port speed, or not more than six times the port speed on a DS3 NNI port.

    - Except as expressly provided herein, this Agreement does not address or alter in any way either Party’s provision of Exchange Access Frame Relay Service or interLATA Frame Relay Service.  All charges by each Party to the other for carriage of Exchange Access Frame Relay Service or interLATA Frame Relay Service are included in the BellSouth access tariff BellSouth Tariff FCC No, 1.

  - NorthPoint will identify and report quarterly to BellSouth the PLCU of the Frame Relay facilities it uses, per section 8.5.3 above.

  - Either Party may request a review or audit of the various service components, consistent with the provisions of section E2 of the BellSouth State Access Services tariffs or Section 2 of the BellSouth FCC No.1 Tariff.

  - If during the term of this Agreement, BellSouth obtains authority to provide interLATA Frame Relay in any State, the Parties agree to renegotiate this arrangement for the exchange of Frame Relay Service Traffic within one hundred eighty (180) days of the date BellSouth receives interLATA authority.  In the event the Parties fail to renegotiate this Section 8 within the one hundred eighty day period, they will submit this matter to the appropriate State commission(s) for resolution.

# 7.		Remote Access Server (RAS) Network Interconnection

- 7.1	The Parties agree that the Remote Access Server (RAS) is a specialized internet traffic concentration device designed to concentrate traffic to specific Internet Service Providers (ISPs), and as such is telecommunications equipment, but is not an end office switch or equivalent facility, and thus is not subject to call transport and termination requirements under FCC Rule 51.711.

- 7.2	The Parties further agree that the purpose of the NorthPoint RAS service offering is to move Internet traffic off the Public Switched Telephone Network (PSTN) terminating end office switch.

- 7.3	NorthPoint shall configure NorthPoint’s RAS device in such a manner as to eliminate the provisioning of enhanced services as defined by the Federal Communications Commission, such as but not limited to, user authentication, security, usage measurement, billing control and protocol conversion.

- 7.4	BellSouth agrees to deliver BellSouth originated and transit traffic intended for NorthPoint’s end users to NorthPoint’s RAS device pursuant to the terms and conditions of this Agreement.

- 7.5		NPA/NXX Code Assignment and Homing

- 7.5.1	NorthPoint shall assign unique NPA/NXXs to NorthPoint’s RAS specifically for Internet traffic routing purposes.

- 7.5.2	NorthPoint shall home its NPA/NXX(s) on the BellSouth Tandem serving the Exchange Rate Center to which NorthPointassigns NorthPoint’s RAS NPA/NXX(s).  The specified association between BellSouth Tandems and Exchange Rate Center Areas is defined in the LERG.

- 7.6		Direct Trunks Between NorthPoint’s RAS And BellSouth End Offices

- 7.6.1	The Parties shall utilize direct end office to RAS trunks pursuant to the following conditions:

- 7.6.1.1	Tandem Exhaust - If a BellSouth tandem through which NorthPoint is interconnected is unable to, or is forecasted to be unable to support additional traffic loads for any period of time, the Parties will mutually agree on an end office trunking plan that will alleviate the BellSouth tandem capacity shortage and ensure completion of traffic between NorthPoint’s and BellSouth’s subscribers.

- 7.6.1.2	NorthPoint agrees to order, install and retain direct end office to RAS trunks sufficient to handle actual and reasonably forecasted traffic exceeding a  single DS1 of traffic per month.

- 7.6.1.3	NorthPoint also agrees to order direct end office to RAS trunks within 30 days of a request by BellSouth if the end office traffic is exceeding or is forecasted to exceed a DS1 of traffic.  If NorthPoint does not order direct trunks within those 30 days, NorthPoint agrees to pay to BellSouth, beginning the following month, the common transport, tandem switching and tandem shared trunk port per minute of use rates reflected in Exhibit A of this Agreement for traffic delivered to NorthPoint’s RAS via BellSouth’s tandem switch until such direct trunks are activated.

- 7.6.1.4	NorthPoint shall install additional capacity between BellSouth end offices and NorthPoint’s RAS devices when overflow traffic between end offices and a RAS device exceeds or is reasonably forecasted to exceed a single DS1 of traffic per month.

- 7.6.1.5		The Parties may install direct end office to RAS trunks upon mutual agreement.

- 7.7		Trunks Between a RAS and Tandem Switches

- 7.7.1	Pursuant to the preceding paragraphs regarding direct trunks between a RAS and the originating end offices, the NorthPoint agrees to order, install and retain trunking to the BellSouth tandem switch sufficient to handle actual and forecasted traffic volumes routed to NorthPoint’s RAS via the BellSouth tandem.

- 7.7.2	NorthPoint shall order and establish the necessary trunk groups to each BellSouth tandem switch on which NorthPoint has homed NorthPoint NPA/NXXs for transit traffic and traffic between NorthPoint and BellSouth and as defined in the LERG.

7.7.3	A RAS Point of Interface is the physical telecommunications interface between BellSouth and NorthPoint’s interconnection functions.  It establishes the technical interface and point of operational responsibility.  The primary function of the RAS Point of Interface is to serve as the terminus for the interconnection service.

7.7.3.1	At a minimum, NorthPoint must establish a RAS Point Of Interface at each BellSouth access tandem serving an Exchange Rate Center to which NorthPoint has assigned a RAS NPA/NXX in the LATA.

7.7.3.2	NorthPoint agrees to establish, within four months of a  request of BellSouth, a Point Of Interface at a BellSouth end office or  tandem switch location where the traffic to NorthPoint’s RAS has reached or is forecasted to reach one DS3 or more within six months.

7.7.3.3		The RAS Point of Interface has the following main characteristics:

7.7.3.3.1	It is a cross-connect point to allow connection, disconnection, transfer or restoration of service.

7.7.3.3.2	It is a point where BellSouth and NorthPoint can verify and maintain specific performance objectives.

7.7.3.3.3	It is specified according to the interface offered in the tariff or interconnection Agreement (for example: for DS1 service, the FCC # 1 tariff specifies that the interface meets the technical specifications detailed in Generic Requirements GR-342-CORE, Issue 1, December 1995.)

- 7.7.3.3.4	The Parties provide their own equipment (CPE) to interface with the DS0, DS1, DS3, STS1 and/or OCn circuits on the customer premises.

- 7.7.4	The RAS Point of Interconnection is the trunk group termination location at which BellSouth delivers BellSouth originated traffic to the NorthPoint’s RAS on NorthPoint’s network. NorthPoint’s requested Point of Interconnection shall also be used for the receipt of transit trunk groups for transit traffic at BellSouth Access and/or Local Tandems pursuant to the terms and conditions of this Agreement.  Points of Interconnection established at the BellSouth Local Tandem apply only to local traffic and local originating transit traffic as defined by BellSouth.

- 7.7.5	Pursuant to the terms and conditions in this Agreement, BellSouth agrees to deliver BellSouth originating traffic to NorthPoint’s RAS Points of Interface as established by NorthPoint.  A Point of Interface may not necessarily be established at a Point of Interconnection.

- 7.7.6	NorthPoint agrees to compensate BellSouth for transport and switching functions performed by BellSouth at the rates reflected in Exhibit A to this Agreement, including third party transit traffic, delivered to NorthPoint’s RAS Point Of Interface.

- 7.7.7	Exhibit A Switching and Transport rates will apply when the BellSouth Rate Center with which NorthPoint has associated its RAS NPA/NXX is not local, pursuant to BellSouth’s flat rated Extended Area Service (EAS) A3 tariffs, to the BellSouth Rate Center in which NorthPoint has placed NorthPoint’s RAS device.

- 7.7.8	NorthPoint shall not deliver switched access traffic to BellSouth via NorthPoint’s RAS interconnection with BellSouth.

- 7.7.9	BellSouth shall not pay reciprocal compensation to NorthPoint for traffic delivered to NorthPoint’s RAS.

7.7.10	Compensation for Switched Access transit traffic shall be pursuant to the Mutual Provision of Switched Access Service section of this Agreement.  Internet Protocol (IP) Telephony traffic shall be considered and treated as switched access traffic by both parties.

# Operational Support Systems (OSS) Rates

8.1	BellSouth has developed and made available the following mechanized systems by which NorthPoint may submit LSRs electronically.

LENS		Local Exchange Navigation System

EDI		Electronic Data Interface

TAG		Telecommunications Access Gateway

8.2	LSRs submitted by means of one of these interactive interfaces will incur an OSS electronic ordering charge as specified in the table below.  An individual LSR will be identified for billing purposes by its Purchase Order Number (PON).  LSRs submitted by means other than one of these interactive interfaces (mail, fax, courier, etc.) will incur a manual order charge as specified in the table below:

| ## OPERATIONAL SUPPORT SYSTEMS | AL, GA, LA, MS, SC | FL, KY, NC, TN |
| --- | --- | --- |
| OSS LSR charge, per LSR received from the CLEC by one of the OSS interactive interfaces | $3.50 SOMEC | $3.50 SOMEC |
| Incremental charge per LSR received from the CLEC by means other than one of the OSS interactive interfaces | See applicable rate element | $19.99 SOMAN |

Note:  In addition to the OSS charges, applicable discounted service order and related          discounted charges apply per the tariff.

8.3	Denial/Restoral OSS Charge

In the event NorthPoint provides a list of customers to be denied and restored, rather than an LSR, each location on the list will require a separate PON and, therefore will be billed as one LSR per location.

8.4		Cancellation OSS Charge

NorthPoint will incur an OSS charge for an accepted LSR that is later canceled by NorthPoint.

Note:  Supplements or clarifications to a previously billed LSR will not incur another OSS charge.

8.5		Network Elements and Other Services Manual Additive

The Commissions in Alabama, Georgia, Louisiana, Mississippi and South Carolina have ordered incremental manual non-recurring charges (NRC) for Network Elements and Other Services ordered by means other than one of the interactive interfaces.  These ordered Network Elements and Other Services manual additive NRCs will apply in these states, rather than the charge per LSR.

8.6		Threshold Billing Plan

8.6.1	The Parties agree that NorthPoint will incur the mechanized rate for all LSRs, both mechanized and manual, if the percentage of mechanized LSRs to total LSRs meets or exceeds the threshold percentages shown below:

Year		Ratio: Mechanized/Total LSRs

2000			80%

2001	90%

The threshold plan will be discontinued in 2002.

8.6.2	BellSouth will track the total LSR volume for each CLEC for each quarter.  At the end of that time period, a Percent Electronic LSR calculation will be made for that quarter based on the LSR data tracked in the LCSC.  If this percentage exceeds the threshold volume, all of that CLECs’ future manual LSRs will be billed at the mechanized LSR rate.  To allow time for obtaining and analyzing the data and updating the billing system, this billing change will take place on the first day of the second month following the end of the quarter (e.g. May 1 for 1Q, Aug 1 for 2Q, etc.).  There will be no adjustments to the amount billed for previously billed LSRs.

Basic Architecture

##### One-Way Trunking Architecture

###### Two-Way Trunking Architecture

SuperGroup Architecture