Attachment 2

Network Elements and Other Services

TABLE OF CONTENTS

1.	Introduction	3

2.	Unbundled Loops	4

3.	Integrated Digital Loop Carriers	4

4.	Network Interface Device	4

5.	Unbundled Loop Concentration (ULC) System	4

6.	Sub-loop Elements	4

7.	Local Switching	22

8.	Transport	29

10.	Operator Systems	37

11.	Signaling	42

12.	Signaling Transfer Points (STPs)	44

13.	Service Control Points/DataBases	48

13.8	Calling Name (CNAM) Database Service	53

14.	DARK FIBER	55

15.	SS7 Network Interconnection	56

16.	Basic 911 and E911	59

17.1.	General	66

17.2.	Operational Support Systems (OSS)	66

1.00  DEFINITIONS	74

2.0	ATTACHMENT	74

3.00  PHYSICAL CONNECTION AND COMPENSATION	75

4.00  CNAM RECORD INITIAL LOAD AND UPDATES	75

EXHIBIT A – LIDB STORAGE AGREEMENT……………………………………EXHIBIT A

EXHIBIT B – CNAM DATABASE SERVICES……………………………………EXHIBIT B

EXHIBIT C – RATES .………….…………………………………………………..EXHIBIT C

ACCESS TO NETWORK ELEMENTS AND OTHER SERVICES

# Introduction

1.1	Network Element is defined to mean a facility or equipment used in the provision of a telecommunications service.  Such term may include, but is not limited to, features, functions, and capabilities that are provided by means of such facility or equipment, including but not limited to, subscriber numbers, databases, signaling systems, and information sufficient for billing and collection or used in the transmission, routing, or other provision of a telecommunications service.  BellSouth offers access to the Network Elements, unbundled loops; network interface device; sub-loop elements; local switching; transport; tandem switching; operator systems; signaling; access to call-related databases; dark fiber as set forth in this Attachment.

### 1.2	BellSouth shall, upon request of PaeTecPaeTec, and to the extent technically feasible, provide to PaeTec access to its network elements for the provision of PaeTec’s telecommunications service.  If no rate is identified in the contract, the rate for the specific service or function will be as set forth in applicable BellSouth tariff or as negotiated by the Parties upon request by either Party.

### 1.3	PaeTec may purchase network elements and other services from BellSouth for the purpose of combining such network elements in any manner PaeTec chooses to provide telecommunication services to its intended users, including recreating existing BellSouth services.  With the exception of the sub-loop elements which are located outside of the central office, BellSouth shall deliver the network elements purchased by PaeTec  for combining to the designated PaeTec collocation space.  The network elements shall be provided as set forth in this Attachment.

### 1.4	BellSouth will provide the following combined  network elements for purchase by PaeTec.  The rate of the following combined  network elements is the sum of the individual element prices as set forth in this Attachment.  Order Coordination as defined in Section 2 of Attachment 2 of this Agreement is available for each of these combinations:

- SL2 loop and cross connect

- Port and cross connect

- Port and cross connect and common (shared) transport

- Port and vertical features

- SL2 Loop with loop concentration

- Port and common (shared) transport

- SL2 Loop and LNP

### 1.5	BellSouth shall comply with the requirements as set forth in the technical references within Attachment 2 to the extent that they are consistent with the greater of BellSouth’s actual performance or applicable industry standards.

1.6	In the event that any final and nonappealable legislative, regulatory, judicial or other legal action modifies or redefines the “Network Elements” in a manner which materially affects the terms of this Attachment or the Network Elements and/or prices set forth herein, either Party may, on thirty (30) days written notice, require renegotiation of such terms, and the Parties shall renegotiate in good faith such new terms in accordance with such legislative, regulatory, judicial or other legal action.  In the event such new terms are not renegotiated within ninety (90) days after the notice for renegotiation, either Party may petition the Commission for resolution of the dispute between the Parties.  Each Party reserves the right to seek judicial review of any Commission ruling concerning this Attachment.

1.7	PaeTec  will adopt and adhere to the reasonable and nondiscriminatory standards contained in the applicable CLEC Work Center Operational Understanding Agreement regarding maintenance and installation of service.

1.8	BellSouth will provide reasonable and nondiscriminatory access to Network Elements on an unbundled basis, pursuant to the terms, conditions and rates set forth in this attachment, and in accordance with all effective rules and decisions of the FCC and the Commission.

# 2.

# Unbundled Loops, Network Interfaces Device, Unbundled Loop Concentration (ULC) System, and Sub loops

All of the negotiated rates, terms and conditions set forth in this Section pertain to the provision of unbundled loops.

## Unbundled Loops

## Definition

### 2.1.2	The local loop network element (“Loop(s)”) is defined as a transmission facility between a distribution frame (or its equivalent) in BellSouth’s central office and the loop demarcation point at an end-user customer premises, including inside wire owned by BellSouth.  The local loop network element includes all features, functions, and capabilities of the transmission facilities, including dark fiber and attached electronics (except those used for the provision of advanced services, such as Digital Subscriber Line Access Multiplexers) and line conditioning.

### 2.1.3	The provisioning of service to a CLEC’s collocation space will require cross-office cabling and cross-connections within the central office to connect the loop to a local switch or to other transmission equipment.  These cross-connects are a separate component, that are not considered a part of the loop, and thus, have a separate charge.

- 2.1.4	BellSouth Order Coordination referenced in Attachment 2 includes two types:  “Order Coordination” (OC) and “Order Coordination - Time Specific” (OC-TS).

- 2.1.5	“Order Coordination” refers to standard BellSouth service order coordination involving the reuse of facilities where PaeTec is requesting that their loop order be provisioned over an existing circuit that is currently providing service to the end user.  Order coordination for physical conversions will be scheduled at BellSouth’s discretion during normal working hours on the committed due date and PaeTec will be advised. OC will be provided as a standard item on SL2 voice grade loops and all Unbundled Digital Loops (UDLs).  OC will be provided as a chargeable option on SL1 voice grade loops, and all Unbundled Copper Loops.

    - “Order Coordination – Time Specific” refers to service order coordination in which PaeTec requests a specific time for a service order conversion to take place.   BellSouth will make every effort to accommodate PaeTec’s specific conversion time request.  However, BellSouth reserves the right to negotiate with PaeTec a conversion time based on load and appointment control when necessary. Loops on a single service order of 14 or more loops will be provisioned on a project basis.  This is a chargeable option for any coordinated order and is billed in addition to the OC charge.  PaeTec may specify a time between 9:00 a.m. and 4:00 p.m. (location time) Monday through Friday (excluding holidays).  If PaeTec specifies a time outside this window, or selects a time or quantity of loops that requires BellSouth technicians to work outside normal work hours, overtime charges will apply in addition to the OC and OC-TS charges.  Overtime charges will be applied according to actual costs based on type of force group required to perform the work, overtime hours worked and any special circumstances.

- If PaeTec requests work to be done for any UNE loop that requires BellSouth technicians to work outside normal work hours, overtime charges will be applied according to actual costs based on type of force group required to perform the work, overtime hours worked and any special circumstances.

|  | - Order Coordination (OC) | - Order Coordination – Time Specific (OC-TS) | - Test Points | - DLR | - Charge for Dispatch and Testing if No Trouble Found |
| --- | --- | --- | --- | --- | --- |
| - SL-1 | - Chargeable option | - Chargeable Option* | - Not available | - Chargeable Option – ordered as Engineering Information Document | - Charged for Dispatch inside & outside Central Office |
| - SL-2 | - Included | - Chargeable Option* | - Included | - Included | - Charged for Dispatch outside Central Office |
| - Unbundled Digital Loop | - Included | - Chargeable Option* (except on Universal Digital Channel) | - Included (where appropriate) | - Included | - Charged for Dispatch outside Central Office |
| - Unbundled Copper Loop | - Chargeable Option | - Not available | - Included | - Included | - Charged for Dispatch outside Central Office |

*Order Coordination-Time Specific charge for orders due on same day at same location will be applied on a per LSR basis.  For UVL-SL1, PaeTec must order OC when requesting OC-TS.

- 2.1.7	Where facilities are available, BellSouth will install loops in compliance with BellSouth’s Interval Guide available at the website at http://www.interconnection.bellsouth.com.  For orders of 14 or more loops, the installation will be handled on a project basis and the intervals will be set by the BellSouth project manager for that order.  Some loops require a Service Inquiry (SI) to determine if facilities are available prior to issuing the order.  The interval for the SI process is separate from the installation interval.  For expedite requests by PaeTec, expedite charges will apply for intervals less than 5 days.  The charges outlined in BellSouth’s FCC No. 1 Tariff, Section 5, will apply.  If PaeTec cancels an order for network elements and other services, any costs incurred by BellSouth in conjunction with the provisioning of that order will be recovered in accordance with FCC No. 1 Tariff, Section 5.

- 2.1.8	If PaeTec modifies an order after being sent a Firm Order Confirmation (FOC) from BellSouth, any costs incurred by BellSouth to accommodate the modification will be paid by PaeTec.

#### 2.1.9	BellSouth will offer Unbundled Voice Loops (UVL) in two different service levels - Service Level One (SL1) and Service Level Two (SL2).

### SL1 loops are 2-wire loop start circuits, will be non-designed, and will not have remote access test points.  OC will be offered as a chargeable option on SLI loops when reuse of existing facilities has been requested by PaeTec.   PaeTec may also order OC-TS when a specified conversion time is requested. OC-TS is a chargeable option for any coordinated order and is billed in addition to the OC charge.  An Engineering Information (EI) document can be ordered as chargeable option. The EI document provides loop make up information which is similar to the information normally provided in a Design Layout Record. Upon issuance of a non-coordinated order in the service order system, SL1 loops will be activated on the due date in the same manner and time frames that BellSouth normally activates POTS-type loops for its customers.

### 2.1.11	SL2 loops may be 2-wire or 4-wire circuits, shall have remote access test points, and will be designed with a design layout record provided to PaeTec.  SL2 circuits can be provisioned with loop start, ground start or reverse battery signaling.  OC is provided as a standard feature on SL2 loops.  The OC feature will allow PaeTec to coordinate the installation of the loop with the disconnect of an existing customer’s service and/or number portability service.  In these cases, BellSouth will perform the order conversion with standard order coordination at its discretion during normal work hours.

### 2.1.12	BellSouth will also offer Unbundled Digital Loops (UDL) to PaeTec.

#### 2.1.12.1          UDLs are service specific and will be designed, will be provisioned with test points (where appropriate), and will come standard with OC and a DLR.   The various UDLs are intended to support a specific digital transmission scheme or service. The UDL loops are as follows:

### 2.1.12.2					UDC/IDSL. Due to technical limitations associated with certain Digital Loop Carrier (DLC) systems, some ISDN-capable loops that are provisioned using DLC systems may not support Integrated Digital Subscriber Line (IDSL) service.  BellSouth will not reconfigure its ISDN-capable loop to support IDSL service.

### 2.1.12.3           Instead, BellSouth agrees to offer the Universal Digital Channel (UDC), which may also be referred to as an IDSL-capable loop as part of its UDL offerings.  The UDC loop is intended to be compatible with IDSL service and has the same physical characteristics and transmission specifications as BellSouth’s ISDN-capable loop. These specifications are listed in BellSouth’s TR73600.

2.1.12. 4 	Like the ISDN-capable loop, the UDC may be provisioned on copper or through a DLC system.  However, when UDCs are provisioned using a DLC system, BellSouth will ensure that they are only provisioned on time slots that are compatible with data-only services such as IDSL.

2.1.12.5	HDSL-capable. This is a designed loop that is provisioned according to Carrier Serving Area (CSA) criteria and may be up to 12,000 feet long and may have up to 2,500 feet of bridged tap (inclusive of loop length).  It may be a 2-wire or 4-wire circuit and will come standard with a test point, Order Coordination, and a DLR.

2.1.12.6	ADSL-capable. This is a designed loop that is provisioned according to Revised Resistance Design (RRD) criteria and may be up to 18kft long and may have up to 6kft of bridged tap (inclusive of loop length).  It may be a 2-wire or 4-wire circuit and will come standard with a test point, Order Coordination, and a DLR.

2.1.12.7	DS1. This is a designed 4-wire loop that is provisioned according to industry standards for DS1 or Primary Rate ISDN services and will come standard with a test point, Order Coordination, and a DLR.

2.1.12.8	DS0. These are designed 4-wire loops that may configured as 64kbps, 56kbps, 19kbps, and other sub-rate speeds associated with digital data services and will come standard with a test point, Order Coordination, and a DLR.

### 2.1.13	As a chargeable option on all loops except the Universal Digital Channel (UDC) and all Unbundled Copper Loops (UCL), BellSouth will offer OC-TS.  This will allow PaeTec the ability to specify the time that the coordinated conversion takes place.  The OC-TS charge for orders due on the same day at the same location will be applied on a per Local Service Request (LSR) basis.

2.1.14	In addition to the UVLs and UDLs, BellSouth shall make available Unbundled Copper Loops (UCLs).  The UCL will be a copper twisted pair loop that is unencumbered by any intervening equipment (e.g., filters, load coils, range extenders, digital loop carrier, or repeaters). The UCL will be offered in two versions - Short and Long.  A short UCL (18 kft or less) will be provisioned according to Resistance Design parameters, may have up to 6kft of bridged tap and will have up to 1300 ohms of resistance.  The long UCL (beyond 18kft) will be any dry copper pair longer than 18kft and may have up to 12kft of bridged tap and up to 2800 ohms of resistance.  Unbundled Loop Modifications (ULM) may be used when a CLEC wants to condition copper loops by removing load coils and other intervening equipment.  In almost every case, the UCL long will require ULM to remove load coils.  BellSouth will only ensure electrical continuity and balance relative to tip and ring on UCLs.

#### 2.1.15	The UCL is a designed circuit, is provisioned with a test point and comes standard with a DLR.  OC will be offered as a chargeable option on all UCL loops.  OC is required on UCLs where a reuse of existing facilities has been requested by PaeTec.  Order Coordination – Time Specific (OC-TS) will not be offered on UCLs.

- 2.1.16	The UCL is a dry copper loop and is not intended to support any particular telecommunications service.  PaeTec may use the UCL loop for a variety of services, including xDSL (e.g., ADSL and HDSL) services, by attaching appropriate terminal equipment of PaeTec’s choosing.  PaeTec will determine the type of service that will be provided over the loop.

- 2.1.17	Because the UCL loop shall be an unbundled loop offering that is separate and distinct from BellSouth’s ADSL and HDSL capable loop offerings, PaeTec agrees that BellSouth’s UCL loop will not be held to the service level and performance expectations that apply to its ADSL and HDSL unbundled loop offerings.  BellSouth shall only be obligated to maintain copper continuity and provide balance relative to tip and ring on UCL loops.

    - All UNE loops offered by BellSouth shall be provided to PaeTec in accordance with BellSouth’s Technical Reference 73600.

### 2.1.19	PaeTec will be responsible for testing and isolating troubles on the loops.  Once PaeTec has isolated a trouble to the BellSouth provided loop, PaeTec will issue a trouble to BellSouth on the loop.  BellSouth will take the actions necessary to repair the loop if a trouble actually exists.  BellSouth will repair these loops in the same time frames that BellSouth repairs similarly situated loops to its customers.

### 2.1.20	If PaeTec reports a trouble on SL1 loops and no trouble actually exists, BellSouth will charge PaeTec for any dispatching and testing (both inside and outside the CO) required by BellSouth in order to confirm the loop’s working status.

    - CLEC must test and isolate trouble to the BellSouth portion of the SL2 loop before reporting repair to UNE Center.  At the time of the trouble report, PaeTec will be required to provide the results of the CLEC test which indicate a problem on the BellSouth provided loop.  	If PaeTec reports a trouble on SL2 loops and no trouble actually exists, BellSouth will charge PaeTec for any dispatching and testing, (outside the CO) required by BellSouth in order to confirm the loop’s working status.

## 2.1.22	Technical Requirements

### 2.1.22.1	To the extent available within BellSouth’s Network at a particular location, BellSouth will offer loops capable of supporting telecommunications services such as: POTS, Centrex, basic rate ISDN, analog PBX, voice grade private line, ADSL, HDSL, DS1 and digital data (up to 64 kb/s).  If a requested loop type is not available, then the CLEC can use the Special Construction process to request that BellSouth place facilities or otherwise modify facilities in order to meet PaeTec’s request.

2.1.22.2	PaeTec will be responsible for providing BellSouth with a Service Profile Identifier (SPID) associated with a particular ISDN-capable loop and end user.  With the SPID, BellSouth will be able to adequately test the circuit and ensure that it properly supports ISDN service.

### 2.1.22.3	The loop will support the transmission, signaling, performance and interface requirements of the services described in 2.1.2 above.  It is recognized that the requirements of different services are different, and that a number of types or grades of loops are required to support these services.  Services provided over the loop by PaeTec will be consistent with industry standards and BellSouth’s TR73600.

2.1.22.4	PaeTec may utilize the unbundled loops to provide any telecommunication service it wishes.  However, BellSouth will only provision, maintain and repair the loops to the standards that are consistent with the type of loop ordered.  For example, if PaeTec orders an ISDN-capable loop but wants to use the loop for a service other than ISDN, BellSouth will only support that the loop is capable of providing ISDN service. For non-service specific loops (e.g. UCL, loops modified by PaeTec using the Special Construction process), BellSouth will only support that the loop has copper continuity and balanced tip-and-ring.

#### 2.1.22.5	In some instances, PaeTec will require access to a copper twisted pair loop unfettered by any intervening equipment (e.g., filters, load coils, range extenders, etc.), so that PaeTec can use the loop for a variety of services by attaching appropriate terminal equipment at the ends.  PaeTec will determine the type of service that will be provided over the loop.  In some cases, PaeTec may be required to pay additional charges for the removal of certain types of equipment.  BellSouth’s Unbundled Loop Modifications (ULM) process will be used to determine the costs and feasibility of these activities.

#### 2.1.22.6	In those cases where PaeTec has requested that BellSouth modify a loop so that it no longer meets the technical parameters of the original loop type (e.g., voice grade, ISDN, ADSL, etc.) the resulting modified loop will be ordered and maintained as a UCL.

### 2.1.22.7	The loop shall be provided to PaeTec in accordance with BellSouth’s TR73600 Unbundled Local Loop Technical Specification and applicable industry standard technical references.

2.2	Unbundled Loop Modifications (Line Conditioning)

    - Subject to applicable and effective FCC rules and orders, BellSouth shall condition loops, as requested by PaeTec, whether or not BellSouth offers advanced services to the End User on that loop.

    - Loop conditioning is defined as the removal from the loop of any devices that may diminish the capability of the loop to deliver high-speed switched wireline telecommunications capability, including xDSL service.  Such devices include, but are not limited to, load coils, bridge taps, low pass filters, and range extenders.

    - The Unbundled Loop Modifications (ULM) offering provides the following elements: 1) removal of equipment on loops equal to or less than 18kft; 2) removal of equipment of loops longer than 18kft; and 3) removal of bridged-taps on loops of any length.

    - BellSouth shall recover the cost of line conditioning requested by PaeTec through a recurring charge and/or nonrecurring charge(s) in accordance with the FCC’s forward-looking pricing principles promulgated pursuant to Section 252 (d) (1) of the Act and in compliance with FCC Rule 52.507 (e).

## Integrated Digital Loop Carriers

### Where BellSouth uses Integrated Digital Loop Carrier (IDLC) systems to provide the local loop and BellSouth has a suitable alternate facility available, BellSouth will make arrangements to permit PaeTec to order a contiguous local loop.  To the extent it is technically feasible, these arrangements will provide PaeTec with the capability to serve end users at a level that is at parity with the level of service BellSouth provides its customers.  If no alternate facility is available, BellSouth will utilize its Special Construction (SC) process to determine the additional costs required to provision the loop facilities.  PaeTec will then have the option of paying the SC rates to place the loop facilities or PaeTec may chose some other method of providing service to the end-user (e.g., Resale, private facilities, etc.).

## Network Interface Device

## Definition

The NID is defined as any means of interconnection of end-user customer inside wire to BellSouth’s distribution plant, such as a cross-connect device used for that purpose.  The NID is a single-line termination device or that portion of a multiple-line termination device required to terminate a single line or circuit at the premises. The NID features two independent chambers or divisions that separate the service provider’s network from the End User’s on-premises wiring.  Each chamber or division contains the appropriate connection points or posts to which the service provider and the end user each make their connections.  The NID provides a protective ground connection and is capable of terminating cables such as twisted pair cable.

    - BellSouth shall permit PaeTec to connect PaeTec’s loop facilities the end-user’s inside wire through the BellSouth NID or at any other technically feasible point.

## Access to Network Interface Device (NID)

2.4.3.1							Due to the wide variety of NIDs utilized by BellSouth (based on subscriber size and environmental considerations), PaeTec may access the end user’s wire by any of the following means:  1)  	BellSouth shall allow PaeTec to connect its loops directly to BellSouth’s multi-line residential NID enclosures that have additional space and are not used by BellSouth or any other telecommunications carriers to provide service to the premise.  It is the responsibility of PaeTec to leave undisturbed the existing form of electrical protection and to maintain the physical integrity of the NID; 2) 	Where an adequate length of the end user’s inside wire is present and environmental conditions permit, either Party may remove the inside wire from the other Party’s NID and connect that wire to that Party’s own NID;  3) Enter the subscriber access chamber or “side” of “dual chamber” NID enclosures for the purpose of extending a connecterized or spliced jumper wire from the inside wiring through a suitable “punch-out” hole of such NID enclosures; or 4)	Request BellSouth to make other rearrangements to the inside wiring terminations or terminal enclosure on a time and materials cost basis to be charged to the requesting Party (i.e., PaeTec, its agent, the building owner or the subscriber).  Such charges will be billed to the requesting Party.

2.4.3.2	In no case shall either Party remove or disconnect the other Party’s loop facilities from either Party’s NIDs, enclosures, or protectors unless the applicable Commission has expressly permitted the same and the disconnecting Party provides prior notice to the other Party.  In such cases, it shall be the responsibility of the Party disconnecting loop facilities to leave undisturbed the existing form of electrical protection and to maintain the physical integrity of the NID.  It will be the CLEC’s responsibility to ensure there is no safety hazard and will hold BellSouth harmless for any liability associated with the removal of the BellSouth loop from the BellSouth NID.  Furthermore, it shall be the responsibility of the disconnecting Party, once the other Party’s loop has been disconnected from the NID, to reconnect the disconnected loop to a nationally- recognized–testing-laboratory-listed station protector, which has been grounded as per Article 800 of the National Electrical Code.  If no spare station protector exists in the NID, the disconnected loop must be appropriately cleared, capped and stored.  If CLEC does not wish to accept these responsibilities, other options exist in which BellSouth installs a NID for the CLEC as a chargeable option.  2.4.3.3	In no case shall either Party remove or disconnect ground wires from BellSouth’s NIDs, enclosures, or protectors.2.4.3.4		In no case shall either Party remove or disconnect NID modules, protectors, or terminals from BellSouth’s NID enclosures. 	Technical Requirements

      - The NID shall provide an accessible point of interconnection and shall maintain a connection to ground.

      - If an existing NID is accessed, it shall be capable of transferring electrical analog or digital signals between the subscriber’s inside wiring and the Distribution Media and/or cross connect to PaeTec’s NID

      - Where a BellSouth NID exists, it is provided in its “as is” condition.  PaeTec may request BellSouth do additional work to the NID on a time and material basis..  	When PaeTec deploys its own local loops with respect to multiple-line termination devices, PaeTec shall specify the quantity of NIDs connections that it requires within such device.

## Interface Requirements

### 2.4.4.1 	The NID shall be equal to or better than all of the requirements for NIDs set forth in the applicable industry standard technical references.

### Unbundled Loop Concentration (ULC) System

### BellSouth will provide to PaeTec Unbundled Loop Concentration (ULC).  Loop concentration systems in the central office concentrate the signals transmitted over local loops onto a digital loop carrier system.  The concentration device is placed inside a BellSouth central office.  BellSouth will offer ULC with a TR008 interface or a TR303 interface.

    - ULC will be offered in two sizes.  System A will allow up to 96 BellSouth loops to be concentrated onto multiple DS1s.  The high-speed connection from the concentrator will be at the electrical DS1 level and will connect to PaeTec at PaeTec’s collocation site.  System B will allow up to 192 BellSouth loops to be concentrated onto multiple DS1s.  System A may be upgraded to a System B.  A minimum of two DS1s is required for each system (i.e., System A requires two DS1s and System B would require an additional two DS1s or four in total).  All DS1 interfaces will terminate to the CLEC’s collocation space.  ULC service is offered with concentration (2 DS1s for 96 channels) or without concentration (4 DS1s for 96 channels) and with or without protection. A Line Interface element will be required for each loop that is terminated onto the ULC system.  Rates for ULC are as set forth in this Attachment.

## Sub-loop Elements

## Where facilities permit and subject to applicable and effective FCC rules and orders, BellSouth shall offer access to its Unbundled Sub Loop (USL) and Unbundled Sub-loop Concentration (USLC) System.  BellSouth shall provide non-discriminatory access, in accordance with FCC Rule 51.311 and Section 251(c) (3) of the Act, to the sub-loop on an unbundled basis and pursuant to the following terms and conditions and the rates approved by the Commission and set forth in this Attachment.

    - Sub-loop components include but are not limited to the following:

      - Unbundled Sub-Loop Distribution;

      - Unbundled Sub-Loop Concentration/Multiplexing Functionality; and

      - Unbundled Sub-Loop Feeder.

  - Unbundled Sub-Loop (distribution facilities)

    - Definition

      - Subject to applicable and effective FCC rules and orders, the unbundled sub-loop distribution facility is dedicated transmission facility that BellSouth provides from a customer’s point of demarcation to a BellSouth cross-connect device.  The BellSouth cross-connect device may be located within a remote terminal (RT) or a stand-alone cross-box in the field or in the equipment room of a building. The unbundled sub-loop distribution media is a copper twisted pair that can be provisioned as a 2 Wire or 4 Wire facility.  Following are the current sub-loop distribution offerings:

        - Voice grade Unbundled Sub-Loop Distribution (USL-D) is a sub-loop facility from the cross-box in the field up to and including the point of demarcation, at the end user’s premises.

#### Unbundled Sub-Loop distribution facilities were originally built as part of the entire voice grade loop from the BellSouth central office to the customer network interface.  Therefore, the voice grade Unbundled Sub-Loop may have load coils, which are necessary for transmission of voice grade services.

        - Unbundled Copper Sub-Loop (UCSL) is a non-loaded copper facility of any length provided from the cross-box in the field up to and including the end-user’s point of demarcation.

          - If available, this facility will not have any intervening equipment such as load coils between the end-user and the cross-box.

    - If PaeTec requests a UCSL and a non-loaded pair is not available, PaeTec may order Unbundled Sub-Loop Modification to remove load coils and/or bridge tap from an existing sub-loop facility.  If load coils are removed from an existing sub-loop, that sub-loop will be classified as a UCSL.  PaeTec may order Loop Make-up to determine what loop modifications will be required.

### Unbundled Sub-Loop distribution facilities shall support functions associated with provisioning, maintenance and testing of the Unbundled Sub-Loop. For access to Voice Grade USL-D and UCSL, PaeTec would be required to deliver a cable to the BellSouth remote terminal or cross-box in the field to provide continuity to PaeTec’s feeder facilities.  This cable would be connected, by a BellSouth technician, within the BellSouth RT/cross-box during the set-up process.  PaeTec’s cable pairs can then be connected to BellSouth’s USL within the BellSouth cross-box by the BellSouth technician.

### Unbundled Sub-Loop – Intrabuilding Network Cable (USL-INC) (a.k.a. riser cable) is the distribution facility inside a subscriber’s building or between buildings on one customer’s same premises (continuous property not separated by a public street or road).  USL-INC includes the facility from the cross-connect device in the building equipment room up to and including the point of demarcation, at the end user’s premises.

### 2.7.4.1	In a scenario that requires connection in a building equipment room, BellSouth will install a cross connect panel for the purpose of accessing USL-INC pairs.  The cross-connect panel will function as a single point of interconnection (SPOI) for USL-INC and will be accessible by multiple carriers as space permits.  BellSouth will place cross-connect blocks in 25-pair increments for PaeTec’s use on this cross-connect panel.  PaeTec will be responsible for connecting its facilities to the 25-pair cross-connect block(s).

    - BellSouth will provide Unbundled Sub-Loops where possible.  Through the firm order Service Inquiry (SI) process, BellSouth will determine if it is feasible to place the required facilities where PaeTec has requested access to Unbundled Sub-Loops.  If existing capacity is sufficient to meet the CLEC demand, then BellSouth will perform the set-up work as described in Section 2.7.6.  If any work must be done to modify existing BellSouth facilities or add new facilities (other than adding the cross-connect panel in a building equipment room as noted in Section 2.7) to accommodate PaeTec’s request for Unbundled Sub-Loops, PaeTec may request BellSouth’s Special Construction (SC) process to determine additional costs required to provision the Unbundled Sub-Loops.  PaeTec will have the option of paying the SC charges to modify the BellSouth facilities.

    - Set-up work must be completed before PaeTec can order sub-loop pairs.  During the set-up in a BellSouth cross-connect box in the field, the BellSouth technician will perform the necessary work to splice the CLEC’s cable into the cross-connect box.  For the set-up inside a building equipment room, BellSouth will perform the necessary work to install the cross-connect panel and the connecting block(s) that will be used to provide access to the requested USLs.

      - Once the set-up is complete, the CLEC will request sub-loop pairs through submission of a Local Service Request (LSR) form to the Local Carrier Service Center (LCSC).  Order Coordination is required with USL pair provisioning when PaeTec requests reuse of an existing facility and is in addition to the USL pair rate.  For expedite requests by PaeTec for sub-loop pairs, expedite charges will apply for intervals less than 5 days.

#### Unbundled Sub-Loop shall be equal to or better than each of the applicable requirements set forth in the applicable industry standard technical references.

#### Unbundled Sub-Loops will be provided in accordance with technical reference TR73600.

## Unbundled Network Terminating Wire (UNTW)

### BellSouth agrees to offer its Unbundled Network Terminating Wire (UNTW) to PaeTec pursuant to the following terms and conditions at rates as set forth in this Attachment.

## Definition

### Subject to applicable and effective FCC rules and orders, UNTW is a dedicated transmission facility that BellSouth provides from the Wiring Closet /Garden Terminal (or other type of cross-connect point) at the point of termination of BellSouth’s loop distribution facilities to the end user’s point of demarcation.   UNTW is the final portion of the loop owned by BellSouth.

## Requirements

### On a multi-unit premises where Provisioning Party owns the network terminating wire, and by request of Requesting Party, Provisioning Party will provide access to UNTW pairs on an Access Terminal that is suitable for use by multiple carriers at each Garden Terminal or Wiring Closet.

      - In new construction where possible, both Parties may at their option and with the property owner’s agreement install their own Network Terminating Wire (NTW).  In existing construction, the Provisioning Party shall not be required to install new or additional NTW beyond existing NTW to provision the services of the Requesting Party.

      - Upon notice from the Requesting Party to the Provisioning Party that the Requesting Party desires access to the Provisioning Party’s UNTW pairs in a multi-unit premises, representatives of both Parties will participate in a meeting at the site of the requested access.  The purpose of the site visit will include discussion of the procedures for Access Terminal installation, location and addresses of the Access Terminals and to discuss an estimated completion date.  Upon completion of site visit, the Requesting Party will submit a Service Inquiry (SI) to the person or organization designated by the Provisioning Party to receive the SI.  The SI will initiate the work for the Provisioning Party to begin the Access Terminal installation.  In multi-tenant unit (MTU) scenarios, Provisioning Party will provide access to UNTW pairs on an Access Terminal(s).  By request of the Requesting Party, an Access Terminal will be installed either adjacent to each Provisioning Party’s Garden Terminal or inside each Wiring Closet on the requested MTU.  All the UNTW pairs served by a Garden Terminal/Wiring Closet will be made available on the Access Terminals.  Requesting Party will deliver and connect its central office facilities to the UNTW pairs within the Access Terminal. Requesting Party may access any available pair on an Access Terminal unless the Provisioning Party or another service provider is using the pair to concurrently provide service.  Prior to connecting Requesting Party’s service on a pair previously used by Provisioning Party, Requesting Party is responsible for ensuring the end-user is no longer using Provisioning Party’s service or another CLEC’s service before accessing UNTW pairs.

      - Access Terminal installation intervals will be established on an individual case basis.

      - Requesting Party is responsible for obtaining the property owner’s permission for Provisioning Party to install an Access Terminal(s) on behalf of the Requesting Party.  The submission of the SI by the Requesting Party will serve as certification by the Requesting Party that such permission has been obtained.  If the property owner objects to Access Terminal installations that are in progress or subsequent to completion and demands removal of Access Terminals, PaeTec will be responsible for costs associated with removing Access Terminals and restoring property to original state prior to Access Terminals being installed.

      - Requesting Party will be billed for non-recurring and recurring charges for accessing UNTW pairs at the time the Requesting Party activates the pair(s).  PaeTec will report use of the UNTW pairs on a Local Service Request (LSR) form submitted to BellSouth’s Local Carrier Service Center (LCSC).

      - Requesting Party will isolate and report repair problems to the UNE center.  Requesting Party must tag the UNTW pair that requires repair.  If Provisioning Party dispatches a technician on a reported trouble call and no UNTW trouble is found, Provisioning Party will charge Requesting Party for time spent on the dispatch and testing the UNTW pair(s).

      - If Requesting Party initiates the Access Terminal installation and the Requesting Party has not activated at least one pair on the Access Terminal installed pursuant to Requesting Party’s request for an Access Terminal within 6 months of installation of the Access Terminal, Provisioning Party will bill Requesting Party a non-recurring charge equal to the actual cost of provisioning the Access Terminal.

      - If Provisioning Party determines that Requesting Party is using the UNTW pairs without reporting such usage to BellSouth, the following charges shall apply in addition to any fines which may be established by state commissions and any other remedies at law or in equity available to the Provisioning Party:

      - If Requesting Party issued a LSR to disconnect an end-user from BellSouth in order to use a UNTW pair, Requesting Party will be billed for the use of the pair back to the disconnect order date.

      - If Requesting Party activated a UNTW pair on which Provisioning Party was not previously providing service, Requesting Party will be billed for the use of that pair back to the date the end-user began receiving service using that pair.  Upon request, Requesting Party will provide copies of its billing record to substantiate such date.  If Requesting Party fails to provide such records, then Provisioning Party will bill the Requesting Party back to the date of the Access Terminal installation.

#### Unbundled Sub-Loop Concentration System (USLC)

### Where facilities permit and where necessary to comply with an effective Commission order, BellSouth will provide PaeTec with the ability to concentrate its sub-loops onto multiple DS1s back to the BellSouth Central Office.  The DS1s will then be terminated into PaeTec’s collocation space. TR-008 and TR303 interface standards are available.

### USLC, using the Lucent Series 5 equipment, will be offered in two different systems.  System A will allow up to 96 of PaeTec’s sub-loops to be concentrated onto multiple DS1s.  System B will allow an additional 96 of PaeTec’s sub-loops to be concentrated onto multiple DS1s.  One System A may be supplemented with one System B and they both must be physically located in a single Series 5 dual channel bank.  A minimum of two DS1s is required for each system (i.e., System A requires two DS1s and System B would require an additional two DS1s or four in total).  The DS1 level facility that connects the RT site with the serving wire center is known as a Feeder Interface.  All DS1 Feeder Interfaces will terminate to the CLEC’s collocation space within the SWC that serves the RT where the CLEC’s sub-loops are connected.  USLC service is offered with or without concentration and with or without a protection DS1.

### In these scenarios PaeTec would be required to place a cross-box, remote terminal (RT), or other similar device and deliver a cable to the BellSouth RT.  This cable would be connected, by a BellSouth technician, to a cross-connect panel within the BellSouth RT/cross-box and would allow PaeTec’s sub-loops to then be placed on the ULSC and transported to their collocation space at a DS1 level.

  - Unbundled Sub-Loop Feeder

    - Definition

      - Unbundled Sub-Loop Feeder (USLF) provides connectivity between BellSouth's central office and its cross-box (or other access point) that serves an end user location.

    - USLF is intended to be utilized for voice traffic and can be configured as 2-wire voice (USLF-2W/V) or 4-wire voice (USLF-4W/V).

    - USLF can also be utilized for digital traffic and can be configured as 2-wire ISDN (USLF-2W/I); 2-wire Copper (USLF-2W/C); 4-wire Copper (USLF-4W/C) facilities:  4-wire DS0 level loop (USLF-4W/D0); or 4-wire DS1 and ISDN (USLF-4W/DI).

    - USLF will provide the facilities needed to provision a 2W or 4W communications pathway from the BellSouth central office to the BellSouth cross-box. This element will allow for the connection of PaeTec’s loop distribution elements onto BellSouth's feeder system.

    - Requirements

      - PaeTec will extend its compatible cable to BellSouth's cross-box. The cable will then be connected to a panel inside the BellSouth cross-box to the requested level of feeder element.  In those cases when there is no room in the BellSouth cross-box to accommodate the additional cross-connect panels mentioned above, BellSouth will utilize its Special Construction process to determine the costs to provide the sub-loop feeder element to PaeTec.  PaeTec will then have the option of paying the special construction charges or canceling the order.

      - USLF will be a designed circuit and BellSouth will provide a Design Layout Record (DLR) for this element.

      - BellSouth will provide USLF elements in accordance with applicable industry standards for these types of facilities.  Where industry standards do not exist, BellSouth's TR73600 will be used to determine performance parameters.

# High Frequency Spectrum Network Element

  - General

    - BellSouth shall provide PaeTec access to the high frequency portion of the local loop as an unbundled network element only where BellSouth is the voice service provider to the end user (“High Frequency Spectrum”) at the rates set forth in Exhibit C.  BellSouth shall provide PaeTec with the High Frequency Spectrum irrespective of whether BellSouth chooses to offer xDSL services on the loop.

    - The High Frequency Spectrum is defined as the frequency range above the voiceband on a copper loop facility carrying analog circuit-switched voiceband transmissions. Access to the High Frequency Spectrum is intended to allow PaeTec the ability to provide Digital Subscriber Line (“xDSL”) data services to the end user for which BellSouth provides voice services.  The High Frequency Spectrum shall be available for any version of xDSL presumed acceptable for deployment pursuant to 47 CFR Section 51.230, including, but not limited to, ADSL, HDSL, and any other xDSL technology that is presumed to be acceptable for deployment pursuant to FCC rules. BellSouth will continue to have access to the low frequency portion of the loop spectrum (from 300 Hertz to at least 3000 Hertz, and potentially up to 3400 Hertz, depending on equipment and facilities) for the purposes of providing voice service. PaeTec shall only use xDSL technology that is within the PSD mask parameters set forth in T1.413 or other applicable industry standards.  PaeTec shall provision xDSL service on the High Frequency Spectrum in accordance with the applicable Technical Specifications and Standards.

    - The following loop requirements are necessary for PaeTec to be able to access the High Frequency Spectrum:  an unconditioned, 2-wire copper loop.  An unconditioned loop is a copper loop with no load coils, low-pass filters, range extenders, DAMLs, or similar devices and minimal bridged taps consistent with ANSI T1.413 and T1.601.  BellSouth will provide PaeTec access to the Unbundled Loop Modification (Line Conditioning), in accordance with Section 2.2 of this Agreement.  BellSouth is not required to condition a loop for access to the high frequency spectrum if conditioning of that loop significantly degrades BellSouth’s voice service. If PaeTec requests that BellSouth condition a loop longer than 18,000 ft. and such conditioning significantly degrades the voice services on the loop, PaeTec shall pay for the loop to be restored to its original state.

    - PaeTec’s termination point is the point of termination for PaeTec on the toll main distributing frame in the central office (“Termination Point”).  BellSouth will use jumpers to connect PaeTec’s connecting block to the splitter.  The splitter will route the High Frequency Spectrum on the circuit to PaeTec’s xDSL equipment in PaeTec’s collocation space.

    - PaeTec shall have access to the splitter for test purposes, irrespective of where the splitter is placed in the BellSouth premises.

- Provisioning of High Frequency Spectrum and Splitter Space

- BellSouth will provide PaeTec with access to the High Frequency Spectrum as follows:

- BellSouth will install splitters within forty-two (42) calendar days of PaeTec’s submission of such order to the BellSouth Complex Resale Support Group; provided, however, that in the event BellSouth did not have reasonable notice that a particular central office was to have a splitter installed therein, the forty-two (42) day interval shall not apply. Collocation itself or an application for collocation will serve as reasonable notice.

- Once a splitter is installed on behalf of PaeTec in a central office, PaeTec shall be entitled to order the High Frequency Spectrum on lines served out of that central office.

- .1         BellSouth will bill and PaeTec shall pay the SOMAN and SOMEC charges as described in Section 2.13 of this Agreement when PaeTec orders High Frequency Spectrum for end-user service.

- BellSouth will select, purchase, install, and maintain a central office POTS splitter and provide PaeTec access to data ports on the splitter. At least 30 days before making a change in splitter suppliers, BellSouth will provide PaeTec with a carrier notification letter, informing PaeTec of change.  PaeTec shall purchase ports on the splitter as set forth more fully below.

- BellSouth will install the splitter in (i) a common area close to the PaeTec collocation area, if possible; or (ii) in a BellSouth relay rack as close to the PaeTec DS0 termination point as possible. For purposes of this section, a common area is defined as an area in the central office in which both Parties have access to a common test access point.   BellSouth will cross-connect the splitter data ports to a specified PaeTec DS0 at such time that a PaeTec end user’s service is established.

- The High Frequency Spectrum shall only be available on loops on which BellSouth is also providing, and continues to provide, analog voice service directly to the end user.  In the event the end-user terminates its BellSouth provided voice service for any reason, and PaeTec desires to continue providing xDSL service on such loop, PaeTec shall be required to purchase a full stand-alone loop unbundled network element. In the event BellSouth disconnects the end-user’s voice service pursuant to its tariffs or applicable law, and PaeTec desires to continue providing xDSL service on such loop, PaeTec shall be permitted to continue using the line by purchasing the full stand-alone loop unbundled network element.  To the extent commercially practicable, BellSouth shall give PaeTec notice in a reasonable time prior to disconnect, which notice shall give PaeTec an adequate opportunity to notify BellSouth of its intent to purchase such loop.  In those cases in which BellSouth no longer provides voice service to the end user and PaeTec purchases the full stand-alone loop, PaeTec may elect the type of loop it will purchase.  PaeTec will pay the appropriate recurring and non-recurring rates for such loop as set forth in Exhibit C to this Attachment.  In the event PaeTec purchases a voice grade loop, PaeTec acknowledges that such loop may not remain xDSL compatible.

- Only one competitive local exchange carrier shall be permitted access to the High Frequency Spectrum of any particular loop.

3.3		           Ordering

    - To order High Frequency Spectrum on a particular loop, PaeTec must have a DSLAM collocated in the central office that serves the end-user of such loop. PaeTec may order splitters in a central office once it has installed its Digital Subscriber Line Access Multiplexer (“DSLAM”) in that central office.   BellSouth will install these splitters within the interval provided in paragraph 3.2.1.1.

    - BellSouth will devise a splitter order form that allows PaeTec to order splitter ports in increments of 24 ports.

      - BellSouth will provide PaeTec the Local Service Request (“LSR”) format to be used when ordering the High Frequency Spectrum.

    - BellSouth will provide access to the High Frequency Spectrum within the following target intervals:  BellSouth will return a manual Firm Order Confirmation (“FOC”) in no more than two (2) business days after receipt of a valid, error free manual LSR.  When PaeTec submits an electronic LSR for High Frequency Spectrum, BellSouth will return a FOC in four (4) hours ninety-five percent (95%) of the time, or, for orders that do not flow-through, in two (2) business days.  BellSouth will provide PaeTec with access to the High Frequency Spectrum at the following target intervals:

      - For 1-5 lines at the same address within three (3) business days from BellSouth’s issuance of a FOC; 6-10 lines at same address within 5 business days from BellSouth’s issuance of a FOC; and more than 10 lines at the same address is to be negotiated.

    - BellSouth will provide to PaeTec BellSouth’s Loop Qualification System that BellSouth uses to qualify loops for its own ADSL offering as described below.

    - BellSouth will provide PaeTec access to the Preordering Loop Makeup (LMU), in accordance with Section 2.14 of this Agreement.  BellSouth shall bill and PaeTec shall pay the rates for such services, as described in Exhibit C.

  - Maintenance and Repair

    - PaeTec shall have access, for test, repair, and maintenance purposes, to any loop as to which it has access to the High Frequency Spectrum.  PaeTec may access the loop at the point where the combined voice and data signal exits the central office splitter.

    - BellSouth will be responsible for repairing voice services and the physical line between the network interface device at the customer’s premises and the Termination Point of demarcation in the central office.  PaeTec will be responsible for repairing data services.  Each Party will be responsible for maintaining its own equipment.

    - PaeTec shall inform its end users to direct data problems to PaeTec, unless both voice and data services are impaired, in which event the end users should call BellSouth.

    - Once a Party has isolated a trouble to the other Party’s portion of the loop, the Party isolating the trouble shall notify the end user that the trouble is on the other Party’s portion of the loop.

    - In the event PaeTec’s deployment of xDSL on the High Frequency Spectrum significantly degrades the performance of other advanced services or of BellSouth’s voice service on the same loop, BellSouth shall notify PaeTec and allow twenty-four (24) hours to cure the trouble.  If PaeTec fails to resolve the trouble, BellSouth may discontinue PaeTec’s access to the High Frequency Spectrum on such loop.

3.5		 Rates

The prices that PaeTec shall pay to BellSouth for Network Elements and Other Services are set forth in Exhibit C to this Attachment.  If PaeTec purchases a service(s) from a tariff, all terms and conditions and rates as set forth in such tariff shall apply.

## 3.6	Operational Support Systems (OSS)

The terms, conditions and rates for OSS are as set forth in the General Terms and Conditions, Section 10.

# 7.	Switching

BellSouth agrees to offer access to local switching pursuant to the following terms and conditions and at the rates set forth in this Attachment.

## 7.1	Definition

## 7.1.1	Subject to applicable and effective FCC rules and orders, BellSouth shall provide nondiscriminatory access, in accordance with FCC Rule 51.311 and Section 251(c)(3) of the Act, to local circuit switching capability and local tandem switching capability on an unbundled basis, except as set forth in FCC Rule 51.319(c)(1)(B), to PaeTec for the provision of a telecommunications service.  Subject to applicable and effective FCC rules and orders, BellSouth shall be required to provide nondiscriminatory access in accordance with FCC Rule 51.311 and Section 251(c)(3) of the Act to packet switching capability on an unbundled basis to PaeTec for the provision of a telecommunications service as described in Rule 51.319(c)(3)(B).

## 7.1.1.1	Local Circuit Switching Capability, including Tandem Switching Capability.  The local circuit switching capability network element is defined as:

- Line-side facilities, which include, but are not limited to, the connection between a loop termination at a main distribution frame and a switch line card;

- Trunk-side facilities, which include, but are not limited to, the connection between trunk termination at a trunk-side cross-connect panel and a switch trunk card; and

- All features, functions and capabilities of the switch, which include, but are not limited to:

a.	The basic switching function of connecting lines to lines, lines to trunks, trunks to lines, and trunks to trunks, as well as the same basic capabilities made available to the incumbent LEC’s customers, such as a telephone number, white page listing and dial tone, and

b.	All other features that the switch is capable of providing, including but not limited to, customer calling, customer local area signaling service features, and Centrex, as well as any technically feasible customized routing functions provided by the switch.

7.1.1.2			Subject to applicable and effective FCC rules and orders, notwithstanding BellSouth’s general duty to unbundle local circuit switching, BellSouth shall not be required to unbundle local circuit switching for PaeTec in cases where PaeTec intends to use such facilities to serve End Users with four or more voice grade (DS0) equivalents or lines, BellSouth provides nondiscriminatory, unrestricted, cost-based access to the enhanced extended link (“EEL”) throughout Density Zone 1, and BellSouth’s local circuit switches are located in:

- The top 50 Metropolitan Statistical Areas as set forth in Appendix B of the Third Report and Order and Fourth Further Notice of Proposed Rulemaking in CC Docket No. 96-98, and

2.	Density Zone I, as defined in FCC Rule 69.123, as of January 1, 1999.

7.1.1.3			Local Tandem Switching Capability.  The tandem switching capability network element is defined as:

(A)	Trunk-connect facilities, which include, but are not limited to, the connection between trunk termination at a cross connect panel and switch trunk card;

(B)	The basic switch trunk function of connecting trunks to trunks; and

(C)	The functions that are centralized in tandem switches (as distinguished from separate end office switches), including but not limited, to call recording, the routing of calls to operator services, and signaling conversion features.

Tandem Switching is the function that establishes a communications path between two switching offices through a third switching office (the Tandem switch).

7.1.1.4	Packet Switching Capability.  The packet switching capability network element is defined as the basic packet switching function of routing or forwarding packets, frames, cells or other data units based on address or other routing information contained in the packets, frames, cells or other data units, and the functions that are performed by Digital Subscriber Line Access Multiplexers, including but not limited to:

- The ability to terminate copper customer loops (which includes both a low band voice channel and a high-band data channel, or solely a data channel);

- The ability to forward the voice channels, if present, to a circuit switch or multiple circuit switches;

- The ability to extract data units from the data channels on the loops, and

4.	The ability to combine data units from multiple loops onto one or more trunks connecting to a packet switch or packet switches.

7.1.1.5			Subject to applicable and effective FCC rules and orders, BellSouth shall provide nondiscriminatory access to unbundled packet switching capability only in cases where each of the following conditions are satisfied:

- BellSouth has deployed digital loop carrier systems, including but not limited to, integrated digital loop carrier or universal digital loop carrier systems; or has deployed any other system in which fiber optic facilities replace copper facilities in the distribution section (e.g., end office to remote terminal, pedestal or environmentally controlled vault);

- There are no spare copper loops capable of supporting the xDSL services the requesting carrier seeks to offer;

- BellSouth has not permitted a requesting carrier to deploy a Digital Subscriber Line Access Multiplexer at the remote terminal, pedestal or environmentally controlled vault or other interconnection point, nor has the requesting carrier obtained a virtual collocation arrangement at these subloop interconnection points as defined by FCC Rule 51.319(b); and

### 4.	BellSouth has deployed packet switching capability for its own use.

### 7.1.2	A featureless port is one that has a line port, switching functionality, and an interoffice port.  A featured port is a port that includes all features then capable or a number of then capable features specifically requested by PaeTec.  Any features that are not currently then capable but are technically feasible through the switch can be requested through the BFR process.

### 7.1.3	Where required to do so in order to comply with an effective Commission order, BellSouth will provide to PaeTec purchasing  local BellSouth switching and reselling BellSouth local exchange service under Attachment 1, selective routing of calls to a requested directory assistance services platform or operator services platform.  PaeTec customers may use the same dialing arrangements as BellSouth customers, but obtain a PaeTec branded service.

## 7.2	Technical Requirements

### 7.2.1	The requirements set forth in this Section apply to Local Switching, but not to the Packet Switching function of Local Switching.

#### 7.2.1.1	Local Switching shall be equal to or better than the requirements for Local Switching set forth in Telcordia (formerly BellCore)’s Local Switching Systems General Requirements (FR-NWT-000064).

#### 7.2.1.2	When applicable, BellSouth shall route calls to the appropriate trunk or lines for call origination or termination.

#### 7.2.1.3	Subject to this section, BellSouth shall route calls on a per line or per screening class basis to (1) BellSouth platforms providing Network Elements or additional requirements (2) Operator Services platforms, (3) Directory Assistance platforms, and (4) Repair Centers.  Any other routing requests by PaeTec will be made pursuant to the Bona Fide Request/ New Business Request Process as set forth in General Terms and Conditions.

#### 7.2.1.4	BellSouth shall provide unbranded recorded announcements and call progress tones to alert callers of call progress and disposition.

#### 7.2.1.5	BellSouth shall activate service for an PaeTec customer or network interconnection on any of the Local Switching interfaces.  This includes provisioning changes to change a customer from BellSouth’s services to PaeTec’s services without loss of switch feature functionality as defined in this Agreement.

#### 7.2.1.6	BellSouth shall perform routine testing (e.g., Mechanized Loop Tests (MLT) and test calls such as 105, 107 and 108 type calls) and fault isolation on a mutually agreed upon schedule.

#### 7.2.1.7	BellSouth shall repair and restore any equipment or any other maintainable component that may adversely impact Local Switching.

#### 7.2.1.8	BellSouth shall control congestion points such as those caused by radio station call-ins, and network routing abnormalities.  All traffic shall be restricted in a non discriminatory manner.

#### 7.2.1.9	BellSouth shall perform manual call trace and permit customer originated call trace.

#### 7.2.1.10	Special Services provided by BellSouth will include the following:

##### 7.2.1.10.1	Telephone Service Prioritization;

##### 7.2.1.10.2	Related services for handicapped;

##### 7.2.1.10.3	Soft dial tone where required by law; and

##### 7.2.1.10.4	Any other service required by law.

#### 7.2.1.11	BellSouth shall provide Switching Service Point (SSP) capabilities and signaling software to interconnect the signaling links destined to the Signaling Transfer Point Switch (STP).  These capabilities shall adhere to Telcordia (formerly BellCore) specifications - TCAP (GR-1432-CORE), ISUP(GR-905-CORE), Call Management (GR-1429-CORE), Switched Fractional DS1 (GR-1357-CORE), Toll Free Service (GR-1428-CORE), Calling Name (GR-1597-CORE), Line Information Database (GR-954-CORE), and Advanced Intelligent Network (GR-2863-CORE).

#### 7.2.1.12	BellSouth shall provide interfaces to adjuncts through Telcordia (formerly BellCore) standard interfaces.  These adjuncts can include, but are not limited to, the Service Circuit Node and Automatic Call Distributors.

#### 7.2.1.13	BellSouth shall provide performance data regarding a customer line, traffic characteristics or other measurable elements to PaeTec, upon a reasonable request from PaeTec.  CLEC will pay BellSouth for all costs incurred to provide such performance data through the Business Opportunity Request process.

#### 7.2.1.14	BellSouth shall offer Local Switching that provides feature offerings at parity to those provided by BellSouth to itself or any other Party.  Such feature offerings shall include but are not limited to:

##### 7.2.1.14.1	Basic and primary rate ISDN;

##### 7.2.1.14.2	Residential features;

##### 7.2.1.14.3	Customer Local Area Signaling Services (CLASS/LASS);

##### 7.2.1.14.4	CENTREX (including equivalent administrative capabilities, such as customer accessible reconfiguration and detailed message recording); and

##### 7.2.1.14.5	Advanced intelligent network triggers supporting PaeTec and BellSouth service applications.

BellSouth shall offer to PaeTec all AIN triggers in connection with its SMS/SCE offering which are supported by BellSouth for offering AIN-based services. Triggers that are currently available are:

###### 7.2.1.14.5.1	Off-Hook Immediate

###### 7.2.1.14.5.2	Off-Hook Delay

###### 7.2.1.14.5.3	Termination Attempt

###### 7.2.1.14.5.4	6/10 Public Office Dialing Plan

###### 7.2.1.14.5.5	Feature Code Dialing

###### 7.2.1.14.5.6	Customer Dialing Plan

##### 7.2.1.14.6	When the following triggers are supported by BellSouth, BellSouth will make these triggers available to PaeTec:

###### 7.2.1.14.6.1	Private EAMF Trunk

###### 7.2.1.14.6.2	Shared Interoffice Trunk (EAMF, SS7)

###### 7.2.1.14.6.3	N11

###### 7.2.1.14.6.4	Automatic Route Selection

#### 7.2.1.15	Where capacity exists, BellSouth shall assign each PaeTec customer line the class of service designated by PaeTec (e.g., using line class codes or other switch specific provisioning methods), and shall route directory assistance calls from PaeTec customers to PaeTec directory assistance operators at PaeTec’s option.

#### 7.2.1.16	Where capacity exists, BellSouth shall assign each PaeTec customer line the class of services designated by PaeTec (e.g., using line class codes or other switch specific provisioning methods) and shall route operator calls from PaeTec customers to PaeTec operators at PaeTec’s option.  For example, BellSouth may translate 0- and 0+ intraLATA traffic, and route the call through appropriate trunks to an PaeTec Operator Services Position System (OSPS).  Calls from Local Switching must pass the ANI-II digits unchanged.

#### 7.2.1.17	Local Switching shall be offered in accordance with the requirements of the following technical references:

##### 7.2.1.17.1	Telcordia (formerly BellCore) GR-1298-CORE, AIN Switching System Generic Requirements, as implemented in BellSouth’s switching equipment;

##### 7.2.1.17.2	Telcordia (formerly BellCore) GR-1299-CORE, AIN Switch-Service Control Point (SCP)/Adjunct Interface Generic Requirements;

##### 7.2.1.17.3	Telcordia (formerly BellCore) TR-NWT-001284, AIN 0.1 Switching System Generic Requirements;

##### 7.2.1.17.4	Telcordia (formerly BellCore) SR-NWT-002247, AIN Release 1 Update.

### 7.2.2	Interface Requirements

#### 7.2.2.1	BellSouth shall provide the following interfaces to loops:

#### 7.2.2.2	Standard Tip/Ring interface including loopstart or groundstart, on-hook signaling (e.g., for calling number, calling name and message waiting lamp);

#### 7.2.2.3	Coin phone signaling;

#### 7.2.2.4	Basic Rate Interface ISDN adhering to appropriate Telcordia (formerly BellCore) Technical Requirements;

#### 7.2.2.5	Two-wire analog interface to PBX;

##### 7.2.2.5.1	Four-wire analog interface to PBX;

#### 7.2.2.6	Four-wire DS1 interface to PBX or customer provided equipment (e.g. computers and voice response systems);

#### 7.2.2.7	Primary Rate ISDN to PBX adhering to ANSI standards Q.931, Q.932 and appropriate Telcordia (formerly BellCore) Technical Requirements;

#### 7.2.2.8	Switched Fractional DS1 with capabilities to configure Nx64 channels (where N = 1 to 24); and

#### 7.2.2.9	Loops adhering to Telcordia (formerly BellCore) TR-NWT-08 and TR-NWT-303 specifications to interconnect Digital Loop Carriers.

#### 7.2.2.10	BellSouth shall provide access to the following but not limited to:

#### 7.2.2.11	SS7 Signaling Network or Multi-Frequency trunking if requested by PaeTec;

#### 7.2.2.12	Interface to PaeTec operator services systems or Operator Services through appropriate trunk interconnections for the system; and

#### 7.2.2.13	Interface to PaeTec directory assistance services through the PaeTec switched network or to Directory Assistance Services through the appropriate trunk interconnections for the system; and 950 access or other PaeTec required access to interexchange carriers as requested through appropriate trunk interfaces.

8.	Interoffice Transmission Facilities

BellSouth shall provide nondiscriminatory access, in accordance with FCC Rule 51.311 and Section 251(c)(3) of the Act, to interoffice transmission facilities on an unbundled basis to PaeTec for the provision of a telecommunications service.

8.1	Interoffice transmission facility network elements include:

- Dedicated transport, defined as BellSouth’s transmission facilities, including all technically feasible capacity-related services including, but not limited to, DS1, DS3 and OCn levels, dedicated to a particular customer or carrier, that provide telecommunications between wire centers or switches owned by BellSouth, or between wire centers and switches owned by BellSouth and PaeTec;

- Dark Fiber transport, defined as BellSouth’s optical transmission facilities without attached multiplexing, aggregation or other electronics;

3. 	Shared transport, defined as transmission facilities shared by more than one carrier, including BellSouth, between end office switches, between end office switches and tandem switches, and between tandem switches, in BellSouth’s network..

8.1.1	BellSouth shall:

- Provide PaeTec exclusive use of interoffice transmission facilities dedicated to a particular customer or carrier, or shared use of the features, functions, and capabilities of interoffice transmission facilities shared by more than one customer or carrier;

- Provide all technically feasible transmission facilities, features, functions, and capabilities that PaeTec could use to provide telecommunications services;

- Permit, to the extent technically feasible, PaeTec to connect such interoffice facilities to equipment designated by PaeTec, including but not limited to, PaeTec’s collocated facilities; and

- Permit, to the extent technically feasible, PaeTec to obtain the functionality provided by BellSouth’s digital cross-connect systems in the same manner that BellSouth provides such functionality to interexchange carriers.

## 8.2	Technical Requirements of Common (Shared) Transport

### 8.2.1	Common (Shared) Transport provided on DS1 or VT1.5 circuits, shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for Central Office to Central Office (“CO to CO”) connections in the appropriate industry standards.

### 8.2.2	Common (Shared) Transport provided on DS3 circuits, STS-1 circuits, and higher transmission bit rate circuits, shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for CO to CO connections in the appropriate industry standards.

### 8.2.3	BellSouth shall be responsible for the engineering, provisioning, and maintenance of the underlying equipment and facilities that are used to provide Common (Shared) Transport.

### 8.2.4	At a minimum, Common (Shared) Transport shall meet all of the requirements set forth in the following technical references (as applicable for the transport technology being used):

#### 8.2.4.1	ANSI T1.101-1994, American National Standard for Telecommunications - Synchronization Interface Standard Performance and Availability;

#### 8.2.4.2	ANSI T1.102-1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces;

#### 8.2.4.3	ANSI T1.102.01-199x, American National Standard for Telecommunications - Digital Hierarchy - VT1.5;

#### 8.2.4.4	ANSI T1.105-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Basic Description including Multiplex Structure, Rates and Formats;

#### 8.2.4.5	ANSI T1.105.01-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Automatic Protection Switching;

#### 8.2.4.6	ANSI T1.105.02-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Payload Mappings;

#### 8.2.4.7	ANSI T1.105.03-1994, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Jitter at Network Interfaces;

#### 8.2.4.8	ANSI T1.105.03a-1995, American National Standard for Telecommunications - Synchronous Optical Network (SONET): Jitter at Network Interfaces - DS1 Supplement;

#### 8.2.4.9	ANSI T1.105.05-1994, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Tandem Connection;

#### 8.2.4.10	ANSI T1.105.06-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Physical Layer Specifications;

#### 8.2.4.11	ANSI T1.105.07-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Sub STS-1 Interface Rates and Formats;

#### 8.2.4.12	ANSI T1.105.09-199x, American National Standard for Telecommunications - Synchronous Optical Network (SONET) - Network Element Timing and Synchronization;

#### 8.2.4.13	ANSI T1.106-1988, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (Single Mode);

#### 8.2.4.14	ANSI T1.107-1988, American National Standard for Telecommunications - Digital Hierarchy - Formats Specifications;

#### 8.2.4.15	ANSI T1.107a-1990 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications (DS3 Format Applications);

#### 8.2.4.16	ANSI T1.107b-1991 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications;

#### 8.2.4.17	ANSI T1.117-1991, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (SONET) (Single Mode - Short Reach);

#### 8.2.4.18	ANSI T1.403-1989, Carrier to Customer Installation, DS1 Metallic Interface Specification;

#### 8.2.4.19	ANSI T1.404-1994, Network-to-Customer Installation - DS3 Metallic Interface Specification;

#### 8.2.4.20	ITU Recommendation G.707, Network node interface for the synchronous digital hierarchy (SDH);

#### 8.2.4.21	ITU Recommendation G.704, Synchronous frame structures used at 1544, 6312, 2048, 8488 and 44736 kbit/s hierarchical levels;

#### 8.2.4.22	Telcordia (formerly BellCore) FR-440 and TR-NWT-000499, Transport Systems Generic Requirements (TSGR):  Common Requirements;

#### 8.2.4.23	Telcordia (formerly BellCore) GR-820-CORE, Generic Transmission Surveillance:  DS1 & DS3 Performance;

#### 8.2.4.24	Telcordia (formerly BellCore) GR-253-CORE, Synchronous Optical Network Systems (SONET);  Common Generic Criteria;

#### 8.2.4.25	Telcordia (formerly BellCore) TR-NWT 000507, Transmission, Section 7, Issue 5 (Telcordia (formerly BellCore), December 1993). (A module of LSSGR, FR-NWT-000064.);

#### 8.2.4.26	Telcordia (formerly BellCore) TR-NWT-000776, Network Interface Description for ISDN Customer Access;

#### 8.2.4.27	Telcordia (formerly BellCore) TR-INS-000342, High-Capacity Digital Special Access Service-Transmission Parameter Limits and lnterface Combinations, Issue 1 February 1991;

#### 8.2.4.28	Telcordia (formerly BellCore) ST-TEC 000052, Telecommunications Transmission Engineering Textbook, Volume 2: Facilities, Third Edition, Issue I May 1989;

#### 8.2.4.29	Telcordia (formerly BellCore) ST-TEC-000051, Telecommunications Transmission Engineering Textbook Volume 1:  Principles, Third Edition. Issue 1 August 1987.

## 8.3	Dedicated Transport

#### 8.3.1.	BellSouth shall offer Dedicated Transport in each of the following ways:

##### 8.3.1.1	As capacity on a shared facility.

##### 8.3.1.2	As a circuit (e.g., DS0, DS1 or DS3) dedicated to PaeTec.

#### 8.3.2	When Dedicated Transport is provided as a system it shall include:

##### 8.3.2.1	Transmission equipment such as multiplexers, line terminating equipment, amplifiers, and regenerators;

##### 8.3.2.2	Inter-office transmission facilities such as optical fiber, copper twisted pair, and coaxial cable.

### 8.3.3	Unbundled Local Channel

8.3.3.1	The Unbundled Local Channel is the dedicated transmission path between PaeTec’s Point of Presence and the BellSouth Serving Wire Center.

8.3.3.2	BellSouth currently offers Unbundled Local Channels for switched traffic.  Rates for these elements are listed in this Attachment.  For those states that do not contain rates in this Attachment for DS1 and DS3 switched Local Channels, the rates in the applicable State Access Tariff will apply as interim rates.  When final rates are developed, these interim rates will be subject to true-up, and the Parties will amend the Agreement to reflect the new rates.

8.3.3.3	BellSouth currently offers Unbundled Local Channels for non-switched traffic at DS1 and DS3 levels at rates as set forth in Exhibit C to this Attachment.

### 8.3.4	Technical Requirements

This Section sets forth technical requirements for all Dedicated Transport.

#### 8.3.4.1	When BellSouth provides Dedicated Transport as a circuit or a system, the entire designated transmission circuit or system (e.g., DS0, DS1,DS3) shall be dedicated to PaeTec designated traffic.

#### 8.3.4.2	BellSouth shall offer Dedicated Transport in all technologies that become available including, but not limited to, DS1 and DS3 transport systems, SONET (or SDH) Bi-directional Line Switched Rings, SONET (or SDH) Unidirectional Path Switched Rings, and SONET (or SDH) point-to-point transport systems (including linear add-drop systems), at all available transmission bit rates.  While SONET Ring facilities are not available in every application, they are typically available in the major metropolitan areas.

#### 8.3.4.3	For DS1 or VT1.5 circuits, Dedicated Transport shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for Customer Interface to Central Office (“CI to CO”) connections in the appropriate industry standards.

#### 8.3.4.4	Where applicable, for DS3 circuits, Dedicated Transport shall, at a minimum, meet the performance, availability, jitter, and delay requirements specified for CI to CO connections in the appropriate industry standards.

#### 8.3.4.5	BellSouth shall offer the following interface transmission rates for Dedicated Transport:

##### 8.3.4.5.1	DS0 Equivalent;

##### 8.3.4.5.2	DS1 (Extended SuperFrame - ESF and D4 channel bank shall be provided);

##### 8.3.4.5.3	DS3 where applicable (M13 multiplexer shall be provided);

##### 8.3.4.5.4	SDH Standard interface rates in accordance with International Telecommunications Union (ITU) Recommendation G.707 and Plesiochronous Digital Hierarchy (PDH) rates per ITU Recommendation G.704.

#### 8.3.4.6	When Dedicated Transport is provided as a system, BellSouth shall design the system according to our network infrastructure to allow for the termination points specified by PaeTec.

8.3.5	At a minimum, Dedicated Transport shall meet each of the requirements set forth in the following technical references:

#### 8.3.5.1	ANSI T1.231-1993 -American National Standard for Telecommunications - Digital Hierarchy - Layer 1 In-Service Digital Transmission performance monitoring.

##### 8.3.5.1.1	ANSI T1.102-1993, American National Standard for Telecommunications - Digital Hierarchy - Electrical Interfaces;

##### 8.3.5.1.2	ANSI T1.106-1988, American National Standard for Telecommunications - Digital Hierarchy - Optical Interface Specifications (Single Mode);

##### 8.3.5.1.3	ANSI T1.107-1988, American National Standard for Telecommunications - Digital Hierarchy - Formats Specifications;

##### 8.3.5.1.4	ANSI T1.107a-1990 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications (DS3 Format Applications);

##### 8.3.5.1.5	ANSI T1.107b-1991 - American National Standard for Telecommunications - Digital Hierarchy - Supplement to Formats Specifications;

##### 8.3.5.1.6	Telcordia (formerly BellCore) FR-440 and TR-NWT-000499, Transport Systems Generic Requirements (TSGR):  Common Requirements;

##### 8.3.5.1.7	Telcordia (formerly BellCore) GR-820-CORE, Generic Transmission Surveillance:  DS1 & DS3 Performance;

##### 8.3.5.1.8	Telcordia (formerly BellCore) TR-NWT 000507, Transmission, Section 7, Issue 5 (Telcordia (formerly BellCore), December 1993). (A module of LSSGR, FR-NWT-000064.);

##### 8.3.5.1.9	Telcordia (formerly BellCore) TR-INS-000342, High-Capacity Digital Special Access Service-Transmission Parameter Limits and lnterface Combinations, Issue 1 February 1991;

##### 8.3.5.1.10	Telcordia (formerly BellCore) ST-TEC 000052, Telecommunications Transmission Engineering Textbook, Volume 2: Facilities, Third Edition, Issue I May 1989;

##### 8.3.5.1.11	Telcordia (formerly BellCore) ST-TEC-000051, Telecommunications Transmission Engineering Textbook Volume 1:  Principles, Third Edition. Issue 1 August 1987;

#### 9.	Tandem Switching

#### 9.1	Tandem Switching is as defined in Section 7.1.1.3 of this Attachment.

#### 9.2 	Technical Requirements

9.2.1 	Tandem Switching shall have the same capabilities or equivalent capabilities as those described in Bell Communications Research TR-TSY-000540 Issue 2R2, Tandem Supplement, 6/1/90. The requirements for Tandem Switching include, but are not limited to the following:

##### 9.2.1.1	Tandem Switching shall provide signaling to establish a tandem connection;

##### 9.2.1.2	Tandem Switching will provide screening as jointly agreed to by PaeTec and BellSouth;

##### 9.2.1.3	Tandem Switching shall provide Advanced Intelligent Network triggers supporting AIN features where such routing is not available from the originating end office switch, to the extent such Tandem switch has such capability;

##### 9.2.1.4	Tandem Switching shall provide access to Toll Free number portability database as designated by PaeTec;

##### 9.2.1.5	Tandem Switching shall provide all trunk interconnections discussed under the “Network Interconnection” section (e.g., SS7, MF, DTMF, DialPulse, PRI-ISDN, DID, and CAMA-ANI (if appropriate for 911));

##### 9.2.1.6	Tandem Switching shall provide connectivity to PSAPs where 911 solutions are deployed and the tandem is used for 911; and

##### 9.2.1.7	Where appropriate, Tandem Switching shall provide connectivity to transit traffic to and from other carriers.

#### 9.2.2	Tandem Switching shall accept connections (including the necessary signaling and trunking interconnections) between end offices, other tandems, IXCs, ICOs, CAPs and CLEC switches.

#### 9.2.3	Tandem Switching shall provide local tandeming functionality between two end offices including two offices belonging to different CLEC's (e.g., between a CLEC end office and the end office of another CLEC).

#### 9.2.4	Tandem Switching shall preserve CLASS/LASS features and Caller ID as traffic is processed.

#### 9.2.5	Tandem Switching shall record billable events and send them to the area billing centers designated by PaeTec.  Tandem Switching will provide recording of all billable events as jointly agreed to by PaeTec and BellSouth.

#### 9.2.6	Upon a reasonable request from PaeTec, BellSouth shall perform routine testing and fault isolation on the underlying switch that is providing Tandem Switching and all its interconnections.  The results and reports of the testing shall be made immediately available to PaeTec.

#### 9.2.7	BellSouth shall maintain PaeTec’s trunks and interconnections associated with Tandem Switching at least at parity to its own trunks and interconnections.

#### 9.2.8	BellSouth shall control congestion points and network abnormalities.  All traffic will be restricted in a non discriminatory manner.

#### 9.2.9	Selective Call Routing through the use of line class codes is not available through the use of tandem switching.  Selective Call Routing through the use of line class codes is an end office capability only.  Detailed primary and overflow routing plans for all interfaces available within BellSouth switching network shall be mutually agreed to by PaeTec and BellSouth.

#### 9.2.10	Tandem Switching shall process originating toll-free traffic received from PaeTec local switch.

#### 9.2.11	In support of AIN triggers and features, Tandem Switching shall provide SSP capabilities when these capabilities are not available from the Local Switching Network Element, to the extent such Tandem Switch has such capability.

### 9.3	Interface Requirements

#### 9.3.1	Tandem Switching shall provide interconnection to the E911 PSAP where the underlying Tandem is acting as the E911 Tandem.

#### 9.3.2	Tandem Switching shall interconnect, with direct trunks, to all carriers with which BellSouth interconnects.

#### 9.3.3	BellSouth shall provide all signaling necessary to provide Tandem Switching with no loss of feature functionality.

#### 9.3.4	Tandem Switching shall interconnect with PaeTec's switch, using two-way trunks, for traffic that is transiting via BellSouth network to interLATA or intraLATA carriers.  At PaeTec’s request, Tandem Switching shall record and keep records of traffic for billing.

#### 9.3.5	Tandem Switching shall provide an alternate final routing pattern for PaeTec traffic overflowing from direct end office high usage trunk groups.

### 9.4	Tandem Switching shall meet or exceed (i.e., be more favorable to PaeTec) each of the requirements for Tandem Switching set forth in the following technical references:

#### 9.4.1	Bell Communications Research TR-TSY-000540 Issue 2R2, Tandem Supplement, 6/1/90;

#### 9.4.2	GR-905-CORE covering CCSNIS;

#### 9.4.3	GR-1429-CORE for call management features; and 
GR-2863-CORE and Telcordia (formerly BellCore) GR-2902-CORE covering CCS AIN interconnection

# 10.	Operator Systems

BellSouth agrees to offer access to operator systems pursuant to the terms and conditions following and at the rates set forth in this Attachment.

## 10.1	Definition

Operator Systems is the Network Element that provides operator and automated call handling and billing, special services, end user telephone listings and optional call completion services.  The Operator Systems, Network Element provides two types of functions:  Operator Service functions and Directory Assistance Service functions, each of which are described in detail below.

## 10.2	Operator Service

### 10.2.1	Definition

Operator Service provides:  (1) operator handling for call completion (for example, collect, third number billing, and manual credit card calls), (2) operator or automated assistance for billing after the end user has dialed the called number (for example, credit card calls); and (3) special services including but not limited to Busy Line Verification and Emergency Line Interrupt (ELI), Emergency Agency Call, Operator-assisted Directory Assistance, and Rate Quotes.

### 11.2.2	Requirements

#### 10.2.2.1	When PaeTec requests BellSouth to provide Operator Services, the following requirements apply:

##### 10.2.2.1.1	BellSouth shall complete 0+ and 0- dialed local calls.

##### 10.2.2.1.2	BellSouth shall complete 0+ intraLATA toll calls.

##### 10.2.2.1.3	BellSouth shall complete calls that are billed to PaeTec end user's calling card that can be validated by BellSouth.

##### 10.2.2.1.4	BellSouth shall complete person-to-person calls.

##### 10.2.2.1.5	BellSouth shall complete collect calls.

##### 10.2.2.1.6	BellSouth shall provide the capability for callers to bill to a third party and complete such calls.

##### 10.2.2.1.7	BellSouth shall complete station-to-station calls.

##### 10.2.2.1.8	BellSouth shall process emergency calls.

##### 10.2.2.1.9	BellSouth shall process Busy Line Verify and Emergency Line Interrupt requests.

##### 10.2.2.1.10	BellSouth shall process emergency call trace, as they do for their End users prior to the Effective Date.  Call must originate from a 911 provider.

##### 10.2.2.1.11	BellSouth shall process operator-assisted directory assistance calls.

#### 10.2.2.2	BellSouth shall adhere to equal access requirements, providing PaeTec local end users the same IXC access as provided to BellSouth end users.

#### 10.2.2.3	BellSouth shall exercise at least the same level of fraud control in providing Operator Service to PaeTec that BellSouth provides for its own operator service.

#### 10.2.2.4	BellSouth shall perform Billed Number Screening when handling Collect, Person-to-Person, and Billed-to-Third-Party calls.

#### 10.2.2.5	BellSouth shall direct customer account and other similar inquiries to the customer service center designated by PaeTec.

#### 10.2.2.6	BellSouth shall provide a feed of customer call records in “EMI” format to PaeTec in accordance with CLECODUF standards specified in Attachment 7.

### 10.2.3	Interface Requirements

With respect to Operator Services for calls that originate on local switching capability provided by or on behalf of PaeTec, the interface requirements shall conform to the then current established system interface specifications for the platform used to provide Operator Service and the interface shall conform to industry standards.

## 10.3	Directory Assistance Service

### 10.3.1	Definition

Directory Assistance Service provides local end user telephone number listings with the option to complete the call at the callers direction separate and distinct from local switching.

### 10.3.2	Requirements

#### 10.3.2.1	Directory Assistance Service shall provide up to two listing requests per call.  If available and if requested by PaeTec’s end user, BellSouth shall provide caller-optional directory assistance call completion service at rates contained in this Attachment to one of the provided listings, equal to that which BellSouth provides its end users.  If not available, PaeTec may request such requirement pursuant to the Bona Fide Request/New Business Process as set forth in General Terms and Conditions.

#### 10.3.2.2	Directory Assistance Service Updates

##### 10.3.2.2.1	BellSouth shall update end user listings changes daily.  These changes include:

###### 10.3.2.2.1.1	New end user connections:  BellSouth will provide service to PaeTec that is equal to the service it provides to itself and its end users;

###### 10.3.2.2.1.2	End user disconnections:  BellSouth will provide service to PaeTec that is equal to the service it provides to itself and its end users; and

###### 10.3.2.2.1.3	End user address changes:  BellSouth will provide service to PaeTec that is equal to the service it provides to itself and its end users;

#### 10.3.2.3	These updates shall also be provided for non-listed and non-published numbers for use in emergencies.

## 10.4	Branding for Operator Call Processing and Directory Assistance

### 10.4.1	The BellSouth Operator Systems Branding Feature provides a definable announcement to PaeTec end users using Directory Assistance (DA)/Operator Call Processing (OCP) prior to placing them in queue or connecting them to an available operator or automated operator system. This feature allows PaeTec to have its calls custom branded with PaeTec name on whose behalf BellSouth is providing Directory Assistance and/or Operator Call Processing.  Rates for Custom Branding, Operator Call Process and Directory Assistance are set forth in this Attachment.

### 10.4.2	BellSouth offers four service levels of branding to PaeTec when ordering Directory Assistance and/or Operator Call Processing.

#### 10.4.2.1	Service Level 1 - BellSouth Branding

#### 10.4.2.2	Service Level 2 - Unbranded

#### 10.4.2.3	Service Level 3 - Custom Branding

#### 10.4.2.4	Service Level 4 - Self Branding (applicable only to PaeTec for Resale or use with an Unbundled Port when routing to an operator service provider other than BellSouth).

### 10.4.3	For Resellers and Use with an Unbundled Port

#### 10.4.3.1	BellSouth Branding is the Default Service Level.

#### 10.4.3.2	Unbranding, Custom Branding, and Self Branding require PaeTec to order selective routing for each originating BellSouth end office identified by PaeTec.  Rates for Selective Routing are set forth in this Attachment.

#### 10.4.3.3	Customer Branding and Self Branding require PaeTec to order dedicated trunking from each BellSouth end office identified by PaeTec, to either the BellSouth Traffic Operator Position System (TOPS) or PaeTec Operator Service Provider.  Rates for trunks are set forth in applicable BellSouth tariffs.

#### 10.4.3.4	Unbranding - Unbranded Directory Assistance and/or Operator Call Processing calls ride common trunk groups provisioned by BellSouth from those end offices identified by PaeTec to the BellSouth TOPS.  These calls are routed to “No Announcement.”

### 10.4.4	For Facilities Based Carriers

#### 10.4.4.1	All Service Levels require PaeTec to order dedicated trunking from their end office(s) point of interface to the BellSouth TOPS Switches. Rates for trunks are set forth in applicable BellSouth tariffs.

#### 10.4.4.2	Customized Branding includes charges for the recording of the branding announcement and the loading of the audio units in each TOPS Switch, IVS and NAV equipment for which PaeTec requires service

Directory Assistance customized branding uses:

- the recording of the name;

- the front-end loading of the Digital Recorded Announcement Machine (DRAM) in each TOPS switch.

Operator Call Processing customized branding uses:

- the recording of the name;

- the front-end loading of the DRAM in the TOPS Switch;

- the back-end loading in the audio units in the Automated Alternate Billing System (AABS) in the Interactive Voice Subsystem (IVS);

- the 0- automation loading for the audio units in the Enhanced Billing and Access Service (EBAS) in the Network Applications Vehicle (NAV).

10.4.4.3	BellSouth will provide to PaeTec purchasing  local BellSouth switching and reselling BellSouth local exchange service, selective routing of calls to a requested directory assistance services platform or operator services platform.  PaeTec end users may use the same dialing arrangements as BellSouth end users, but obtain a PaeTec branded service.

## 10.5	Directory Assistance Database Service (DADS)

### 10.5.1	BellSouth shall make its Directory Assistance Database Service (DADS) available solely for the expressed purpose of providing Directory Assistance type services to PaeTec end users.  The term “end user” denotes any entity which obtains Directory Assistance type services for its own use from a DADS customer.  Directory Assistance type service is defined as Voice Directory Assistance (DA Operator assisted and Electronic Directory Assistance (Data System assisted)).  PaeTec agrees that Directory Assistance Database Service (DADS) will not be used for any purpose which violates federal or state laws, statutes, regulatory orders or tariffs.  Except for the permitted users, PaeTec agrees not to disclose DADS to others and shall provide due care in providing for the security and confidentiality of DADS. Further, PaeTec authorizes the inclusion of PaeTec Subscriber listings in the BellSouth Directory Assistance products.

### 10.5.2	BellSouth shall provide PaeTec initially with a base file of subscriber listings which reflect all listing change activity occurring since PaeTec’s most recent update via magnetic tape, and subsequently using electronic connectivity such as Network Data Mover to be developed mutually by PaeTec and BellSouth.  PaeTec agrees to assume the costs associated with CONNECT: Direct TM connectivity, which will vary depending upon volume and mileage.

### 10.5.3	BellSouth will require approximately one month after receiving an order to prepare the Base File.  BellSouth will provide daily updates which will reflect all listing change activity occurring since CLEC’s most recent update.  BellSouth shall provide updates to PaeTec on a Business, Residence, or combined Business and Residence basis.  PaeTec agrees that the updates shall be used solely to keep the information current.  Delivery of Daily Updates will commence the day after PaeTec receives the Base File.

### 10.5.4	BellSouth is authorized to include PaeTec Subscriber List Information in its Directory Assistance Database Service (DADS) and its Directory Publishers Database Service (DPDS).  Any other use by BellSouth of PaeTec Subscriber List Information is not authorized and with the exception of a request for DADS or DPDS, BellSouth shall refer any request for such information to PaeTec.

### 10.5.5	Rates for DADS are as set forth in this Attachment.

## 10.6	Direct Access to Directory Assistance Service

### 10.6.1	Direct Access to Directory Assistance Service (DADAS) will provide PaeTec’s directory assistance operators with the ability to search all available BellSouth’s subscriber listings using the Directory Assistance search format.  Subscription to DADAS will allow PaeTec to utilize its own switch, operator workstations and optional audio subsystems.

### 10.6.2	BellSouth will provide DADAS from its DA location.  PaeTec will access the DADAS system via a telephone company provided point of availability.  PaeTec has the responsibility of providing the physical links required to connect to the point of availability.  These facilities may be purchased from the telephone company as rates and charges billed separately from the charges associated with this offering.

### 10.6.3	A specified interface to each PaeTec subsystem will be provided by BellSouth.  Interconnection between PaeTec system and a specified BellSouth location will be pursuant to the use of PaeTec owned or PaeTec leased facilities and shall be appropriate sized based upon the volume of queries being generated by PaeTec.

### 10.6.4	The specifications for the three interfaces necessary for interconnection are available in the following documents:

#### 10.6.4.1	DADAS to Subscriber Operator Position System—Northern Telecom Document CSI-2300-07; Universal Gateway/ Position Message Interface Format Specification

#### 10.6.4.2	DADAS to Subscriber Switch—Northern Telecom Document Q210-1 Version A107; NTDMS/CCIDAS System Application Protocol; and AT&T Document 250-900-535 Operator Services Position System Listing Service and Application Call Processing Data Link Interface Specification

#### 10.6.4.3	DADAS to Audio Subsystem (Optional)—Directory One Call Control to Audio Response Unit system interface specifications are available through Northern Telecom as a licensed access protocol—Northern Telecom Document 355-004424 and Gateway/Interactive Voice subsystem Protocol Specification

### 10.6.5	Rates for DADAS are as set forth in this Attachment.

# 11.	Signaling

BellSouth agrees to offer access to  signaling and access to BellSouth’s signaling databases subject to compatibility testing and at the rates set forth in this Attachment.  BellSouth may provide mediated access to BellSouth signaling systems and databases.  Available signaling elements include signaling links, signal transfer points and service control points.  Signaling functionality will be available with both A-link and B-link connectivity.

## 11.1	Definition of Signaling Link Transport

Signaling Link Transport is a set of two or four dedicated 56 Kbps. transmission paths between CLEC-designated Signaling Points of Interconnection (SPOI) that provides appropriate physical diversity.

## 11.2	Technical Requirements

### 11.2.1	Signaling Link Transport shall consist of full duplex mode 56 kbps transmission paths.

### 11.2.2	Of the various options available, Signaling Link Transport shall perform in the following two ways:

#### 11.2.2.1	As an “A-link” which is a connection between a switch or SCP and a home Signaling Transfer Point Switch (STP) pair; and

#### 11.2.2.2	As a “B-link” which is a connection between two STP pairs in different company networks (e.g., between two STP pairs for two Competitive Local Exchange Carriers (CLECs)).

### 11.2.3	Signaling Link Transport shall consist of two or more signaling link layers as follows:

#### 11.2.3.1	An A-link layer shall consist of two links.

#### 11.2.3.2	A B-link layer shall consist of four links.

### 11.2.4	A signaling link layer shall satisfy a performance objective such that:

#### 11.2.4.1	There shall be no more than two minutes down time per year for an A-link layer; and

#### 11.2.4.2	There shall be negligible (less than 2 seconds) down time per year for a B-link layer.

### 11.2.5	A signaling link layer shall satisfy interoffice and intraoffice diversity of facilities and equipment, such that:

#### 11.2.5.1	No single failure of facilities or equipment causes the failure of both links in an A-link layer (i.e., the links should be provided on a minimum of two separate physical paths end-to-end); and

#### 11.2.5.2	No two concurrent failures of facilities or equipment shall cause the failure of all four links in a B-link layer (i.e., the links should be provided on a minimum of three separate physical paths end-to-end).

## 11.3	Interface Requirements

### 11.3.1	There shall be a DS1 (1.544 Mbps) interface at the PaeTec-designated SPOIs.  Each 56 kbps transmission path shall appear as a DS0 channel within the DS1 interface.

# 12.	Signaling Transfer Points (STPs)

## 12.1	Definition - Signaling Transfer Points is a signaling network function that includes all of the capabilities provided by the signaling transfer point switches (STPs) and their associated signaling links which enable the exchange of SS7 messages among and between switching elements, database elements and signaling transfer point switches

## 12.2	Technical Requirements

### 12.2.1	STPs shall provide access to Network Elements connected to BellSouth SS7 network. These include:

#### 12.2.1.1	BellSouth Local Switching or Tandem Switching;

#### 12.2.1.2	BellSouth Service Control Points/DataBases;

#### 12.2.1.3	Third-party local or tandem switching;

#### 12.2.1.4	Third-party-provided STPs.

### 12.2.2	The connectivity provided by STPs shall fully support the functions of all other Network Elements connected to BellSouth SS7 network.  This explicitly includes the use of BellSouth SS7 network to convey messages which neither originate nor terminate at a signaling end point directly connected to BellSouth SS7 network (i.e., transient messages).  When BellSouth SS7 network is used to convey transient messages, there shall be no alteration of the Integrated Services Digital Network User Part (ISDNUP) or Transaction Capabilities Application Part (TCAP) user data that constitutes the content of the message.

### 12.2.3	If a BellSouth tandem switch routes calling traffic, based on dialed or translated digits, on SS7 trunks between an PaeTec local switch and third party local switch, BellSouth SS7 network shall convey the TCAP messages that are necessary to provide Call Management features (Automatic Callback, Automatic Recall, and Screening List Editing) between PaeTec local STPs and the STPs that provide connectivity with the third party local switch, even if the third party local switch is not directly connected to BellSouth STPs.

### 12.2.4	STPs shall provide all functions of the MTP as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.  This includes:

#### 12.2.4.1	Signaling Data Link functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements,

#### 12.2.4.2	Signaling Link functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements, and

#### 12.2.4.3	Signaling Network Management functions, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.

### 12.2.5	STPs shall provide all functions of the SCCP necessary for Class 0 (basic connectionless) service, as defined in Telcordia (formerly BellCore) ANSI Interconnection Requirements.  In particular, this includes Global Title Translation (GTT) and SCCP Management procedures, as specified in T1.112.4.  In cases where the destination signaling point is a PaeTec or third party local or tandem switching system directly connected to BellSouth SS7 network, BellSouth shall perform final GTT of messages to the destination and SCCP Subsystem Management of the destination.  In all other cases, BellSouth shall perform intermediate GTT of messages to a gateway pair of STPs in an SS7 network connected with BellSouth SS7 network, and shall not perform SCCP Subsystem Management of the destination.  If BellSouth performs final GTT to a PaeTec database, then PaeTec agrees to provide BellSouth with the Destination Point Code for the PaeTec database.

### 12.2.6	STPs shall provide on a non-discriminatory basis all functions of the OMAP commonly provided by STPs, as specified in the reference in Section 12.4.5 of this Attachment.  All OMAP functions will be on a "where available" basis and can include:

#### 12.2.6.1	MTP Routing Verification Test (MRVT) and

#### 12.2.6.2	SCCP Routing Verification Test (SRVT).

### 12.2.7	In cases where the destination signaling point is a BellSouth local or tandem switching system or database, or is an PaeTec or third party local or tandem switching system directly connected to the BellSouth SS7 network, STPs shall perform MRVT and SRVT to the destination signaling point.  In all other cases, STPs shall perform MRVT and SRVT to a gateway pair of STPs in an SS7 network connected with the BellSouth SS7 network.  This requirement shall be superseded by the specifications for Internetwork MRVT and SRVT if and when these become approved ANSI standards and available capabilities of BellSouth STPs, and if mutually agreed upon by PaeTec and BellSouth.

### 12.2.8	STPs shall be on parity with BellSouth.

### 12.2.9	SS7 Advanced Intelligent Network (AIN) Access

#### 12.2.9.1	When technically feasible and upon request by PaeTec, SS7 Access shall be made available in association with  switching.  SS7 AIN Access is the provisioning of AIN 0.1 triggers in an equipped BellSouth local switch and interconnection of the BellSouth SS7 network with the PaeTec SS7 network to exchange TCAP queries and responses with an PaeTec SCP.

#### 12.2.9.2	SS7 AIN Access shall provide PaeTec SCP access to BellSouth local switch in association with  switching via interconnection of BellSouth SS7 and PaeTec SS7 Networks.  BellSouth shall offer SS7 access through its STPs.  If BellSouth requires a mediation device on any part of its network specific to this form of access, BellSouth must route its messages in the same manner.  The interconnection arrangement shall result in the BellSouth local switch recognizing the PaeTec SCP as at least at parity with BellSouth’s SCPs in terms of interfaces, performance and capabilities.

## 12.3	Interface Requirements

### 12.3.1	BellSouth shall provide the following STPs options to connect PaeTec or PaeTec-designated local switching systems or STPs to BellSouth SS7 network:

#### 12.3.1.1	An A-link interface from PaeTec local switching systems; and,

#### 12.3.1.2	A B-link interface from PaeTec local STPs.

### 12.3.2	Each type of interface shall be provided by one or more sets (layers) of signaling links.

### 12.3.3	The Signaling Point of Interconnection (SPOI) for each link shall be located at a cross-connect element, such as a DSX-1, in the Central Office (CO) where BellSouth STP is located.  There shall be a DS1 or higher rate transport interface at each of the SPOIs.  Each signaling link shall appear as a DS0 channel within the DS1 or higher rate interface. BellSouth shall offer higher rate DS1 signaling for interconnecting PaeTec local switching systems or STPs with BellSouth STPs as soon as these become approved ANSI standards and available capabilities of BellSouth STPs.  BellSouth and PaeTec will work jointly to establish mutually acceptable SPOIs.

### 12.3.4	BellSouth CO shall provide intraoffice diversity between the SPOIs and BellSouth STPs, so that no single failure of intraoffice facilities or equipment shall cause the failure of both B-links in a layer connecting to a BellSouth STP.  BellSouth and PaeTec will work jointly to establish mutually acceptable SPOIs.

### 12.3.5	BellSouth shall provide MTP and SCCP protocol interfaces that shall conform to all sections relevant to the MTP or SCCP in the following specifications:

#### 12.3.5.1	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

#### 12.3.5.2	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

### 12.3.6	Message Screening

#### 12.3.6.1	BellSouth shall set message screening parameters so as to accept valid messages from PaeTec local or tandem switching systems destined to any signaling point within BellSouth’s SS7 network where the PaeTec switching system has a legitimate signaling relation.

#### 12.3.6.2	BellSouth shall set message screening parameters so as to pass valid messages from PaeTec local or tandem switching systems destined to any signaling point or network accessed through BellSouth’s SS7 network where the PaeTec switching system has a legitimate signaling relation.

#### 12.3.6.3	BellSouth shall set message screening parameters so as to accept and pass/send valid messages destined to and from PaeTec from any signaling point or network interconnected through BellSouth’s SS7 network where the PaeTec SCP has a legitimate signaling relation.

## 12.4	STPs shall be equal to or better than all of the requirements for STPs set forth in the following technical references:

### 12.4.1	ANSI T1.111-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP);

### 12.4.2	ANSI T1.111A-1994 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP) Supplement;

### 12.4.3	ANSI T1.112-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Signaling Connection Control Part (SCCP);

### 12.4.4	ANSI T1.115-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Monitoring and Measurements for Networks;

### 12.4.5	ANSI T1.116-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Operations, Maintenance and Administration Part (OMAP);

### 12.4.6	ANSI T1.118-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Intermediate Signaling Network Identification (ISNI);

### 12.4.7	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP); and

### 12.4.8	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

# 13.	Service Control Points/DataBases

## 13.1	Definition

### 13.1.1	Databases are the Network Elements that provide the functionality for storage of, access to, and manipulation of information required to offer a particular service and/or capability.  Databases include, but are not limited to:  Local Number Portability, LIDB, Toll Free Number Database, Automatic Location Identification/Data Management System, Calling Name Database, access to Service Creation Environment and Service Management System (SCE/SMS) application databases and Directory Assistance.

### 13.1.2	A Service Control Point (SCP) is a specific type of Database functionality deployed in a Signaling System 7 (SS7) network that executes service application logic in response to SS7 queries sent to it by a switching system also connected to the SS7 network.  Service Management Systems provide operational interfaces to allow for provisioning, administration and maintenance of subscriber data and service application data stored in SCPs.

## 13.2	Technical Requirements for SCPs/Databases

Requirements for SCPs/Databases within this section address storage of information, access to information (e.g. signaling protocols, response times), and administration of information (e.g., provisioning, administration, and maintenance).  All SCPs/Databases shall be provided to PaeTec in accordance with the following requirements.

### 13.2.1	BellSouth shall provide physical access to SCPs through the SS7 network and protocols with TCAP as the application layer protocol.

### 13.2.2	BellSouth shall provide physical interconnection to databases via industry standard interfaces and protocols (e.g. SS7, ISDN and X.25).

### 13.2.3	The reliability of interconnection options shall be consistent with requirements for diversity and survivability.

### 13.2.4	Database Availability

Call processing databases shall have a maximum unscheduled availability of 30 minutes per year.  Unavailability due to software and hardware upgrades shall be scheduled during minimal usage periods and only be undertaken upon proper notification to providers which might be impacted.  Any downtime associated with the provision of call processing related databases will impact all service providers, including BellSouth, equally.

### 13.2.5	The operational interface provided by BellSouth shall complete Database transactions (i.e., add, modify, delete) for PaeTec customer records stored in BellSouth databases within 3 days, or sooner where BellSouth provisions its own customer records within a shorter interval.

## 13.3	Local Number Portability Database

### 13.3.1	Definition

The Permanent Number Portability (PNP) database supplies routing numbers for calls involving numbers that have been ported from one local service provider to another.  PNP is currently being worked in industry forums. The results of these forums will dictate the industry direction of PNP.  BellSouth agrees to provide access to the PNP database at rates, terms and conditions as set forth by BellSouth and in accordance with an effective FCC or Commission directive.

## 13.4	Line Information Database (LIDB)

BellSouth will store in its LIDB only records relating to service in the BellSouth region.   The LIDB Storage Agreement is included in this Attachment.

### 13.4.1	Definition

The Line Information Database (LIDB) is a transaction-oriented database accessible through Common Channel Signaling (CCS) networks.  It contains records associated with end user Line Numbers and Special Billing Numbers.  LIDB accepts queries from other Network Elements and provides appropriate responses.  The query originator need not be the owner of LIDB data.  LIDB queries include functions such as screening billed numbers that provides the ability to accept Collect or Third Number Billing calls and validation of Telephone Line Number based non-proprietary calling cards.  The interface for the LIDB functionality is the interface between BellSouth CCS network and other CCS networks.  LIDB also interfaces to administrative systems.

### 13.4.2	Technical Requirements

BellSouth will offer to PaeTec any additional capabilities that are developed for LIDB during  the life of this Agreement.

#### 13.4.2.1	BellSouth shall process PaeTec’s Customer records in LIDB at least at parity with BellSouth customer records, with respect to other LIDB functions.  BellSouth shall indicate to PaeTec what additional functions (if any) are performed by LIDB in the BellSouth network.

#### 13.4.2.2	Within two (2) weeks after a request by PaeTec, BellSouth shall provide PaeTec with a list of the customer data items which PaeTec would have to provide in order to support each required LIDB function.  The list shall indicate which data items are essential to LIDB function, and which are required only to support certain services.  For each data item, the list shall show the data formats, the acceptable values of the data item and the meaning of those values.

#### 13.4.2.3	BellSouth shall provide LIDB systems for which operating deficiencies that would result in calls being blocked, shall not exceed 30 minutes per year.

#### 13.4.2.4	BellSouth shall provide LIDB systems for which operating deficiencies that would not result in calls being blocked shall not exceed 12 hours per year.

#### 13.4.2.5	BellSouth shall provide LIDB systems for which the LIDB function shall be in overload no more than 12 hours per year.

#### 13.4.2.6	All additions, updates and deletions of PaeTec data to the LIDB shall be solely at the direction of PaeTec.  Such direction from PaeTec will not be required where the addition, update or deletion is necessary to perform standard fraud control measures (e.g., calling card auto-deactivation).

#### 13.4.2.7	BellSouth shall provide priority updates to LIDB for PaeTec data upon PaeTec’s request (e.g., to support fraud detection), via password-protected telephone card, facsimile, or electronic mail within one hour of notice from the established BellSouth contact.

#### 13.4.2.8	BellSouth shall provide LIDB systems such that no more than 0.01% of PaeTec customer records will be missing from LIDB, as measured by PaeTec audits.  BellSouth will audit PaeTec records in LIDB against DBAS to identify record mismatches and provide this data to a designated PaeTec contact person to resolve the status of the records and BellSouth will update system appropriately.  BellSouth will refer record of mis-matches to PaeTec within one business day of audit.  Once reconciled records are received back from PaeTec, BellSouth will update LIDB the same business day if less than 500 records are received before 1:00PM Central Time.  If more than 500 records are received, BellSouth will contact PaeTec to negotiate a time frame for the updates, not to exceed three business days.

#### 13.4.2.9	BellSouth shall perform backup and recovery of all of PaeTec’s data in LIDB including sending to LIDB all changes made since the date of the most recent backup copy, in at least the same time frame BellSouth performs backup and recovery of BellSouth data in LIDB for itself.  Currently, BellSouth performs backups of the LIDB for itself on a weekly basis and when a new software release is scheduled, a backup is performed prior to loading the new release.

#### 13.4.2.10	BellSouth shall provide PaeTec with LIDB reports of data which are missing or contain errors, as well as any misrouted errors, within a reasonable time period as negotiated between PaeTec and BellSouth.

#### 13.4.2.11	BellSouth shall prevent any access to or use of PaeTec data in LIDB by BellSouth personnel that are outside of established administrative and fraud control personnel, or by any other Party that is not authorized by PaeTec in writing.

#### 13.4.2.12	BellSouth shall provide PaeTec performance of the LIDB Data Screening function, which allows a LIDB to completely or partially deny specific query originators access to LIDB data owned by specific data owners, for Customer Data that is part of an NPA-NXX or RAO-0/1XX wholly or partially owned by PaeTec at least at parity with BellSouth Customer Data. BellSouth shall obtain from PaeTec the screening information associated with LIDB Data Screening of PaeTec data in accordance with this requirement.  BellSouth currently does not have LIDB Data Screening capabilities.  When such capability is available, BellSouth shall offer it to PaeTec under the Bona Fide Request/New Business Process as set forth in General Terms and Conditions .

#### 13.4.2.13	BellSouth shall accept queries to LIDB associated with PaeTec customer records, and shall return responses in accordance with industry standards.

#### 13.4.2.14	BellSouth shall provide mean processing time at the LIDB within 0.50 seconds under normal conditions as defined in industry standards.

#### 13.4.2.15	BellSouth shall provide processing time at the LIDB within 1 second for 99% of all messages under normal conditions as defined in industry standards.

### 13.4.3	Interface Requirements

BellSouth shall offer LIDB in accordance with the requirements of this subsection.

#### 13.4.3.1	The interface to LIDB shall be in accordance with the technical references contained within.

#### 13.4.3.2	The CCS interface to LIDB shall be the standard interface described herein.

#### 13.4.3.3	The LIDB Data Base interpretation of the ANSI-TCAP messages shall comply with the technical reference herein.  Global Title Translation shall be maintained in the signaling network in order to support signaling network routing to the LIDB.

## 13.5	Toll Free Number Database

The Toll Free Number Database is a SCP that provides functionality necessary for toll free (e.g., 800 and 888) number services by providing routing information and additional so-called vertical features during call set-up in response to queries from SSPs. BellSouth shall provide the Toll Free Number Database in accordance with the following:

### 13.5.1	Technical Requirements

#### 13.5.1.1	BellSouth shall make BellSouth Toll Free Number Database available for PaeTec to query with a toll-free number and originating information.

#### 13.5.1.2	The Toll Free Number Database shall return carrier identification and, where applicable, the queried toll free number, translated numbers and instructions as it would in response to a query from a BellSouth switch.

#### 13.5.1.3	The SCP shall also provide, at PaeTec's option, such additional feature as described in SR-TSV-002275 (BOC Notes on BellSouth Networks, SR-TSV-002275, Issue 2, (Telcordia (formerly BellCore), April 1994)) as are available to BellSouth.  These may include but are not limited to:

##### 13.5.1.3.1	Network Management;

##### 13.5.1.3.2	Customer Sample Collection; and

##### 13.5.1.3.3	Service Maintenance

## 13.6	Automatic Location Identification/Data Management System (ALI/DMS)

The ALI/DMS Database contains end user information (including name, address, telephone information, and sometimes special information from the local service provider or end user) used to determine to which Public Safety Answering Point  (PSAP) to route the call.  The ALI/DMS database is used to provide more routing flexibility for E911 calls than Basic 911.  BellSouth shall provide the Emergency Services Database in accordance with the following:

### 13.6.1	Technical Requirements

#### 13.6.1.1	BellSouth shall offer PaeTec a data link to the ALI/DMS database or permit PaeTec to provide its own data link to the ALI/DMS database. BellSouth shall provide error reports from the ALI/DMS database to PaeTec immediately after PaeTec inputs information into the ALI/DMS database.  Alternately, PaeTec may utilize BellSouth, to enter end user information into the data base on a demand basis, and validate end user information on a demand basis.

#### 13.6.1.2	The ALI/DMS database shall contain the following end user information:

##### 13.6.1.2.1	Name;

##### 13.6.1.2.2	Address;

##### 13.6.1.2.3	Telephone number; and

##### 13.6.1.2.4	Other information as appropriate (e.g., whether a end user is blind or deaf or has another disability).

#### 13.6.1.3	When BellSouth is responsible for administering the ALI/DMS database in its entirety, ported number NXXs entries for the ported numbers should be maintained unless PaeTec requests otherwise and shall be updated if PaeTec requests, provided PaeTec supplies BellSouth with the updates.

#### 13.6.1.4	When Remote Call Forwarding (RCF) is used to provide number portability to the local end user and a remark or other appropriate field information is available in the database, the shadow or “forwarded-to” number and an indication that the number is ported shall be added to the customer record.

#### 13.6.1.5	If BellSouth is responsible for configuring PSAP features (for cases when the PSAP or BellSouth supports an ISDN interface) it shall ensure that CLASS Automatic Recall (Call Return) is not used to call back to the ported number.  Although BellSouth currently does not have ISDN interface, BellSouth agrees to comply with this requirement once ISDN interfaces are in place.

### 13.6.2	Interface Requirements

The interface between the E911 Switch or Tandem and the ALI/DMS database for PaeTec end users shall meet industry standards.

# 13.7	Calling Name (CNAM) Database Service.  The Agreement for Calling Name (CNAM) with standard pricing is included as Exhibit B to this Attachment.  PaeTec must provide to its account manager a written request with a requested activation date to activate this service.  If PaeTec is interested in requesting CNAM with volume and term pricing, PaeTec must contact its account manager to request a separate CNAM volume and term Agreement.

## 13.8	SCPs/Databases shall be equal to or better than all of the requirements for SCPs/Databases set forth in the following technical references:

### 13.8.1	GR-246-CORE, Bell Communications Research Specification of  Signaling System Number 7, ISSUE 1 (Telcordia (formerly BellCore), December 199);

### 13.8.2	GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP). (Telcordia (formerly BellCore), March 1994);

### 13.8.3	GR-954-CORE, CCS Network Interface Specification (CCSNIS) Supporting Line Information Database (LIDB) Service 6, Issue 1, Rev. 1 (Telcordia (formerly BellCore), October 1995);

### 13.8.4	GR-1149-CORE, OSSGR Section 10: System Interfaces, Issue 1 (Telcordia (formerly BellCore), October 1995)  (Replaces TR-NWT-001149);

### 13.8.5	Telcordia (formerly BellCore) GR-1158-CORE, OSSGR Section 22.3: Line Information Database 6, Issue (Telcordia (formerly BellCore), October 1995);

### 13.8.6	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service (Telcordia (formerly BellCore), May 1995); and

### 13.8.7	BOC Notes on BellSouth Networks, SR-TSV-002275, ISSUE 2, (Telcordia (formerly BellCore), April 1994).

## 13.9	Service Creation Environment and Service Management System (SCE/SMS) Advanced Intelligent Network (AIN) Access.

### 13.9.1	BellSouth’s Service Creation Environment and Service Management System (SCE/SMS) Advanced Intelligent Network (AIN) Access shall provide PaeTec the capability that will allow PaeTec and other third parties to create service applications in a BellSouth Service Creation Environment and deploy those applications in a BellSouth SMS to a BellSouth SCP.  The third party service applications interact with AIN triggers provisioned on a BellSouth SSP.

### 13.9.2	BellSouth’s SCE/SMS AIN Access shall provide access to SCE hardware, software, testing and technical support (e.g., help desk, system administrator) resources available to PaeTec.  Scheduling procedures shall provide PaeTec equivalent priority to these resources

### 13.9.3	BellSouth SCP shall partition and protect PaeTec service logic and data from unauthorized access, execution or other types of compromise.

### 13.9.4	When PaeTec selects SCE/SMS AIN Access, BellSouth shall provide training, documentation, and technical support to enable PaeTec to use BellSouth’s SCE/SMS AIN Access to create and administer applications.  Training, documentation, and technical support will address use of SCE and SMS access and administrative functions, but will not include support for the creation of a specific service application.

### 13.9.5	When PaeTec selects SCE/SMS AIN Access, BellSouth shall provide for a secure, controlled access environment in association with its internal use of AIN components.  PaeTec access will be provided via remote data connection (e.g., dial-in, ISDN).

### 13.9.6	When PaeTec selects SCE/SMS AIN Access, BellSouth shall allow PaeTec to download data forms and/or tables to BellSouth SCP via BellSouth SMS without intervention from BellSouth (e.g., service customization and end user subscription).

# 14.	DARK FIBER

BellSouth agrees to offer access to Dark Fiber pursuant to the terms and conditions following and at the rates set forth in this Attachment.

### 14.1.1	Dark Fiber is defined as BellSouth’s optical transmission facilities without attached multiplexing, aggregation or other electronics.  PaeTec may utilize Dark Fiber for either loops or transport. Dark Fiber does not have electronics on either end of the Dark Fiber segment.   It may be strands of optical fiber existing in aerial or underground structure. No regeneration or optical amplification will be included with this element.

## 14.2	Requirements

### 14.2.1	BellSouth shall make available Dark Fiber where it exists in BellSouth’s network and where, as a result of future building or deployment, it becomes available.  BellSouth shall offer all Dark Fiber to PaeTec pursuant to the prices set forth in this Attachment.

### 14.2.2	PaeTec may test the quality of the Dark Fiber to confirm its usability and performance specifications.

### 14.2.3	BellSouth shall use its best efforts to provide to PaeTec information regarding the location, availability and performance of Dark Fiber within ten (10) business days for a records based answer and twenty (20) business days for a field based answer, after receiving a request from PaeTec ("Request").  Within such time period, BellSouth shall send written confirmation of availability of the Dark Fiber ("Confirmation").

### 14.2.4	BellSouth shall use its best efforts to make Dark Fiber available to PaeTec within thirty (30) business days after it receives written confirmation from PaeTec that the Dark Fiber previously deemed available by BellSouth is wanted for use by PaeTec.  This includes identification of appropriate connection points (e.g., Light Guide Interconnection (LGX) or splice points) to enable PaeTec to connect or splice PaeTec provided transmission media (e.g., optical fiber) or equipment to the Dark Fiber.

# 15.	SS7 Network Interconnection

### 15.1.1	Definition

SS7 Network Interconnection is the interconnection of PaeTec local Signaling Transfer Point Switches (STP) and PaeTec local or tandem switching systems with BellSouth STPs.  This interconnection provides connectivity that enables the exchange of SS7 messages among BellSouth switching systems and databases (DBs), PaeTec local or tandem switching systems, and other third-party switching systems directly connected to the BellSouth SS7 network.

### 15.1.2	Technical Requirements

#### 15.1.2.1	SS7 Network Interconnection shall provide connectivity to all components of the BellSouth SS7 network.  These include:

##### 15.1.2.1.1	BellSouth local or tandem switching systems;

##### 15.1.2.1.2	BellSouth DBs; and

##### 15.1.2.1.3	Other third-party local or tandem switching systems.

#### 15.1.2.2	The connectivity provided by SS7 Network Interconnection shall fully support the functions of BellSouth switching systems and DBs and PaeTec or other third-party switching systems with A-link access to the BellSouth SS7 network.

If traffic is routed based on dialed or translated digits between an PaeTec local switching system and a BellSouth or other third-party local switching system, either directly or via a BellSouth tandem switching system, then it is a requirement that the BellSouth SS7 network convey via SS7 Network Interconnection the TCAP messages that are necessary to provide Call Management services (Automatic Callback, Automatic Recall, and Screening List Editing) between the PaeTec local STPs and BellSouth or other third-party local switch.

#### 15.1.2.3	When the capability to route messages based on Intermediate Signaling Network Identifier (ISNI) is generally available on BellSouth STPs,  the BellSouth SS7 Network shall also convey TCAP messages using SS7 Network Interconnection in similar circumstances where the BellSouth switch routes traffic based on a Carrier Identification Code (CIC).

#### 15.1.2.4	SS7 Network Interconnection shall provide all functions of the MTP as specified in ANSI T1.111. This includes:

##### 15.1.2.4.1	Signaling Data Link functions, as specified in ANSI T1.111.2;

##### 15.1.2.4.2	Signaling Link functions, as specified in ANSI T1.111.3; and

##### 15.1.2.4.3	Signaling Network Management functions, as specified in ANSI T1.111.4.

#### 15.1.2.5	SS7 Network Interconnection shall provide all functions of the SCCP necessary for Class 0 (basic connectionless) service, as specified in ANSI T1.112.  In particular, this includes Global Title Translation (GTT) and SCCP Management procedures, as specified in T1.112.4.  Where the destination signaling point is a BellSouth switching system or DB, or is another third-party local or tandem switching system directly connected to the BellSouth SS7 network, SS7 Network Interconnection shall include final GTT of messages to the destination and SCCP Subsystem Management of the destination.  Where the destination signaling point is an PaeTec local or tandem switching system, SS7 Network Interconnection shall include intermediate GTT of messages to a gateway pair of PaeTec local STPs, and shall not include SCCP Subsystem Management of the destination.

#### 15.1.2.6	SS7 Network Interconnection shall provide all functions of the Integrated Services Digital Network User Part (ISDNUP), as specified in ANSI T1.113.

#### 15.1.2.7	SS7 Network Interconnection shall provide all functions of the TCAP, as specified in ANSI T1.114.

#### 15.1.2.8	If and when Internetwork MTP Routing Verification Test (MRVT) and SCCP Routing Verification Test (SRVT) become approved ANSI standards and available capabilities of BellSouth STPs, SS7 Network Interconnection shall provide these functions of the OMAP.

#### 15.1.2.9	SS7 Network Interconnection shall be equal to or better than the following performance requirements:

##### 15.1.2.9.1	MTP Performance, as specified in ANSI T1.111.6;

##### 15.1.2.9.2	SCCP Performance, as specified in ANSI T1.112.5; and

##### 15.1.2.9.3	ISDNUP Performance, as specified in ANSI T1.113.5.

### 15.1.3	Interface Requirements

#### 15.1.3.1	BellSouth shall offer the following SS7 Network Interconnection options to connect PaeTec or PaeTec-designated local or tandem switching systems or STPs to the BellSouth SS7 network:

##### 15.1.3.1.1	A-link interface from PaeTec local or tandem switching systems; and

##### 15.1.3.1.2	B-link interface from PaeTec STPs.

#### 15.1.3.2	The Signaling Point of Interconnection (SPOI) for each link shall be located at a cross-connect element, such as a DSX-1, in the Central Office (CO) where the BellSouth STP is located.  There shall be a DS1 or higher rate transport interface at each of the SPOIs.  Each signaling link shall appear as a DS0 channel within the DS1 or higher rate interface.  BellSouth shall offer higher rate DS1 signaling links for interconnecting PaeTec local switching systems or STPs with BellSouth STPs as soon as these become approved ANSI standards and available capabilities of BellSouth STPs.  BellSouth and PaeTec will work jointly to establish mutually acceptable SPOI.

#### 15.1.3.3	BellSouth CO shall provide intraoffice diversity between the SPOIs and the BellSouth STP, so that no single failure of intraoffice facilities or equipment shall cause the failure of both B-links in a layer connecting to a BellSouth STP. BellSouth and PaeTec will work jointly to establish mutually acceptable SPOI.

#### 15.1.3.4	The protocol interface requirements for SS7 Network Interconnection include the MTP, ISDNUP, SCCP, and TCAP. These protocol interfaces shall conform to the following specifications:

##### 15.1.3.4.1	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

##### 15.1.3.4.2	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service;

##### 15.1.3.4.3	Telcordia (formerly BellCore) GR-1429-CORE, CCS Network Interface Specification (CCSNIS) Supporting Call Management Services; and

##### 15.1.3.4.4	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

#### 15.1.3.5	BellSouth shall set message screening parameters to block accept messages from PaeTec local or tandem switching systems destined to any signaling point in the BellSouth SS7 network with which the PaeTec switching system has a legitimate signaling relation.

### 15.1.4	SS7 Network Interconnection shall be equal to or better than all of the requirements for SS7 Network Interconnection set forth in the following technical references:

#### 15.1.4.1	ANSI T1.110-1992 American National Standard Telecommunications - Signaling System Number 7 (SS7) - General Information;

#### 15.1.4.2	ANSI T1.111-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP);

#### 15.1.4.3	ANSI T1.111A-1994 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Message Transfer Part (MTP) Supplement;

#### 15.1.4.4	ANSI T1.112-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Signaling Connection Control Part (SCCP);

#### 15.1.4.5	ANSI T1.113-1995 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Integrated Services Digital Network (ISDN) User Part;

#### 15.1.4.6	ANSI T1.114-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Transaction Capabilities Application Part (TCAP);

#### 15.1.4.7	ANSI T1.115-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Monitoring and Measurements for Networks;

#### 15.1.4.8	ANSI T1.116-1990 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Operations, Maintenance and Administration Part (OMAP);

#### 15.1.4.9	ANSI T1.118-1992 American National Standard for Telecommunications - Signaling System Number 7 (SS7) - Intermediate Signaling Network Identification (ISNI);

#### 15.1.4.10	Telcordia (formerly BellCore) GR-905-CORE, Common Channel Signaling Network Interface Specification (CCSNIS) Supporting Network Interconnection, Message Transfer Part (MTP), and Integrated Services Digital Network User Part (ISDNUP);

#### 15.1.4.11	Telcordia (formerly BellCore) GR-954-CORE, CCS Network Interface Specification (CCSNIS) Supporting Line Information Database (LIDB) Service;

#### 15.1.4.12	Telcordia (formerly BellCore) GR-1428-CORE, CCS Network Interface Specification (CCSNIS) Supporting Toll Free Service;

#### 15.1.4.13	Telcordia (formerly BellCore) GR-1429-CORE, CCS Network Interface Specification (CCSNIS) Supporting Call Management Services; and,

#### 15.1.4.14	Telcordia (formerly BellCore) GR-1432-CORE, CCS Network Interface Specification (CCSNIS) Supporting Signaling Connection Control Part (SCCP) and Transaction Capabilities Application Part (TCAP).

# 16.	Basic 911 and E911

If PaeTec orders network elements and other services, then PaeTec is also responsible for providing E911 to its end users.  BellSouth agrees to offer access to the 911/E911 network pursuant to the following terms and conditions set forth in this Attachment.

## 16.1	Definition

Basic 911 and E911 is an additional requirement that provides a caller access to the applicable emergency service bureau by dialing a 3-digit universal telephone number (911).

## 16.2	Requirements

### 16.2.1	Basic 911 Service Provisioning.  For Basic 911 service, BellSouth will provide to PaeTec a list consisting of each municipality that subscribes to Basic 911 service.  The list will also provide, if known, the E911 conversion date for each municipality and, for network routing purposes, a ten-digit directory number representing the appropriate emergency answering position for each municipality subscribing to 911.  PaeTec will be required to arrange to accept 911 calls from its end users in municipalities that subscribe to Basic 911 service and translate the 911 call to the appropriate 10-digit directory number as stated on the list provided by BellSouth.  PaeTec will be required to route that call to BellSouth at the appropriate tandem or end office.  When a municipality converts to E911 service, PaeTec will be required to discontinue the Basic 911 procedures and being using E911 procedures.

### 16.2.2	E911 Service Provisioning.  For E911 service, PaeTec will be required to install a minimum of two dedicated trunks originating from the PaeTec serving wire center and terminating to the appropriate E911 tandem.  The dedicated trunks shall be, at a minimum, DS-0 level trunks configured either as a 2-wire analog interface or as part of a digital (1.544 Mb/s) interface.  Either configuration shall use CAMA-type signaling with multifrequency (“MF”) pulsing that will deliver automatic number identification (“ANI”) with the voice portion of the call.  If the user interface is digital, MF pulses, as well as other AC signals, shall be encoded per the u-255 Law convention.  PaeTec will be required to provide BellSouth daily updates to the E911 database.  PaeTec will be required to forward 911 calls to the appropriate E911 tandem, along with ANI, based upon the current E911 end office to tandem homing arrangement as provided by BellSouth.  If the E911 tandem trunks are not available, PaeTec will be required to route the call to a designated 7-digit local number residing in the appropriate Public Service Answering Point (“PSAP”).  This call will be transported over BellSouth’s interoffice network and will not carry the ANI of the calling party.  PaeTec shall be responsible for providing BellSouth with complete and accurate data for submission to the 911/E911 database for the purpose of providing 911/E911 to its end users.

### 16.2.3	Rates.  Charges for 911/E911 service are borne by the municipality purchasing the service.  BellSouth will impose no charge on PaeTec beyond applicable charges for BellSouth trunking arrangements.

### 16.2.4	Basic 911 and E911 functions provided to PaeTec shall be at least at parity with the support and services that BellSouth provides to its end users for such similar functionality.

### Detailed Practices and Procedures.  The detailed practices and procedures contained in the E911 Local Exchange Carrier Guide For Facility-Based Providers as amended from time to time during the term of this Agreement will determine the appropriate practices and procedures for BellSouth and PaeTec to follow in providing 911/E911 services.

# 17.	Combinations

17.1	For purposes of this Section, references to “Already Combined” network elements shall mean that such network elements are in fact already combined by BellSouth in the BellSouth network to provide service to a particular end user at a particular location.

17.2	EELs

17.2.1	Where necessary to comply with an effective FCC and/or State Commission order, or as otherwise mutually agreed by the Parties, BellSouth shall offer access to loop and transport combinations, also known as the Enhanced Extended Link (“EEL”) as defined in Section 17.3 below.

17.2.2	Subject to Section 17.2.3 below, BellSouth will provide access to the EEL in the  combinations set forth in 17.3 following.  This offering is intended to provide connectivity from an end user’s location through that end user’s SWC to PaeTec’s POP serving wire center.  The circuit must be connected to PaeTec’s switch for the purpose of provisioning telecommunications services, including telephone exchange service, to PaeTec’s end-user customers.  Except as provided for in paragraph 22 of the FCC’s Supplemental Order Clarification, released June 2, 2000, in CC Docket No. 96-98 (“June 2, 2000 Order”), the EEL will be connected to PaeTec’s facilities in PaeTec’s collocation space at the POP SWC.  PaeTec may purchase BellSouth’s access facilities between PaeTec’s POP and PaeTec’s collocation space at the POP SWC.

17.2.3	BellSouth shall provide EEL combinations to PaeTec in the state of Georgia regardless of whether or not such EELs are Already Combined. In all other states, BellSouth shall make available to PaeTec those EEL combinations described in Section 4.3 below only to the extent such combinations are Already Combined.

17.2.4	BellSouth will make available EEL combinations to PaeTec in density Zone 1, as defined in 47 C.F.R. 69.123 as of January 1, 1999, in the  Miami, Orlando, Fort Lauderdale, Charlotte, New Orleans, Greensboro and Nashville MSAs, regardless of whether or not such EELs are Already Combined.

17.2.5	Additionally, BellSouth shall make available to PaeTec a combination of an unbundled loop and tariffed special access interoffice facilities.  To the extent PaeTec will require multiplexing functionality in connection with such combination, BellSouth will provide access to multiplexing within the central office pursuant to the terms, conditions and rates set forth in its Access Services Tariffs. The combination of an unbundled loop and tariffed special access interoffice facilities and any associated tariffed services, including but not limited to multiplexing, shall not be eligible for conversion to UNEs as described in Section 17.5 below.  Where multiplexing functionality is required in connection with loop and transport combinations, such multiplexing will be provided at the rates and on the terms set forth in this Agreement.

17.3	EEL Combinations

17.3.1	DS1 Interoffice Channel + DS1 Channelization + 2-wire VG Local Loop

17.3.2	DS1 Interoffice Channel + DS1 Channelization + 4-wire VG Local Loop

17.3.3	DS1 Interoffice Channel + DS1 Channelization + 2-wire ISDN Local Loop

17.3.4	DS1 Interoffice Channel + DS1 Channelization + 4-wire 56 kbps Local Loop

17.3.5	DS1 Interoffice Channel + DS1 Channelization + 4-wire 64 kbps Local Loop

17.3.6	DS1 Interoffice Channel + DS1 Local Loop

17.3.7	DS3 Interoffice Channel + DS3 Local Loop

17.3.8	STS-1 Interoffice Channel + STS-1 Local Loop

17.3.9	DS3 Interoffice Channel + DS3 Channelization + DS1 Local Loop

17.3.10	STS-1 Interoffice Channel + DS3 Channelization + DS1 Local Loop

17.3.11	2-wire VG Interoffice Channel + 2-wire VG Local Loop

17.3.12	4wire VG Interoffice Channel + 4-wire VG Local Loop

17.3.13	4-wire 56 kbps Interoffice Channel + 4-wire 56 kbps Local Loop

17.3.14	4-wire 64 kbps Interoffice Channel + 4-wire 64 kbps Local Loop

17.4	Other Network Element Combinations

In the state of Georgia, BellSouth shall make available to PaeTec, in accordance with Section 17.6 below: (1) combinations of network elements other than EELs that are Already Combined; and (2) combinations of network elements other than EELs that are not Already Combined but that BellSouth ordinarily combines in its network.  In all other states, BellSouth shall make available to PaeTec, in accordance with Section 17.6 below, combinations of network elements other than EELs only to the extent such combinations are Already Combined.

17.5	Special Access Service Conversions

17.5.1	PaeTec may not convert special access services to combinations of loop and transport network elements, whether or not PaeTec self-provides its entrance facilities (or obtains entrance facilities from a third party), unless PaeTec uses the combination to provide a “significant amount of local exchange service” (as described in Section 17.5.2 below), in addition to exchange access service, to a particular customer.

17.5.2	For the purpose of special access conversions, a “significant amount of local exchange service” is as defined in the FCC’s June 2, 2000 Order.  The Parties agree to incorporate by reference paragraph 22 of the June 2, 2000 Order.  When  PaeTec requests conversion of special access circuits, PaeTec will self-certify to BellSouth in the manner specified in paragraph 29 of the June 2, 2000 Order that the circuits to be converted qualify for conversion.  In addition there may be extraordinary circumstances where  PaeTec is providing a significant amount of local exchange service, but does not qualify under any of the three options set forth in paragraph 22 of June 2, 2000 Order.  In such case,  PaeTec may petition the FCC for a waiver of the local usage options set forth in the June 2, 2000 Order.  If a waiver is granted, then upon PaeTec’s request the Parties shall amend this Agreement to the extent necessary to incorporate the terms of such waiver for such extraordinary circumstance.

17.5.3 	Upon request for conversions of up to 15 circuits from special access to EELs, BellSouth shall perform such conversions within seven (7) days from BellSouth’s receipt of a valid, error free service order from PaeTec.  Requests for conversions of fifteen (15) or more circuits from special access to EELs will be provisioned on a project basis.  Conversions should not require the special access circuit to be disconnected and reconnected because only the billing information or other administrative information associated with the circuit will change when PaeTec requests a conversion.  The Access Service Request process will be used for conversion requests.

17.5.4	BellSouth may, at its sole expense, and upon thirty (30) days notice to PaeTec, audit PaeTec’s records not more than once in any twelve month period, unless an audit finds non-compliance with the local usage options referenced in the June 2, 2000 Order, in order to verify the type of traffic being transmitted over combinations of loop and transport network elements.  If, based on its audits, BellSouth concludes that PaeTec is not providing a significant amount of local exchange traffic over the combinations of loop and transport network elements, BellSouth may file a complaint with the appropriate Commission, pursuant to the dispute resolution process set forth in this Agreement.  In the event that BellSouth prevails, BellSouth may convert such combinations of loop and transport network elements to special access services and may seek appropriate retroactive reimbursement from PaeTec.

17.6	Rates

17.6.1	Georgia

17.6.1.1	The non-recurring and recurring rates for the EEL Combinations of network elements set forth in 17.3, whether Already Combined or new, are as set forth in this Attachment.

17.6.1.2	On an interim basis, for combinations of loop and transport network elements not set forth in Section 17.3, where the elements are not Already Combined but are ordinarily combined in BellSouth’s network, the non-recurring and recurring charges for such UNE combinations shall be the sum of the stand-alone non-recurring and recurring charges of the network elements which make up the combination.  These interim rates shall be subject to true-up based on the Commission’s review of BellSouth’s cost studies.

17.6.1.3	To the extent that PaeTec seeks to obtain other combinations of network elements that BellSouth ordinarily combines in its network which have not been specifically priced by the Commission when purchased in combined form, PaeTec, at its option, can request that such rates be determined pursuant to the Bona Fide Request/New Business Request (NBR) process set forth in this Agreement.

## 17.6.2	All Other States

## 17.6.2.1	Subject to Section 17.2.3 and 17.4 preceding, all other states, the  rates for (1) Already Combined EEL combinations set forth in Section 17.3, and (2) other combinations of network elements that are Already Combined in the network will be the sum of the recurring rates for the individual network elements plus a nonrecurring charge as specified in Exhibit A hereto.

## 17.6.2.2	Rates for new EEL combinations in Density Zone 1 in the Miami, Orlando, Fort Lauderdale, Charlotte, New Orleans, Greensboro and Nashville MSAs shall be as set forth in Exhibit A hereto; provided, however, that to the extent a rate is not established in Exhibit A, the rate shall be the sum of the recurring and nonrecurring charges for the individual network elements as set forth in Exhibit A to this Attachment, unless otherwise established by the Commission.

# 17.7	Port/Loop Combinations

## 17.7.1	At PaeTec’s request, BellSouth shall provide access to combinations of port and loop network elements, as set forth in Section 17.7.4 below, that are Already Combined in BellSouth’s network except as specified in Sections 17.7.1.1 and 17.7.1.2 below, consistent with the requirements of 47 C.F.R. 315(b) and all applicable FCC and Commission rules and policies.

## 17.7.1.1	BellSouth shall not provide access to combinations of unbundled port and loop network elements in locations where, pursuant to FCC rules,  BellSouth is not required to provide circuit switching as an unbundled network element.

## 17.7.1.2	In accordance with effective and applicable FCC rules, BellSouth shall not provide unbundled circuit switching in density Zone 1, as defined in 47 C.F.R. 69.123 as of January 1, 1999, of the Atlanta, Miami, Orlando, Fort Lauderdale, Charlotte, New Orleans, Greensboro and Nashville MSAs to PaeTec if PaeTec’s customer has 4 or more DS0 equivalent lines.

## 17.7.2 	In Georgia, BellSouth shall provide combinations of port and loop network elements to PaeTec regardless of whether or not such combinations are Already Combined except in those locations where BellSouth is not required to provide unbundled circuit switching, as set forth in Section 17.7.1.1 above. In all other states, and subject to Sections 17.7.1.1 and 17.7.1.2 above, BellSouth shall provide combinations of port and loop network elements to PaeTec only to the extent such elements are Already Combined.

## 17.7.3	Rates for Combinations of Loop and Port Network Elements

## 17.7.3.1	Rates for combinations of loop and port network elements, as set forth in Section 17.7.4, are provided in Exhibit A of this Attachment.

## 17.7.3.2	Rates for Circuit Switching

## 17.7.3.2.1	Rates for circuit switching, where BellSouth is not required, pursuant to Sections 17.7.1.1 and 17.7.1.2, to provide circuit switching are as set forth in Exhibit A of this Attachment.

## 17.7.4	Combination Offerings

17.7.4.1	2-wire voice grade port, voice grade loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

17.7.4.2	2-wire voice grade DID port, voice grade loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

17.7.4.3	2-wire CENTREX port, voice grade loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

17.7.4.4.	2-wire ISDN Basic Rate Interface, voice grade loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

17.7.4.5	2-wire ISDN Primary Rate Interface, DS1 loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

17.7.4.6	4-wire DS1 Trunk port, DS1 Loop, unbundled end office switching, unbundled end office trunk port, common transport per mile per MOU, common transport facilities termination, tandem switching, and tandem trunk port.

18.		Rates

# 18.1.	General

The prices that PaeTec shall pay to BellSouth for  Network Elements and Other Services are set forth in Exhibit C to this Attachment.

# 18.2.	Operations Support Systems (“OSS”)

BellSouth shall provide nondiscriminatory access in accordance with FCC Rule 51.311 and Section 251(c)(3) of the Act to OSS on an unbundled basis to PaeTec for the provision of a telecommunications service. OSS functions consist of pre-ordering, ordering, provisioning, maintenance and repair, and billing functions supported by BellSouth’s databases and information.  Subject to applicable and effective FCC rules and orders, BellSouth, as part of its duty to provide access to the pre-ordering function, shall provide PaeTec with nondiscriminatory access to the same detailed information about the loop that is available to its own retail unit.  The rate associated with BellSouth’s provision of loop qualification information shall be as set forth in this Attachment.

All Local Service Requests (“LSRs”) submitted for products and services under this Attachment will be subject to the OSS charges set forth in the General Terms and Conditions of this Agreement.

18.3 	True-up

This section applies only to Tennessee.

The interim prices for Network Elements and Other Services and Local Interconnection shall be subject to true-up according to the following procedures:

1.	The interim prices shall be trued-up, either up or down, based on final prices determined either by further agreement between the Parties, or by a final order (including any appeals) of the Commission which final order meets the criteria of (3) below.  The Parties shall implement the true-up by comparing the actual volumes and demand for each item, together with interim prices for each item, with the final prices determined for each item.  Each Party shall keep its own records upon which the true-up can be based, and any final payment from one Party to the other shall be in an amount agreed upon by the Parties based on such records.  In the event of any disagreement as between the records or the Parties regarding the amount of such true-up, the Parties agree that the body having jurisdiction over the matter shall be called upon to resolve such differences, or the Parties may mutually agree to submit the matter to the Dispute Resolution process in accordance with the provisions of Section 16 of the General Terms and Conditions and Attachment 1 of the Agreement.

2.	The Parties may continue to negotiate toward final prices, but in the event that no such Agreement is reached within nine (9) months, either Party may petition the Commission to resolve such disputes and to determine final prices for each item.  Alternatively, upon mutual agreement, the Parties may submit the matter to  the Dispute Resolution Process set forth in Section 16 of the General

Terms and Conditions and Attachment 1 of the Agreement, so long as they file the resulting Agreement with the Commission as a “negotiated Agreement” under Section 252(e) of the Act.

3.	A final order of this Commission that forms the basis of a true-up shall be the final order as to prices based on appropriate cost studies, or potentially may be a final order in any other Commission proceeding which meets the following criteria:

(a)	BellSouth and CLEC is entitled to be a full Party to the proceeding;

(b)	It shall apply the provisions of the federal Telecommunications Act of 1996, including but not limited to Section 252(d)(1) (which contains pricing standards) and all then-effective implementing rules and regulations; and,

(c)	It shall include as an issue the geographic deaveraging of network element and other services prices, which deaveraged prices, if any are required by said final order, shall form the basis of any true-up.

18.4	Geographic Deaveraging

The Parties acknowledge that this Agreement contains geographically

deaveraged rates consistent with 47 C.F.R. 51.507(f) for all states except

North Carolina.  Upon adoption of geographically deaveraged rates by the North Carolina Utilities Commission, the Parties shall amend this

Agreement to include such rates.  Prior thereto, rates for the state of

North Carolina shall be as set forth in this Agreement.

EXHIBIT A

LINE INFORMATION DATA BASE (LIDB)

STORAGE AGREEMENT

I.	SCOPE

A.	This Agreement sets forth the terms and conditions pursuant to which BST agrees to store in its LIDB certain information at the request of the Local Exchange Company and pursuant to which BST, its LIDB customers and Local Exchange Carrier shall have access to such information.  Local Exchange Carrier understands that BST provides access to information in its LIDB to various telecommunications service providers pursuant to applicable tariffs and agrees that information stored at the request of Local Exchange Carrier, pursuant to this Agreement, shall be available to those telecommunications service providers.  The terms and conditions contained in the attached Addendum(s)  are hereby made a part of this Agreement as if fully incorporated herein.

B.	LIDB is accessed for the following purposes:

1.	Billed Number Screening

2.	Calling Card Validation

3.	Fraud Control

C.	BST will provide seven days per week, 24-hours per day, fraud monitoring on Calling Cards, bill-to-third and collect calls made to numbers in BST’s LIDB, provided that such information is included in the LIDB query.  BST will establish fraud alert thresholds and will notify the Local Exchange Company of fraud alerts so that the Local Exchange Company may take action it deems appropriate.  Local Exchange Company understands and agrees BST will administer all data stored in the LIDB, including the data provided by Local Exchange Company pursuant to this Agreement, in the same manner as BST’s data for BST’s end user customers.  BST shall not be responsible to Local Exchange Company for any lost revenue which may result from BST’s administration of the LIDB pursuant to its established practices and procedures as they exist and as they may be changed by BST in its sole discretion from time to time.

Local Exchange Company understands that BST currently has in effect numerous billing and collection agreements with various interexchange carriers and billing clearing houses.  Local Exchange Company further understands that these billing and collection customers of BST query BST’s LIDB to determine whether to accept various billing options from end users.  Additionally, Local Exchange Company understands that presently BST has no method to differentiate between BST’s own billing and line data in the LIDB and such data which it includes in the LIDB on Local Exchange Company’s behalf pursuant to this Agreement.  Therefore, until such time as BST can and does implement in its LIDB and its supporting systems the means to differentiate Local Exchange Company’s data from BST’s data and the Parties to this Agreement execute appropriate amendments hereto, the following terms and conditions shall apply:

(a)	The Local Exchange Company agrees that it will accept responsibility for telecommunications services billed by BST for its billing and collection customers for Local Exchange Customer’s end user accounts which are resident in LIDB pursuant to this Agreement.  Local Exchange Company authorizes BST to place such charges on Local Exchange Company’s bill from BST and agrees that it shall pay all such charges.  Charges for which Local Exchange Company hereby takes responsibility include, but are not limited to, collect and third number calls.

(b)	Charges for such services shall appear on a separate BST bill page identified with the name of the entity for which BST is billing the charge.

(c)	Local Exchange Company shall have the responsibility to render a billing statement to its end users for these charges, but Local Exchange Company’s obligation to pay BST for the charges billed shall be independent of whether Local Exchange Company is able or not to collect from the Local Exchange Company’s end users.

- BST shall not become involved in any disputes between Local Exchange Company and the entities for which BST performs billing and collection.  BellSouth will not issue adjustments for charges billed on behalf of an entity to Local Exchange Company.  It shall be the responsibility of the Local Exchange Company and the other entity to negotiate and arrange for any appropriate adjustments.

II.	TERM

This Agreement will be effective as of ____________, 199__, and will continue in effect for one year, and thereafter may be continued until terminated by either Party upon thirty (30) days written notice to the other Party.

III.	FEES FOR SERVICE AND TAXES

A.	The Local Exchange Company will not be charged a fee for storage services provided by BST to the Local Exchange Company, as described in Section I of this Agreement.

B.	Sales, use and all other taxes (excluding taxes on BST’s income) determined by BST or any taxing authority to be due to any federal, state or local taxing jurisdiction with respect to the provision of the service set forth herein will be paid by the Local Exchange Company.  The Local Exchange Company shall have the right to have BST contest with the imposing jurisdiction, at the Local Exchange Company’s expense, any such taxes that the Local Exchange Company deems are improperly levied.

IV.	INDEMNIFICATION

To the extent not prohibited by law, each Party will indemnify the other and hold the other harmless against any loss, cost, claim, injury, or liability relating to or arising out of negligence or willful misconduct by the indemnifying Party or its agents or contractors in connection with the indemnifying Party’s provision of services, provided, however, that any indemnity for any loss, cost, claim, injury or liability arising out of or relating to errors or omissions in the provision of services under this Agreement shall be limited as otherwise specified in this Agreement.  The indemnifying Party under this Section agrees to defend any suit brought against the other Party for any such loss, cost, claim, injury or liability.  The indemnified Party agrees to notify the other Party promptly, in writing, of any written claims, lawsuits, or demands for which the other Party is responsible under this Section and to cooperate in every reasonable way to facilitate defense or settlement of claims.  The indemnifying Party shall not be liable under this Section for settlement by the indemnified Party of any claim, lawsuit, or demand unless the defense of the claim, lawsuit, or demand has been tendered to it in writing and the indemnifying Party has unreasonably failed to assume such defense.

V.	LIMITATION OF LIABILITY

Neither Party shall be liable to the other Party for any lost profits or revenues or for any indirect, incidental or consequential damages incurred by the other Party arising from this Agreement or the services performed or not performed hereunder, regardless of the cause of such loss or damage.

VI.	MISCELLANEOUS

A.  It is understood and agreed to by the Parties that BST may provide similar services to other companies.

B.	All terms, conditions and operations under this Agreement shall be performed in accordance with, and subject to, all applicable local, state or federal legal and regulatory tariffs, rulings, and other requirements of the federal courts, the U. S. Department of Justice and state and federal regulatory agencies.  Nothing in this Agreement shall be construed to cause either Party to violate any such legal or regulatory requirement and either Party’s obligation to perform shall be subject to all such requirements.

C.	The Local Exchange Company agrees to submit to BST all advertising, sales promotion, press releases, and other publicity matters relating to this Agreement wherein BST’s corporate or trade names, logos, trademarks or service marks or those of BST’s affiliated companies are mentioned or language from which the connection of said names or trademarks therewith may be inferred or implied; and the Local Exchange Company further agrees not to publish or use advertising, sales promotions, press releases, or publicity matters without BST’s prior written approval.

D.	This Agreement constitutes the entire Agreement between the Local Exchange Company and BST which supersedes all prior Agreements or contracts, oral or written representations, statements, negotiations, understandings, proposals and undertakings with respect to the subject matter hereof.

E.	Except as expressly provided in this Agreement, if any part of this Agreement is held or construed to be invalid or unenforceable, the validity of any other Section of this Agreement shall remain in full force and effect to the extent permissible or appropriate in furtherance of the intent of this Agreement.

F.	Neither Party shall be held liable for any delay or failure in performance of any part of this Agreement for any cause beyond its control and without its fault or negligence, such as acts of God, acts of civil or military authority, government regulations, embargoes, epidemics, war, terrorist acts, riots, insurrections, fires, explosions, earthquakes, nuclear accidents, floods, strikes, power blackouts, volcanic action, other major environmental disturbances, unusually severe weather conditions, inability to secure products or services of other persons or transportation facilities, or acts or omissions of transportation common carriers.

G.	This Agreement shall be deemed to be a contract made under the laws of the State of Georgia, and the construction, interpretation and performance of this Agreement and all transactions hereunder shall be governed by the domestic law of such State.

FACILITIES BASED ADDENDUM

TO LINE INFORMATION DATA BASE (LIDB)

STORAGE AGREEMENT

This is a Facilities Based Addendum to the Line Information Data Base Storage Agreement dated ________________________________, 199 __, between BellSouth Telecommunications, Inc. (“BST”), and _________________________________________(“Local Exchange Company”), effective the _____ day of _________________, 199 ___.

I.	GENERAL

This Addendum sets forth the terms and conditions for Local Exchange Company’s provision of billing number information to BST for inclusion in BST’s LIDB.  BST will store in its LIDB the billing number information provided by Local Exchange Company, and BST will provide responses to on-line, call-by-call queries to this information for purposes specified in Section I.B. of the Agreement.

II.	DEFINITIONS

A.	Billing number - a number that the Local Exchange Company creates for the purpose of identifying an account liable for charges.  This number may be a line or a special billing number.

B.	Line number - a ten digit number that identifies a telephone line administered by the Local Exchange Company.

C.	Special billing number - a ten digit number that identifies a billing account established by the Local Exchange Company.

D.	Calling Card number - a billing number plus PIN number.

E.	PIN number - a four digit security code assigned by the Local Exchange Company which is added to a billing number to compose a fourteen digit calling card number.

F.	Toll billing exception indicator - associated with a billing number to indicate that it is considered invalid for billing of collect calls or third number calls or both, by the Local Exchange Company.

G.	Billed Number Screening - refers to the activity of determining whether a toll billing exception indicator is present for a particular billing number.

H.	Calling Card Validation - refers to the activity of determining whether a particular calling card number exists as stated or otherwise provided by a caller.

I.	Billing number information - information about billing number, Calling Card number and toll billing exception indicator provided to BST by the Local Exchange Company.

III.	RESPONSIBILITIES OF PARTIES

A.	The Local Exchange Company will  provide its billing number information to BST’s LIDB each business day by a method that has been mutually agreed upon by both Parties.

B.	BST will store in its LIDB the billing number information provided by the Local Exchange Company.  Under normal operating conditions, BST shall include the Local Exchange Company’s billing number information in its LIDB no later than two business days following BST’s receipt of such billing number information, provided that BST shall not be held responsible for any delay or failure in performance to the extent such delay or failure is caused by circumstances or conditions beyond BST’s reasonable control.  BST will store in its LIDB an unlimited volume of the Local Exchange Company’s working telephone numbers.

C.	BST will provide responses to on-line, call-by-call queries to the stored information for the specific purposes listed in the next paragraph.

D.	BST is authorized to use the billing number information provided by the Local Exchange Company to perform the following functions for authorized users on an on-line basis:

1.	Validate a 14 digit Calling Card number where the first 10 digits are a line number or special billing number assigned by the Local Exchange Company, and where the last four digits (PIN) are a security code assigned by the Local Exchange Company.

2.	Determine whether the Local Exchange Company or the subscriber has identified the billing number as one which should not be billed for collect or third number calls, or both.

E.	The Local Exchange Company will provide its own billing number information to BST for storage and to be used for Billed Number Screening and Calling Card Validation.  The Local Exchange Company will arrange and pay for transport of updates to BST.

- COMPLIANCE

Unless expressly authorized in writing by the Local Exchange Company, all billing number information provided pursuant to this Addendum shall be used for no purposes other than those set forth in this Addendum.

EXHIBIT B

CALLING NAME DELIVERY (CNAM) DATABASE SERVICES

# 1.00  DEFINITIONS

For the purpose of this Attachment, the following terms shall be defined as:

CALLING NAME DELIVERY DATABASE SERVICE (CNAM) - The ability to associate a name with the calling party number, allowing the end user subscriber (to which a call is being terminated) to view the calling party's name before the call is answered.  This service also provides PaeTec the opportunity to load and store its subscriber names in the BellSouth CNAM SCPs.

CALLING PARTY NUMBER (CPN) - The number of the calling party that is delivered to the terminating switch using common channel signaling system 7 (CCS7) technology, and that is contained in the Initial Address Message (IAM) portion of the CCS7 call setup.

COMMON CHANNEL SIGNALING SYSTEM 7 (CCS7) - A network signaling

technology in which all signaling information between two or more nodes is transmitted over high-speed data links, rather than over voice circuits.

SERVICE CONTROL POINTs (SCPs) - The real-time data base systems that contain the names to be provided in response to queries received from CNAM SSPs.

SERVICE MANAGEMENT SYSTEM (SMS) - The main operations support system of CNAM DATABASE SERVICE.  CNAM records are loaded into the SMS, which in turn downloads into the CNAM SCP.

SERVICE SWITCHING POINTs (SSPs) - Features of computerized switches in the telephone network that determine that a terminating line has subscribed to CNAM service, and then communicate with CNAM SCPs in order to provide the name associated with the calling party number.

SUBSYSTEM NUMBER (SSN) - The address used in the Signaling Connection Control Part (SCCP) layer of the SS7 protocol to designate an application at an end signaling point.  A SSN for CNAM at the end office designates the CNAM application within the end office.  BellSouth uses the CNAM SSN of 232.

# 2.0	ATTACHMENT

2.01   This Attachment  contains  the terms and conditions where BellSouth will provide to the PaeTec access to the BellSouth CNAM SCP for query or record storage purposes.

2.02	PaeTec shall submit to BellSouth a notice of its intent to access and utilize BellSouth CNAM Database Services pursuant to the terms and conditions of this Attachment.  Said notice shall be in writing, no less than 60 days prior to PaeTec’s access to BellSouth’s CNAM Database Services and shall be addressed to PaeTec’s Account Manager.

# 3.00  PHYSICAL CONNECTION AND COMPENSATION

3.01	BellSouth's provision of CNAM Database Services to  PaeTec requires interconnection from PaeTec to BellSouth CNAM Service Control Points (SCPs).  Such interconnections shall be established pursuant to Attachment 3 of this Agreement .  The appropriate charge for access to and use of the BellSouth CNAM Database service shall be as set forth in this Attachment.

3.02	In order to formulate a CNAM query to be sent to the BellSouth CNAM SCP, PaeTec PaeTec shall  provide its own CNAM SSP.  PaeTec’s CNAM SSPs must be compliant with TR-NWT-001188, "CLASS Calling Name Delivery Generic Requirements".

3.03	If PaeTec elects to access the BellSouth CNAM SCP via a third party CCS7 transport provider, the third party CCS7 provider shall interconnect with the BellSouth CCS7 network according to BellSouth's Common Channel Signaling Interconnection Guidelines and Telcordia (formerly BellCore)'s CCS Network Interface Specification document, TR-TSV-000905.  In addition, the third party provider shall establish CCS7 interconnection at the BellSouth Local Signal Transfer Points (LSTPs) serving the BellSouth CNAM SCPs that PaeTec desires to query.

3.04	Out-Of-Region Customers

If the customer queries the BellSouth CNAM SCP via a third party national SS7 transport provider, the third party SS7 provider shall interconnect with the BellSouth CCS7 network according to BellSouth's Common Channel Signaling Interconnection Guidelines and Bellcore's CCS Network Interface Specification document, TR-TSV-000905. In addition, the third party provider shall establish SS7 interconnection at one or more of the BellSouth Gateway Signal Transfer Points (STPs).  The payment of all costs associated with the transport of SS7 signals via a third party will be established by mutual agreement of the Parties and writing shall, by this reference become an integral part of this Agreement.

# 4.00  CNAM RECORD INITIAL LOAD AND UPDATES

4.01	The mechanism to be used by PaeTec for initial CNAM record load and/or updates shall be determined by mutual agreement.    The initial load and all updates shall be provided by PaeTec in the BellSouth specified format and shall contain records for every working telephone number that can originate phone calls.  It is the responsibility of PaeTec to provide accurate information to BellSouth on a current basis.

4.02	Updates to the SMS shall occur no less than once a week, reflect service order activity affecting either name or telephone number, and involve only record additions, deletions or changes.

4.03	PaeTec CNAM records provided for storage in the BellSouth CNAM SCP shall be available, on a SCP query basis only, to all Parties querying the BellSouth CNAM SCP.  Further, CNAM service shall be provided by each Party consistent with state and/or federal regulation.