Storm Preparedness Report

City of Wauchula Implementation Plan

June 2, 2006

A.	Introduction

This is the Storm Preparedness Report prepared by the City of Wauchula,

located in Hardee County, Florida. Information contact:

Ray McClellan

Director of Public Works

126 S. 7th Avenue

Wauchula, FL 33873

(863) 773-3535

Wauchula is an inland community not normally directly affected by hurricane 	strikes. However in 2004, Wauchula was hit by 3 Hurricanes: Charley, Frances 	and Jeanne.

The eye of Charley passed over the City of Wauchula. Charley, a Category 4 	hurricane and being the most destructive of the three, put customers out of power 	for as long as 10 days in Wauchula. It took only hours to restore power after 	Hurricane Frances and Jeanne. During the 2005 season, Wauchula was not 	directly affected by any Hurricanes.

Wauchula’s distribution system covers approximately 2 ¼ square miles. We serve 	2773 customer meters at this time. We have no subdivisions under construction at 	this time. Wauchula utility customers have the option for utilities to be placed 	either underground or overhead.

Three-Year Vegetation Management Cycle

The City of Wauchula trims its distribution system every three-four years by grid. 	Hanging limbs and deteriorated trees in the rights-of-way are removed when 	needed.

B.	Transmission and Distribution Geographic Information System

Wauchula’s entire distribution system is mapped on paper. We are currently in the 	process of installing a GIS database. The database will contain pole information 	including, age, material condition, type and attachments. Currently, any work 	done on poles is tracked by work order.

C.	Wooden Transmission vs. Concrete Transmission Structures

This section is not applicable to the City of Wauchula – the City has no 	transmission system. The City of Wauchula receives transmission service through 	Progress Energy.

D.	Post-Storm Data Gathering, Data Retention and Forensic Analysis

Outage work orders are generated for every outage on our system. An outage 	report is created and contains the following information; cause, corrective actions 	taken and any recommended actions to prevent a recurrence.

E.	Audit of Joint-Use Pole Attachment Agreements

Utility pole attachments are audited every three by utility personnel. During a 	field audit, our facilities are examined by knowledgeable personnel to identify 	overloaded poles. Wauchula has not experienced any failures of poles due to 	overloading.

F.	Six-Year Transmission Inspection Program

Wauchula does not have any transmission facilities. The City of Wauchula 	receives transmission service through Progress Energy.

G.	Collection of Outage Data Differentiating Between the Reliability 	Performance of Overhead and Underground Systems

The City of Wauchula does not currently do this. However, it would be relatively 	easy to calculate for overhead and underground outages.

H.	Coordination with Local Governments

The City of Wauchula coordinates with Hardee County Emergency Management 	and FMEA for mutual aid during times of approaching severe weather.

I.	Collaborative Research through the Public Research Center at the 	University of Florida

The City of Wauchula, through its membership in the Florida Municipal Electric 	Association (FMEA) and its involvement with the Public Utility Research Center 	(PURC) at the University of Florida, participates in PURC activities related to s	storm hardening research.