# A.10.0  Consistency with Peninsular Florida Needs

This section describes the consistency of TEC with the power requirements of peninsular Florida.  The information in this section is based in part on the 2006 Regional Load and Resource Plan (2006 L&RP) for the State of Florida, compiled by the Florida Reliability Coordinating Council (FRCC) and published in July 2006.  The FRCC is responsible for coordinating power supply reliability in peninsular Florida for the North American Electric Reliability Council (NERC).  The 2006 L&RP summarizes forecast utility loads and resources, by type of capacity, through the year 2015.  The report also includes proposed generation expansion plans, retirements of existing generating capacity, and capacity re-rates.

This section also presents a discussion of the existing and projected generation by fuel type throughout the State of Florida.  The FPSC’s Review of Electric Utility 2005 Ten-Year Site Plans was used as the basis for the information discussed herein.

A.10.1  Peninsular Florida Capacity and Reliability Needs

The need for TEC can be evaluated by comparing the existing and planned capacity in peninsular Florida with the capacity resources required to meet forecast peak loads plus reserve requirements.  As shown in Table A.10-1, the weighted average summer and winter target reserve margins for the peninsular Florida utilities are 19.0 and 18.9 percent, respectively, as of January 1, 2006.  The projections of reserve margins in peninsular Florida in Table A.10-2 should be compared to those weighted average target reserve margins.

The FRCC 2006 L&RP was developed based on the information submitted by FRCC members in the 2006 Load and Resource Database (LRDB).  In developing Table A.10-2, all units that were listed with either “Regulatory Approval Pending” or “Planned but not Authorized” status were not included as committed additions.  Committed capacity additions and reductions are defined as changes to existing units, such as re-rates, planned retirements, units that are currently under construction, and units that have received approval under the Florida Electrical Power Plant Siting Act, but are not yet under construction.  Capacity additions that have received FPSC approval subsequent to the FRCC LRDB process, such as FPL’s West County units, OUC’s Stanton Energy Center Unit B, and Seminole Electric Cooperative’s (SEC’s) Seminole Generating Station Unit 3, have been included in the projection of installed capacity.  It should be noted that while these three units have received approval from the FPSC, they have yet to receive approval from the Governor and Cabinet.

| Table A.10-1 Peninsular Florida Weighted Average Reserve Requirement (as of January 1, 2006) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Utility | Net Capacity (MW) (1) |  | Reserve Requirement (2) |  |
|  | Summer | Winter | Summer | Winter |
| Florida Keys Electric Cooperative Association (3) | 21 | 21 | 15% | 15% |
| Florida Municipal Power Agency (4) | 1,409 | 1,475 | 18% | 15% |
| Florida Power and Light Company | 20,777 | 22,099 | 20% | 20% |
| Gainesville Regional Utilities | 612 | 632 | 15% | 15% |
| JEA | 3,387 | 3,552 | 15% | 15% |
| Lakeland, City of | 913 | 995 | 15% | 15% |
| New Smyrna Beach, Commission of (3) | 66 | 70 | 15% | 15% |
| Orlando Utilities Commission | 1,199 | 1,257 | 15% | 15% |
| Progress Energy Florida | 8,842 | 9,760 | 20% | 20% |
| Reedy Creek Improvement District | 43 | 44 | 15% | 15% |
| Seminole Electric Cooperative | 1,819 | 1,886 | 15% | 15% |
| St. Cloud, City of | 21 | 21 | 15% | 15% |
| Tallahassee, City of | 744 | 795 | 17% | 17% |
| Tampa Electric Company | 4,071 | 4,383 | 20% | 20% |
| US Corps of Engineers – Mobile (3) | 44 | 44 | 15% | 15% |
| Total Net Capacity (MW) | 43,966 | 47,033 |  |  |
| Weighted Average Reserve Requirement |  |  | 19.0% | 18.9% |
| (1)Source: 2006 FRCC Load and Resource Plan. (2)Source: 2006 Ten-Year Site Plans. (3)Reserve requirement has not been confirmed and is estimated at 15 percent. (4)Includes members of the All-Requirements Project. |  |  |  |  |

| Table A.10-2 Peninsular Florida Installed Capacity and Reserve Margins of Existing Facilities and Committed Capacity Addition and Reductions(1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (1) Calendar Year | (2) Projection of Installed Capacity (MW) |  |  | (3) Net Contracted Firm Interchange (MW) |  |  | (4) Projected Firm Net to Grid from Non-Utility Generator (NUG) (MW) |  |  | (5) Total Available Capacity (MW) |  |  | (6) Total Peak Demand (MW) |  |  | (7) Reserve Margin w/o Load Management and Int. Load |  |  |  |  |  | (8) Load Management and Interruptible Load (MW) |  |  | (9) Firm Peak Demand (MW) |  |  |  |  | (10) Reserve Margin w/Load Management and Int. Load |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MW |  |  | Percent of Peak |  |  |  |  |  |  |  |  |  |  | MW |  |  | Percent of Peak |  |
| Summer Peak Demand |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2006 |  |  | 44,212 |  |  | 1,552 |  |  | 5,498 |  |  | 51,262 |  |  | 45,520 |  |  | 5,742 |  |  | 12.6 |  |  | 2,759 |  |  | 42,761 | 8,501 |  |  | 19.9 |  |  |  |
| 2007 |  |  | 46,005 |  |  | 1,552 |  |  | 5,272 |  |  | 52,829 |  |  | 46,725 |  |  | 6,104 |  |  | 13.1 |  |  | 2,947 |  |  | 43,778 | 9,051 |  |  | 20.7 |  |  |  |
| 2008 |  |  | 46,842 |  |  | 1,552 |  |  | 5,378 |  |  | 53,772 |  |  | 48,030 |  |  | 5,742 |  |  | 12.0 |  |  | 3,001 |  |  | 45,029 | 8,743 |  |  | 19.4 |  |  |  |
| 2009 |  |  | 47,237 |  |  | 1,552 |  |  | 5,528 |  |  | 54,317 |  |  | 49,233 |  |  | 5,084 |  |  | 10.3 |  |  | 3,023 |  |  | 46,210 | 8,107 |  |  | 17.5 |  |  |  |
| 2010 |  |  | 48,687 |  |  | 1,342 |  |  | 4,818 |  |  | 54,847 |  |  | 50,221 |  |  | 4,626 |  |  | 9.2 |  |  | 3,006 |  |  | 47,215 | 7,632 |  |  | 16.2 |  |  |  |
| 2011 |  |  | 48,596 |  |  | 1,342 |  |  | 4,611 |  |  | 54,549 |  |  | 51,343 |  |  | 3,206 |  |  | 6.2 |  |  | 3,025 |  |  | 48,318 | 6,231 |  |  | 12.9 |  |  |  |
| 2012 |  |  | 49,346 |  |  | 1,342 |  |  | 4,530 |  |  | 55,218 |  |  | 52,490 |  |  | 2,728 |  |  | 5.2 |  |  | 3,048 |  |  | 49,442 | 5,776 |  |  | 11.7 |  |  |  |
| 2013 |  |  | 49,346 |  |  | 1,342 |  |  | 3,876 |  |  | 54,564 |  |  | 53,686 |  |  | 878 |  |  | 1.6 |  |  | 3,075 |  |  | 50,611 | 3,953 |  |  | 7.8 |  |  |  |
| 2014 |  |  | 49,346 |  |  | 1,342 |  |  | 3,841 |  |  | 54,529 |  |  | 54,830 |  |  | (301) |  |  | (0.5) |  |  | 3,104 |  |  | 51,726 | 2,803 |  |  | 5.4 |  |  |  |
| 2015 |  |  | 49,188 |  |  | 1,342 |  |  | 4,169 |  |  | 54,699 |  |  | 56,130 |  |  | (1,431) |  |  | (2.5) |  |  | 3,112 |  |  | 53,018 | 1,681 |  |  | 3.2 |  |  |  |
| Winter Peak Demand |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2006/07 |  | 47,632 |  |  | 1,552 |  |  | 5,494 |  |  | 54,678 |  |  | 48,296 |  |  | 6,382 |  |  | 13.2 |  |  | 3,504 |  |  | 44,792 |  |  | 9,886 |  |  | 22.1 |  |  |
| 2007/08 |  | 49,760 |  |  | 1,552 |  |  | 5,899 |  |  | 57,211 |  |  | 49,464 |  |  | 7,747 |  |  | 15.7 |  |  | 3,559 |  |  | 45,905 |  |  | 11,306 |  |  | 24.6 |  |  |
| 2008/09 |  | 50,130 |  |  | 1,552 |  |  | 5,707 |  |  | 57,389 |  |  | 50,732 |  |  | 6,657 |  |  | 13.1 |  |  | 3,605 |  |  | 47,127 |  |  | 10,262 |  |  | 21.8 |  |  |
| 2009/10 |  | 50,584 |  |  | 1,552 |  |  | 5,177 |  |  | 57,313 |  |  | 51,678 |  |  | 5,635 |  |  | 10.9 |  |  | 3,590 |  |  | 48,088 |  |  | 9,225 |  |  | 19.2 |  |  |
| 2010/11 |  | 52,132 |  |  | 1,342 |  |  | 5,159 |  |  | 58,633 |  |  | 52,869 |  |  | 5,764 |  |  | 10.9 |  |  | 3,612 |  |  | 49,257 |  |  | 9,376 |  |  | 19.0 |  |  |
| 2011/12 |  | 52,109 |  |  | 1,412 |  |  | 5,080 |  |  | 58,601 |  |  | 53,923 |  |  | 4,678 |  |  | 8.7 |  |  | 3,635 |  |  | 50,288 |  |  | 8,313 |  |  | 16.5 |  |  |
| 2012/13 |  | 52,859 |  |  | 1,342 |  |  | 4,273 |  |  | 58,474 |  |  | 55,086 |  |  | 3,388 |  |  | 6.1 |  |  | 3,666 |  |  | 51,420 |  |  | 7,054 |  |  | 13.7 |  |  |
| 2013/14 |  | 52,859 |  |  | 1,342 |  |  | 4,669 |  |  | 58,870 |  |  | 56,271 |  |  | 2,599 |  |  | 4.6 |  |  | 3,700 |  |  | 52,571 |  |  | 6,299 |  |  | 12.0 |  |  |
| 2014/15 |  | 52,670 |  |  | 1,342 |  |  | 4,378 |  |  | 58,390 |  |  | 57,674 |  |  | 716 |  |  | 1.2 |  |  | 3,734 |  |  | 53,940 |  |  | 4,450 |  |  | 8.2 |  |  |
| 2015/16 |  | 52,669 |  |  | 930 |  |  | 4,273 |  |  | 57,872 |  |  | 59,162 |  |  | (1,290) |  |  | (2.2) |  |  | 3,730 |  |  | 55,432 |  |  | 2,440 |  |  | 4.4 |  |  |
| (1)Represents existing generating resources, planned retirements, planned capacity re-rates and de-rates, and planned generating facilities which have received approval to proceed with construction. Subsequent to the data collection period of the 2006 L&RP, FPL’s West County units, OUC’s Stanton B, and SEC’s Seminole Generating Station Unit 3 received FPSC approval and are therefore included in the projected installed capacity. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.10-2 illustrates that if only the committed additions presented in the 2006 L&RP are considered in the projections of installed capacity, forecasted capacity reserve margins will not satisfy target seasonal reserve margins beginning in the summer of 2009 and winter of 2011/12.  Column (10) of Table A.10-2 shows that summer capacity reserve margins decrease below the target reserve margin of 19 percent beginning in 2009, with reserves projected at 17.5 percent, and decrease further to 3.2 percent in 2015.  Similarly, winter reserve margins are projected to be only 16.5 percent by 2011/12, and decrease to 4.4 percent in 2015/16.  Note that these reserve margins include the expected demand reductions associated with load management and interruptible load.  If the expected demand reductions associated with load management and interruptible loads do not materialize as projected, Column (7) of Table A.10-2 indicates that the summer reserve margins would be below the target reserve margin in all years, with a low of negative 2.5 percent in 2015.  Likewise, without load management and interruptible loads, winter reserve margins would be below the target reserve margin in all years as well, reaching a low of negative 2.2 percent in 2015/16.  Thus, approval and construction of the TEC will help fill the capacity shortfall projected in the state that emerges after accounting for projects that are not yet committed.

A.10.2  Existing Fuel Mix

The need for TEC is seen not only through comparison of existing generating capacity and capacity resource additions with forecast peak demand, but also through an evaluation of the existing and projected fuel mix throughout the State of Florida.  Florida is already heavily dependent upon natural gas and is projected to grow more dependent.  The FPSC’s Department of Economic Regulation published its Review of Florida Electric Utility 2005 Ten-Year Site Plans in December 2005.  Figure A.10-1, extracted from the FPSC’s Review, indicates that in 2004, natural gas accounted for 29.9 percent of Florida’s energy generation.  Projections for 2014 indicate an increased reliance on natural gas, because natural gas is projected to account for 44.4 percent of total generation.  Coal usage in Florida is projected to increase only slightly from 29.4 percent in 2004 to 30.7 percent in 2014, in spite of the addition of six planned coal units in that period of time, only two of which (Stanton B and Seminole Generating Station Unit 3) have been approved by the FPSC subsequent to the Review of Florida Electric Utility 2005 Ten-Year Site Plans.

This growing dependence upon natural gas exposes the state to the high price volatility associated with natural gas.  This conclusion is bolstered by the rapid price escalation for natural gas supply encountered beginning in late August of 2005, as a result of Hurricane Katrina.  Following this event, Henry Hub spot prices for natural gas rose to a September average of $11.96/MBtu and further rose to an average of $13.35/MBtu in December 2005 (oilnergy.com).

2004

2014

Figure A.10-1

Energy Generation by Fuel Type

(Source:  Review of Florida Electric Utility 2005 Ten-Year Site Plans)