FMPA CAPACITY EMERGENCY PLAN

Last Revision: 2008

PURPOSE

The purpose of this plan is to provide personnel of the Florida Municipal Power Agency (FMPA) with a specific set of guidelines and procedures to use for the All-Requirements Project (ARP) when responding to a generating capacity shortage.  The All-Requirements Project is the wholesale supplier of electricity to the City of Bushnell, City of Clewiston, City of Fort Meade, Fort Pierce Utilities Authority, City of Green Cove Springs, Town of Havana, Keys Energy Services (Utility Board of the City of Key West), Beaches Energy Services (City of Jacksonville Beach), Kissimmee Utility Authority, Lake Worth Utilities, City of Leesburg, City of Newberry, City of Ocala, City of Starke, and City of Vero Beach.

The purpose of this plan is to coordinate with the individual All-Requirements Project participant's emergency plans and with the Florida Reliability Coordinating Council (FRCC) plan when responding to generating capacity shortages in the State of Florida.

This plan provides Florida Municipal Power Agency personnel with procedures to contact and inform All-Requirement Project participants' operation and management personnel of a Generating Capacity Advisory, Generating Capacity Alert, Generating Capacity Emergency, or System Load Restoration.

A generating capacity shortage exists when any one of the electric utilities in the State of Florida has, or is forecast to have, inadequate generating capability, including purchased power, to supply its firm load obligations.

The FMPA ALL-REQUIREMENTS PROJECT CAPACITY EMERGENCY PLAN is designed to address the timely notification of project participants so they can notify their own emergency and public information personnel, customers, news media, local government personnel, municipal emergency agencies, fire, police and the Public Service Commission.

The Florida Municipal Power Pool Balancing Authority (FMPP BA) will be notified of a Generating Capacity Advisory, Alert, Emergency or System Load Restoration by the State Capacity Emergency Coordinator via the State messaging system.  FMPP BA System Operators will notify FMPA personnel of the Generating Capacity Advisory, Alert, Emergency or System Load Restoration.

FMPA personnel will notify specified people at the cities of Bushnell, Clewiston, Fort Meade, Fort Pierce, Green Cove Springs, Havana, Jacksonville Beach, Key West, Kissimmee, Lake Worth, Leesburg, Newberry, Ocala, Starke and Vero Beach.  If FMPA personnel cannot be reached, the FMPP BA System Operators will notify specified people at the cities of Bushnell, Clewiston, Fort Meade, Fort Pierce, Green Cove Springs, Havana, Jacksonville Beach, Key West, Kissimmee, Lake Worth, Leesburg, Newberry, Ocala, Starke and Vero Beach.

GENERATING CAPACITY ADVISORY

A Generating Capacity Advisory anticipates conditions which may affect operations and is primarily for information purposes. An Advisory will be issued when: (1) temperature projections for up to three days in advance of the current date exceed temperature criteria in a prescribed number of cities; (2) one or more utilities in an area are issuing or planning to issue public appeals for conservation, (3) notification by an individual utility that their generation fuel supplies may be impacted and may decrease below a level adequate to provide for continuous, uninterrupted service to its firm customers, or (4) the fuel supplies and deliveries, on a state-wide basis may be impacted by weather, natural gas production disruptions, natural gas pipeline delivery disruptions, or any other fuel infrastructure impacts within the FRCC. An Advisory issued for conditions (3) or (4) will be issued as:

Generating Capacity Advisory /

Short-Term Generation Fuel Availability Concern

An Advisory does not indicate an imminent threat of an Alert or an Emergency, and minimal action would normally be required by utilities or governmental agencies. An Advisory declared on the basis of forecasted temperatures will not be rescinded even if the temperature forecast changes.

The State Capacity Emergency Coordinator will, via the State messaging system, notify the FMPP BA that an Advisory has been declared.  The FMPP BA System Operators will immediately notify the Florida Municipal Power Agency.  FMPA personnel will immediately notify the participants of the All-Requirements Project.

FMPA shall notify the State Capacity Emergency Coordinator if any of the All-Requirements participants are issuing or planning to issue public appeals for conservation.

When a Generating Capacity Advisory has been issued, FMPP BA System Operators will immediately contact one of the FMPA personnel listed in Appendix A.  FMPA personnel (or FMPP BA System Operators if FMPA cannot be contacted) will contact All-Requirements participants' office and fax as listed in Appendix A.  FMPA personnel will fax a GENERATING CAPACITY ADVISORY FOR AREA 1 sheet in Appendix B, or a GENERATING CAPACITY ADVISORY FOR AREA 2 sheet in Appendix C.

FMPA will provide participants with the reason a Generating Capacity Advisory is being declared:

- Temperature projections for up to three days in advance of the current date exceed temperature criteria in a prescribed number of cities.

- One or more utilities in an area are issuing or planning to issue public appeals for conservation.

- Notification by an individual utility that their generation fuel supplies may be impacted and may decrease below a level adequate to provide for continuous, uninterrupted service to its firm customers.

- The fuel supplies and deliveries, on a state-wide basis may be impacted by weather, natural gas production disruptions, natural gas pipeline delivery disruptions, or any other fuel infrastructure impacts within the FRCC.

Recommended participant action:

- Notify utility emergency personnel, if appropriate.

- Notify local emergency agencies, if appropriate.

- Prepare a Generating Capacity Advisory announcement for the news media.

- Implement utility public awareness programs.

GENERATING CAPACITY ALERT

A Generating Capacity Alert will be issued

when: (1) the FRCC operating margin is such that the loss of the largest generating unit will necessitate interruption of firm load

in Florida; (2) the fuel supplies of an individual utility have decreased below a level adequate to provide for continuous, uninterrupted service to its firm customers, or (3) the fuel supplies and

deliveries on a state-wide basis have decreased and may be below a level adequate to provide for continuous, uninterrupted service to firm customers. An Alert issued for conditions (2) or (3) will be issued as a:

Generating Capacity Alert /

Short-Term Generation Fuel Shortage

The issuance of a Generating Capacity Alert does not indicate an imminent threat of a Generating Capacity Emergency and is used to increase situational awareness and heighten the coordination and response efforts between and among utilities and the appropriate

governmental agencies, to a potential generating capacity shortage.

The State Capacity Emergency Coordinator will, via the State messaging system, notify the FMPP BA that an Alert has been declared.  FMPP BA System Operators will immediately notify the Florida Municipal Power Agency.  FMPA personnel will immediately notify the participants of the All-Requirements Project.

When a Generating Capacity Advisory has been issued, FMPP BA System Operators will immediately contact one of the FMPA personnel listed in Appendix A.  FMPA personnel (or FMPP BA System Operators if FMPA cannot be contacted) will contact All-Requirements participants' office and fax as listed in Appendix A.  FMPA personnel will fax a GENERATING CAPACITY ALERT  sheet in Appendix D.

FMPA will provide participants with the reason a Generating Capacity Advisory is being declared:

- The FRCC operating margin is such that the loss of the largest generating unit will necessitate interruption of firm load in Florida.

- The fuel supplies of an individual utility have decreased below a level adequate to provide for continuous, uninterrupted service to its firm customers.

- The fuel supplies and deliveries on a state-wide basis have decreased and may be below a level adequate to provide for continuous, uninterrupted service to firm customers.

Recommended participant action:

- Notify Utility emergency personnel, if appropriate.

- Notify local emergency agencies, if appropriate.

- Prepare a Generating Capacity Alert announcement for the news media.

- Implement utility public awareness programs.

- Implement Load Management/Interruptible Service.

- Implement procedures to reduce utility and city use of power.

GENERATING CAPACITY EMERGENCY

A Generating Capacity Emergency will be declared when (1) one of the electric utilities in the FRCC Region has inadequate generating capacity, including purchased power, to supply its firm load obligations, or (2) the fuel supplies and deliveries on a state-wide basis have decreased to a level that is not adequate to provide for continuous, uninterrupted service

to firm customers.  The loss of firm load in a localized area due to a transmission or

distribution outage, temporary problem or isolated event may be reported but would not cause the implementation of the plan.  The loss of firm load in a localized area due to automatic underfrequency relay operation would not cause the implementation of the plan unless it is anticipated that the outage will extend over several hours. The declaration of a Generating Capacity Emergency for condition (2) above will be declared as a:

Generating Capacity Emergency /

Short-Term Generation Fuel Shortage

A Generating Capacity Emergency declaration indicates an immediate or imminent threat to the reliability of the overall FRCC bulk power system. The declaration of a Generating Capacity Emergency will specify a time period and date that denotes the emergency period. If an emergency has been declared more than one day in advance based on forecasted data, it

will not be rescinded unless the revised data indicates that the operating margin and availability of generation fuel is sufficient to be “out of” an Alert phase as well.

The State Capacity Emergency Coordinator will, via the State messaging system, notify the FMPP BA that an Emergency has been issued.  FMPP BA System Operators will immediately notify FMPA.  FMPA personnel will immediately notify the participants of the All-Requirements Project.

FMPP BA System Operators shall monitor the capability of FMPA generating resources and the FMPA All-Requirements participant load.  FMPA shall be notified by the FMPP BA System Operators if FMPA generating resources are not sufficient to serve the FMPA load and emergency purchases may not be available.

FMPA shall notify the State Capacity Emergency Coordinator if any of the All-Requirements participants have implemented firm load reductions.

When a Generating Capacity Emergency has been issued or FMPA generating resources are not sufficient to serve the FMPA load and emergency purchases may not be available, FMPP BA System Operators will immediately contact one of the FMPA personnel listed in Appendix A.

FMPA personnel (or FMPP BA System Operators if FMPA cannot be contacted) will contact All-Requirements participants' office and fax as listed in Appendix A.  FMPA personnel will fax a GENERATING CAPACITY EMERGENCY  sheet in Appendix E.

FMPA will provide participants with the reason a Generating Capacity Advisory is being declared:

- One of the electric utilities in the FRCC Region has inadequate generating capacity, including purchased power, to supply its firm load obligations.

- The fuel supplies and deliveries on a state-wide basis have decreased to a level that is not adequate to provide for continuous, uninterrupted service to firm customers.

Recommended participant action:

- Notify utility emergency personnel, if appropriate.

- Notify local emergency agencies, if appropriate.

- Prepare a Generating Capacity Alert announcement for the news media.

- Implement  utility public awareness programs.

- Implement Load Management/Interruptible Service.

- Implement procedures to reduce utility and city use of power.

- Implement voltage reduction if available

- Implement voltage reduction if available

- Request the removal or relaxation of environmental restraints

- Shed firm load as directed by the FMPP

SYSTEM LOAD RESTORATION

System Load Restoration is complete when firm load reduction has been terminated and power supply is adequate.

The State Capacity Emergency Coordinator will, via the State messaging system, notify the FMPP BA that all firm load has been restored.  FMPP BA System Operators will immediately notify the Florida Municipal Power Agency.  FMPA personnel will immediately notify the participants of the All-Requirements Project.

FMPA shall notify the State Capacity Emergency Coordinator when firm load has been restored if any of the All-Requirements participants have implemented firm load reductions.

When a System Load Restoration has been issued, FMPP BA System Operators will immediately contact one of the FMPA personnel listed in Appendix A.  FMPA personnel (or FMPP BA System Operators if FMPA cannot be contacted) will contact the All-Requirements participants' office and fax as listed in Appendix A.  FMPA personnel will fax a SYSTEM LOAD RESTORATION  sheet as shown in Appendix F.

Recommended participant action:

- Restore load as directed by the FMPP BA System Operator

- Notify utility emergency personnel, if appropriate.

- Notify local emergency agencies, if appropriate.

- Prepare a System Load Restoration announcement for the news media.

- Implement  utility public awareness programs.

NOTIFICATION TO DEPARTMENT OF ENERGY

FMPA shall notify the DOE Emergency Operations Center if FMPA requests meet any of the conditions outlined in Form EIA-417.

Also, FMPA shall notify the DOE Emergency Operations Center for any issuance of a public appeal by All-Requirements project participant(s) to reduce the use of electricity due to a Generating Capacity Advisory, Generating Capacity Alert, or Generating Capacity Emergency.

FMPA will complete the United States Department of Energy, Office of Energy Emergency Operations, Power System Emergency Report Form EIA-417.

REVIEW CAPACITY EMERGENCY PLAN

This plan and attached messages will be reviewed once a year by the Energy Supply and Fuels Manager at FMPA.

The Energy Supply and Fuels Manager at FMPA will issue revisions of the plan to the following:

- All-Requirements participants

- Florida Reliability Coordinating Council

- Florida Public Service Commission

- Florida Municipal Power Pool

Appendix A

FMPA Personnel Contact List

|  | Name |  | Office |  | Dispatch |  | Home |  | Cell Phone |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Joe McKinney |  | (407) 355-7767 |  | NA |  | (352) 867-7179 |  | (407) 947-5038 |  |  |
|  | Steve Kirchoff |  | (407) 355-7767 |  | NA |  | (407) 593-1280 |  | (239) 218-4865 |  |  |
|  | Richard Montgomery |  | (407) 355-7767 |  | NA |  | (407) 362-7661 |  | (407) 637-1929 |  |  |

# All-Requirements Contact List

|  |  |  | Office |  | Dispatch |  | FAX |  | Cell Phone |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | City of Bushnell Bruce Hickle |  | (352) 793-8012 |  | Contact Bruce Hiclke |  | (352) 793-8036 |  | (352) 303-1090 |  |  |
|  | City of Clewiston Kevin McCarthy |  | (863) 983-1454 |  | Contact Kevin McCarthy |  | (863) 983-3406 |  | (863) 228-0360 |  |  |
|  | City of Fort Meade Fred Hilliard |  | (863) 285-1100 Ext. 232 |  | Contact Fred Hilliard |  | (863) 285-1124 |  | (863) 226-8350 |  |  |
|  | Fort Pierce Utilities Authority Tom Richards |  | (772) 466-1600 Ext. 3400 |  | (772) 461-5875 * |  | (772) 595-9841 |  | (772) 528-0075 |  |  |
|  | Green Cove Springs Greg Griffin |  | (904) 529-2249 |  | (904) 529-2229 |  | (904) 529-2232 |  | (904) 298-4342 |  |  |
|  | Town of Havana Howard McKinnon |  | (850) 539-2820 |  | Contact Paul Bert @ (850) 556-3285 |  | (850) 539-2830 |  | (850) 524-2268 |  |  |
|  | Jacksonville Beach Don Ouchley |  | (904) 247-6259 |  | (904) 247-6171 * (904) 247-6204 |  | (904) 247-6120 |  | (904) 226-3859 |  |  |
|  | Keys Energy Fred Culpepper |  | (305) 295-1062 |  | (305) 295-1059 * |  | (305) 295-1060 |  |  |  |  |
|  | Kissimmee Utility Authority Ken Davis |  | (407) 933-7777 Ext 61210 |  | (407) 847-7893 * (407) 847-7627 |  | (407) 933-4178 |  | (321) 624-0901 |  |  |
|  | Lake Worth Utilities Walt Gill |  | (561) 586-1706 |  | (561) 586-1704 * |  | (561) 586-1759 |  |  |  |  |
|  | City of Leesburg Paul Kalv |  | (352) 728-9834 |  | (352) 728-9830 * |  | (352) 728-9809 |  | (352) 516-2052 |  |  |
|  | City of Newberry Blaine Suggs |  | (352) 472-2161 |  | Contact Blaine Suggs |  | (352) 472-7026 |  | (352) 258-4486 |  |  |
|  | City of Ocala Matt Brower |  | (352) 351-6600 |  | (352) 351-6609 * |  | (352) 351-8263 |  |  |  |  |
|  | City of Starke Ricky Thompson |  | (904) 964-2011 |  | Contact Ricky Thompson |  | (904) 966-0584 |  | (352) 494-3288 |  |  |
|  | City of Vero Beach Jim Stevens |  | (772) 978-5051 |  | (772) 978-5041 * |  | (772) 978-5090 |  | (772) 473-3202 |  |  |

*  Note: The dispatch offices of these cities are operated 24/7

Appendix B

FLORIDA MUNICIPAL POWER AGENCY

ALL-REQUIRMENTS PROJECT

CAPACITY EMERGENCY PLAN

GENERATING CAPACITY ADVISORY FOR AREA 1*

FOR ADDITIONAL INFORMATION CALL

FMPA OFFICE (407) 355-7767

* (Area 1 includes Jacksonville, Pensacola & Tallahassee)

NOTE:   Green Cove Springs, Havana, Jacksonville Beach, Newberry, and Starke are in AREA 1.

Generating Capacity Advisory declared for ___ / ___ / ___ through ___ / ___ / ___

Definition of Advisory:

- Temperature projections for up to three days in advance of the current date exceed temperature criteria in a prescribed number of cities.

- One or more utilities in an area are issuing or planning to issue public appeals for conservation.

- Notification by an individual utility that their generation fuel supplies may be impacted and may decrease below a level adequate to provide for continuous, uninterrupted service to its firm customers.

- The fuel supplies and deliveries, on a state-wide basis may be impacted by weather, natural gas production disruptions, natural gas pipeline delivery disruptions, or any other fuel infrastructure impacts within the FRCC.

Recommended participant action:

- Notify utility emergency personnel, if appropriate.

- Notify local emergency agencies, if appropriate.

- Prepare a Generating Capacity Advisory announcement for the news media.

- Implement  utility public awareness programs.

Date & Time issued   ___ / ___ / ___                             ___ : ___

By:___________________

Appendix C

FLORIDA MUNICIPAL POWER AGENCY

ALL-REQUIRMENTS PROJECT

CAPACITY EMERGENCY PLAN

GENERATING CAPACITY ADVISORY FOR AREA 2*

FOR ADDITIONAL INFORMATION CALL

FMPA OFFICE (407) 355-7767

---------------------------------------------------------------------------------------------------------------

* (Area 2 includes Miami, Orlando, St. Petersburg & Tampa)

NOTE: Bushnell, Clewiston, Fort Meade, Fort Pierce, Key West, Kissimmee, Lake Worth, Leesburg, Ocala, and Vero Beach are in AREA 2.

Generating Capacity Advisory declared for ___ / ___ / ___ through ___ / ___ / ___

Definition of Advisory:

- Temperature projections for up to three days in advance of the current date exceed temperature criteria in a prescribed number of cities.

- One or more utilities in an area are issuing or planning to issue public appeals for conservation.

- Notification by an individual utility that their generation fuel supplies may be impacted and may decrease below a level adequate to provide for continuous, uninterrupted service to its firm customers.

- The fuel supplies and deliveries, on a state-wide basis may be impacted by weather, natural gas production disruptions, natural gas pipeline delivery disruptions, or any other fuel infrastructure impacts within the FRCC.

Recommended participant action:

- Notify utility emergency personnel, if appropriate.

- Notify local emergency agencies, if appropriate.

- Prepare a Generating Capacity Advisory announcement for the news media.

- Implement  utility public awareness programs.

Date & Time issued   ___ / ___ / ___                             ___ : ___

By:___________________

Appendix D

FLORIDA MUNICIPAL POWER AGENCY

ALL-REQUIRMENTS PROJECT

CAPACITY EMERGENCY PLAN

GENERATORING CAPACITY ALERT

FOR ADDITIONAL INFORMATION CALL

FMPA OFFICE (407) 355-7767

---------------------------------------------------------------------------------------------------------------

Generating Capacity Alert is declared for ___ / ___ / ___.

Definition of Alert:

- The FRCC operating margin is such that the loss of the largest generating unit will necessitate interruption of firm load in Florida.

- The fuel supplies of an individual utility have decreased below a level adequate to provide for continuous, uninterrupted service to its firm customers.

- The fuel supplies and deliveries on a state-wide basis have decreased and may be below a level adequate to provide for continuous, uninterrupted service to firm customers.

ALL-REQUIREMENTS PROJECT SITUATION:

- FMPA projects ___% reserves during the peak on ___ / ___ / ___.

- Due to FMPA resources out of service, FMPA is purchasing power that can be recalled by the seller.

- Due to unexpected high loads, FMPA is purchasing power that can be recalled by the seller.

Recommended participant action:

- Notify Utility emergency personnel, if appropriate.

- Notify local emergency agencies, if appropriate.

- Prepare a Generating Capacity Alert announcement for the news media.

- Implement utility public awareness programs.

- Implement Load Management/Interruptible Service.

- Implement procedures to reduce utility and city use of power.

Date & Time issued   ___ / ___ / ___                              ___ : ___

By: __________________

Appendix E

FLORIDA MUNICIPAL POWER AGENCY

ALL-REQUIRMENTS PROJECT

CAPACITY EMERGENCY PLAN

GENERATING CAPACITY EMERGENCY

FOR ADDITIONAL INFORMATION CALL

FMPA OFFICE (407) 355-7767

---------------------------------------------------------------------------------------------------------------

Generating Capacity Emergency is issued for ___ / ___ / ___.

Definition of Emergency:

- One of the electric utilities in the FRCC Region has inadequate generating capacity, including purchased power, to supply its firm load obligations.

- The fuel supplies and deliveries on a state-wide basis have decreased to a level that is not adequate to provide for continuous, uninterrupted service to firm customers.

Recommended participant action:

- Notify utility emergency personnel, if appropriate.

- Notify local emergency agencies, if appropriate.

- Prepare a Generating Capacity Alert announcement for the news media.

- Implement  utility public awareness programs.

- Implement Load Management/Interruptible Service.

- Implement procedures to reduce utility and city use of power.

- Implement voltage reduction if available

- Implement voltage reduction if available

- Request the removal or relaxation of environmental restraints

- Shed firm load as directed by the FMPP

ALL-REQUIREMENTS PROJECT SITUATION:

- FMPA projects ___% reserves during the peak on ___ / ___ / ___.

- Due to FMPA resources out of service, FMPA is purchasing power that can be recalled by the seller.

- Due to unexpected high loads, FMPA is purchasing power that can be recalled by the seller.

- FMPA projects to be deficient by ___ MW during the peak on ___/ ___ /___ and, if purchase power is not available, will be contacting participants to reduce firm load.

- FMPA has requested firm load reductions of ___ MW in the

- cities of _____________________________________________________________

- Date & Time issued   ___ / ___ / ___                               ___ : ___

By: _____________

Appendix F

FLORIDA MUNICIPAL POWER AGENCY

ALL-REQUIRMENTS PROJECT

CAPACITY EMERGENCY PLAN

SYSTEM LOAD RESTORATION

FOR ADDITIONAL INFORMATION CALL

FMPA OFFICE (407) 355-7767

----------------------------------------------------------------------------------------------------------------

System Load Restoration is issued for ___ / ___ / ___.

Definition of Restoration:

A System Load Restoration is complete when firm load reduction has been terminated and power supply is adequate.

Recommended participant action:

- Restore load as directed by the FMPP BA System Operator

- Notify utility emergency personnel, if appropriate.

- Notify local emergency agencies, if appropriate.

- Prepare a System Load Restoration announcement for the news media.

- Implement  utility public awareness programs.

Date & Time issued   ___ / ___ / ___                               ___ : ___

By:_________________

FMPA Fuel Emergency Plan

### Last Revision: December 2008

The Florida Municipal Power Agency (FMPA) is a non-profit joint action agency made of 30 municipalities in Florida.  The FMPA All-Requirements Project (ARP) is the wholesale supplier of electricity to the City of Bushnell, City of Clewiston, City of Fort Meade, Fort Pierce Utilities Authority, City of Green Cove Springs, Town of Havana, Keys Energy Services (Utility Board of the City of Key West), Beaches Energy Services (City of Jacksonville Beach), Kissimmee Utility Authority, Lake Worth Utilities, City of Leesburg, City of Newberry, City of Ocala, City of Starke, and City of Vero Beach.  FMPA utilizes a variety of resources in its portfolio to supply the energy requirements of the ARP cities including purchases from other utilities, FMPA-owned generation and generation FMPA leases from ARP generating cities.  The generating resources that FMPA controls are those located at Key West, KUA, Lake Worth, Vero Beach, the Treasure Coast Energy Center (TCEC) in Ft. Pierce, and the Oleander #5 unit in Cocoa.  The unit identification, the capacity in megawatts and available fuel type(s) for each unit is listed in Attachment A.  For jointly owned generation where FMPA owns a minority share, the majority Owner/Operator is responsible for the fuel supply.

This Fuel Emergency Plan details how FMPA anticipates handling different fuel emergencies for the FMPA fleet while serving the electrical needs of the ARP municipalities.  There are two types of fuel that can be used in these generating units: natural gas and fuel oil.  FMPA is developing this plan to address constraints on either of these fuels types.  This plan will be enacted if the Florida Reliability Coordinating Council (FRCC) declares a fuel emergency.

- Natural Gas Emergency Plan

When the natural gas pipeline is severely constrained, both Florida Gas Transmission, Inc. (FGT) and Gulfstream Natural Gas System, LLC (Gulfstream) are required to notify the Florida Gas Utility (FGU), as the agent for FMPA, of the constraint and inform FGU of the amount of natural gas available.  Immediately upon notification of any constraint, FGU will notify FMPA of anticipated available gas volumes available for FMPA generating units.  FMPA along with FGU will implement its plan to reduce its natural gas flow to FMPA units as follows:

FMPA will notify the Florida Municipal Power Pool and request a modified commitment or redispatch that will consider the following:

- Any available purchase power shall be utilized to the extent appropriate, taking into account relevant surrounding factors.  All non-firm sales will be terminated.

- If additional natural gas reduction is required, all units at Vero Beach, Cane Island (KUA), TCEC, and Oleander #5 shall start switching to the appropriate fuel oil.

- If further natural gas reduction is required and there is available Key West and Key West Tie line capacity, Key West shall start all units on fuel oil.  Lake Worth shall take all units using natural gas off-line up to the MW amount that equals the amount supplied by the Key West units.

- If additional natural gas reduction is required, acquire the proper authorization from Florida Department of Environmental Protection (DEP) to allow the generating units to exceed their stated permits.

- If all the above options have been utilized and FMPA is still using too much natural gas, FMPA shall call on other utilities for emergency power.  If emergency power is available, FMPA shall purchase the necessary amount of emergency power and reduce the natural gas fired units by that amount of megawatts purchased.

- If FMPA is still above the natural gas restriction after completion of all of these steps, FMPA shall declare an emergency and go to its Capacity Emergency Plan to prepare for the implementation of load curtailment.

- Fuel Oil Emergency Plan

Fuel oil is stored at all the generating sites mentioned previously.  In general, for generation that can utilize natural gas, enough fuel oil is on site to run a unit at a 50% capacity factor for approximately 5 days. If fuel oil shipments are delayed due to unforeseen circumstances, FMPA has several days to implement a change to alternate fuels.  During that period, FMPA will analyze the situation and determine the best plan in consideration of reliability and cost.  FMPA will utilize all alternate fuels and aggressively seek out purchase power to prevent power interruption.

Under any fuel emergency, FMPA plans to work with all utilities to prevent power interruption to any customer.  If alternative fuels and purchase power are not available, FMPA will implement its Capacity Emergency Plan to prepare for the possibility of load curtailment.  The Capacity Emergency Plan provides for notification to the State of Florida and FRCC and details the method that will be used in the event that load curtailment becomes necessary.

ATTACHMENT A

| # * Approval required by # Operating # Utility | # Unit | Summer Capacity | Winter Capacity | Primary Fuel | Alternate Fuel |
| --- | --- | --- | --- | --- | --- |
| Vero Beach | Vero #1 | 11MW | 12 MW | Natural Gas | None |
|  | Vero #2 (Heat Recovery unit) | 14 MW | 21 MW | None | None |
|  | Vero #3 | 28 MW | 30 MW | Natural Gas | Oil |
|  | Vero #4 | 48 MW | 50 MW | Natural Gas | Oil |
|  | Vero #5 | 31 MW | 31 MW | Natural Gas | Oil |
| Ft. Pierce | TCEC | 315 MW | 315 MW | Natural Gas | Oil |
| Key West | Key West Ct #1 | 17 MW | 18 MW | Oil | None |
|  | Key West Ct #2, #3, #4 | 75 MW | 77 MW | Oil | None |
|  | Medium Speed Diesels #1, #2 | 16 MW | 18 MW | Oil | None |
|  | High Speed Diesels #1, #2, #3 | 5 MW | 5 MW | Oil | None |
| Lake Worth | T. G. Smith S-2 (Heat Recovery Unit) | 12 MW | 13 MW | None | None |
|  | T. G. Smith S-3 | 26 MW | 26 MW | Natural Gas | Oil |
|  | T. G. Smith GT 5 | 18 MW | 20 MW | Natural Gas | Oil |
|  | T. G. Smith GT 1 | 30 MW | 30 MW | Oil | None |
|  | T. G. Smith MUs | 10 MW | 10 MW | Oil | None |
| KUA | Hansel CC | 45 MW | 48 MW | Natural Gas | Oil |
|  | Cane Island CT #1 | 30 MW | 38 MW | Natural Gas | Oil |
|  | Cane Island CC #2 | 112 MW | 120 MW | Natural Gas | Oil |
|  | Cane Island CC #3 | 250 MW | 257 MW | Natural Gas | Oil |
| Southern Co. | Oleander #5 | 160 MW | 160 MW | Natural Gas | Oil |

* Due to the Florida Department of Environmental Protection constraints, oil will only be used if there is no natural gas available.